2024

A guidebook intended for use by first responders during the initial phase of a transportation incident involving hazardous materials/dangerous goods

EMERGENCY RESPONSE GUIDEBOOK

##### SHIPPING PAPERS (DOCUMENTS)

For the purpose of this guidebook, shipping documents and shipping papers are synonymous. Shipping papers provide vital information regarding the hazardous materials/ dangerous goods to initiate protective actions. A consolidated version of the information found on shipping papers may be found as follows:

- Road – kept in the cab of a motor vehicle

- Rail – kept in possession of a crew member

- Aviation – kept in possession of the pilot or aircraft employees

- Marine – kept in a holder on the bridge of a vessel

Information provided:

- 4-digit identification number, UN or NA (go to yellow section)

- Proper shipping name (go to blue section)

- Hazard class or division number of material

- Packing group

- Emergency response telephone number

- Information describing the hazards of the material (entered on or attached to the shipping paper)*

CONTRACT #: XX-XXXX-X **

PACKING GROUP

SHIPPING NAME

ID NUMBER

12 000 LITERS	1 TANKTRUCK

II

ISOPROPANOL	3

UN1219

NO. & TYPE OF PACKAGES

QUANTITY

HAZARD CLASS OR DIVISION NO.

EMERGENCY CONTACT 1-000-000-0000

EXAMPLE OF EMERGENCY CONTACT TELEPHONE NUMBER

##### EXAMPLE OF PLACARD AND PANEL WITH ID NUMBER

The 4-digit ID Number may be shown on the diamond-shaped placard or on an adjacent orange panel displayed on the ends and sides of a cargo tank, vehicle or rail car.

A Numbered Placard

A Placard

OR	and an

Orange Panel

* Inthe United States, this requirement may besatisfied by attachingaguidefromthe ERG2024totheshipping paper, or by having the entire guidebook available for reference.

** Inthe United States, aregistration or contract number may be required on ashipping paper.

##### HOWTO USE THIS GUIDEBOOK

RESIST RUSHING IN!

APPROACH INCIDENT FROM UPWIND, AND UPHILL AND/OR UPSTREAM STAY CLEAR OF ALL SPILLS, VAPORS, FUMES, SMOKE, AND POTENTIAL HAZARDS

WARNING: DO NOT USE THIS FLOWCHART if more than one hazardous material/dangerous good is involved. Immediately call the appropriate emergency response agency telephone number listed on the inside back cover of this guidebook. For chemical or biological warfare agents, refer to the “Criminal or Terrorist Use of Chemical, Biological and Radiological Agents” section.

Determine the Guide Number

Refer to Pages 10-15

Do you know the name of the material?

NO

Search for the ID number

in the Yellow Pages

NO

For Divisions 1.1, 1.2, 1.3, or 1.5

For Divisions 1.4 or 1.6

Use GUIDE 112

Use GUIDE 114

YES

Is the material highlighted in green?

NO

NO

Do you see a placard or label?

NO

Do you know the UN/NA ID number?

Do you see a railcar or road trailer?

NO

Use GUIDE 111

Do you see an explosive placard or label?

YES

YES

YES

Refer to Pages 8-9

YES

Search for the material in the Blue Pages

CONSULT the appropriate Orange Guide

Use the appropriate Orange Guide

If the Guide Number has a “P” next to it, the material may suffer a violent polymerization

PROTECT in downwind direction according to TABLE 1 for residual material release

If product includes the reference (when spilled in water), consult TABLE 2 for a list of the generated gases (for information purposes only), AND do not use water as extinguishing agent.

YES

YES

USE the appropriate Orange Guide for

EVACUATION distances, THEN

NO

USE TABLE 1 for

isolation and protective action distances, AND

BEFORE AN EMERGENCY - BECOME FAMILIAR WITH THIS GUIDEBOOK!

First responders must be trained in the use of this guidebook.

Page 1

##### LOCAL EMERGENCY TELEPHONE NUMBERS

Please populate this section with emergency telephone numbers for local assistance:

HAZMAT CONTRACTORS

RAIL COMPANIES

FEDERAL/STATE/PROVINCIAL AGENCIES

OTHERS

Page 2

##### TABLE OF CONTENTS

Shipping Papers (Documents)	Inside front cover

How to Use this Guidebook	1

Local Emergency Telephone Numbers	2

Safety Precautions	4

Notification and Request for Technical Information	5

Hazard Classification System	6

Introduction to the Table of Markings, Labels and Placards	7

Table of Markings, Labels and Placards and Initial Response Guide to Use On-scene	8

Rail Car Identification Chart	10

Road Trailer Identification Chart	13

Globally Harmonized System of Classification and Labeling of Chemicals (GHS)	16

Hazard Identification Numbers Displayed on Some Intermodal Containers	18

Pipeline Transportation	22

ID Number Index (yellow section)	28

Name of Material Index (blue section)	88

How to Use the Orange Guides	148

General First Aid	150

Guides (orange section)	152

Introduction to Green Tables	280

Protective Actions	283

Protective Action Decision Factors to Consider	284

Background on Table 1 – Initial Isolation and Protective Action Distances	286

Table 1 – Initial Isolation and Protective Action Distances	288

Table 2 – Water-Reactive Materials which Produce Toxic Gases	334

Table 3 – Initial Isolation and Protective Action Distances for Large Spills

for Different Quantities of Six Common TIH (PIH in the US) Gases	340

ERG2024 User’s Guide	344

Protective Clothing	350

Decontamination	352

Fire and Spill Control	353

Considerations for Lithium Battery and Electric Vehicle (EV) Fires	356

BLEVE and Heat Induced Tear	357

BLEVE – Safety Precautions	358

Criminal or Terrorist Use of Chemical, Biological and Radiological Agents	360

Improvised Explosive Device (IED) Safe Stand-Off Distance	367

Glossary	369

Publication Data	380

Canada and United States National Response Centers	384

ERAP – Emergency Response Assistance Plans	385

24-Hour Emergency Response Telephone Numbers	388

Page 3

##### SAFETY PRECAUTIONS RESIST RUSHINGIN!

APPROACH CAUTIOUSLY FROM UPWIND, UPHILL AND/OR UPSTREAM:

- Stay clear of Vapor, Fumes, Smoke and Spills.

- Keep vehicle at a safe distance from the scene.

##### SECURE THE SCENE:

- Isolate the area and protect yourself and others.

##### IDENTIFY THE HAZARDS USING ANY OF THE FOLLOWING:

- Placards

- Container labels

- Shipping papers

- Rail Car and Road Trailer Identification Chart

- Safety Data Sheets (SDS)

- Knowledge of persons on scene

- Consult applicable orange guide

##### ASSESS THE SITUATION:

- Is there a fire, a spill or a leak?

- What are the weather conditions?

- What is the terrain like?

- Who/what is at risk: people, property or the environment?

- What actions should be taken – evacuation, shelter-in-place or dike?

- What resources (human and equipment) are required?

- What can be done immediately?

##### OBTAIN HELP:

- Advise your headquarters to notify responsible agencies and call for assistance from qualified personnel.

##### RESPOND:

- Enter only when wearing appropriate protective gear.

- Rescue attempts and protecting property must be weighed against you becoming part of the problem.

- Establish a command post and lines of communication.

- Continually reassess the situation and modify response accordingly.

- Consider safety of people in the immediate area first, including your own safety.

ABOVE ALL: Do not assume that gases or vapors are harmless because of lack of a smell – odorless gases or vapors may be harmful. Use CAUTION when handling empty containers because they may still present hazards until they are cleaned and purged of all residues.

Page 4

##### NOTIFICATION AND REQUEST FOR TECHNICAL INFORMATION

Follow the steps outlined in your organization’s standard operating procedures and/or local emergency response plan for obtaining qualified assistance. Generally, the notification sequence and requests for technical information beyond what is available in this guidebook should occur in the following order:

##### NOTIFY YOUR ORGANIZATION/AGENCY

  - Based on information provided, this will set in motion a series of events. Actions may range from dispatching additional trained personnel to the scene, to activating the local emergency response plan.

  - Ensure that local fire and police departments have been notified.

##### CALL THE EMERGENCY RESPONSE TELEPHONE NUMBER ON THE SHIPPING PAPER

  - If shipping paper is not available, use guidance under next section “NATIONAL ASSISTANCE”.

##### NATIONAL ASSISTANCE

  - Contact the appropriate emergency response agency listed on the inside back cover of this guidebook.

  - Provide as much information about the hazardous material/dangerous good and the nature of the incident.

  - Theagency will provideimmediateadviceonhandlingtheearly stages of theincident.

  - The agency will also contact the shipper or manufacturer of the material for more detailed information if necessary.

  - The agency will request on-scene assistance when necessary.

##### PROVIDE AS MUCH OF THE FOLLOWING INFORMATION AS POSSIBLE:

  - Your name, call-back telephone number, fax number

  - Location and nature of problem (spill, fire, etc.)

  - Name and identification number of material(s) involved

  - Shipper/consignee/point-of-origin

  - Carrier name, rail car or truck number

  - Container type and size

  - Quantity of material transported/released

  - Local conditions (weather, terrain)

  - Proximity to schools, hospitals, waterways, etc.

  - Injuries and exposures

  - Local emergency services that have been notified

Page 5

##### HAZARD CLASSIFICATION SYSTEM

The hazard class of hazardous materials/dangerous goods is indicated either by its class (or division) number or name. Placards are used to identify the class or division of a material. The hazard class or division number must be displayed in the lower corner of a placard and is required for both primary and subsidiary hazard classes and divisions, if applicable. For other than Class 7 placards, text indicating a hazard (for example, "CORROSIVE") is not required. Text is shown only in the U.S. The hazard class or division number and subsidiary hazard classes or division numbers placed in parentheses (when applicable), must appear on the shipping paper after each proper shipping name.

| Class 1 - | Explosives |  |
| --- | --- | --- |
|  | Division 1.1 | Explosives which have a mass explosion hazard |
|  | Division 1.2 | Explosives which have a projection hazard but not a mass |
|  |  | explosion hazard |
|  | Division 1.3 | Explosives which have afire hazard and either a minor blast |
|  |  | hazard or a minor projection hazard or both, but not a mass |
|  |  | explosion hazard |
|  | Division 1.4 | Explosives which present no significant hazard |
|  | Division 1.5 | Very insensitive explosives with a mass explosion hazard |
|  | Division 1.6 | Extremely insensitive articles which do not have a mass |
|  |  | explosion hazard |
| Class 2 - | Gases |  |
|  | Division 2.1 | Flammable gases |
|  | Division 2.2 | Non-flammable, non-toxic* gases |
|  | Division 2.3 | Toxic* gases |

###### Class 3 -	Flammable liquids (and Combustible liquids [U.S.])

Class 4 -	Flammable solids; Substances liable to spontaneous combustion; Substances which, on contact with water, emit flammable gases Division 4.1	Flammable solids, self-reactive substances and solid

desensitized explosives

Division 4.2	Substances liable to spontaneous combustion

Division 4.3	Substances which in contact with water emit flammable gases

Class 5 -	Oxidizing substances and Organic peroxides Division 5.1	Oxidizing substances Division 5.2	Organic peroxides

###### Class 6 -	Toxic* substances and Infectious substances

Division 6.1	Toxic* substances

Division 6.2	Infectious substances

###### Class 7 -	Radioactive materials Class 8 -	Corrosive substances

Class 9 -	Miscellaneous hazardous materials/dangerous goods and articles

* The words “poison” or “poisonous” are synonymous with the word “toxic”.

Page 6

##### INTRODUCTION TO THE TABLE OF MARKINGS, LABELS AND PLACARDS

USE THIS TABLE ONLY WHEN THE ID NUMBER OR PROPER SHIPPING NAME IS NOT AVAILABLE.

This section displays the placards used on transport vehicles carrying hazardous materials/ dangerous goods with the applicable reference GUIDE circled. Follow these steps:

- Approach scene fromupwind, uphill and/or upstreamat a safe distance to safely identify and/or read the placard or orange panel. Use binoculars if available.

- Match the vehicle placard(s) with one of the placards displayed in this section.

- Consult the circled guide number associated with the placard. Use that guide information for now. For example:

  - Use GUIDE 127 for a FLAMMABLE (Class 3) placard

  - Use GUIDE 153 for a CORROSIVE (Class 8) placard

  - UseGUIDE 111 when the DANGER or DANGEROUS placard is displayed or the nature of the spilled, leaking or burning material is not known. Also use this GUIDE when the presence of hazardous materials/dangerous goods is suspected but no placards can be seen.

If multiple placards point to more than one guide, initially use the most conservative guide (i.e., the guide requiring the greatest degree of protective actions).

- Guides associated with the placards provide the most significant risk and/or hazard information.

- Whenspecificinformation, suchasIDnumber or proper shippingname, becomes available, the more specific Guide recommended for that material must be consulted.

- A single asterisk (*) on orange placards represents an explosive's compatibility group letter. The asterisk must be replaced with the appropriate compatibility group letter. Refer to the “Glossary” section.

- Double asterisks (**) on orange placards represent the division of the explosive. The double asterisks must be replaced with the appropriate division number.

Page 7

Air only

All other

modes

TABLE OF MARKINGS, LABELS AND PLACARDS

111

112

For Divisions 1.1,

1.2, 1.3 and 1.5, enter division number (**) and compatibility group letter (*), when required.

114

118

For Divisions

1.4 and 1.6, enter compatibility group letter (*), when required.

123

USE THIS TABLE ONLY IF MATERIALS CANNOT BE SPECIFICALLY IDENTIFIED BY

120

122

128

125

127

Page 8

143

148

AND INITIAL RESPONSE GUIDE TO USE ON-SCENE

USING THE SHIPPING PAPER, NUMBERED PLACARD, OR ORANGE PANEL NUMBER

134

136

139

153

158

163

153

138

147

171

Lithium metal batteries (UN3090, UN3091)

Lithium ion batteries (UN3480, UN3481)

Page 9

##### RAIL CAR IDENTIFICATION CHART

CAUTION: Emergency response personnel must be aware that rail tank cars vary widely in construction, fittings and purpose. Tank cars could transport products that may be solids, liquids or gases. The products may be under pressure. It is essential that products be identified by consulting shipping papers or train consist or contacting dispatch centers before emergency response is initiated. The information stenciled on the sides or ends of tank cars, as illustrated below, may be used to identify the product utilizing:

- the commodity name shown;

- the other information shown, especially reporting marks and car number which, when supplied to a dispatch center, will facilitate the identification of the product.

###### The recommended guides should be considered as last resort if the material cannot be identified by any other means.

117

Pressure tank car

  - For flammable, non-flammable, toxic and/or liquefied compressed gases

  - Protective housing

  - No bottom fittings

  - Pressures usually above 40 psi

131

Non-pressure / low pressure tank car

  - Known as general service tank car

  - For variety of hazardous and non-hazardous materials

  - Fittings and valves normally visible at the top of the tank

  - Some may have bottom outlet valve

  - Pressures usually below 25 psi

Page 10

##### RAIL CARIDENTIFICATIONCHART

128

Non-pressure / low pressure tank car (TC117, DOT117)

- For flammable liquids

(e.g., petroleum crude oil, ethanol)

- Protective housing separate from manway

- Bottom outlet valve

- Pressures usually below 25 psi

115

Cryogenic tank car

  - For refrigerated liquefied gases (cryogenic liquids)

  - Vacuum-insulated railcars (tank within a tank)

  - Fittings compartment located in a cabinet at ground level of the tank

  - Pressures range from 10 - 75 psi

Page 11

##### RAIL CARIDENTIFICATIONCHART

111

Box car

140

Hopper car

- For general freight that carry bulk or non-bulk packages

- May transport hazardous materials/dangerous goods in small packages or "totes"

- Single or double sliding door

- For bulk commodities and bulk cargo (e.g., coal, ore, cement and solid granular materials)

- Bulk lading discharged by gravity through the hopper bottom doors when doors opened

COMMONMARKINGSONRAILCARS: reporting marks and car number, load limit (pounds or kilograms), empty weight of car, placard, tank qualification and pressure relief device information, car specification, and commodity name.

Page 12

##### ROAD TRAILER IDENTIFICATION CHART

CAUTION: Thischart depictsonlythemost general shapesof roadtrailersandcargotransport units. Emergency response personnel must be aware that there are many variations of road trailers, not illustrated below, that are used for shipping chemical products. Many intermodal tanks that transport liquids, solids, liquefied compressed gases, and refrigerated liquefied gases have similar silhouettes. The suggested guides are for the most hazardous products that may be transported in these trailer types.

WARNING: Road trailers may be jacketed, the cross-section may look different than shown and external ring stiffeners would be invisible.

NOTE: Anemergencyshut-off valveiscommonlyfoundat thefront of thetank, near thedriver door.

The recommended guides should be considered as last resort if the material cannot be identified by any other means.

MAWP: Maximum Allowable Working Pressure.

117

MC331, TC331, SCT331

  - For liquefied compressed gases (e.g., LPG, ammonia)

  - Rounded heads

  - Design pressure between 100-500 psi

  - Different configurations exist

117

MC338, TC338, SCT338, TC341, CGA341

- For refrigerated liquefied gases (cryogenic liquids)

- Similar to a "giant thermo-bottle"

- Fittings compartment located in a cabinet at the rear of the tank

- MAWP between 25-500 psi

137

DOT407, TC407, SCT307, MC307, TC307

- For toxic, corrosive, and flammable liquids

- Circular cross-section

- May have external ring stiffeners

- MAWP of at least 25 psi

Page 13

##### ROADTRAILERIDENTIFICATIONCHART

131

DOT406, TC406, SCT306/406, MC306, TC306

  - For flammable liquids (e.g., gasoline, diesel)

  - Elliptical cross-section

  - Rollover protection at the top

  - Bottom outlet valves

  - MAWP between 3-5 psi (US + Mexico: MC306, DOT406, SCT306/406)

  - MAWP between 3-15 psi (Canada: TC306/406)

137

DOT412, TC412, SCT312, MC312, TC312

  - Usually for corrosive liquids

  - Circular cross-section

  - External ring stiffeners

  - Tank diameter is relatively small

  - MAWP of at least 15 psi

112

TC423

  - For emulsion and water-gel explosives

  - Hopper-style configuration

  - MAWP between 5-15 psi

117

Compressed Gas/Tube Trailer

- For pressurized gases (e.g., air, helium, oxygen)

- Long horizontal tubes permanently mounted on a trailer

- Filling and discharge manifold typically located at the rear of trailer

Page 14

##### ROADTRAILERIDENTIFICATIONCHART

134

Dry Bulk Cargo Trailer

137

Vacuum Tanker

  - For dry bulk cargo (e.g., oxidizers, corrosive solids, cement, plastic pellets, fertilizers)

  - Also known as hopper tanks or hopper trailers

  - Shape can vary but always contains one or more cone shaped bins

  - For cleanup of chemical/oil spills or for hauling crude oil, water

  - Large hinged rear door used for dumping

111

Mixed Cargo

111

Intermodal Freight Container

117

Intermodal Tank

- For general freight that carry bulk or non-bulk packages

- May transport hazardous materials/ dangerous goods in small packages (e.g., bags, boxes, drums) or intermediate bulk containers (IBCs), sometimes referred to as “totes"

- Cargo door(s) in the rear

- For liquefied compressed gases, refrigerated liquefied gases, solids and liquids

- Working pressure may range from 20 to 500 psi

- Tank capacity can range from 50 to 12,000 gallons (200 to 45,000 L), and tank dimensions may vary

Page 15

##### GLOBALLY HARMONIZED SYSTEM OF CLASSIFICATION AND LABELING OF CHEMICALS (GHS)

###### (May be found on means of containment during transport)

The Globally Harmonized System of Classification and Labeling of Chemicals (GHS) is an international guideline published by the United Nations. The GHS aims to harmonize the classification and labeling systems for all sectors involved in the life cycle of a chemical (production, storage, transport, workplace use, consumer use and presence in the environment).

The GHS has nine symbols used to convey specific physical, health and environmental hazard information. These symbols are part of a pictogram that is diamond shaped and includes the GHS symbol in black on a white background with a red frame. The pictogram is part of the GHS label, which also includes the following information:

- Signal word

- Hazard statement

- Precautionary statements

- Product identifier

- Supplier identification

GHS pictograms are similar in shape to transport labels; however, transport labels have backgrounds of different colors.

The elements of the GHS that address signal words and hazard statements are not expected to be adopted in the transport sector. For substances and mixtures covered by the UN Recommendations on the Transport of Dangerous Goods, Model Regulations, the transport labels for physical hazards will have precedence. In transport, a GHS pictogram for the same (or lesser) hazard as the one reflected by the transport label or placard should not be present, but it could exist on the package.

###### Examples of GHS labeling:

Outer Packaging: Box with flammable liquid transport label

Inner Packaging: Plastic bottle with GHS hazard warning label

Single Packaging: 200 L (55 US gallons) drum with a flammable liquid transport label combined with GHS hazard warning label

Page 16

In some cases, such as on drums or intermediate bulk containers (IBCs), which must address information for all sectors, the GHS label may be found in addition to the required transport labels and placards. Both types of labels (GHS and transport) will differ in a way that will make them easy to identify during an emergency.

| GHS Pictograms | Physical hazards | GHS Pictograms | Health and Environmental hazards |
| --- | --- | --- | --- |
| Explosive; Self-reactive; Organic peroxide |  |  | Skin corrosion; Serious eye damage |
| Flammable; Pyrophoric; Self-reactive; Organic peroxide; Self-heating; Emits flammable gases when in contact with water |  |  | Acute toxicity (harmful); Skin sensitizer; Irritant (skin and eye); Narcotic effect; Respiratory tract irritant; Hazardous to ozone layer (environment) |
| Oxidizer |  |  | Respiratory sensitizer; Mutagen; Carcinogen; Reproductive toxicity; Target organ toxicity; Aspiration hazard |
| Gas under pressure |  |  | Hazardous to aquatic environment |
| Corrosive to metals |  |  | Acute toxicity (fatal or toxic) |

Page 17

##### HAZARD IDENTIFICATION NUMBERS DISPLAYED ON SOME INTERMODAL CONTAINERS

Hazardidentificationnumbers, utilized under European andsomeSouthAmericanregulations, may be found in the top half of an orange panel on some intermodal bulk containers. The 4-digit ID number is in the bottom half of the orange panel.

The hazard identification number in the top half of the orange panel consists of two or three digits. In general, the digits indicate the following hazards:

- - Emission of gas due to pressure or to chemical reaction

- - Flammability of liquids (vapors) and gases or self-heating liquid

- - Flammability of solids or self-heating solid

- - Oxidizing (fire-intensifying) effect

- - Toxicity or risk of infection

- - Radioactivity

- - Corrosivity

- - Risk of spontaneous violent reaction

NOTE: Therisk of spontaneousviolent reactionwithinthemeaningof digit 9includesthe possibility, duetothe natureof asubstance, of arisk of explosion, disintegration and polymerization reaction followed by the release of considerable heat or flammable and/or toxic gases.

  - Doubling of a digit indicates an intensification of that particular hazard (e.g., 33, 66, 88).

  - Where the hazard associated with a substance can be adequately indicated by a single digit, the digit is followed by a zero (e.g., 30, 40, 50).

  - A hazard identification number prefixed by the letter “X” indicates that the substance will react dangerously with water (e.g., X88).

  - For the transport of Class 1 materials, the hazard identification number will be replaced by the explosive’s division number and compatibility group.

Page 18

##### HAZARDIDENTIFICATIONNUMBERS DISPLAYEDONSOMEINTERMODAL CONTAINERS

The hazard identification numbers listed below have the following meanings:

20	Asphyxiant gas or gas with no subsidiary hazard

22	Refrigerated liquefied gas, asphyxiant

223	Refrigerated liquefied gas, flammable

225	Refrigerated liquefied gas, oxidizing (fire-intensifying)

23	Flammable gas

238	Gas, flammable corrosive

239	Flammable gas which can spontaneously lead to violent reaction

25	Oxidizing (fire-intensifying) gas

26	Toxic gas

263	Toxic gas, flammable

265	Toxic gas, oxidizing (fire-intensifying)

268	Toxic gas, corrosive

28	Gas, corrosive

30 Flammableliquid(flash-point between23°Cand60°C, inclusive), or flammableliquid or solid in the molten state with a flash-point above 60°C, heated to a temperature equal to or above its flash-point, or self-heating liquid

323  Flammable liquid which reacts with water, emitting flammable gases

X323  Flammable liquid which reacts dangerously with water, emitting flammable gases

33   Highly flammable liquid (flash-point below 23°C)

333  Pyrophoric liquid

X333  Pyrophoric liquid which reacts dangerously with water

336  Highly flammable liquid, toxic

338  Highly flammable liquid, corrosive

X338  Highly flammable liquid, corrosive, which reacts dangerously with water

339	Highly flammable liquid which can spontaneously lead to violent reaction

36	Flammable liquid (flash-point between 23°C and 60°C, inclusive), slightly toxic, or self-heating liquid, toxic

362	Flammable liquid, toxic, which reacts with water, emitting flammable gases

X362	Flammable liquid, toxic, which reacts dangerously with water, emitting flammable gases

368	Flammable liquid, toxic, corrosive

- Flammable liquid (flash-point between 23°Cand 60°C, inclusive), slightly corrosive or self-heating liquid, corrosive

382	Flammable liquid, corrosive, which reacts with water, emitting flammable gases X382	Flammable liquid, corrosive, which reacts dangerously with water, emitting

flammable gases

- Flammable liquid, which can spontaneously lead to violent reaction

- Flammable solid, or self-reactive substance, or self-heating substance, or polymerizing substance

Page 19

##### HAZARDIDENTIFICATIONNUMBERS DISPLAYEDONSOMEINTERMODAL CONTAINERS

423 Solid which reacts with water, emitting flammable gases, or flammable solid which reacts with water, emitting flammable gases, or self-heating solid which reacts with water, emitting flammable gases

X423 Solid which reacts dangerously with water, emitting flammable gases, or flammable solidwhichreactsdangerouslywithwater, emittingflammablegases, or self-heating solid which reacts dangerously with water, emitting flammable gases

- Spontaneously flammable (pyrophoric) solid

X432	Spontaneously flammable (pyrophoric) solid which reacts dangerously with water, emitting flammable gases

- Flammable solid, in the molten state at an elevated temperature

446	Flammable solid, toxic, in the molten state at an elevated temperature

46	Flammable or self-heating solid, toxic

462	Toxic solid which reacts with water, emitting flammable gases X462	Solid which reacts dangerously with water, emitting toxic gases

48	Flammable or self-heating solid, corrosive

482	Corrosive solid which reacts with water, emitting flammable gases X482	Solid which reacts dangerously with water, emitting corrosive gases

50	Oxidizing (fire-intensifying) substance

539	Flammable organic peroxide

- Strongly oxidizing (fire-intensifying) substance

556	Strongly oxidizing (fire-intensifying) substance, toxic

558	Strongly oxidizing (fire-intensifying) substance, corrosive

559	Strongly oxidizing (fire-intensifying) substance which can spontaneously lead to violent reaction

- Oxidizing substance (fire-intensifying), toxic

568	Oxidizing substance (fire-intensifying), toxic, corrosive

- Oxidizing substance (fire-intensifying), corrosive

- Oxidizing substance (fire-intensifying) which can spontaneously lead to violent reaction

- Toxic or slightly toxic substance 606	Infectious substance

623	Toxic liquid, which reacts with water, emitting flammable gases

- Toxic substance, flammable (flash-point between 23°C and 60°C, inclusive)

638	Toxicsubstance, flammable(flash-point between23°Cand60°C, inclusive), corrosive

639	Toxic substance, flammable (flash-point not above 60°C) which can spontaneously lead to violent reaction

- Toxic solid, flammable or self-heating

642	Toxic solid which reacts with water, emitting flammable gases

- Toxic substance, oxidizing (fire-intensifying)

- Highly toxic substance

663	Highly toxic substance, flammable (flash-point not above 60°C)

Page 20

##### HAZARDIDENTIFICATIONNUMBERS DISPLAYEDONSOMEINTERMODAL CONTAINERS

664	Highly toxic solid, flammable or self-heating

665	Highly toxic substance, oxidizing (fire-intensifying) 668	Highly toxic substance, corrosive

X668	Highly toxic substance, corrosive, which reacts dangerously with water 669	Highly toxic substance which can spontaneously lead to violent reaction

- Toxic substance, corrosive

- Toxic or slightly toxic substance which can spontaneously lead to violent reaction

- Radioactive material

768	Radioactive material, toxic, corrosive

78	Radioactive material, corrosive

80	Corrosive or slightly corrosive substance

X80	Corrosive or slightly corrosive substance which reacts dangerously with water 823	Corrosive liquid which reacts with water, emitting flammable gases

83 Corrosiveor slightly corrosivesubstance, flammable(flash-point between23°Cand 60°C, inclusive)

X83 Corrosiveor slightly corrosivesubstance, flammable(flash-point between23°Cand 60°C, inclusive), which reacts dangerously with water

836 Corrosiveor slightly corrosivesubstance, flammable(flash-point between23°Cand 60°C, inclusive) and toxic

839 Corrosiveor slightly corrosivesubstance, flammable(flash-point between23°Cand 60°C, inclusive), which can spontaneously lead to violent reaction

X839 Corrosiveor slightly corrosivesubstance, flammable(flash-point between23°Cand 60°C, inclusive), which can spontaneously lead toviolent reaction and which reacts dangerously with water

84   Corrosive solid, flammable or self-heating

842   Corrosive solid which reacts with water, emitting flammable gases

85   Corrosive or slightly corrosive substance, oxidizing (fire-intensifying)

856   Corrosive or slightly corrosive substance, oxidizing (fire-intensifying) and toxic

86   Corrosive or slightly corrosive substance, toxic

- Highly corrosive substance

X88	Highly corrosive substance which reacts dangerously with water

883	Highly corrosive substance, flammable (flash-point between 23°C and 60°C, inclusive)

884	Highly corrosive solid, flammable or self-heating

885	Highly corrosive substance, oxidizing (fire-intensifying)

886	Highly corrosive substance, toxic

X886	Highly corrosive substance, toxic, which reacts dangerously with water

- Corrosive or slightly corrosive substance which can spontaneously lead to violent reaction

- Environmentally hazardous substance; miscellaneous dangerous substances

99	Miscellaneous dangerous substance carried at an elevated temperature

Page 21

##### PIPELINE TRANSPORTATION

In North America, hazardous materials/dangerous goods are commonly transported through millions of miles of pipelines and related structures. Products transported include natural gas, natural gas liquids, crude oil, gasoline, diesel fuel, anhydrous ammonia, carbon dioxide, jet fuel, andother commodities. Althoughmost pipelinesareburied, oftenthereareabove-ground structures and markers indicating the presence of pipelines. First responders should be aware of the pipelines in their jurisdictions, the products they transport, and the operators responsible for those pipelines. Proactive relationships can be beneficial in the safe and effective management of pipeline emergencies.

Types of Pipelines

Natural Gas Pipelines

###### Natural Gas Transmission Pipelines

Large-diameter, steel pipelines transport flammable natural gas (toxic and non-toxic) at very high pressures ranging from 200 to 1,500 psi*. Natural gas in transmission pipelines is odorless —generally not odorizedwith mercaptan (the “rotten egg” smell); however, natural gas containing hydrogen sulfide (H2S) will have a distinct “rotten egg” odor.

###### Natural Gas Distribution Pipelines

Natural gas is delivered directly to customers via distribution pipelines. These pipelines are typically smaller-diameter, lower-pressure pipelines constructed of steel, plastic, or cast iron. Natural gas in distribution pipelines is odorized with mercaptan (the “rotten egg” smell).

###### Natural Gas-Gathering and Natural Gas Well Production Pipelines

Natural gas-gathering/well production pipelines collect “raw” natural gas fromwellheads and transport the product to gas-processing and/or gas-treating plants. These gathering pipelines carry natural gas mixed with some quantity of natural gas liquids, water, and, insome areas, contaminants such as toxic hydrogen sulfide (H2S). Natural gas in these pipelines is not odorizedwithmercaptan(the“rottenegg” smell); however, natural gasthat containshydrogen sulfide (H2S) will have a distinct “rotten egg” odor.

###### Hazardous Liquid and Highly Volatile Liquid Pipelines Hazardous Liquid Pipelines

Crudeoil, refinedpetroleumproducts(e.g. gasoline, kerosene, jet fuel or diesel) andhazardous liquids (e.g. anhydrous ammonia or ethanol) are often transported by pipelines.

Many liquid petroleum pipelines transport different types of liquid petroleum in the same pipeline. To do so, the pipeline operator sends different products in “batches.” For example, an operator could send gasoline for several hours, and then switch to jet fuels, before switching to diesel fuel.

* Data fromhttp://naturalgas.org/naturalgas/transport/

Page 22

###### Highly Volatile Liquid (HVL) Pipelines

HVL pipelines transport hazardous liquids which will form a vapor cloud when released to the atmosphere and which have a vapor pressure exceeding 276 KPa (40 psia) at 37.8°C (100°F). An example of an HVL is liquid propane.

Pipeline Markers

Since pipelines are usually buried underground, pipeline markers are used to indicate their presence in an area along the pipeline route. Of the three types of pipelines typically buried underground —distribution, gathering, and transmission —only transmission pipelines are marked with the following above-ground markers used to indicate their route.

Markers warn that a transmission pipeline is located in the area, identify the product transported in the line, and provide the name and telephone number of the pipeline operator to call. Markers and warning signs are located at frequent intervals along natural gas and liquid transmission pipeline rights-of-way, and are located at prominent points such as where pipelines intersect streets, highways, railways, or waterways.

Pipeline markers only indicate the presence of a pipeline—they do not indicate the exact location of the pipeline. Pipeline locations within aright-of-way may vary along its length and there may be multiple pipelines located in the same right-of-way.

##### NOTE:

- Markers for pipelines transporting materials containing dangerous levels of hydrogen sulfide (H2S) may have markers that say: “Sour” or “Poison.”

- Natural gas distribution pipelines are not marked with above-ground signs.

- Gathering/production pipelines are often not marked with above-ground signs.

Page 23

Pipeline Structures(Above Ground)

| Natural Gas Transmission Pipelines: | Compressor stations, valves, metering stations. |
| --- | --- |
| Natural Gas Distribution Pipelines: | Regulator stations, customer meters and regulators, valve box covers. |
| Natural Gas Gathering/Well Production Pipelines: | Compressor stations, valves, metering stations, wellheads, piping, manifolds. |
| Petroleum and Hazardous Liquids Pipelines: | Storage tanks, valves, pump stations, loading racks. |

Indications of Pipeline Leaks and Ruptures

Pipelinereleases canrangefromrelatively minor leaks tocatastrophic ruptures. It is important toremember that gases and liquids behavedifferently oncethey arereleasedfromapipeline. Generally, the following could be indications of a pipeline leak or rupture:

- Hissing, roaring, or explosive sound

- Flames appearing from the ground or water (perhaps very large flames)

- Vapor cloud/fog/mist

- Dirt/debris/water blowing out of the ground

- Liquids bubbling up from the ground or bubbling in water

- Distinctive, unusually strong odor of rotten eggs, mercaptan (an odorant in some natural gas pipelines), skunk, or petroleum

- Discolored/dead vegetation or discolored snow above a pipeline right-of-way

- Oil slick or sheen on flowing/standing water

- An area of frozen ground in the summer

- An unusual area of melted snow in the winter

Page 24

General Considerationsfor Respondingtoa Pipeline Emergency

  - Safety First! Your safety and the safety of the community you protect is top priority. Remember toapproachapipelineincident fromupwind, uphill, andupstreamwhileusing air monitoring equipment to detect for the presence of explosive and/or toxic levels of hazardous materials/dangerous goods.

    - Always wear proper personal protectiveequipment. Bepreparedfor aflashfire. Use shielding to protect first responders in the event of an explosion. Use respiratory protection.

    - Never operate pipeline valves (except in coordination with the pipeline operator); this could make the incident worse and put you and others in danger.

    - Never attempt to extinguish a pipeline fire before supply isshut off; this could result in the accumulation of a large flammable/explosive vapor cloud or liquid pool that could make the incident worse and put you and others in danger.

    - Do not walk or drive into a vapor cloud in an attempt to identify the product(s) involved.

    - Do not park over manholes or storm drains.

    - Donot approachthescenewithvehicles or mechanical equipment until theisolation zones have been established (vehicles are a potential ignition source).

  - Secure the site and determine a plan to evacuate or shelter-in-place. Work with other responders to deny entry to an area.

  - Identify the product and the operator. If safe to do so, you may be able to identify the product based on its characteristics or other external clues. Look for pipeline markers indicatingthe product, operator of the pipeline, andtheir emergency contact information. Pipelines transport many different types of products, including gases, liquids, and highly volatile liquids that are in a liquid state inside the pipeline but in a gaseous state if released fromthe pipeline. The vapor density of gases determines if they rise or sink in air. Viscosity and specific gravity also are important characteristics of hazardous liquids to consider. Identification of the product also will help you determine the appropriate distance for isolation of the affected area.

  - Notify the pipeline operator using the emergency contact information on the pipeline marker or other contact information you may have received fromthe pipeline operator. The pipeline operator will be a resource to you in the response.

  - Establish a command post. Implement the Incident Command Structure, as needed, and be prepared to implement a Unified Command as additional stakeholders and resources arrive.

Page 25

###### Other Important Considerations

- If no flames are present, do not introduce ignition sources such as open flames, running vehicles, or electrical equipment (cell phones, pagers, two-way radios, lights, garage door openers, fans, door bells, etc.).

- Abandon any equipment used in or near the area of the pipeline release.

- If there is no risk to your safety or the safety of others, move far enough away fromany noise coming from the pipeline to allow for normal conversation.

- Pipelines often are close to other public utilities, railroads, and highways; these can be impacted by pipeline releases or may be potential ignition sources.

- Natural gas can migrate underground fromthe source of arelease to other areas viathe pathof least resistance(includingthroughsewers, water lines, and geologic formations).

###### Considerations for Establishing Protective Action Distances

- Type of product

– If you know the material involved, identify the three-digit guide number by looking up the name in the alphabetical list (blue section). The relevant guide will provide an idea of the risks posed by the material.

- Pressure and diameter of pipe (the pipeline operator can tell you this if you don’t already know it)

- Timing of valve closure by the pipeline operator (quickly for automated valves; longer for manually operated valves)

- Dissipation time of the product in the pipeline once valves are closed

- Ability to conduct atmospheric monitoring and/or air sampling

- Weather (wind direction, etc.)

- Local variables such as topography, population density, demographics, and fire suppression methods available

- Nearby building construction material/density

- Natural and man-made barriers (such as highways, railroads, rivers, etc.)

Page 26

U.S. Pipeline Resources

U.S. Pipeline Locations: The National Pipeline Mapping System (NPMS) https://www.npms.phmsa.dot.gov indicates the general locations of hazardous liquids and natural gas transmission pipelines found within the U.S. The pipelines depicted in the NPMS are within 500 feet of their actual locations. Emergency responders may apply for an NPMS web viewer account that will allowaccess to more detailed information than is available tothe general public. The NPMS does not contain gathering/production or natural gas distribution pipelines.

U.S. Pipeline Emergency Response Training: Where appropriate, reference pipeline emergencies training materials produced by the Pipeline and Hazardous Materials Safety Administration. Your state or jurisdiction also may provide training on how to handle the response to a pipeline incident.

Other Resources:

Pipeline Association for Public Awareness

https://www.pipelineawareness.org/

U.S. DOT, Pipeline and Hazardous Materials Safety Administration

https://www.phmsa.dot.gov/safety-awareness/pipeline/safety-awareness-overview

Pipeline Emergency Responders Initiative (PERI)

https://www.phmsa.dot.gov/pipeline/peri/pipeline-emergency-responders-initiative-peri

Canadian Pipeline Resources

For more information about pipelines in Canada, please consult the Canada Energy Regulator website:

https://www.cer-rec.gc.ca/en/index.html

Page 27

##### INTRODUCTION TO YELLOWSECTION

For entries highlighted in green follow these steps:

##### IF THERE IS NO FIRE:

- Go directly to Table 1 (green section)

- Look up the ID number and name of material

- Identify initial isolation and protective action distances

- Also consult the appropriate Orange Guide

##### IF A FIRE IS INVOLVED:

- Use the appropriate Orange Guide for EVACUATION distances

- Also protect in downwind direction according to Table 1 for residual material release

Note 1: If the name in Table 1 is shown with (when spilled in water), these materials produce large amounts of Toxic Inhalation Hazard (TIH) (PIH in the US) gases when spilled in water. Some water-reactive materials are also TIH materials themselves (e.g., UN1746 (Bromine trifluoride), UN1836 (Thionyl chloride)). In these instances, two entries are provided in Table 1 for land-based and water- based spills. If a water-reactive material only has one entry in Table 1 for (when spilled in water) and the product is NOT spilled in water, Table 1 and Table 2 do not apply. You will find safe distances in the appropriate Orange Guide.

Note 2: Explosivesarenot individuallylistedbytheir IDnumber becauseinanemergency situation, the response will be based only on the division of the explosive, not on the individual explosive.

###### For divisions 1.1, 1.2, 1.3 and 1.5, refer to GUIDE 112. For divisions 1.4 and 1.6, refer to GUIDE 114.

Note 3: Chemical and biological warfareagents arenowfoundinthe“Criminal or Terrorist Use of Chemical, Biological and Radiological Agents” section.

Page 28

| ID No. | Guide No. | Name of Material | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| — — | 112 | Ammonium nitrate-fuel oil mixtures |  | 1022 | 126 | Chlorotrifluoromethane |  |
|  |  |  |  | 1022 | 126 | Refrigerant gas R-13 |  |
| — — | 112 | Blasting agent, n.o.s. |  | 1023 | 119 | Coal gas, compressed |  |
| — — | 112 | Explosives, division 1.1, 1.2, 1.3 or 1.5 |  |  |  |  |  |
|  |  |  |  | 1026 | 119 | Cyanogen |  |
| — — | 114 | Explosives, division 1.4 or 1.6 |  | 1027 | 115 | Cyclopropane |  |
| 1001 | 116 | Acetylene, dissolved |  | 1028 | 126 | Dichlorodifluoromethane |  |
| 1002 | 122 | Air, compressed |  | 1028 | 126 | Refrigerant gas R-12 |  |
| 1003 | 122 | Air, refrigerated liquid (cryogenic liquid) |  | 1029 | 126 | Dichlorofluoromethane |  |
|  |  |  |  | 1029 | 126 | Refrigerant gas R-21 |  |
| 1005 | 125 | Ammonia, anhydrous |  | 1030 | 115 | 1,1-Difluoroethane |  |
| 1005 | 125 | Anhydrous ammonia |  | 1030 | 115 | Refrigerant gas R-152a |  |
| 1006 | 120 | Argon |  | 1032 | 118 | Dimethylamine, anhydrous |  |
| 1006 | 120 | Argon, compressed |  | 1033 | 115 | Dimethyl ether |  |
| 1008 | 125 | Boron trifluoride |  | 1035 | 115 | Ethane |  |
| 1008 | 125 | Boron trifluoride, compressed |  | 1035 | 115 | Ethane, compressed |  |
| 1009 | 126 | Bromotrifluoromethane |  | 1036 | 118 | Ethylamine |  |
| 1009 | 126 | Refrigerant gas R-13B1 |  | 1037 | 115 | Ethyl chloride |  |
| 1010 | 116P | Butadienes, stabilized |  | 1038 | 115 | Ethylene, refrigerated liquid (cryogenic liquid) |  |
| 1010 | 116P | Butadienes and hydrocarbon mixture, stabilized |  |  |  |  |  |
|  |  |  |  | 1039 | 115 | Ethyl methyl ether |  |
| 1011 | 115 | Butane |  | 1039 | 115 | Methyl ethyl ether |  |
| 1012 | 115 | Butylene |  | 1040 | 119P | Ethylene oxide |  |
| 1013 | 120 | Carbon dioxide |  | 1040 | 119P | Ethylene oxide with nitrogen |  |
| 1013 | 120 | Carbon dioxide, compressed |  | 1041 | 115 | Ethylene oxide and carbon dioxide mixture, with more than 9% but not more than 87% ethylene oxide |  |
| 1016 | 119 | Carbon monoxide, compressed |  |  |  |  |  |
| 1017 | 124 | Chlorine |  |  |  |  |  |
| 1018 | 126 | Chlorodifluoromethane |  | 1043 | 125 | Fertilizer, ammoniating solution, with free ammonia |  |
| 1018 | 126 | Refrigerant gas R-22 |  |  |  |  |  |
| 1020 | 126 | Chloropentafluoroethane |  | 1044 | 126 | Fire extinguishers with compressed or liquefied gas |  |
| 1020 | 126 | Refrigerant gas R-115 |  | 1045 | 124 | Fluorine, compressed |  |
| 1021 | 126 | 1-Chloro-1,2,2,2- tetrafluoroethane |  | 1046 | 120 | Helium, compressed |  |
| 1021 | 126 | Refrigerant gas R-124 |  | 1048 | 125 | Hydrogen bromide, anhydrous |  |

Page 29

ID	Guide	Name of Material No. No.

ID	Guide	Name of Material No. No.

|  | 1049 115 Hydrogen, compressed |  | 1075 115 Butane 1075 115 Butylene 1075 115 Isobutane 1075 115 Isobutylene 1075 115 Liquefied petroleum gas 1075 115 LPG 1075 115 Petroleum gases, liquefied 1075 115 Propane 1075 115 Propylene 1076 125 Phosgene 1077 115 Propylene 1078 126 Refrigerant gas, n.o.s. 1079 125 Sulfur dioxide 1079 125 Sulphur dioxide 1080 126 Sulfur hexafluoride 1080 126 Sulphur hexafluoride 1081 116P Tetrafluoroethylene, stabilized 1082 119P Refrigerant gas R-1113 1082 119P Trifluorochloroethylene, stabilized 1083 118 Trimethylamine, anhydrous 1085 116P Vinyl bromide, stabilized 1086 116P Vinyl chloride, stabilized 1087 116P Vinyl methyl ether, stabilized 1088 127 Acetal 1089 129P Acetaldehyde 1090 127 Acetone 1091 127 Acetone oils |  |
| --- | --- | --- | --- | --- |
|  | 1050 125 Hydrogen chloride, anhydrous |  |  |  |
|  | 1051 117P Hydrogen cyanide, stabilized |  |  |  |
|  | 1052 125 Hydrogen fluoride, anhydrous |  |  |  |
|  | 1053 117 Hydrogen sulfide |  |  |  |
|  | 1053 117 Hydrogen sulphide |  |  |  |
|  | 1055 115 Isobutylene 1056 120 Krypton, compressed 1057 115 Lighter refills containing flammable gas 1057 115 Lighters containing flammable gas 1057 128 Lighters, non-pressurized, containing flammable liquid 1058 120 Liquefied gases, non- flammable, charged with nitrogen, carbon dioxide or air 1060 116P Methylacetylene and propadiene mixture, stabilized |  |  |  |
|  | 1061 118 Methylamine, anhydrous |  |  |  |
|  | 1062 123 Methyl bromide |  |  |  |
|  | 1063 115 Methyl chloride 1063 115 Refrigerant gas R-40 |  |  |  |
|  | 1064 117 Methyl mercaptan |  |  |  |
|  | 1065 120 Neon, compressed 1066 120 Nitrogen, compressed |  |  |  |
|  | 1067 124 Dinitrogen tetroxide |  |  |  |
|  | 1067 124 Nitrogen dioxide |  |  |  |
|  | 1069 125 Nitrosyl chloride |  |  |  |
|  | 1070 122 Nitrous oxide |  |  |  |
|  | 1070 122 Nitrous oxide, compressed 1071 119 Oil gas, compressed 1072 122 Oxygen, compressed 1073 122 Oxygen, refrigerated liquid (cryogenic liquid) |  |  | 1092 131P Acrolein, stabilized |
|  |  |  |  | 1093 131P Acrylonitrile, stabilized |
|  |  |  |  | 1098 131 Allyl alcohol |
|  |  |  |  | 1099 131P Allyl bromide |

Page 30

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1100 | 131P | Allyl chloride |  | 1143 | 131P | Crotonaldehyde |  |
| 1104 | 129 | Amyl acetates |  | 1143 | 131P | Crotonaldehyde, stabilized |  |
| 1105 | 129 | Pentanols |  | 1144 | 128 | Crotonylene |  |
| 1106 | 132 | Amylamine |  | 1145 | 128 | Cyclohexane |  |
| 1107 | 129 | Amyl chloride |  | 1146 | 128 | Cyclopentane |  |
| 1108 | 128 | n-Amylene |  | 1147 | 130 | Decahydronaphthalene |  |
| 1108 | 128 | 1-Pentene |  | 1148 | 129 | Diacetone alcohol |  |
| 1109 | 129 | Amyl formates |  | 1149 | 128 | Butyl ethers |  |
| 1110 | 127 | n-Amyl methyl ketone |  | 1149 | 128 | Dibutyl ethers |  |
| 1110 | 127 | Methyl amyl ketone |  | 1150 | 130P | 1,2-Dichloroethylene |  |
| 1111 | 130 | Amyl mercaptan |  | 1152 | 130 | Dichloropentanes |  |
| 1112 | 128 | Amyl nitrate |  | 1153 | 127 | Ethylene glycol diethyl ether |  |
| 1113 | 129 | Amyl nitrite |  | 1154 | 132 | Diethylamine |  |
| 1114 | 130 | Benzene |  | 1155 | 127 | Diethyl ether |  |
| 1120 | 129 | Butanols |  | 1155 | 127 | Ethyl ether |  |
| 1123 | 129 | Butyl acetates |  | 1156 | 127 | Diethyl ketone |  |
| 1125 | 132 | n-Butylamine |  | 1157 | 128 | Diisobutyl ketone |  |
| 1126 | 130 | 1-Bromobutane |  | 1158 | 132 | Diisopropylamine |  |
| 1126 | 130 | n-Butyl bromide |  | 1159 | 127 | Diisopropyl ether |  |
| 1127 | 130 | n-Butyl chloride |  | 1160 | 132 | Dimethylamine, aqueous solution |  |
| 1127 | 130 | Chlorobutanes |  |  |  |  |  |
| 1128 | 129 | n-Butyl formate |  | 1160 | 132 | Dimethylamine, solution |  |
| 1129 | 129P | Butyraldehyde |  | 1161 | 129 | Dimethyl carbonate |  |
|  |  |  |  | 1162 | 155 | Dimethyldichlorosilane |  |
| 1130 | 128 | Camphor oil |  |  |  |  |  |
|  |  |  |  | 1163 | 131 | Dimethylhydrazine, unsymmetrical |  |
| 1131 | 131 | Carbon bisulfide |  |  |  |  |  |
| 1131 | 131 | Carbon disulfide |  | 1164 | 130 | Dimethyl sulfide |  |
| 1131 | 131 | Carbon disulphide |  | 1164 | 130 | Dimethyl sulphide |  |
| 1133 | 128 | Adhesives (flammable) |  | 1165 | 127 | Dioxane |  |
| 1134 | 130 | Chlorobenzene |  | 1166 | 127 | Dioxolane |  |
| 1135 | 131 | Ethylene chlorohydrin |  | 1167 | 128P | Divinyl ether, stabilized |  |
| 1136 | 128 | Coal tar distillates, flammable |  | 1169 | 127 | Extracts, aromatic, liquid |  |
| 1139 | 127 | Coating solution |  | 1170 | 127 | Ethanol |  |

Page 31

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1170 | 127 | Ethanol, solution |  | 1197 | 127 | Extracts, liquid |
|  | 1170 | 127 | Ethyl alcohol |  | 1198 | 132 | Formaldehyde, solution, flammable |
|  | 1170 | 127 | Ethyl alcohol, solution |  |  |  |  |
|  | 1171 | 127 | Ethylene glycol monoethyl ether |  | 1198 | 132 | Formalin (flammable) |
|  | 1172 | 129 | Ethylene glycol monoethyl ether acetate |  | 1199 | 153P | Furaldehydes |
|  |  |  |  |  | 1201 | 127 | Fusel oil |
|  | 1173 | 129 | Ethyl acetate |  | 1202 | 128 | Diesel fuel |
|  | 1175 | 130 | Ethylbenzene |  | 1202 | 128 | Gas oil |
|  | 1176 | 129 | Ethyl borate |  | 1202 | 128 | Heating oil, light |
|  | 1177 | 130 | 2-Ethylbutyl acetate |  | 1203 | 128 | Gasoline |
|  | 1178 | 130 | 2-Ethylbutyraldehyde |  | 1203 | 128 | Motor spirit |
|  | 1179 | 127 | Ethyl butyl ether |  | 1203 | 128 | Petrol |
|  | 1180 | 130 | Ethyl butyrate |  | 1204 | 127 | Nitroglycerin, solution in alcohol, with not more than 1% nitroglycerin |
|  | 1181 | 155 | Ethyl chloroacetate |  |  |  |  |
|  | 1182 | 155 | Ethyl chloroformate |  | 1206 | 128 | Heptanes |
|  | 1183 | 139 | Ethyldichlorosilane |  | 1207 | 130 | Hexaldehyde |
|  | 1184 | 131 | Ethylene dichloride |  | 1208 | 128 | Hexanes |
|  | 1185 | 131P | Ethyleneimine, stabilized |  | 1208 | 128 | Neohexane |
|  | 1188 | 127 | Ethylene glycol monomethyl ether |  | 1210 | 129 | Printing ink, flammable |
|  | 1189 | 129 | Ethylene glycol monomethyl ether acetate |  | 1210 | 129 | Printing ink related material, flammable |
|  | 1190 | 129 | Ethyl formate |  | 1212 | 129 | Isobutanol |
|  | 1191 | 129 | Ethylhexaldehyde |  | 1212 | 129 | Isobutyl alcohol |
|  | 1191 | 129 | Octyl aldehydes |  | 1213 | 129 | Isobutyl acetate |
|  | 1192 | 129 | Ethyl lactate |  | 1214 | 132 | Isobutylamine |
|  | 1193 | 127 | Ethyl methyl ketone |  | 1216 | 128 | Isooctenes |
|  | 1193 | 127 | Methyl ethyl ketone |  | 1218 | 130P | Isoprene, stabilized |
|  | 1194 | 131 | Ethyl nitrite, solution |  | 1219 | 129 | Isopropanol |
|  | 1195 | 129 | Ethyl propionate |  | 1219 | 129 | Isopropyl alcohol |
|  |  |  |  |  | 1220 | 129 | Isopropyl acetate |
|  | 1196 | 155 | Ethyltrichlorosilane |  |  |  |  |
|  |  |  |  |  | 1221 | 132 | Isopropylamine |
|  | 1197 | 127 | Extracts, flavoring, liquid |  |  |  |  |
|  | 1197 | 127 | Extracts, flavouring, liquid |  | 1222 | 130 | Isopropyl nitrate |
|  |  |  |  |  | 1223 | 128 | Kerosene |

Page 32

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1224 | 127 | Ketones, liquid, n.o.s. |  | 1262 | 128 | Octanes |  |
| 1228 | 131 | Mercaptan mixture, liquid, flammable, poisonous, n.o.s. |  | 1263 | 128 | Paint (flammable) |  |
|  |  |  |  | 1263 | 128 | Paint related material (flammable) |  |
| 1228 | 131 | Mercaptan mixture, liquid, flammable, toxic, n.o.s. |  |  |  |  |  |
|  |  |  |  | 1264 | 129 | Paraldehyde |  |
| 1228 | 131 | Mercaptans, liquid, flammable, poisonous, n.o.s. |  | 1265 | 128 | Isopentane |  |
| 1228 | 131 | Mercaptans, liquid, flammable, toxic, n.o.s. |  | 1265 | 128 | Pentanes |  |
|  |  |  |  | 1266 | 127 | Perfumery products, with flammable solvents |  |
| 1229 | 129 | Mesityl oxide |  |  |  |  |  |
| 1230 | 131 | Methanol |  | 1267 | 128 | Petroleum crude oil |  |
| 1230 | 131 | Methyl alcohol |  | 1268 | 128 | Petroleum distillates, n.o.s. |  |
| 1231 | 129 | Methyl acetate |  | 1268 | 128 | Petroleum products, n.o.s. |  |
| 1233 | 130 | Methylamyl acetate |  | 1270 | 128 | Petroleum oil |  |
| 1234 | 127 | Methylal |  | 1272 | 129 | Pine oil |  |
| 1235 | 132 | Methylamine, aqueous solution |  | 1274 | 129 | n-Propanol |  |
| 1237 | 129 | Methyl butyrate |  | 1274 | 129 | Propyl alcohol, normal |  |
| 1238 | 155 | Methyl chloroformate |  | 1275 | 129P | Propionaldehyde |  |
| 1239 | 131 | Methyl chloromethyl ether |  | 1276 | 129 | n-Propyl acetate |  |
| 1242 | 139 | Methyldichlorosilane |  | 1277 | 132 | Propylamine |  |
| 1243 | 129 | Methyl formate |  | 1278 | 129 | 1-Chloropropane |  |
| 1244 | 131 | Methylhydrazine |  | 1278 | 129 | Propyl chloride |  |
| 1245 | 127 | Methyl isobutyl ketone |  | 1279 | 130 | 1,2-Dichloropropane |  |
| 1246 | 127P | Methyl isopropenyl ketone, stabilized |  | 1280 | 127P | Propylene oxide |  |
|  |  |  |  | 1281 | 129 | Propyl formates |  |
| 1247 | 129P | Methyl methacrylate monomer, stabilized |  | 1282 | 129 | Pyridine |  |
| 1248 | 129 | Methyl propionate |  | 1286 | 127 | Rosin oil |  |
| 1249 | 127 | Methyl propyl ketone |  | 1287 | 127 | Rubber solution |  |
|  |  |  |  | 1288 | 128 | Shale oil |  |
| 1250 | 155 | Methyltrichlorosilane |  |  |  |  |  |
|  |  |  |  | 1289 | 132 | Sodium methylate, solution in alcohol |  |
| 1251 | 131P | Methyl vinyl ketone, stabilized |  |  |  |  |  |
| 1259 | 131 | Nickel carbonyl |  | 1292 | 129 | Ethyl silicate |  |
| 1261 | 129 | Nitromethane |  | 1292 | 129 | Tetraethyl silicate |  |
| 1262 | 128 | Isooctane |  | 1293 | 127 | Tinctures, medicinal |  |

Page 33

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

1294 130  Toluene

1295 139  Trichlorosilane

1296 132  Triethylamine

1297 132 Trimethylamine, aqueous solution

1298 155  Trimethylchlorosilane

1299 128  Turpentine

1300 128 Turpentine substitute 1301 129P Vinyl acetate, stabilized

1302 127P Vinyl ethyl ether, stabilized 1303 130P Vinylidene chloride, stabilized 1304 127P Vinyl isobutyl ether, stabilized 1305 155P Vinyltrichlorosilane

1306 129 Wood preservatives, liquid

1307 130  Xylenes

1308 170  Zirconium suspended in a

flammable liquid

1308 170 Zirconium suspended in a liquid (flammable)

1309 170 Aluminum powder, coated

1310 113 Ammonium picrate, wetted with

not less than 10% water 1312 133 Borneol

1313 133 Calcium resinate

1314 133 Calcium resinate, fused

1318 133 Cobalt resinate, precipitated

1320 113 Dinitrophenol, wetted with not

less than 15% water

1321 113 Dinitrophenolates, wetted with

not less than 15% water

1322 113 Dinitroresorcinol, wetted with

not less than 15% water 1323 170 Ferrocerium

1324 133 Films, nitrocellulose base

1325 133 Flammable solid, organic, n.o.s.

Page 34

1325 133 Fusee (railway or highway)

1326 170 Hafnium powder, wetted with

not less than 25% water

1327 133 Bhusa, wet, damp or

contaminated with oil

1327 133 Hay, wet, damp or contaminated

with oil

1327 133 Straw, wet, damp or

contaminated with oil 1328 133 Hexamethylenetetramine

1330 133 Manganese resinate

1331 133 Matches, “strike anywhere”

1332 133  Metaldehyde

1333 170 Cerium, slabs, ingots or rods 1334 133 Naphthalene, crude

1334 133 Naphthalene, refined

1336 113  Nitroguanidine, wetted with not

less than 20% water

1336 113 Picrite, wetted with not less

than 20% water

1337 113 Nitrostarch, wetted with not less than 20% water

1338 133 Phosphorus, amorphous

1338 133 Red phosphorus

1339 139 Phosphorus heptasulfide,

free from yellow and white phosphorus

1339 139 Phosphorus heptasulphide,

free from yellow and white phosphorus

1340 139 Phosphorus pentasulfide,

free from yellow and white phosphorus

1340 139 Phosphorus pentasulphide,

free from yellow and white phosphorus

1341 139 Phosphorus sesquisulfide,

free from yellow and white phosphorus

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1341 | 139 | Phosphorus sesquisulphide, free from yellow and white phosphorus |  | 1356 | 113 | TNT, wetted with not less than 30% water |  |
|  |  |  |  | 1356 | 113 | Trinitrotoluene, wetted with not less than 30% water |  |
| 1343 | 139 | Phosphorus trisulfide, free from yellow and white phosphorus |  |  |  |  |  |
|  |  |  |  | 1357 | 113 | Urea nitrate, wetted with not less than 20% water |  |
| 1343 | 139 | Phosphorus trisulphide, free from yellow and white phosphorus |  |  |  |  |  |
|  |  |  |  | 1358 | 170 | Zirconium powder, wetted with not less than 25% water |  |
| 1344 | 113 | Picric acid, wetted with not less than 30% water |  |  |  |  |  |
|  |  |  |  | 1360 | 139 | Calcium phosphide |  |
| 1344 | 113 | Trinitrophenol, wetted with not less than 30% water |  | 1361 | 133 | Carbon, animal or vegetable origin |  |
| 1345 | 133 | Rubber scrap, powdered or granulated |  | 1361 | 133 | Charcoal |  |
|  |  |  |  | 1362 | 133 | Carbon, activated |  |
| 1345 | 133 | Rubber shoddy, powdered or granulated |  | 1363 | 135 | Copra |  |
| 1346 | 170 | Silicon powder, amorphous |  | 1364 | 133 | Cotton waste, oily |  |
| 1347 | 113 | Silver picrate, wetted with not less than 30% water |  | 1365 | 133 | Cotton |  |
|  |  |  |  | 1365 | 133 | Cotton, wet |  |
| 1348 | 113 | Sodium dinitro-o-cresolate, wetted with not less than 15% water |  | 1369 | 135 | p-Nitrosodimethylaniline |  |
|  |  |  |  | 1372 | 133 | Fibers, animal or vegetable, burnt, wet or damp |  |
| 1349 | 113 | Sodium picramate, wetted with not less than 20% water |  | 1372 | 133 | Fibres, animal or vegetable, burnt, wet or damp |  |
| 1350 | 133 | Sulfur |  | 1373 | 133 | Fabrics, animal or vegetable or synthetic, n.o.s. with oil |  |
| 1350 | 133 | Sulphur |  |  |  |  |  |
| 1352 | 170 | Titanium powder, wetted with not less than 25% water |  | 1373 | 133 | Fibers, animal or vegetable or synthetic, n.o.s. with oil |  |
| 1353 | 133 | Fabrics impregnated with weakly nitrated nitrocellulose, n.o.s. |  | 1373 | 133 | Fibres, animal or vegetable or synthetic, n.o.s. with oil |  |
|  |  |  |  | 1374 | 133 | Fish meal, unstabilized |  |
| 1353 | 133 | Fibers impregnated with weakly nitrated nitrocellulose, n.o.s. |  | 1374 | 133 | Fish scrap, unstabilized |  |
| 1353 | 133 | Fibres impregnated with weakly nitrated nitrocellulose, n.o.s. |  | 1376 | 135 | Iron oxide, spent |  |
|  |  |  |  | 1376 | 135 | Iron sponge, spent |  |
| 1354 | 113 | Trinitrobenzene, wetted with not less than 30% water |  | 1378 | 170 | Metal catalyst, wetted |  |
| 1355 | 113 | Trinitrobenzoic acid, wetted with not less than 30% water |  | 1379 | 133 | Paper, unsaturated oil treated |  |
|  |  |  |  | 1380 | 135 | Pentaborane |  |

Page 35

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

1381 136 Phosphorus, white, dry or under

water or in solution

1381 136 Phosphorus, yellow, dry or

under water or in solution

1381 136 White phosphorus, dry or under

water or in solution

1381 136 Yellow phosphorus, dry or under

water or in solution

1382 135 Potassium sulfide, anhydrous

1382 135 Potassium sulfide, with

less than 30% water of crystallization

1382 135 Potassium sulphide, anhydrous

1382 135 Potassium sulphide, with

less than 30% water of crystallization

| 1383 | 135 | Aluminum powder, pyrophoric |  |
| --- | --- | --- | --- |
| 1383 | 135 | Pyrophoric alloy, n.o.s. |  |
| 1383 | 135 | Pyrophoric metal, n.o.s. |  |
| 1384 | 135 | Sodium dithionite |  |
| 1384 | 135 | Sodium hydrosulfite |  |
| 1384 | 135 | Sodium hydrosulphite |  |
| 1385 | 135 | Sodium sulfide, anhydrous |  |
| 1385 | 135 | Sodium sulfide, with less than |  |
| 30% water of crystallization 1385 135 Sodium sulphide, anhydrous 1385 135 Sodium sulphide, with less than 30% water of crystallization 1386 135 Seed cake, with more than 1.5% oil and not more than 11% moisture |  |  |  |
| 1387 | 133 | Wool waste, wet |  |
| 1389 | 138 | Alkali metal amalgam, liquid |  |
| 1390 | 139 | Alkali metal amides |  |
| 1391 | 138 | Alkali metal dispersion |  |
| 1391 | 138 | Alkaline earth metal dispersion |  |

Page 36

1392 138 Alkaline earth metal amalgam,

liquid

1393 138 Alkaline earth metal alloy, n.o.s. 1394 138 Aluminum carbide

1395 139 Aluminum ferrosilicon powder

1396 138 Aluminum powder, uncoated

1397 139 Aluminum phosphide

1398 138 Aluminum silicon powder, uncoated

1400 138  Barium

1401 138  Calcium

1402 138 Calcium carbide

1403 138 Calcium cyanamide, with more

than 0.1% calcium carbide 1404 138 Calcium hydride

1405 138 Calcium silicide

1407 138  Caesium

1407 138  Cesium

1408 139  Ferrosilicon

1409 138 Metal hydrides, water-reactive,

n.o.s.

1410 138 Lithium aluminum hydride

1411 138 Lithium aluminum hydride, ethereal

1413 138 Lithium borohydride

1414 138 Lithium hydride

1415 138  Lithium

1417 138 Lithium silicon

1418 138 Magnesium alloys powder

1418 138 Magnesium powder

1419 139 Magnesium aluminum

phosphide

1420 138 Potassium metal alloys, liquid 1421 138 Alkali metal alloy, liquid, n.o.s.

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

1422 138 Potassium sodium alloys, liquid

1423 138  Rubidium

1426 138 Sodium borohydride

1427 138 Sodium hydride

1428 138  Sodium

1432 139 Sodium phosphide

1431 138 Sodium methylate, dry

1433 139 Stannic phosphides

1435 138 Zinc ashes

1435 138 Zinc dross

1435 138 Zinc residue

1435 138 Zinc skimmings

1436 138 Zinc dust

1436 138 Zinc powder

1437 138 Zirconium hydride

1438 140 Aluminum nitrate

1439 141 Ammonium dichromate

1442 143 Ammonium perchlorate

1444 140 Ammonium persulfate

1444 140 Ammonium persulphate

1445 141 Barium chlorate, solid

1446 141 Barium nitrate

1447 141 Barium perchlorate, solid

1448 141 Barium permanganate

1449 141 Barium peroxide

1450 140 Bromates, inorganic, n.o.s.

1451 140 Caesium nitrate

1451 140 Cesium nitrate

1452 140 Calcium chlorate

1453 140 Calcium chlorite

1454 140 Calcium nitrate

1455 140 Calcium perchlorate

1456 140 Calcium permanganate

1457 140 Calcium peroxide

1458 140 Chlorate and borate mixture

1459 140 Chlorate and magnesium

chloride mixture, solid 1461 140 Chlorates, inorganic, n.o.s.

1462 143 Chlorites, inorganic, n.o.s.

1463 141 Chromium trioxide, anhydrous

1465 140 Didymium nitrate

1466 140  Ferric nitrate

1467 143 Guanidine nitrate

1469 141 Lead nitrate

1470 141 Lead perchlorate, solid

1471 140 Lithium hypochlorite, dry

1471 140 Lithium hypochlorite mixture

1472 143 Lithium peroxide

1473 140 Magnesium bromate

1474 140 Magnesium nitrate

1475 140 Magnesium perchlorate

1476 140 Magnesium peroxide

1477 140 Nitrates, inorganic, n.o.s.

1479 140 Oxidizing solid, n.o.s.

1481 140 Perchlorates, inorganic, n.o.s.

1482 140 Permanganates, inorganic,

n.o.s.

1483 140 Peroxides, inorganic, n.o.s.

1484 140 Potassium bromate

1485 140 Potassium chlorate

1486 140 Potassium nitrate

1487 140 Potassium nitrate and sodium nitrite mixture

1488 140 Potassium nitrite

1489 140 Potassium perchlorate

Page 37

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

1490 140 Potassium permanganate

1491 144 Potassium peroxide

1492 140 Potassium persulfate

1492 140 Potassium persulphate

1493 140 Silver nitrate

1494 140 Sodium bromate

1495 140 Sodium chlorate

1496 143 Sodium chlorite

1498 140 Sodium nitrate

1499 140 Sodium nitrate and potassium nitrate mixture

| 1500 | 141 | Sodium nitrite |  |
| --- | --- | --- | --- |
| 1502 | 140 | Sodium perchlorate |  |
| 1503 | 140 | Sodium permanganate |  |
| 1504 | 144 | Sodium peroxide |  |
| 1505 | 140 | Sodium persulfate |  |
| 1505 | 140 | Sodium persulphate |  |
| 1506 | 143 | Strontium chlorate |  |
| 1507 | 140 | Strontium nitrate |  |
| 1508 | 140 | Strontium perchlorate |  |
| 1509 | 143 | Strontium peroxide |  |
| 1510 | 143 | Tetranitromethane |  |
| 1511 | 140 | Urea hydrogen peroxide |  |
| 1512 | 140 | Zinc ammonium nitrite |  |
| 1513 | 140 | Zinc chlorate |  |
| 1514 | 140 | Zinc nitrate |  |
| 1515 | 140 | Zinc permanganate |  |
| 1516 | 143 | Zinc peroxide |  |
| 1517 | 113 | Zirconium picramate, wetted |  |
| with not less than 20% water 1541 156 Acetone cyanohydrin, stabilized 1544 151 Alkaloids, solid, n.o.s. (poisonous) |  |  |  |

Page 38

1544 151 Alkaloid salts, solid, n.o.s.

(poisonous)

1545 131 Allyl isothiocyanate, stabilized

1546 151 Ammonium arsenate

1547 153  Aniline

1548 153 Aniline hydrochloride

1549 157 Antimony compound, inorganic, solid, n.o.s.

1550 151 Antimony lactate

1551 151 Antimony potassium tartrate

1553 154 Arsenic acid, liquid

1554 154 Arsenic acid, solid

1555 151 Arsenic bromide

1556 152 Arsenic compound, liquid, n.o.s.

1556 152  Methyldichloroarsine

1557 152 Arsenic compound, solid, n.o.s.

1558 152  Arsenic

1559 151 Arsenic pentoxide

1560 157 Arsenic chloride

1560 157 Arsenic trichloride

1561 151 Arsenic trioxide

1562 152 Arsenical dust

1564 154 Barium compound, n.o.s.

1565 157 Barium cyanide

1566 154 Beryllium compound, n.o.s.

1567 134 Beryllium powder

1569 131  Bromoacetone

1570 151  Brucine

1571 113 Barium azide, wetted with not

less than 50% water 1572 151 Cacodylic acid

1573 151 Calcium arsenate

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1574 | 151 | Calcium arsenate and calcium arsenite mixture, solid |  |  | 1601 | 151 | Disinfectant, solid, toxic, n.o.s. |  |
|  |  |  |  |  | 1602 | 151 | Dye, liquid, poisonous, n.o.s. |  |
| 1575 | 157 | Calcium cyanide |  |  | 1602 | 151 | Dye, liquid, toxic, n.o.s. |  |
| 1577 | 153 | Chlorodinitrobenzenes, liquid |  |  | 1602 | 151 | Dye intermediate, liquid, poisonous, n.o.s. |  |
| 1578 | 152 | Chloronitrobenzenes, solid |  |  |  |  |  |  |
| 1579 | 153 | 4-Chloro-o-toluidine hydrochloride, solid |  |  | 1602 | 151 | Dye intermediate, liquid, toxic, n.o.s. |  |
| 1580 | 154 | Chloropicrin |  |  | 1603 | 155 | Ethyl bromoacetate |  |
| 1581 | 123 | Chloropicrin and methyl bromide mixture |  |  | 1604 | 132 | Ethylenediamine |  |
|  |  |  |  |  | 1605 | 154 | Ethylene dibromide |  |
| 1582 | 119 | Chloropicrin and methyl chloride mixture |  |  | 1606 | 151 | Ferric arsenate |  |
| 1583 | 154 | Chloropicrin mixture, n.o.s. |  |  | 1607 | 151 | Ferric arsenite |  |
| 1585 | 151 | Copper acetoarsenite |  |  | 1608 | 151 | Ferrous arsenate |  |
| 1586 | 151 | Copper arsenite |  |  | 1611 | 151 | Hexaethyl tetraphosphate |  |
| 1587 | 151 | Copper cyanide |  |  | 1612 | 123 | Hexaethyl tetraphosphate and compressed gas mixture |  |
| 1588 | 157 | Cyanides, inorganic, solid, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 1613 | 154 | Hydrocyanic acid, aqueous solution, with less than 5% hydrogen cyanide |  |
| 1589 | 125 | Cyanogen chloride, stabilized |  |  |  |  |  |  |
| 1590 | 153 | Dichloroanilines, liquid |  |  | 1613 | 154 | Hydrocyanic acid, aqueous solution, with not more than 20% hydrogen cyanide |  |
| 1591 | 152 | o-Dichlorobenzene |  |  |  |  |  |  |
| 1593 | 160 | Dichloromethane |  |  | 1613 | 154 | Hydrogen cyanide, aqueous solution, with not more than 20% hydrogen cyanide |  |
| 1593 | 160 | Methylene chloride |  |  |  |  |  |  |
| 1594 | 152 | Diethyl sulfate |  |  |  |  |  |  |
|  |  |  |  |  | 1614 | 152 | Hydrogen cyanide, stabilized (absorbed) |  |
| 1594 | 152 | Diethyl sulphate |  |  |  |  |  |  |
| 1595 | 156 | Dimethyl sulfate |  |  | 1616 | 151 | Lead acetate |  |
| 1595 | 156 | Dimethyl sulphate |  |  | 1617 | 151 | Lead arsenates |  |
| 1596 | 153 | Dinitroanilines |  |  | 1618 | 151 | Lead arsenites |  |
| 1597 | 152 | Dinitrobenzenes, liquid |  |  | 1620 | 151 | Lead cyanide |  |
| 1598 | 153 | Dinitro-o-cresol |  |  | 1621 | 151 | London purple |  |
| 1599 | 153 | Dinitrophenol, solution |  |  | 1622 | 151 | Magnesium arsenate |  |
| 1600 | 152 | Dinitrotoluenes, molten |  |  | 1623 | 151 | Mercuric arsenate |  |
| 1601 | 151 | Disinfectant, solid, poisonous, n.o.s. |  |  | 1624 | 154 | Mercuric chloride |  |
|  |  |  |  |  | 1625 | 141 | Mercuric nitrate |  |

Page 39

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

| 1626 | 157 | Mercuric potassium cyanide | 1656 | 151 | Nicotine hydrochloride, solution |
| --- | --- | --- | --- | --- | --- |
| 1627 | 141 | Mercurous nitrate | 1657 | 151 | Nicotine salicylate |
| 1629 | 151 | Mercury acetate | 1658 | 151 | Nicotine sulfate, solution |
| 1630 | 151 | Mercury ammonium chloride | 1658 | 151 | Nicotine sulphate, solution |
| 1631 | 154 | Mercury benzoate | 1659 | 151 | Nicotine tartrate |
| 1634 | 154 | Mercury bromides | 1660 | 124 | Nitric oxide, compressed |
| 1636 | 154 | Mercury cyanide | 1661 | 153 | Nitroanilines |
| 1637 | 151 | Mercury gluconate | 1662 | 152 | Nitrobenzene |
| 1638 | 151 | Mercury iodide | 1663 | 153 | Nitrophenols |
| 1639 | 151 | Mercury nucleate | 1664 | 152 | Nitrotoluenes, liquid |
| 1640 | 151 | Mercury oleate | 1665 | 152 | Nitroxylenes, liquid |
| 1641 | 151 | Mercury oxide | 1669 | 151 | Pentachloroethane |
| 1642 | 151 | Mercury oxycyanide, desensitized | 1670 | 157 | Perchloromethyl mercaptan |
|  |  |  | 1671 | 153 | Phenol, solid |
| 1643 | 151 | Mercury potassium iodide | 1672 | 151 | Phenylcarbylamine chloride |
| 1644 | 151 | Mercury salicylate | 1673 | 153 | Phenylenediamines |
| 1645 | 151 | Mercury sulfate | 1674 | 151 | Phenylmercuric acetate |
| 1645 | 151 | Mercury sulphate | 1677 | 151 | Potassium arsenate |
| 1646 | 151 | Mercury thiocyanate | 1678 | 154 | Potassium arsenite |
| 1647 | 151 | Methyl bromide and ethylene dibromide mixture, liquid | 1679 | 157 | Potassium cuprocyanide |
| 1648 | 127 | Acetonitrile | 1680 | 157 | Potassium cyanide, solid |
| 1649 | 152 | Motor fuel anti-knock mixture | 1683 | 151 | Silver arsenite |
| 1650 | 153 | beta-Naphthylamine, solid | 1684 | 151 | Silver cyanide |
| 1650 | 153 | Naphthylamine (beta), solid | 1685 | 151 | Sodium arsenate |
| 1651 | 153 | Naphthylthiourea | 1686 | 154 | Sodium arsenite, aqueous solution |
| 1652 | 153 | Naphthylurea | 1687 | 153 | Sodium azide |
| 1653 | 151 | Nickel cyanide | 1688 | 152 | Sodium cacodylate |
| 1654 | 151 | Nicotine | 1689 | 157 | Sodium cyanide, solid |
| 1655 | 151 | Nicotine compound, solid, n.o.s. | 1690 | 154 | Sodium fluoride, solid |
| 1655 | 151 | Nicotine preparation, solid, n.o.s. | 1691 | 151 | Strontium arsenite |
| 1656 | 151 | Nicotine hydrochloride, liquid | 1692 | 151 | Strychnine |

Page 40

| ID No. | Guide Name of Material No. |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1692 | 151 | Strychnine salts |  | 1722 | 155 | Allyl chloroformate |  |
| 1693 | 159 | Tear gas devices |  | 1723 | 132 | Allyl iodide |  |
| 1693 | 159 | Tear gas substance, liquid, n.o.s. |  | 1724 | 155 | Allyltrichlorosilane, stabilized |  |
|  |  |  |  | 1725 | 137 | Aluminum bromide, anhydrous |  |
| 1694 | 159 | Bromobenzyl cyanides, liquid |  |  |  |  |  |
|  |  |  |  | 1726 | 137 | Aluminum chloride, anhydrous |  |
| 1695 | 131 | Chloroacetone, stabilized |  |  |  |  |  |
|  |  |  |  | 1727 1727 | 154 154 | Ammonium bifluoride, solid Ammonium hydrogendifluoride, solid |  |
| 1697 | 153 | Chloroacetophenone, solid |  |  |  |  |  |
| 1698 | 154 | Diphenylamine chloroarsine |  |  |  |  |  |
| 1699 | 151 | Diphenylchloroarsine, liquid |  | 1728 | 156 | Amyltrichlorosilane |  |
| 1700 | 159 | Tear gas candles |  | 1729 1730 1731 | 156 157 157 | Anisoyl chloride Antimony pentachloride, liquid Antimony pentachloride, solution |  |
| 1700 | 159 | Tear gas grenades |  |  |  |  |  |
| 1701 | 152 | Xylyl bromide, liquid |  |  |  |  |  |
| 1702 | 151 | 1,1,2,2-Tetrachloroethane |  |  |  |  |  |
|  |  |  |  | 1732 | 157 | Antimony pentafluoride |  |
| 1704 | 153 | Tetraethyl dithiopyrophosphate |  |  |  |  |  |
|  |  |  |  | 1733 1733 1733 1736 1737 1738 1739 1740 | 157 157 157 137 156 156 137 154 | Antimony trichloride Antimony trichloride, liquid Antimony trichloride, solid Benzoyl chloride Benzyl bromide Benzyl chloride Benzyl chloroformate Hydrogendifluorides, solid, n.o.s. |  |
| 1707 | 151 | Thallium compound, n.o.s. |  |  |  |  |  |
| 1708 | 153 | Toluidines, liquid |  |  |  |  |  |
| 1709 | 151 | 2,4-Toluenediamine, solid |  |  |  |  |  |
| 1709 | 151 | 2,4-Toluylenediamine, solid |  |  |  |  |  |
| 1710 | 160 | Trichloroethylene |  |  |  |  |  |
| 1711 | 153 | Xylidines, liquid |  |  |  |  |  |
| 1712 | 151 | Zinc arsenate |  |  |  |  |  |
| 1712 | 151 | Zinc arsenate and zinc arsenite mixture |  |  |  |  |  |
|  |  |  |  | 1741 | 125 | Boron trichloride |  |
| 1712 | 151 | Zinc arsenite |  |  |  |  |  |
|  |  |  |  | 1742 1743 | 157 157 | Boron trifluoride acetic acid complex, liquid Boron trifluoride propionic acid complex, liquid |  |
| 1713 | 151 | Zinc cyanide |  |  |  |  |  |
| 1714 | 139 | Zinc phosphide |  |  |  |  |  |
| 1715 | 137 | Acetic anhydride |  |  |  |  |  |
| 1716 | 156 | Acetyl bromide |  | 1744 | 154 | Bromine |  |
| 1717 | 155 | Acetyl chloride |  | 1744 | 154 | Bromine, solution |  |
| 1718 | 153 | Acid butyl phosphate |  | 1744 | 154 | Bromine, solution (Inhalation Hazard Zone A) |  |
| 1718 | 153 | Butyl acid phosphate |  |  |  |  |  |
|  |  |  |  | 1744 | 154 | Bromine, solution (Inhalation Hazard Zone B) |  |
| 1719 | 154 | Caustic alkali liquid, n.o.s. |  |  |  |  |  |
| 1722 | 155 | Allyl chlorocarbonate |  | 1745 | 144 | Bromine pentafluoride |  |

Page 41

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

1746 144 Bromine trifluoride

1747 155  Butyltrichlorosilane

1748 140 Calcium hypochlorite, dry

1748 140 Calcium hypochlorite mixture,

dry, with more than 39% available chlorine (8.8% available oxygen)

1749 124 Chlorine trifluoride

1750 153 Chloroacetic acid, solution

1751 153 Chloroacetic acid, solid

1752 156 Chloroacetyl chloride

1753 156  Chlorophenyltrichlorosilane

1754 137 Chlorosulfonic acid (with or

without sulfur trioxide)

1754 137  Chlorosulphonic acid (with or

without sulphur trioxide) 1755 154 Chromic acid, solution

1756 154 Chromic fluoride, solid

1757 154 Chromic fluoride, solution

1758 137 Chromium oxychloride

1759 154 Corrosive solid, n.o.s.

1759 154 Ferrous chloride, solid

1760 154 Chemical kit

1760 154 Compounds, cleaning liquid (corrosive)

1760 154 Compounds, tree or weed

killing, liquid (corrosive) 1760 154 Corrosive liquid, n.o.s.

1760 154 Ferrous chloride, solution

1761 154 Cupriethylenediamine, solution

1762 156  Cyclohexenyltrichlorosilane

1763 156  Cyclohexyltrichlorosilane

1764 153 Dichloroacetic acid

1765 156 Dichloroacetyl chloride

1766 156  Dichlorophenyltrichlorosilane

Page 42

1767 155  Diethyldichlorosilane

1768 154 Difluorophosphoric acid,

anhydrous

1769 156  Diphenyldichlorosilane

1770 153 Diphenylmethyl bromide

1771 156  Dodecyltrichlorosilane

1773 157 Ferric chloride, anhydrous

1774 154 Fire extinguisher charges,

corrosive liquid 1775 154 Fluoroboric acid

1776 154 Fluorophosphoric acid,

anhydrous

1777 137 Fluorosulfonic acid

1777 137 Fluorosulphonic acid

1778 154 Fluorosilicic acid

1778 154 Hydrofluorosilicic acid

1779 153 Formic acid

1779 153 Formic acid, with more than 85% acid

1780 156 Fumaryl chloride

1781 156  Hexadecyltrichlorosilane

1782 154 Hexafluorophosphoric acid

1783 153 Hexamethylenediamine, solution

1784 156  Hexyltrichlorosilane

1786 157 Hydrofluoric acid and sulfuric acid mixture

1786 157 Hydrofluoric acid and sulphuric acid mixture

1786 157 Sulfuric acid and hydrofluoric acid mixture

1786 157 Sulphuric acid and hydrofluoric acid mixture

1787 154 Hydriodic acid

1788 154 Hydrobromic acid

1789 157 Hydrochloric acid

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1789 | 157 | Muriatic acid |  |  | 1813 | 154 | Potassium hydroxide, solid |  |
| 1790 | 157 | Hydrofluoric acid |  |  | 1814 | 154 | Caustic potash, solution |  |
| 1791 | 154 | Hypochlorite solution |  |  | 1814 | 154 | Potassium hydroxide, solution |  |
| 1791 | 154 | Sodium hypochlorite |  |  | 1815 | 155 | Propionyl chloride |  |
| 1792 | 157 | Iodine monochloride, solid |  |  | 1816 | 155 | Propyltrichlorosilane |  |
| 1793 | 153 | Isopropyl acid phosphate |  |  | 1817 | 137 | Pyrosulfuryl chloride |  |
| 1794 | 154 | Lead sulfate, with more than 3% free acid |  |  | 1817 | 137 | Pyrosulphuryl chloride |  |
|  |  |  |  |  | 1818 | 157 | Silicon tetrachloride |  |
| 1794 | 154 | Lead sulphate, with more than 3% free acid |  |  |  |  |  |  |
|  |  |  |  |  | 1819 | 154 | Sodium aluminate, solution |  |
| 1796 | 157 | Nitrating acid mixture with more than 50% nitric acid |  |  | 1823 | 154 | Caustic soda, solid |  |
|  |  |  |  |  | 1823 | 154 | Sodium hydroxide, solid |  |
| 1796 | 157 | Nitrating acid mixture with not more than 50% nitric acid |  |  | 1824 | 154 | Caustic soda, solution |  |
| 1798 | 157 | Aqua regia |  |  | 1824 | 154 | Sodium hydroxide, solution |  |
| 1798 | 157 | Nitrohydrochloric acid |  |  | 1825 | 157 | Sodium monoxide |  |
| 1799 | 156 | Nonyltrichlorosilane |  |  | 1826 | 157 | Nitrating acid mixture, spent, with more than 50% nitric acid |  |
| 1800 | 156 | Octadecyltrichlorosilane |  |  |  |  |  |  |
| 1801 | 156 | Octyltrichlorosilane |  |  | 1826 | 157 | Nitrating acid mixture, spent, with not more than 50% nitric acid |  |
| 1802 | 157 | Perchloric acid, with not more than 50% acid |  |  |  |  |  |  |
|  |  |  |  |  | 1827 | 137 | Stannic chloride, anhydrous |  |
| 1803 | 153 | Phenolsulfonic acid, liquid |  |  | 1827 | 137 | Tin tetrachloride |  |
| 1803 | 153 | Phenolsulphonic acid, liquid |  |  | 1828 | 137 | Sulfur chlorides |  |
| 1804 | 156 | Phenyltrichlorosilane |  |  |  |  |  |  |
|  |  |  |  |  | 1828 | 137 | Sulphur chlorides |  |
| 1805 | 154 | Phosphoric acid, solution |  |  |  |  |  |  |
|  |  |  |  |  | 1829 | 137 | Sulfur trioxide, stabilized |  |
| 1806 | 137 | Phosphorus pentachloride |  |  |  |  |  |  |
|  |  |  |  |  | 1829 | 137 | Sulphur trioxide, stabilized |  |
| 1807 | 137 | Phosphorus pentoxide |  |  |  |  |  |  |
|  |  |  |  |  | 1830 | 137 | Sulfuric acid |  |
| 1808 | 137 | Phosphorus tribromide |  |  | 1830 | 137 | Sulfuric acid, with more than 51% acid |  |
| 1809 | 137 | Phosphorus trichloride |  |  |  |  |  |  |
| 1810 | 137 | Phosphorus oxychloride |  |  | 1830 | 137 | Sulphuric acid |  |
| 1811 | 154 | Potassium hydrogen difluoride, solid |  |  | 1830 | 137 | Sulphuric acid, with more than 51% acid |  |
| 1812 | 154 | Potassium fluoride, solid |  |  | 1831 | 137 | Sulfuric acid, fuming |  |
| 1813 | 154 | Caustic potash, solid |  |  | 1831 | 137 | Sulphuric acid, fuming |  |

Page 43

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1832 | 137 | Sulfuric acid, spent |  | 1851 | 151 | Medicine, liquid, poisonous, n.o.s. |
|  | 1832 | 137 | Sulphuric acid, spent |  |  |  |  |
|  | 1833 | 154 | Sulfurous acid |  | 1851 | 151 | Medicine, liquid, toxic, n.o.s. |
|  | 1833 | 154 | Sulphurous acid |  | 1854 | 135 | Barium alloys, pyrophoric |
|  |  |  |  |  | 1855 | 135 | Calcium, pyrophoric |
|  | 1834 | 137 | Sulfuryl chloride |  |  |  |  |
|  |  |  |  |  | 1855 | 135 | Calcium alloys, pyrophoric |
|  | 1834 | 137 | Sulphuryl chloride |  |  |  |  |
|  |  |  |  |  | 1856 | 133 | Rags, oily |
|  | 1835 | 153 | Tetramethylammonium hydroxide aqueous solution with more than 2.5% but less than 25% tetramethylammonium hydroxide |  |  |  |  |
|  |  |  |  |  | 1857 | 133 | Textile waste, wet |
|  |  |  |  |  | 1858 | 126 | Hexafluoropropylene |
|  |  |  |  |  | 1858 | 126 | Hexafluoropropylene, compressed |
|  | 1835 | 153 | Tetramethylammonium hydroxide, solution |  |  |  |  |
|  |  |  |  |  | 1858 | 126 | Refrigerant gas R-1216 |
|  | 1836 | 137 | Thionyl chloride |  | 1859 | 125 | Silicon tetrafluoride |
|  |  |  |  |  |  |  | Silicon tetrafluoride, compressed |
|  | 1837 | 157 | Thiophosphoryl chloride |  | 1859 | 125 |  |
|  | 1838 | 137 | Titanium tetrachloride |  |  |  |  |
|  | 1839 | 153 | Trichloroacetic acid |  | 1860 | 116P | Vinyl fluoride, stabilized |
|  | 1840 | 154 | Zinc chloride, solution |  | 1862 | 130 | Ethyl crotonate |
|  | 1841 | 171 | Acetaldehyde ammonia |  | 1863 | 128 | Fuel, aviation, turbine engine |
|  | 1843 | 141 | Ammonium dinitro-o-cresolate, solid |  | 1865 | 128 | n-Propyl nitrate |
|  |  |  |  |  | 1866 | 128 | Resin solution |
|  | 1845 | 120 | Carbon dioxide, solid |  | 1868 | 134 | Decaborane |
|  | 1845 | 120 | Dry ice |  | 1869 | 138 | Magnesium |
|  | 1846 | 151 | Carbon tetrachloride |  | 1869 | 138 | Magnesium, in pellets, turnings or ribbons |
|  | 1847 | 153 | Potassium sulfide, hydrated, with not less than 30% water of crystallization |  |  |  |  |
|  |  |  |  |  | 1869 | 138 | Magnesium alloys, with more than 50% magnesium, in pellets, turnings or ribbons |
|  | 1847 | 153 | Potassium sulphide, hydrated, with not less than 30% water of crystallization |  |  |  |  |
|  |  |  |  |  | 1870 | 138 | Potassium borohydride |
|  | 1848 | 153 | Propionic acid |  | 1871 | 170 | Titanium hydride |
|  | 1848 | 153 | Propionic acid, with not less than 10% and less than 90% acid |  | 1872 | 140 | Lead dioxide |
|  |  |  |  |  | 1873 | 143 | Perchloric acid, with more than 50% but not more than 72% acid |
|  | 1849 | 153 | Sodium sulfide, hydrated, with not less than 30% water |  |  |  |  |
|  |  |  |  |  | 1884 | 157 | Barium oxide |
|  | 1849 | 153 | Sodium sulphide, hydrated, with not less than 30% water |  | 1885 | 153 | Benzidine |

Page 44

| ID Guide Name of Material No. No. | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- |
| 1886 156 Benzylidene chloride 1887 160 Bromochloromethane 1888 151 Chloroform 1889 157 Cyanogen bromide 1891 131 Ethyl bromide 1892 151 Ethyldichloroarsine 1894 151 Phenylmercuric hydroxide 1895 151 Phenylmercuric nitrate 1897 160 Perchloroethylene 1897 160 Tetrachloroethylene 1898 156 Acetyl iodide 1902 153 Diisooctyl acid phosphate 1903 153 Disinfectant, liquid, corrosive, n.o.s. 1905 154 Selenic acid 1906 153 Acid, sludge 1906 153 Sludge acid 1907 154 Soda lime, with more than 4% sodium hydroxide 1908 154 Chlorite solution 1910 157 Calcium oxide 1911 119 Diborane 1911 119 Diborane mixtures 1912 115 Methyl chloride and methylene chloride mixture 1913 120 Neon, refrigerated liquid (cryogenic liquid) 1914 130 Butyl propionates 1915 127 Cyclohexanone 1916 152 2,2'-Dichlorodiethyl ether 1916 152 Dichloroethyl ether 1917 129P Ethyl acrylate, stabilized 1918 130 Cumene 1918 130 Isopropylbenzene | 1919 129P Methyl acrylate, stabilized 1920 128 Nonanes 1921 131P Propyleneimine, stabilized 1922 132 Pyrrolidine |  |  |  |
|  |  | 1923 135 | Calcium dithionite |  |
|  |  | 1923 135 | Calcium hydrosulfite |  |
|  |  | 1923 135 | Calcium hydrosulphite |  |
|  |  | 1928 138 | Methyl magnesium bromide in ethyl ether |  |
|  |  | 1929 135 | Potassium dithionite |  |
|  |  | 1929 135 | Potassium hydrosulfite |  |
|  |  | 1929 135 | Potassium hydrosulphite |  |
|  |  | 1931 171 | Zinc dithionite |  |
|  |  | 1931 171 | Zinc hydrosulfite |  |
|  |  | 1931 171 | Zinc hydrosulphite |  |
|  |  | 1932 135 1935 157 1938 156 1939 137 1940 153 1941 171 1941 171 1942 140 1944 133 1945 133 1950 126 1951 120 1952 126 | Zirconium scrap Cyanide solution, n.o.s. Bromoacetic acid, solution Phosphorus oxybromide, solid Thioglycolic acid Dibromodifluoromethane Refrigerant gas R-12B2 Ammonium nitrate, with not more than 0.2% combustible substances Matches, safety Matches, wax “vesta” Aerosols Argon, refrigerated liquid (cryogenic liquid) Ethylene oxide and carbon dioxide mixture, with not more than 9% ethylene oxide |  |
|  |  | 1953 119 | Compressed gas, poisonous, flammable, n.o.s. |  |

Page 45

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1953 | 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  | 1955 | 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) |
|  | 1953 | 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 1955 | 123 | Compressed gas, toxic, n.o.s. |
|  |  |  |  |  |  | 1955 | 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone A) |
|  | 1953 | 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  | 1955 | 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone B) |
|  | 1953 | 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  | 1955 | 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone C) |
|  |  |  |  |  |  | 1955 | 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone D) |
|  | 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 1955 | 123 | Organic phosphate compound mixed with compressed gas |
|  | 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  |  |  | 1955 | 123 | Organic phosphate mixed with compressed gas |
|  | 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 1955 | 123 | Organic phosphorus compound mixed with compressed gas |
|  | 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  | 1956 | 126 | Compressed gas, n.o.s. |
|  |  |  |  |  |  | 1957 | 115 | Deuterium, compressed |
|  |  |  |  |  |  | 1958 | 126 | 1,2-Dichloro-1,1,2,2- tetrafluoroethane |
|  | 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |
|  |  |  |  |  |  | 1958 | 126 | Refrigerant gas R-114 |
|  | 1954 | 115 | Compressed gas, flammable, n.o.s. |  |  | 1959 | 116P | 1,1-Difluoroethylene |
|  |  |  |  |  |  | 1959 | 116P | Refrigerant gas R-1132a |
|  | 1954 | 115 | Dispersant gases, n.o.s. (flammable) |  |  | 1961 | 115 | Ethane, refrigerated liquid |
|  | 1954 | 115 | Refrigerant gases, n.o.s. (flammable) |  |  | 1961 | 115 | Ethane-propane mixture, refrigerated liquid |
|  |  |  |  |  |  | 1962 | 116P | Ethylene |
|  | 1955 | 123 | Compressed gas, poisonous, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 1962 | 116P | Ethylene, compressed |
|  | 1955 | 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone A) |  |  | 1963 | 120 | Helium, refrigerated liquid (cryogenic liquid) |
|  |  |  |  |  |  | 1964 | 115 | Hydrocarbon gas mixture, compressed, n.o.s. |
|  | 1955 | 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  |  |  | 1965 | 115 | Hydrocarbon gas mixture, liquefied, n.o.s. |
|  | 1955 | 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |  |
|  |  |  |  |  |  | 1966 | 115 | Hydrogen, refrigerated liquid (cryogenic liquid) |

Page 46

| ID No. 1967 | Guid No. 123 | e Name of Material Insecticide gas, poisonous, n.o.s. |  | ID No. 1983 | Guide No. 126 | Name of Material Refrigerant gas R-133a |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1984 | 126 | Refrigerant gas R-23 |
| 1967 | 123 | Insecticide gas, toxic, n.o.s. |  | 1984 | 126 | Trifluoromethane |
| 1967 | 123 | Parathion and compressed gas mixture |  | 1986 | 131 | Alcohols, flammable, poisonous, n.o.s. |
| 1968 | 126 | Insecticide gas, n.o.s. |  | 1986 | 131 | Alcohols, flammable, toxic, n.o.s. |
| 1969 | 115 | Isobutane |  |  |  |  |
| 1970 | 120 | Krypton, refrigerated liquid (cryogenic liquid) |  | 1987 | 127 | Alcohols, n.o.s. |
|  |  |  |  | 1987 | 127 | Denatured alcohol |
| 1971 | 115 | Methane, compressed |  | 1988 | 131P | Aldehydes, flammable, poisonous, n.o.s. |
| 1971 | 115 | Natural gas, compressed |  |  |  |  |
| 1972 | 115 | Liquefied natural gas (cryogenic liquid) |  | 1988 | 131P | Aldehydes, flammable, toxic, n.o.s. |
| 1972 | 115 | LNG (cryogenic liquid) |  | 1989 | 129P | Aldehydes, n.o.s. |
| 1972 | 115 | Methane, refrigerated liquid (cryogenic liquid) |  | 1990 | 171 | Benzaldehyde |
|  |  |  |  | 1991 | 131P | Chloroprene, stabilized |
| 1972 | 115 | Natural gas, refrigerated liquid (cryogenic liquid) |  | 1992 | 131 | Flammable liquid, poisonous, n.o.s. |
| 1973 | 126 | Chlorodifluoromethane and chloropentafluoroethane mixture |  | 1992 | 131 | Flammable liquid, toxic, n.o.s. |
|  |  |  |  | 1993 | 128 | Combustible liquid, n.o.s. |
| 1973 | 126 | Refrigerant gas R-502 |  | 1993 | 128 | Compounds, cleaning liquid (flammable) |
| 1974 | 126 | Chlorodifluorobromomethane |  |  |  |  |
| 1974 | 126 | Refrigerant gas R-12B1 |  | 1993 | 128 | Compounds, tree or weed killing, liquid (flammable) |
| 1975 | 124 | Nitric oxide and dinitrogen tetroxide mixture |  | 1993 | 128 | Diesel fuel |
| 1975 | 124 | Nitric oxide and nitrogen dioxide mixture |  | 1993 | 128 | Flammable liquid, n.o.s. |
|  |  |  |  | 1993 | 128 | Fuel oil |
| 1976 | 126 | Octafluorocyclobutane |  | 1994 | 136 | Iron pentacarbonyl |
| 1976 | 126 | Refrigerant gas RC-318 |  | 1999 | 130 | Asphalt |
| 1977 | 120 | Nitrogen, refrigerated liquid (cryogenic liquid) |  | 1999 | 130 | Asphalt, cut back |
| 1978 | 115 | Propane |  | 1999 | 130 | Tars, liquid |
| 1982 | 126 | Refrigerant gas R-14 |  | 2000 | 133 | Celluloid, in block, rods, rolls, sheets, tubes, etc., except scrap |
| 1982 | 126 | Tetrafluoromethane |  |  |  |  |
| 1983 | 126 | 1-Chloro-2,2,2-trifluoroethane |  | 2001 | 133 | Cobalt naphthenates, powder |
|  |  |  |  | 2002 | 135 | Celluloid, scrap |

Page 47

| ID No. 2004 | Guide No. 135 | Name of Material Magnesium diamide |  |  | ID No. 2028 | Guide No. 153 | Name of Material Bombs, smoke, non-explosive, with corrosive liquid, without initiating device |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 | 135 | Plastics, nitrocellulose-based, self-heating, n.o.s. |  |  |  |  |  |
| 2008 | 135 | Zirconium powder, dry |  |  | 2029 | 132 | Hydrazine, anhydrous |
| 2009 | 135 | Zirconium, dry, finished sheets, strip or coiled wire |  |  | 2030 | 153 | Hydrazine, aqueous solution, with more than 37% hydrazine |
| 2010 | 138 | Magnesium hydride |  |  | 2031 | 157 | Nitric acid, other than red fuming, with more than 65% nitric acid |
| 2011 | 139 | Magnesium phosphide |  |  |  |  |  |
| 2012 | 139 | Potassium phosphide |  |  | 2031 | 157 | Nitric acid, other than red fuming, with not more than 65% nitric acid |
| 2013 | 139 | Strontium phosphide |  |  |  |  |  |
| 2014 | 140 | Hydrogen peroxide, aqueous solution, with not less than 20% but not more than 60% hydrogen peroxide (stabilized as necessary) |  |  |  |  |  |
|  |  |  |  |  | 2032 | 157 | Nitric acid, red fuming |
|  |  |  |  |  | 2033 | 154 | Potassium monoxide |
|  |  |  |  |  | 2034 | 115 | Hydrogen and methane mixture, compressed |
| 2015 | 143 | Hydrogen peroxide, aqueous solution, stabilized, with more than 60% hydrogen peroxide |  |  |  |  |  |
|  |  |  |  |  | 2035 | 115 | Refrigerant gas R-143a |
| 2015 | 143 | Hydrogen peroxide, stabilized |  |  | 2035 | 115 | 1,1,1-Trifluoroethane |
| 2016 | 151 | Ammunition, poisonous, non-explosive |  |  | 2036 | 120 | Xenon |
|  |  |  |  |  | 2036 | 120 | Xenon, compressed |
| 2016 | 151 | Ammunition, toxic, non-explosive |  |  | 2037 | 115 | Gas cartridges |
|  |  |  |  |  | 2037 | 115 | Receptacles, small, containing gas |
| 2017 | 159 | Ammunition, tear-producing, non-explosive |  |  |  |  |  |
|  |  |  |  |  | 2038 | 152 | Dinitrotoluenes, liquid |
| 2018 | 152 | Chloroanilines, solid |  |  | 2044 | 115 | 2,2-Dimethylpropane |
| 2019 | 152 | Chloroanilines, liquid |  |  | 2045 | 130 | Isobutyl aldehyde |
| 2020 | 153 | Chlorophenols, solid |  |  | 2045 | 130 | Isobutyraldehyde |
| 2021 | 153 | Chlorophenols, liquid |  |  | 2046 | 130 | Cymenes |
| 2022 | 153 | Cresylic acid |  |  | 2047 | 129 | Dichloropropenes |
| 2023 | 131P | Epichlorohydrin |  |  | 2048 | 130P | Dicyclopentadiene |
| 2024 | 151 | Mercury compound, liquid, n.o.s. |  |  | 2049 | 130 | Diethylbenzene |
| 2025 | 151 | Mercury compound, solid, n.o.s. |  |  | 2050 | 128 | Diisobutylene, isomeric compounds |
| 2026 | 151 | Phenylmercuric compound, n.o.s. |  |  | 2051 | 132 | 2-Dimethylaminoethanol |
| 2027 | 151 | Sodium arsenite, solid |  |  | 2052 | 128 | Dipentene |

Page 48

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2053 | 129 | Methyl isobutyl carbinol |  |  | 2195 | 125 | Tellurium hexafluoride |  |
| 2054 | 132 | Morpholine |  |  | 2196 | 125 | Tungsten hexafluoride |  |
| 2055 | 128P | Styrene monomer, stabilized |  |  | 2197 | 125 | Hydrogen iodide, anhydrous |  |
| 2056 | 127 | Tetrahydrofuran |  |  | 2198 | 125 | Phosphorus pentafluoride |  |
| 2057 | 128 | Tripropylene |  |  | 2198 | 125 | Phosphorus pentafluoride, compressed |  |
| 2058 | 129 | Valeraldehyde |  |  |  |  |  |  |
|  |  |  |  |  | 2199 | 119 | Phosphine |  |
| 2059 | 127 | Nitrocellulose solution, flammable |  |  |  |  |  |  |
|  |  |  |  |  | 2200 | 116P | Propadiene, stabilized |  |
| 2067 | 140 | Ammonium nitrate based fertilizer |  |  | 2201 | 122 | Nitrous oxide, refrigerated liquid |  |
|  |  |  |  |  | 2202 | 117 | Hydrogen selenide, anhydrous |  |
| 2071 | 140 | Ammonium nitrate based fertilizer |  |  |  |  |  |  |
|  |  |  |  |  | 2203 | 116 | Silane |  |
| 2073 | 125 | Ammonia solution, with more than 35% but not more than 50% ammonia |  |  | 2204 | 119 | Carbonyl sulfide |  |
|  |  |  |  |  | 2204 | 119 | Carbonyl sulphide |  |
|  |  |  |  |  | 2205 | 153 | Adiponitrile |  |
| 2074 | 153P | Acrylamide, solid |  |  |  |  |  |  |
| 2075 | 153 | Chloral, anhydrous, stabilized |  |  | 2206 | 156 | Isocyanate solution, poisonous, n.o.s. |  |
| 2076 | 153 | Cresols, liquid |  |  | 2206 | 156 | Isocyanate solution, toxic, n.o.s. |  |
| 2077 | 153 | alpha-Naphthylamine |  |  |  |  |  |  |
| 2077 | 153 | Naphthylamine (alpha) |  |  | 2206 | 156 | Isocyanates, poisonous, n.o.s. |  |
| 2078 | 156 | Toluene diisocyanate |  |  | 2206 | 156 | Isocyanates, toxic, n.o.s. |  |
| 2079 | 154 | Diethylenetriamine |  |  | 2208 | 140 | Bleaching powder |  |
| 2186 | 125 | Hydrogen chloride, refrigerated liquid |  |  | 2208 | 140 | Calcium hypochlorite mixture, dry, with more than 10% but not more than 39% available chlorine |  |
| 2187 | 120 | Carbon dioxide, refrigerated liquid |  |  |  |  |  |  |
|  |  |  |  |  | 2209 | 153 | Formaldehyde, solution |  |
| 2188 | 119 | Arsine |  |  |  |  | (corrosive) |  |
| 2189 | 119 | Dichlorosilane |  |  | 2209 | 153 | Formalin (corrosive) |  |
| 2190 | 124 | Oxygen difluoride, compressed |  |  | 2210 | 135 | Maneb |  |
| 2191 | 123 | Sulfuryl fluoride |  |  | 2210 | 135 | Maneb preparation, with not less than 60% maneb |  |
| 2191 | 123 | Sulphuryl fluoride |  |  |  |  |  |  |
|  |  |  |  |  | 2211 | 171 | Polymeric beads, expandable |  |
| 2192 | 119 | Germane |  |  |  |  |  |  |
|  |  |  |  |  | 2212 | 171 | Asbestos |  |
| 2193 | 126 | Hexafluoroethane |  |  |  |  |  |  |
| 2193 | 126 | Refrigerant gas R-116 |  |  | 2212 | 171 | Asbestos, amphibole |  |
| 2194 | 125 | Selenium hexafluoride |  |  | 2213 | 133 | Paraformaldehyde |  |

Page 49

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2214 | 156 | Phthalic anhydride |  | 2246 | 128 | Cyclopentene |
|  | 2215 | 156 | Maleic anhydride |  | 2247 | 128 | n-Decane |
|  | 2215 | 156 | Maleic anhydride, molten |  | 2248 | 132 | Di-n-butylamine |
|  | 2216 | 171 | Fish meal, stabilized |  | 2249 | 131 | Dichlorodimethyl ether, symmetrical |
|  | 2216 | 171 | Fish scrap, stabilized |  |  |  |  |
|  | 2217 | 135 | Seed cake, with not more than 1.5% oil and not more than 11% moisture |  | 2250 | 156 | Dichlorophenyl isocyanates |
|  |  |  |  |  | 2251 | 128P | Bicyclo[2.2.1]hepta-2,5-diene, stabilized |
|  | 2218 | 132P | Acrylic acid, stabilized |  | 2251 | 128P | 2,5-Norbornadiene, stabilized |
|  | 2219 | 129 | Allyl glycidyl ether |  | 2252 | 127 | 1,2-Dimethoxyethane |
|  | 2222 | 128 | Anisole |  | 2253 | 153 | N,N-Dimethylaniline |
|  | 2224 | 152 | Benzonitrile |  | 2254 | 133 | Matches, fusee |
|  | 2225 | 156 | Benzenesulfonyl chloride |  | 2256 | 130 | Cyclohexene |
|  | 2225 | 156 | Benzenesulphonyl chloride |  | 2257 | 138 | Potassium |
|  | 2226 | 156 | Benzotrichloride |  | 2258 | 132 | 1,2-Propylenediamine |
|  | 2227 | 130P | n-Butyl methacrylate, stabilized |  | 2259 | 153 | Triethylenetetramine |
|  | 2232 | 153 | Chloroacetaldehyde |  | 2260 | 132 | Tripropylamine |
|  | 2232 | 153 | 2-Chloroethanal |  | 2261 | 153 | Xylenols, solid |
|  | 2233 | 152 | Chloroanisidines |  | 2262 | 156 | Dimethylcarbamoyl chloride |
|  | 2234 | 130 | Chlorobenzotrifluorides |  | 2263 | 128 | Dimethylcyclohexanes |
|  | 2235 | 153 | Chlorobenzyl chlorides, liquid |  | 2264 | 132 | N,N-Dimethylcyclohexylamine |
|  | 2236 | 156 | 3-Chloro-4-methylphenyl isocyanate, liquid |  | 2264 | 132 | Dimethylcyclohexylamine |
|  |  |  |  |  | 2265 | 129 | N,N-Dimethylformamide |
|  | 2237 | 153 | Chloronitroanilines |  | 2266 | 132 | Dimethyl-N-propylamine |
|  | 2238 | 129 | Chlorotoluenes |  | 2267 | 156 | Dimethyl thiophosphoryl chloride |
|  | 2239 | 153 | Chlorotoluidines, solid |  |  |  |  |
|  | 2240 | 154 | Chromosulfuric acid |  | 2269 | 153 | 3,3'-Iminodipropylamine |
|  | 2240 | 154 | Chromosulphuric acid |  | 2270 | 132 | Ethylamine, aqueous solution, with not less than 50% but not more than 70% ethylamine |
|  | 2241 | 128 | Cycloheptane |  |  |  |  |
|  | 2242 | 128 | Cycloheptene |  |  |  |  |
|  | 2243 | 130 | Cyclohexyl acetate |  | 2271 | 128 | Ethyl amyl ketone |
|  | 2244 | 129 | Cyclopentanol |  | 2272 | 153 | N-Ethylaniline |
|  | 2245 | 128 | Cyclopentanone |  | 2273 | 153 | 2-Ethylaniline |

Page 50

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

2274 153 N-Ethyl-N-benzylaniline

2275 129 2-Ethylbutanol

2276 132 2-Ethylhexylamine

2277 130P Ethyl methacrylate, stabilized 2278 128 n-Heptene

2279 151  Hexachlorobutadiene

2280 153 Hexamethylenediamine, solid

2281 156 Hexamethylene diisocyanate

2282 129  Hexanols

2283 130P Isobutyl methacrylate, stabilized

| 2284 | 131 | Isobutyronitrile |
| --- | --- | --- |
| 2285 | 155 | Isocyanatobenzotrifluorides |
| 2286 | 128 | Pentamethylheptane |
| 2287 | 128 | Isoheptenes |
| 2288 | 128 | Isohexenes |
| 2289 | 153 | Isophoronediamine |
| 2290 | 156 | Isophorone diisocyanate |
| 2291 | 151 | Lead compound, soluble, n.o.s. |
| 2293 | 128 | 4-Methoxy-4-methylpentan-2- |

one

2294 153 N-Methylaniline

2295 131 Methyl chloroacetate

2296 128  Methylcyclohexane

2297 128  Methylcyclohexanone

2298 128  Methylcyclopentane

2299 156 Methyl dichloroacetate

2300 153 2-Methyl-5-ethylpyridine

2301 128 2-Methylfuran

2302 127 5-Methylhexan-2-one

2303 128  Isopropenylbenzene

2304 133 Naphthalene, molten

2305 153 Nitrobenzenesulfonic acid

2305 153 Nitrobenzenesulphonic acid

2306 152 Nitrobenzotrifluorides, liquid

2307 152 3-Nitro-4-chlorobenzotrifluoride

2308 157 Nitrosylsulfuric acid, liquid

2308 157 Nitrosylsulphuric acid, liquid

2309 128P Octadiene

2310 131 Pentane-2,4-dione

2311 153  Phenetidines

2312 153 Phenol, molten

2313 129  Picolines

2315 171 PCB, liquid

2315 171 Polychlorinated biphenyls, liquid

2316 157 Sodium cuprocyanide, solid

2317 157 Sodium cuprocyanide, solution

2318 135 Sodium hydrosulfide, with

less than 25% water of crystallization

2318 135 Sodium hydrosulphide, with

less than 25% water of crystallization

2319 128 Terpene hydrocarbons, n.o.s.

2320 153  Tetraethylenepentamine

2321 153 Trichlorobenzenes, liquid

2322 152  Trichlorobutene

2323 130 Triethyl phosphite

2324 128  Triisobutylene

2325 129 1,3,5-Trimethylbenzene

2326 153  Trimethylcyclohexylamine

2327 153  Trimethylhexamethylenediamines

2328 156  Trimethylhexamethylene

diisocyanate 2329 130 Trimethyl phosphite

2330 128  Undecane

2331 154 Zinc chloride, anhydrous

Page 51

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2332 | 129 | Acetaldehyde oxime |  |  | 2366 | 128 | Diethyl carbonate |
| 2333 | 131 | Allyl acetate |  |  | 2367 | 130 | alpha-Methylvaleraldehyde |
| 2334 | 131 | Allylamine |  |  | 2367 | 130 | Methyl valeraldehyde (alpha) |
| 2335 | 131 | Allyl ethyl ether |  |  | 2368 | 128 | alpha-Pinene |
| 2336 | 131 | Allyl formate |  |  | 2368 | 128 | Pinene (alpha) |
| 2337 | 131 | Phenyl mercaptan |  |  | 2370 | 128 | 1-Hexene |
| 2338 | 127 | Benzotrifluoride |  |  | 2371 | 128 | Isopentenes |
| 2339 | 130 | 2-Bromobutane |  |  | 2372 | 129 | 1,2-Di-(dimethylamino) ethane |
| 2340 | 130 | 2-Bromoethyl ethyl ether |  |  | 2373 | 127 | Diethoxymethane |
| 2341 | 130 | 1-Bromo-3-methylbutane |  |  | 2374 | 127 | 3,3-Diethoxypropene |
| 2342 | 130 | Bromomethylpropanes |  |  | 2375 | 129 | Diethyl sulfide |
| 2343 | 130 | 2-Bromopentane |  |  | 2375 | 129 | Diethyl sulphide |
| 2344 | 129 | Bromopropanes |  |  | 2376 | 127 | 2,3-Dihydropyran |
| 2345 | 130 | 3-Bromopropyne |  |  | 2377 | 127 | 1,1-Dimethoxyethane |
| 2346 | 127 | Butanedione |  |  | 2378 | 131 | 2-Dimethylaminoacetonitrile |
| 2346 | 127 | Diacetyl |  |  | 2379 | 132 | 1,3-Dimethylbutylamine |
| 2347 | 130 | Butyl mercaptan |  |  | 2380 | 127 | Dimethyldiethoxysilane |
| 2348 | 129P | Butyl acrylates, stabilized |  |  | 2381 | 131 | Dimethyl disulfide |
| 2350 | 127 | Butyl methyl ether |  |  | 2381 | 131 | Dimethyl disulphide |
| 2351 | 129 | Butyl nitrites |  |  | 2382 | 131 | Dimethylhydrazine, symmetrical |
| 2352 | 127P | Butyl vinyl ether, stabilized |  |  | 2383 | 132 | Dipropylamine |
| 2353 | 155 | Butyryl chloride |  |  | 2384 | 127 | Di-n-propyl ether |
| 2354 | 131 | Chloromethyl ethyl ether |  |  | 2385 | 129 | Ethyl isobutyrate |
| 2356 | 129 | 2-Chloropropane |  |  | 2386 | 132 | 1-Ethylpiperidine |
| 2357 | 132 | Cyclohexylamine |  |  | 2387 | 130 | Fluorobenzene |
| 2358 | 128P | Cyclooctatetraene |  |  | 2388 | 130 | Fluorotoluenes |
| 2359 | 132 | Diallylamine |  |  | 2389 | 128 | Furan |
| 2360 | 131P | Diallyl ether |  |  | 2390 | 129 | 2-Iodobutane |
| 2361 | 132 | Diisobutylamine |  |  | 2391 | 129 | Iodomethylpropanes |
| 2362 | 130 | 1,1-Dichloroethane |  |  | 2392 | 129 | Iodopropanes |
| 2363 | 129 | Ethyl mercaptan |  |  | 2393 | 129 | Isobutyl formate |
| 2364 | 128 | n-Propylbenzene |  |  | 2394 | 129 | Isobutyl propionate |

Page 52

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2395 | 155 | Isobutyryl chloride |  |  | 2427 | 140 | Potassium chlorate, aqueous solution |  |
| 2396 | 131P | Methacrylaldehyde, stabilized |  |  |  |  |  |  |
| 2397 | 127 | 3-Methylbutan-2-one |  |  | 2428 | 140 | Sodium chlorate, aqueous solution |  |
| 2398 | 127 | Methyl tert-butyl ether |  |  | 2429 | 140 | Calcium chlorate, aqueous solution |  |
| 2399 | 132 | 1-Methylpiperidine |  |  |  |  |  |  |
| 2400 | 130 | Methyl isovalerate |  |  | 2430 | 153 | Alkylphenols, solid, n.o.s. (including C2-C12 homologues) |  |
| 2401 | 132 | Piperidine |  |  |  |  |  |  |
| 2402 | 130 | Propanethiols |  |  | 2431 | 153 | Anisidines |  |
| 2403 | 129P | Isopropenyl acetate |  |  | 2432 | 153 | N,N-Diethylaniline |  |
| 2404 | 131 | Propionitrile |  |  | 2433 | 152 | Chloronitrotoluenes, liquid |  |
| 2405 | 129 | Isopropyl butyrate |  |  | 2434 | 156 | Dibenzyldichlorosilane |  |
| 2406 | 127 | Isopropyl isobutyrate |  |  | 2435 | 156 | Ethylphenyldichlorosilane |  |
| 2407 | 155 | Isopropyl chloroformate |  |  | 2436 | 129 | Thioacetic acid |  |
| 2409 | 129 | Isopropyl propionate |  |  | 2437 | 156 | Methylphenyldichlorosilane |  |
| 2410 | 129 | 1,2,3,6-Tetrahydropyridine |  |  | 2438 | 131 | Trimethylacetyl chloride |  |
| 2411 | 131 | Butyronitrile |  |  | 2439 | 154 | Sodium hydrogendifluoride |  |
| 2412 | 130 | Tetrahydrothiophene |  |  | 2440 | 154 | Stannic chloride, pentahydrate |  |
| 2413 | 128 | Tetrapropyl orthotitanate |  |  | 2441 | 135 | Titanium trichloride, pyrophoric |  |
| 2414 | 130 | Thiophene |  |  | 2441 | 135 | Titanium trichloride mixture, pyrophoric |  |
| 2416 | 129 | Trimethyl borate |  |  |  |  |  |  |
| 2417 | 125 | Carbonyl fluoride |  |  | 2442 | 156 | Trichloroacetyl chloride |  |
| 2418 | 125 | Sulfur tetrafluoride |  |  | 2443 | 137 | Vanadium oxytrichloride |  |
| 2418 | 125 | Sulphur tetrafluoride |  |  | 2444 | 137 | Vanadium tetrachloride |  |
|  |  |  |  |  | 2446 | 153 | Nitrocresols, solid |  |
| 2419 | 116 | Bromotrifluoroethylene |  |  |  |  |  |  |
| 2420 | 125 | Hexafluoroacetone |  |  | 2447 | 136 | Phosphorus, white, molten |  |
| 2421 | 124 | Nitrogen trioxide |  |  | 2447 | 136 | White phosphorus, molten |  |
| 2422 | 126 | Octafluorobut-2-ene |  |  | 2448 | 133 | Molten sulfur |  |
| 2422 | 126 | Refrigerant gas R-1318 |  |  | 2448 | 133 | Molten sulphur |  |
| 2424 | 126 | Octafluoropropane |  |  | 2448 | 133 | Sulfur, molten |  |
| 2424 | 126 | Refrigerant gas R-218 |  |  | 2448 | 133 | Sulphur, molten |  |
| 2426 | 140 | Ammonium nitrate, liquid (hot concentrated solution) |  |  | 2451 | 122 | Nitrogen trifluoride |  |
|  |  |  |  |  | 2452 | 116P | Ethylacetylene, stabilized |  |

Page 53

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2453 | 115 | Ethyl fluoride |  |  | 2480 | 155P | Methyl isocyanate |
|  | 2453 | 115 | Refrigerant gas R-161 |  |  | 2481 | 155 | Ethyl isocyanate |
|  | 2454 | 115 | Methyl fluoride |  |  | 2482 | 155P | n-Propyl isocyanate |
|  | 2454 | 115 | Refrigerant gas R-41 |  |  | 2483 | 155P | Isopropyl isocyanate |
|  | 2455 | 116 | Methyl nitrite |  |  | 2484 | 155 | tert-Butyl isocyanate |
|  | 2456 | 130P | 2-Chloropropene |  |  | 2485 | 155P | n-Butyl isocyanate |
|  | 2457 | 128 | 2,3-Dimethylbutane |  |  | 2486 | 155P | Isobutyl isocyanate |
|  | 2458 | 130 | Hexadiene |  |  | 2487 | 155 | Phenyl isocyanate |
|  | 2459 | 128 | 2-Methyl-1-butene |  |  | 2488 | 155 | Cyclohexyl isocyanate |
|  | 2460 | 128 | 2-Methyl-2-butene |  |  | 2490 | 153 | Dichloroisopropyl ether |
|  | 2461 | 128 | Methylpentadiene |  |  | 2491 | 153 | Ethanolamine |
|  | 2463 | 138 | Aluminum hydride |  |  | 2491 | 153 | Ethanolamine, solution |
|  | 2464 | 141 | Beryllium nitrate |  |  | 2491 | 153 | Monoethanolamine |
|  | 2465 | 140 | Dichloroisocyanuric acid, dry |  |  | 2493 | 132 | Hexamethyleneimine |
|  | 2465 | 140 | Dichloroisocyanuric acid salts |  |  | 2495 | 144 | Iodine pentafluoride |
|  | 2465 | 140 | Sodium dichloroisocyanurate |  |  | 2496 | 156 | Propionic anhydride |
|  | 2465 | 140 | Sodium dichloro-s-triazinetrione |  |  | 2498 | 129 | 1,2,3,6-Tetrahydrobenzaldehyde |
|  | 2466 | 143 | Potassium superoxide |  |  | 2501 | 152 | Tris-(1-aziridinyl)phosphine oxide, solution |
|  | 2468 | 140 | Trichloroisocyanuric acid, dry |  |  |  |  |  |
|  | 2469 | 140 | Zinc bromate |  |  | 2502 | 132 | Valeryl chloride |
|  | 2470 | 152 | Phenylacetonitrile, liquid |  |  | 2503 | 137 | Zirconium tetrachloride |
|  | 2471 | 154 | Osmium tetroxide |  |  | 2504 | 159 | Acetylene tetrabromide |
|  | 2473 | 154 | Sodium arsanilate |  |  | 2504 | 159 | Tetrabromoethane |
|  | 2474 | 156 | Thiophosgene |  |  | 2505 | 154 | Ammonium fluoride |
|  |  |  |  |  |  | 2506 | 154 | Ammonium hydrogen sulfate |
|  | 2475 | 157 | Vanadium trichloride |  |  |  |  |  |
|  |  |  |  |  |  | 2506 | 154 | Ammonium hydrogen sulphate |
|  | 2477 | 131 | Methyl isothiocyanate |  |  |  |  |  |
|  |  |  |  |  |  | 2507 | 154 | Chloroplatinic acid, solid |
|  | 2478 | 155 | Isocyanate solution, flammable, poisonous, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 2508 | 156 | Molybdenum pentachloride |
|  | 2478 | 155 | Isocyanate solution, flammable, toxic, n.o.s. |  |  | 2509 | 154 | Potassium hydrogen sulfate |
|  |  |  |  |  |  | 2509 | 154 | Potassium hydrogen sulphate |
|  | 2478 | 155 | Isocyanates, flammable, poisonous, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 2511 | 153 | 2-Chloropropionic acid |
|  | 2478 | 155 | Isocyanates, flammable, toxic, n.o.s. |  |  | 2512 | 152 | Aminophenols |

Page 54

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

2513 156 Bromoacetyl bromide

2514 130  Bromobenzene

2515 159  Bromoform

2516 151 Carbon tetrabromide

2517 115 1-Chloro-1,1-difluoroethane

2517 115  Difluorochloroethanes

2517 115 Refrigerant gas R-142b

2518 153 1,5,9-Cyclododecatriene

2521 131P Diketene, stabilized

2520 130P Cyclooctadienes

2522 153P 2-Dimethylaminoethyl

methacrylate, stabilized 2524 129 Ethyl orthoformate

2525 156 Ethyl oxalate

2526 132  Furfurylamine

2527 129P Isobutyl acrylate, stabilized 2528 130 Isobutyl isobutyrate

2529 132 Isobutyric acid

2534 119  Methylchlorosilane

2531 153P Methacrylic acid, stabilized 2533 156 Methyl trichloroacetate

2535 132 4-Methylmorpholine

2535 132 N-Methylmorpholine

2536 127  Methyltetrahydrofuran

2538 133  Nitronaphthalene

2541 128  Terpinolene

2542 153  Tributylamine

2545 135 Hafnium powder, dry

2546 135 Titanium powder, dry

2548 124 Chlorine pentafluoride

2547 143 Sodium superoxide

2552 151 Hexafluoroacetone hydrate,

liquid

2554 130P Methylallyl chloride

2555 113 Nitrocellulose with water, not

less than 25% water

2556 113 Nitrocellulose with alcohol, not

less than 25% alcohol

2557 133 Nitrocellulose mixture, with or

without pigment

2557 133 Nitrocellulose mixture, with or

without plasticizer 2558 131 Epibromohydrin

2560 129 2-Methylpentan-2-ol

2561 128 3-Methyl-1-butene

2564 153 Trichloroacetic acid, solution

2565 153  Dicyclohexylamine

2567 154 Sodium pentachlorophenate

2570 154 Cadmium compound

2571 156 Alkylsulfuric acids

2571 156 Alkylsulphuric acids

2572 153  Phenylhydrazine

2573 141 Thallium chlorate

2574 151 Tricresyl phosphate

2576 137 Phosphorus oxybromide, molten

2577 156 Phenylacetyl chloride

2578 157 Phosphorus trioxide

2579 153  Piperazine

2580 154 Aluminum bromide, solution

2581 154 Aluminum chloride, solution

2582 154 Ferric chloride, solution

2583 153 Alkyl sulfonic acids, solid, with

more than 5% free sulfuric acid

2583 153 Alkyl sulphonic acids, solid, with

more than 5% free sulphuric acid

Page 55

| ID No. 2583 | Guid No. 153 | e Name of Material Aryl sulfonic acids, solid, with more than 5% free sulfuric acid | ID No. 2588 | Guide No. 151 | Name of Material Pesticide, solid, poisonous, n.o.s. |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 2588 | 151 | Pesticide, solid, toxic, n.o.s. |
| 2583 | 153 | Aryl sulphonic acids, solid, with more than 5% free sulphuric acid | 2589 | 155 | Vinyl chloroacetate |
|  |  |  | 2590 | 171 | Asbestos, chrysotile |
| 2584 | 153 | Alkyl sulfonic acids, liquid, with more than 5% free sulfuric acid | 2591 | 120 | Xenon, refrigerated liquid (cryogenic liquid) |
| 2584 | 153 | Alkyl sulphonic acids, liquid, with more than 5% free sulphuric acid | 2599 | 126 | Chlorotrifluoromethane and trifluoromethane azeotropic mixture with approximately 60% chlorotrifluoromethane |
| 2584 | 153 | Aryl sulfonic acids, liquid, with more than 5% free sulfuric acid | 2599 | 126 | Refrigerant gas R-503 |
|  |  |  | 2601 | 115 | Cyclobutane |
| 2584 | 153 | Aryl sulphonic acids, liquid, with more than 5% free sulphuric acid | 2602 | 126 | Dichlorodifluoromethane and difluoroethane azeotropic mixture with approximately 74% dichlorodifluoromethane |
| 2585 | 153 | Alkyl sulfonic acids, solid, with not more than 5% free sulfuric acid |  |  |  |
|  |  |  | 2602 | 126 | Refrigerant gas R-500 |
|  |  |  | 2603 | 131 | Cycloheptatriene |
| 2585 | 153 | Alkyl sulphonic acids, solid, with not more than 5% free sulphuric acid | 2604 | 132 | Boron trifluoride diethyl etherate |
| 2585 | 153 | Aryl sulfonic acids, solid, with not more than 5% free sulfuric acid | 2605 | 155 | Methoxymethyl isocyanate |
|  |  |  | 2606 | 155 | Methyl orthosilicate |
| 2585 | 153 | Aryl sulphonic acids, solid, with not more than 5% free sulphuric acid | 2607 | 129P | Acrolein dimer, stabilized |
|  |  |  | 2608 | 129 | Nitropropanes |
|  |  |  | 2609 | 156 | Triallyl borate |
| 2586 | 153 | Alkyl sulfonic acids, liquid, with not more than 5% free sulfuric acid | 2610 | 132 | Triallylamine |
|  |  |  | 2611 | 131 | Propylene chlorohydrin |
| 2586 | 153 | Alkyl sulphonic acids, liquid, with not more than 5% free sulphuric acid | 2612 | 127 | Methyl propyl ether |
|  |  |  | 2614 | 129 | Methallyl alcohol |
| 2586 | 153 | Aryl sulfonic acids, liquid, with not more than 5% free sulfuric acid | 2615 | 127 | Ethyl propyl ether |
|  |  |  | 2616 | 129 | Triisopropyl borate |
| 2586 | 153 | Aryl sulphonic acids, liquid, with not more than 5% free sulphuric acid | 2617 | 129 | Methylcyclohexanols |
|  |  |  | 2618 | 130P | Vinyltoluenes, stabilized |
| 2587 | 153 | Benzoquinone | 2619 | 132 | Benzyldimethylamine |
|  |  |  | 2620 | 130 | Amyl butyrates |

Page 56

| ID No. | Guide Name of Material No. |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2621 | 127 | Acetyl methyl carbinol |  | 2667 | 152 | Butyltoluenes |  |
| 2622 131P Glycidaldehyde |  |  |  | 2668 | 131 | Chloroacetonitrile |  |
| 2623 | 133 | Firelighters, solid, with |  | 2669 2670 2671 2672 2672 2673 2674 | 152 157 153 154 154 151 154 | Chlorocresols, solution Cyanuric chloride Aminopyridines Ammonia solution, with more than 10% but not more than 35% ammonia Ammonium hydroxide, with more than 10% but not more than 35% ammonia 2-Amino-4-chlorophenol Sodium fluorosilicate |  |
|  |  | flammable liquid |  |  |  |  |  |
| 2624 | 138 | Magnesium silicide |  |  |  |  |  |
| 2626 | 140 | Chloric acid, aqueous solution, |  |  |  |  |  |
|  |  | with not more than 10% |  |  |  |  |  |
|  |  | chloric acid |  |  |  |  |  |
| 2627 | 140 | Nitrites, inorganic, n.o.s. |  |  |  |  |  |
| 2628 | 151 | Potassium fluoroacetate |  |  |  |  |  |
| 2629 | 151 | Sodium fluoroacetate |  |  |  |  |  |
| 2630 | 151 | Selenates |  |  |  |  |  |
| 2630 | 151 | Selenites |  |  |  |  |  |
|  |  |  |  | 2676 | 119 | Stibine |  |
| 2642 | 154 | Fluoroacetic acid |  |  |  |  |  |
|  |  |  |  | 2677 | 154 | Rubidium hydroxide, solution Rubidium hydroxide, solid Lithium hydroxide, solution Lithium hydroxide Caesium hydroxide, solution Cesium hydroxide, solution Caesium hydroxide Cesium hydroxide Ammonium sulfide, solution Ammonium sulphide, solution 3-Diethylaminopropylamine N,N-Diethylethylenediamine 2-Diethylaminoethanol Dicyclohexylammonium nitrite 1-Bromo-3-chloropropane Glycerol alpha- monochlorohydrin N,n-Butylimidazole |  |
| 2643 | 153 | Methyl bromoacetate |  |  |  |  |  |
|  |  |  |  | 2678 | 154 |  |  |
| 2644 | 151 | Methyl iodide |  |  |  |  |  |
|  |  |  |  | 2679 | 154 |  |  |
| 2645 | 153 | Phenacyl bromide |  |  |  |  |  |
|  |  |  |  | 2680 | 154 |  |  |
| 2646 | 151 | Hexachlorocyclopentadiene |  |  |  |  |  |
|  |  |  |  | 2681 | 154 |  |  |
| 2647 | 153 | Malononitrile |  |  |  |  |  |
|  |  |  |  | 2681 | 154 |  |  |
| 2648 | 154 | 1,2-Dibromobutan-3-one |  |  |  |  |  |
|  |  |  |  | 2682 | 157 |  |  |
| 2649 | 153 | 1,3-Dichloroacetone |  |  |  |  |  |
|  |  |  |  | 2682 | 157 |  |  |
| 2650 | 153 | 1,1-Dichloro-1-nitroethane |  |  |  |  |  |
|  |  |  |  | 2683 | 132 |  |  |
| 2651 | 153 | 4,4'-Diaminodiphenylmethane |  |  |  |  |  |
|  |  |  |  | 2683 | 132 |  |  |
| 2653 | 156 | Benzyl iodide |  |  |  |  |  |
|  |  |  |  | 2684 | 132 |  |  |
| 2655 | 151 | Potassium fluorosilicate |  |  |  |  |  |
|  |  |  |  | 2685 | 132 |  |  |
| 2656 | 154 | Quinoline |  |  |  |  |  |
|  |  |  |  | 2686 | 132 |  |  |
| 2657 | 153 | Selenium disulfide |  |  |  |  |  |
|  |  |  |  | 2687 | 133 |  |  |
| 2657 | 153 | Selenium disulphide |  |  |  |  |  |
|  |  |  |  | 2688 | 159 |  |  |
| 2659 | 151 | Sodium chloroacetate |  |  |  |  |  |
|  |  |  |  | 2689 | 153 |  |  |
| 2660 | 153 | Mononitrotoluidines |  |  |  |  |  |
| 2660 | 153 | Nitrotoluidines (mono) |  | 2690 | 152 |  |  |
| 2661 | 153 | Hexachloroacetone |  | 2691 | 137 | Phosphorus pentabromide |  |
| 2664 | 160 | Dibromomethane |  | 2692 | 157 | Boron tribromide |  |

Page 57

| ID No. 2693 | Guide No. 154 | Name of Material Bisulfites, aqueous solution, n.o.s. | ID No. 2734 | Guid No. 132 | e Name of Material Polyamines, liquid, corrosive, flammable, n.o.s. |
| --- | --- | --- | --- | --- | --- |
| 2693 | 154 | Bisulphites, aqueous solution, n.o.s. | 2735 | 153 | Amines, liquid, corrosive, n.o.s. |
|  |  |  | 2735 | 153 | Polyamines, liquid, corrosive, n.o.s. |
| 2698 | 156 | Tetrahydrophthalic anhydrides |  |  |  |
| 2699 | 154 | Trifluoroacetic acid | 2738 | 153 | N-Butylaniline |
| 2705 | 153P | 1-Pentol | 2739 | 156 | Butyric anhydride |
| 2707 | 127 | Dimethyldioxanes | 2740 | 155 | n-Propyl chloroformate |
| 2709 | 128 | Butylbenzenes | 2741 | 141 | Barium hypochlorite, with more than 22% available chlorine |
| 2710 | 128 | Dipropyl ketone |  |  |  |
| 2713 | 153 | Acridine | 2742 | 155 | Chloroformates, poisonous, corrosive, flammable, n.o.s. |
| 2714 | 133 | Zinc resinate | 2742 | 155 | Chloroformates, toxic, corrosive, flammable, n.o.s. |
| 2715 | 133 | Aluminum resinate |  |  |  |
| 2716 | 153 | 1,4-Butynediol | 2743 | 155 | n-Butyl chloroformate |
| 2717 | 133 | Camphor, synthetic | 2744 | 155 | Cyclobutyl chloroformate |
| 2719 | 141 | Barium bromate | 2745 | 157 | Chloromethyl chloroformate |
| 2720 | 141 | Chromium nitrate | 2746 | 156 | Phenyl chloroformate |
| 2721 | 140 | Copper chlorate | 2747 | 156 | tert-Butylcyclohexyl chloroformate |
| 2722 | 140 | Lithium nitrate | 2748 | 156 | 2-Ethylhexyl chloroformate |
| 2723 | 140 | Magnesium chlorate | 2749 | 130 | Tetramethylsilane |
| 2724 | 140 | Manganese nitrate | 2750 | 153 | 1,3-Dichloropropanol-2 |
| 2725 | 140 | Nickel nitrate | 2751 | 156 | Diethylthiophosphoryl chloride |
| 2726 | 140 | Nickel nitrite | 2752 | 127 | 1,2-Epoxy-3-ethoxypropane |
| 2727 | 141 | Thallium nitrate | 2753 | 153 | N-Ethylbenzyltoluidines, liquid |
| 2728 | 140 | Zirconium nitrate | 2754 | 153 | N-Ethyltoluidines |
| 2729 | 152 | Hexachlorobenzene | 2757 | 151 | Carbamate pesticide, solid, poisonous |
| 2730 | 152 | Nitroanisoles, liquid |  |  |  |
| 2732 | 152 | Nitrobromobenzenes, liquid | 2757 | 151 | Carbamate pesticide, solid, toxic |
| 2733 | 132 | Amines, flammable, corrosive, n.o.s. |  |  |  |
|  |  |  | 2758 | 131 | Carbamate pesticide, liquid, flammable, poisonous |
| 2733 | 132 | Polyamines, flammable, corrosive, n.o.s. |  |  |  |
|  |  |  | 2758 | 131 | Carbamate pesticide, liquid, flammable, toxic |
| 2734 | 132 | Amines, liquid, corrosive, flammable, n.o.s. |  |  |  |

Page 58

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2759 | 151 | Arsenical pesticide, solid, poisonous | 2777 | 151 | Mercury based pesticide, solid, poisonous |  |
| 2759 | 151 | Arsenical pesticide, solid, toxic | 2777 | 151 | Mercury based pesticide, solid, toxic |  |
| 2760 | 131 | Arsenical pesticide, liquid, flammable, poisonous |  |  |  |  |
|  |  |  | 2778 | 131 | Mercury based pesticide, liquid, flammable, poisonous |  |
| 2760 | 131 | Arsenical pesticide, liquid, flammable, toxic |  |  |  |  |
|  |  |  | 2778 | 131 | Mercury based pesticide, liquid, flammable, toxic |  |
| 2761 | 151 | Organochlorine pesticide, solid, poisonous |  |  |  |  |
|  |  |  | 2779 | 153 | Substituted nitrophenol pesticide, solid, poisonous |  |
| 2761 | 151 | Organochlorine pesticide, solid, toxic |  |  |  |  |
|  |  |  | 2779 | 153 | Substituted nitrophenol pesticide, solid, toxic |  |
| 2762 | 131 | Organochlorine pesticide, liquid, flammable, poisonous |  |  |  |  |
|  |  |  | 2780 | 131 | Substituted nitrophenol pesticide, liquid, flammable, poisonous |  |
| 2762 | 131 | Organochlorine pesticide, liquid, flammable, toxic |  |  |  |  |
| 2763 | 151 | Triazine pesticide, solid, poisonous | 2780 | 131 | Substituted nitrophenol pesticide, liquid, flammable, toxic |  |
| 2763 | 151 | Triazine pesticide, solid, toxic | 2781 | 151 | Bipyridilium pesticide, solid, poisonous |  |
| 2764 | 131 | Triazine pesticide, liquid, flammable, poisonous |  |  |  |  |
|  |  |  | 2781 | 151 | Bipyridilium pesticide, solid, toxic |  |
| 2764 | 131 | Triazine pesticide, liquid, flammable, toxic |  |  |  |  |
|  |  |  | 2782 | 131 | Bipyridilium pesticide, liquid, flammable, poisonous |  |
| 2771 | 151 | Thiocarbamate pesticide, solid, poisonous |  |  |  |  |
|  |  |  | 2782 | 131 | Bipyridilium pesticide, liquid, flammable, toxic |  |
| 2771 | 151 | Thiocarbamate pesticide, solid, toxic |  |  |  |  |
|  |  |  | 2783 | 152 | Organophosphorus pesticide, solid, poisonous |  |
| 2772 | 131 | Thiocarbamate pesticide, liquid, flammable, poisonous |  |  |  |  |
|  |  |  | 2783 | 152 | Organophosphorus pesticide, solid, toxic |  |
| 2772 | 131 | Thiocarbamate pesticide, liquid, flammable, toxic |  |  |  |  |
|  |  |  | 2784 | 131 | Organophosphorus pesticide, liquid, flammable, poisonous |  |
| 2775 | 151 | Copper based pesticide, solid, poisonous |  |  |  |  |
|  |  |  | 2784 | 131 | Organophosphorus pesticide, liquid, flammable, toxic |  |
| 2775 | 151 | Copper based pesticide, solid, toxic |  |  |  |  |
|  |  |  | 2785 | 152 | 4-Thiapentanal |  |
| 2776 | 131 | Copper based pesticide, liquid, flammable, poisonous | 2786 | 153 | Organotin pesticide, solid, poisonous |  |
| 2776 | 131 | Copper based pesticide, liquid, flammable, toxic | 2786 | 153 | Organotin pesticide, solid, toxic |  |
|  |  |  | 2787 | 131 | Organotin pesticide, liquid, flammable, poisonous |  |

Page 59

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

| 2787 | 131 | Organotin pesticide, liquid, flammable, toxic | 2810 | 153 | Compounds, tree or weed killing, liquid (toxic) |
| --- | --- | --- | --- | --- | --- |
| 2788 | 153 | Organotin compound, liquid, n.o.s. | 2810 | 153 | Poisonous liquid, organic, n.o.s. |
|  |  |  | 2810 | 153 | Toxic liquid, organic, n.o.s. |
| 2789 | 132 | Acetic acid, glacial | 2811 | 154 | Poisonous solid, organic, n.o.s. |
| 2789 | 132 | Acetic acid, solution, more than 80% acid | 2811 | 154 | Toxic solid, organic, n.o.s. |
| 2790 | 153 | Acetic acid, solution, more than 10% but not more than 80% acid | 2812 | 154 | Sodium aluminate, solid |
|  |  |  | 2813 | 138 | Water-reactive solid, n.o.s. |
|  |  |  | 2814 | 158 | Infectious substance, affecting humans |
| 2793 | 170 | Ferrous metal borings, shavings, turnings or cuttings |  |  |  |
|  |  |  | 2815 | 153 | N-Aminoethylpiperazine |
| 2794 | 154 | Batteries, wet, filled with acid | 2817 | 154 | Ammonium bifluoride, solution |
| 2795 | 154 | Batteries, wet, filled with alkali | 2817 | 154 | Ammonium hydrogendifluoride, solution |
| 2796 | 157 | Battery fluid, acid |  |  |  |
| 2796 | 157 | Sulfuric acid, with not more than 51% acid | 2818 | 154 | Ammonium polysulfide, solution |
|  |  |  | 2818 | 154 | Ammonium polysulphide, solution |
| 2796 | 157 | Sulphuric acid, with not more than 51% acid |  |  |  |
|  |  |  | 2819 | 153 | Amyl acid phosphate |
| 2797 | 154 | Battery fluid, alkali | 2820 | 153 | Butyric acid |
| 2798 | 137 | Benzene phosphorus dichloride | 2821 | 153 | Phenol solution |
| 2798 | 137 | Phenylphosphorus dichloride | 2822 | 153 | 2-Chloropyridine |
| 2799 | 137 | Benzene phosphorus thiodichloride | 2823 | 153 | Crotonic acid, solid |
| 2799 | 137 | Phenylphosphorus thiodichloride | 2826 | 155 | Ethyl chlorothioformate |
|  |  |  | 2829 | 153 | Caproic acid |
| 2800 | 154 | Batteries, wet, non-spillable | 2829 | 153 | Hexanoic acid |
| 2801 | 154 | Dye, liquid, corrosive, n.o.s. | 2830 | 139 | Lithium ferrosilicon |
| 2801 | 154 | Dye intermediate, liquid, corrosive, n.o.s. | 2831 | 160 | 1,1,1-Trichloroethane |
| 2802 | 154 | Copper chloride | 2834 | 154 | Phosphorous acid |
| 2803 | 172 | Gallium | 2835 | 138 | Sodium aluminum hydride |
| 2805 | 138 | Lithium hydride, fused solid | 2837 | 154 | Bisulfates, aqueous solution |
| 2806 | 139 | Lithium nitride | 2837 | 154 | Bisulphates, aqueous solution |
| 2807 | 171 | Magnetized material | 2837 | 154 | Sodium bisulfate, solution |
| 2809 | 172 | Mercury | 2837 | 154 | Sodium bisulphate, solution |
|  |  |  | 2838 | 129P | Vinyl butyrate, stabilized |

Page 60

| ID No. 2839 | Guid No. 153 | e Name of Material Aldol |  |  | ID No. 2861 | Guid No. 151 | e Name of Material Ammonium polyvanadate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2840 | 129 | Butyraldoxime |  |  | 2862 | 151 | Vanadium pentoxide |
| 2841 | 131 | Di-n-amylamine |  |  | 2863 | 154 | Sodium ammonium vanadate |
| 2842 | 129 | Nitroethane |  |  | 2864 | 151 | Potassium metavanadate |
| 2844 | 138 | Calcium manganese silicon |  |  | 2865 | 154 | Hydroxylamine sulfate |
| 2845 | 135 | Ethyl phosphonous dichloride, anhydrous |  |  | 2865 | 154 | Hydroxylamine sulphate |
|  |  |  |  |  | 2869 | 157 | Titanium trichloride mixture |
| 2845 | 135 | Methyl phosphonous dichloride |  |  | 2870 | 135 | Aluminum borohydride |
| 2845 | 135 | Pyrophoric liquid, organic, n.o.s. |  |  | 2870 | 135 | Aluminum borohydride in devices |
| 2846 | 135 | Pyrophoric solid, organic, n.o.s. |  |  | 2871 | 170 | Antimony powder |
| 2849 | 153 | 3-Chloropropanol-1 |  |  | 2872 | 159 | Dibromochloropropanes |
| 2850 | 128 | Propylene tetramer |  |  | 2873 | 153 | Dibutylaminoethanol |
| 2851 | 157 | Boron trifluoride, dihydrate |  |  | 2874 | 153 | Furfuryl alcohol |
| 2852 | 113 | Dipicryl sulfide, wetted with not less than 10% water |  |  | 2875 | 151 | Hexachlorophene |
| 2852 | 113 | Dipicryl sulphide, wetted with not less than 10% water |  |  | 2876 | 153 | Resorcinol |
|  |  |  |  |  | 2878 | 170 | Titanium sponge granules |
| 2853 | 151 | Magnesium fluorosilicate |  |  | 2878 | 170 | Titanium sponge powders |
| 2854 | 151 | Ammonium fluorosilicate |  |  | 2879 | 157 | Selenium oxychloride |
| 2854 | 151 | Ammonium silicofluoride |  |  | 2880 | 140 | Calcium hypochlorite, hydrated, with not less than 5.5% but not more than 16% water |
| 2855 | 151 | Zinc fluorosilicate |  |  |  |  |  |
| 2855 | 151 | Zinc silicofluoride |  |  | 2880 | 140 | Calcium hypochlorite, hydrated mixture, with not less than 5.5% but not more than 16% water |
| 2856 | 151 | Fluorosilicates, n.o.s. |  |  |  |  |  |
| 2857 | 126 | Refrigerating machines, containing ammonia solutions (UN2672) |  |  |  |  |  |
|  |  |  |  |  | 2881 | 135 | Metal catalyst, dry |
| 2857 | 126 | Refrigerating machines, containing non-flammable, non-poisonous gases |  |  | 2881 | 135 | Nickel catalyst, dry |
|  |  |  |  |  | 2900 | 158 | Infectious substance, affecting animals only |
| 2857 | 126 | Refrigerating machines, containing non-flammable, non-toxic gases |  |  | 2901 | 124 | Bromine chloride |
|  |  |  |  |  | 2902 | 151 | Pesticide, liquid, poisonous, n.o.s. |
| 2858 | 170 | Zirconium, dry, coiled wire, finished metal sheets, strip |  |  |  |  |  |
|  |  |  |  |  | 2902 | 151 | Pesticide, liquid, toxic, n.o.s. |
| 2859 | 154 | Ammonium metavanadate |  |  | 2903 | 131 | Pesticide, liquid, poisonous, flammable, n.o.s. |

Page 61

| ID No. 2903 | Guid No. 131 | e Name of Material Pesticide, liquid, toxic, flammable, n.o.s. | ID No. 2917 | Guid No. 163 | e Name of Material Radioactive material, Type B(M) package, non fissile or fissile-excepted |
| --- | --- | --- | --- | --- | --- |
| 2904 | 154 | Chlorophenolates, liquid |  |  |  |
| 2904 | 154 | Phenolates, liquid | 2919 | 163 | Radioactive material, transported under special arrangement, non fissile or fissile-excepted |
| 2905 | 154 | Chlorophenolates, solid |  |  |  |
| 2905 | 154 | Phenolates, solid | 2920 | 132 | Corrosive liquid, flammable, n.o.s. |
| 2907 | 133 | Isosorbide dinitrate mixture |  |  |  |
| 2908 | 161 | Radioactive material, excepted package, empty packaging | 2921 | 134 | Corrosive solid, flammable, n.o.s. |
| 2909 | 161 | Radioactive material, excepted package, articles manufactured from depleted uranium | 2922 | 154 | Corrosive liquid, poisonous, n.o.s. |
|  |  |  | 2922 | 154 | Corrosive liquid, toxic, n.o.s. |
| 2909 | 161 | Radioactive material, excepted package, articles manufactured from natural thorium | 2923 | 154 | Corrosive solid, poisonous, n.o.s. |
|  |  |  | 2923 | 154 | Corrosive solid, toxic, n.o.s. |
| 2909 | 161 | Radioactive material, excepted package, articles manufactured from natural uranium | 2924 | 132 | Flammable liquid, corrosive, n.o.s. |
|  |  |  | 2925 | 134 | Flammable solid, corrosive, organic, n.o.s. |
| 2910 | 161 | Radioactive material, excepted package, limited quantity of material | 2926 | 134 | Flammable solid, poisonous, organic, n.o.s. |
|  |  |  | 2926 | 134 | Flammable solid, toxic, organic, n.o.s. |
| 2911 | 161 | Radioactive material, excepted package, articles |  |  |  |
|  |  |  | 2927 | 154 | Ethyl phosphonothioic dichloride, anhydrous |
| 2911 | 161 | Radioactive material, excepted package, instruments |  |  |  |
|  |  |  | 2927 | 154 | Ethyl phosphorodichloridate |
| 2912 | 162 | Radioactive material, low specific activity (LSA-I), non fissile or fissile-excepted | 2927 | 154 | Poisonous liquid, corrosive, organic, n.o.s. |
| 2913 | 162 | Radioactive material, surface contaminated objects (SCO-I, SCO-II or SCO-III), non fissile or fissile-excepted | 2927 | 154 | Toxic liquid, corrosive, organic, n.o.s. |
|  |  |  | 2928 | 154 | Poisonous solid, corrosive, organic, n.o.s. |
| 2915 | 163 | Radioactive material, Type A package, non-special form, non fissile or fissile-excepted | 2928 | 154 | Toxic solid, corrosive, organic, n.o.s. |
| 2916 | 163 | Radioactive material, Type B(U) package, non fissile or fissile-excepted | 2929 | 131 | Poisonous liquid, flammable, organic, n.o.s. |
|  |  |  | 2929 | 131 | Toxic liquid, flammable, organic, n.o.s. |

Page 62

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2930 | 134 | Poisonous solid, flammable, organic, n.o.s. |  | 2956 | 149 | 5-tert-Butyl-2,4,6-trinitro- m-xylene |  |
| 2930 | 134 | Toxic solid, flammable, organic, n.o.s. |  | 2956 | 149 | Musk xylene |  |
|  |  |  |  | 2965 | 139 | Boron trifluoride dimethyl etherate |  |
| 2931 | 151 | Vanadyl sulfate |  |  |  |  |  |
| 2931 | 151 | Vanadyl sulphate |  | 2966 | 153 | Thioglycol |  |
| 2933 | 129 | Methyl 2-chloropropionate |  | 2967 | 154 | Sulfamic acid |  |
| 2934 | 129 | Isopropyl 2-chloropropionate |  | 2967 | 154 | Sulphamic acid |  |
| 2935 | 129 | Ethyl 2-chloropropionate |  | 2968 | 135 | Maneb, stabilized |  |
| 2936 | 153 | Thiolactic acid |  | 2968 | 135 | Maneb preparation, stabilized |  |
| 2937 | 153 | alpha-Methylbenzyl alcohol, liquid |  | 2969 | 171 | Castor beans, meal, pomace or flake |  |
| 2937 | 153 | Methylbenzyl (alpha) alcohol, liquid |  | 2977 | 166 | Radioactive material, uranium hexafluoride, fissile |  |
| 2940 | 135 | Cyclooctadiene phosphines |  | 2977 | 166 | Uranium hexafluoride, radioactive material, fissile |  |
| 2940 | 135 | 9-Phosphabicyclononanes |  |  |  |  |  |
|  |  |  |  | 2978 | 166 | Radioactive material, uranium hexafluoride, non fissile or fissile-excepted |  |
| 2941 | 153 | Fluoroanilines |  |  |  |  |  |
| 2942 | 153 | 2-Trifluoromethylaniline |  |  |  |  |  |
| 2943 | 129 | Tetrahydrofurfurylamine |  | 2978 | 166 | Uranium hexafluoride, radioactive material, non fissile or fissile-excepted |  |
| 2945 | 132 | N-Methylbutylamine |  |  |  |  |  |
| 2946 | 153 | 2-Amino-5-diethylaminopentane |  | 2983 | 131P | Ethylene oxide and propylene oxide mixture, with not more than 30% ethylene oxide |  |
| 2947 | 127 | Isopropyl chloroacetate |  |  |  |  |  |
| 2948 | 153 | 3-Trifluoromethylaniline |  | 2984 | 140 | Hydrogen peroxide, aqueous solution, with not less than 8% but less than 20% hydrogen peroxide |  |
| 2949 | 154 | Sodium hydrosulfide, hydrated, with not less than 25% water of crystallization |  |  |  |  |  |
| 2949 | 154 | Sodium hydrosulfide, with not less than 25% water of crystallization |  | 2985 | 155 | Chlorosilanes, flammable, corrosive, n.o.s. |  |
|  |  |  |  | 2986 | 155 | Chlorosilanes, corrosive, flammable, n.o.s. |  |
| 2949 | 154 | Sodium hydrosulphide, hydrated, with not less than 25% water of crystallization |  |  |  |  |  |
|  |  |  |  | 2987 | 156 | Chlorosilanes, corrosive, n.o.s. |  |
|  |  |  |  | 2988 | 139 | Chlorosilanes, water-reactive, flammable, corrosive, n.o.s. |  |
| 2949 | 154 | Sodium hydrosulphide, with not less than 25% water of crystallization |  |  |  |  |  |
|  |  |  |  | 2989 | 133 | Lead phosphite, dibasic |  |
| 2950 | 138 | Magnesium granules, coated |  | 2990 | 171 | Life-saving appliances, self- inflating |  |

Page 63

| ID No. 2991 | Guid No. 131 | e Name of Material Carbamate pesticide, liquid, poisonous, flammable | ID No. 3006 | Guid No. 151 | e Name of Material Thiocarbamate pesticide, liquid, poisonous |
| --- | --- | --- | --- | --- | --- |
| 2991 | 131 | Carbamate pesticide, liquid, toxic, flammable | 3006 | 151 | Thiocarbamate pesticide, liquid, toxic |
| 2992 | 151 | Carbamate pesticide, liquid, poisonous | 3009 | 131 | Copper based pesticide, liquid, poisonous, flammable |
| 2992 | 151 | Carbamate pesticide, liquid, toxic | 3009 | 131 | Copper based pesticide, liquid, toxic, flammable |
| 2993 | 131 | Arsenical pesticide, liquid, poisonous, flammable | 3010 | 151 | Copper based pesticide, liquid, poisonous |
| 2993 | 131 | Arsenical pesticide, liquid, toxic, flammable | 3010 | 151 | Copper based pesticide, liquid, toxic |
| 2994 | 151 | Arsenical pesticide, liquid, poisonous | 3011 | 131 | Mercury based pesticide, liquid, poisonous, flammable |
| 2994 | 151 | Arsenical pesticide, liquid, toxic | 3011 | 131 | Mercury based pesticide, liquid, toxic, flammable |
| 2995 | 131 | Organochlorine pesticide, liquid, poisonous, flammable |  |  |  |
|  |  |  | 3012 | 151 | Mercury based pesticide, liquid, poisonous |
| 2995 | 131 | Organochlorine pesticide, liquid, toxic, flammable |  |  |  |
|  |  |  | 3012 | 151 | Mercury based pesticide, liquid, toxic |
| 2996 | 151 | Organochlorine pesticide, liquid, poisonous |  |  |  |
|  |  |  | 3013 | 131 | Substituted nitrophenol pesticide, liquid, poisonous, flammable |
| 2996 | 151 | Organochlorine pesticide, liquid, toxic |  |  |  |
| 2997 | 131 | Triazine pesticide, liquid, poisonous, flammable | 3013 | 131 | Substituted nitrophenol pesticide, liquid, toxic, flammable |
| 2997 | 131 | Triazine pesticide, liquid, toxic, flammable | 3014 | 153 | Substituted nitrophenol pesticide, liquid, poisonous |
| 2998 | 151 | Triazine pesticide, liquid, poisonous | 3014 | 153 | Substituted nitrophenol pesticide, liquid, toxic |
| 2998 | 151 | Triazine pesticide, liquid, toxic | 3015 | 131 | Bipyridilium pesticide, liquid, poisonous, flammable |
| 3002 | 151 | Phenyl urea pesticides, liquid, poisonous |  |  |  |
|  |  |  | 3015 | 131 | Bipyridilium pesticide, liquid, toxic, flammable |
| 3002 | 151 | Phenyl urea pesticides, liquid, toxic |  |  |  |
|  |  |  | 3016 | 151 | Bipyridilium pesticide, liquid, poisonous |
| 3005 | 131 | Thiocarbamate pesticide, liquid, poisonous, flammable |  |  |  |
|  |  |  | 3016 | 151 | Bipyridilium pesticide, liquid, toxic |
| 3005 | 131 | Thiocarbamate pesticide, liquid, toxic, flammable |  |  |  |
|  |  |  | 3017 | 131 | Organophosphorus pesticide, liquid, poisonous, flammable |

Page 64

| ID Guide Name of Material No. No. |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3017 131 3018 152 | Organophosphorus pesticide, liquid, toxic, flammable Organophosphorus pesticide, liquid, poisonous |  | 3054 3055 3056 | 129 154 129 | Cyclohexyl mercaptan 2-(2-Aminoethoxy)ethanol n-Heptaldehyde |  |
|  |  |  | 3057 | 125 | Trifluoroacetyl chloride |  |
| 3018 152 | Organophosphorus pesticide, liquid, toxic |  |  |  |  |  |
|  |  | 3064 127 Nitroglycerin, solution in alcohol, with more than 1% but not more than 5% nitroglycerin 3065 127 Alcoholic beverages 3066 153 Paint (corrosive) 3066 153 Paint related material (corrosive) 3070 126 Ethylene oxide and dichlorodifluoromethane mixture, with not more than 12.5% ethylene oxide 3071 131 Mercaptan mixture, liquid, poisonous, flammable, n.o.s. 3071 131 Mercaptan mixture, liquid, toxic, flammable, n.o.s. 3071 131 Mercaptans, liquid, poisonous, flammable, n.o.s. 3071 131 Mercaptans, liquid, toxic, flammable, n.o.s. 3072 171 Life-saving appliances, not self- inflating 3073 131P Vinylpyridines, stabilized 3077 171 Environmentally hazardous substance, solid, n.o.s. 3077 171 Hazardous waste, solid, n.o.s. 3077 171 Other regulated substances, solid, n.o.s. 3078 138 Cerium, turnings or gritty powder |  |  |  |  |
| 3019 131 | Organotin pesticide, liquid, poisonous, flammable |  |  |  |  |  |
| 3019 131 | Organotin pesticide, liquid, toxic, flammable |  |  |  |  |  |
| 3020 153 | Organotin pesticide, liquid, poisonous |  |  |  |  |  |
| 3020 153 | Organotin pesticide, liquid, toxic |  |  |  |  |  |
| 3021 131 | Pesticide, liquid, flammable, poisonous, n.o.s. |  |  |  |  |  |
| 3021 131 | Pesticide, liquid, flammable, toxic, n.o.s. |  |  |  |  |  |
| 3022 127P 1,2-Butylene oxide, stabilized |  |  |  |  |  |  |
| 3023 131 | 2-Methyl-2-heptanethiol |  |  |  |  |  |
| 3024 131 | Coumarin derivative pesticide, liquid, flammable, poisonous |  |  |  |  |  |
| 3024 131 | Coumarin derivative pesticide, liquid, flammable, toxic |  |  |  |  |  |
| 3025 131 | Coumarin derivative pesticide, liquid, poisonous, flammable |  |  |  |  |  |
| 3025 131 | Coumarin derivative pesticide, liquid, toxic, flammable |  |  |  |  |  |
| 3026 151 | Coumarin derivative pesticide, liquid, poisonous |  |  |  |  |  |
| 3026 151 | Coumarin derivative pesticide, liquid, toxic |  |  |  |  |  |
| 3027 151 | Coumarin derivative pesticide, solid, poisonous |  |  |  |  |  |
| 3027 151 | Coumarin derivative pesticide, solid, toxic |  |  |  |  |  |
|  |  |  | 3079 131P Methacrylonitrile, stabilized |  |  |  |
| 3028 154 | Batteries, dry, containing potassium hydroxide solid |  |  |  |  |  |
|  |  |  | 3080 | 155 | Isocyanate solution, poisonous, flammable, n.o.s. Isocyanate solution, toxic, flammable, n.o.s. |  |
| 3048 157 3054 129 | Aluminum phosphide pesticide Cyclohexanethiol |  | 3080 | 155 |  |  |

Page 65

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

| 3080 | 155 | Isocyanates, poisonous, flammable, n.o.s. | 3096 | 138 | Corrosive solid, water-reactive, n.o.s. |
| --- | --- | --- | --- | --- | --- |
| 3080 | 155 | Isocyanates, toxic, flammable, n.o.s. | 3097 | 140 | Flammable solid, oxidizing, n.o.s. |
| 3082 | 171 | Environmentally hazardous substance, liquid, n.o.s. | 3098 | 140 | Oxidizing liquid, corrosive, n.o.s. |
| 3082 | 171 | Hazardous waste, liquid, n.o.s. | 3099 | 142 | Oxidizing liquid, poisonous, n.o.s. |
| 3082 | 171 | Other regulated substances, liquid, n.o.s. |  |  |  |
|  |  |  | 3099 | 142 | Oxidizing liquid, toxic, n.o.s. |
| 3083 | 124 | Perchloryl fluoride | 3100 | 135 | Oxidizing solid, self-heating, n.o.s. |
| 3084 | 157 | Corrosive solid, oxidizing, n.o.s. |  |  |  |
| 3085 | 140 | Oxidizing solid, corrosive, n.o.s. | 3101 | 146 | Organic peroxide type B, liquid |
| 3086 | 141 | Poisonous solid, oxidizing, n.o.s. | 3102 | 146 | Organic peroxide type B, solid |
|  |  |  | 3103 | 146 | Organic peroxide type C, liquid |
| 3086 | 141 | Toxic solid, oxidizing, n.o.s. | 3104 | 146 | Organic peroxide type C, solid |
| 3087 | 141 | Oxidizing solid, poisonous, n.o.s. | 3105 | 145 | Organic peroxide type D, liquid |
|  |  |  | 3106 | 145 | Organic peroxide type D, solid |
| 3087 | 141 | Oxidizing solid, toxic, n.o.s. | 3107 | 145 | Organic peroxide type E, liquid |
| 3088 | 135 | Self-heating solid, organic, n.o.s. | 3108 | 145 | Organic peroxide type E, solid |
| 3089 | 170 | Metal powder, flammable, n.o.s. | 3109 | 145 | Organic peroxide type F, liquid |
| 3090 | 138 | Lithium metal batteries (including lithium alloy batteries) | 3110 | 145 | Organic peroxide type F, solid |
|  |  |  | 3111 | 148 | Organic peroxide type B, liquid, temperature controlled |
| 3091 | 138 | Lithium metal batteries contained in equipment (including lithium alloy batteries) | 3112 | 148 | Organic peroxide type B, solid, temperature controlled |
|  |  |  | 3113 | 148 | Organic peroxide type C, liquid, temperature controlled |
| 3091 | 138 | Lithium metal batteries packed with equipment (including lithium alloy batteries) |  |  |  |
|  |  |  | 3114 | 148 | Organic peroxide type C, solid, temperature controlled |
| 3092 | 129 | 1-Methoxy-2-propanol | 3115 | 148 | Organic peroxide type D, liquid, temperature controlled |
| 3093 | 157 | Corrosive liquid, oxidizing, n.o.s. |  |  |  |
|  |  |  | 3116 | 148 | Organic peroxide type D, solid, temperature controlled |
| 3094 | 138 | Corrosive liquid, water-reactive, n.o.s. |  |  |  |
|  |  |  | 3117 | 148 | Organic peroxide type E, liquid, temperature controlled |
| 3095 | 136 | Corrosive solid, self-heating, n.o.s. |  |  |  |
|  |  |  | 3118 | 148 | Organic peroxide type E, solid, temperature controlled |

Page 66

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3119 | 148 | Organic peroxide type F, liquid, temperature controlled | 3133 | 138 | Water-reactive solid, oxidizing, n.o.s. |  |
| 3120 | 148 | Organic peroxide type F, solid, temperature controlled | 3134 | 139 | Water-reactive solid, poisonous, n.o.s. |  |
| 3121 | 144 | Oxidizing solid, water-reactive, n.o.s. | 3134 | 139 | Water-reactive solid, toxic, n.o.s. |  |
| 3122 | 142 | Poisonous liquid, oxidizing, n.o.s. | 3135 | 138 | Water-reactive solid, self- heating, n.o.s. |  |
| 3122 | 142 | Toxic liquid, oxidizing, n.o.s. | 3136 | 120 | Trifluoromethane, refrigerated liquid |  |
| 3123 | 139 | Poisonous liquid, water- reactive, n.o.s. |  |  |  |  |
|  |  |  | 3137 | 140 | Oxidizing solid, flammable, n.o.s. |  |
| 3123 | 139 | Toxic liquid, water-reactive, n.o.s. |  |  |  |  |
|  |  |  | 3138 | 115 | Ethylene, acetylene and propylene mixture, refrigerated liquid containing at least 71.5% ethylene with not more than 22.5% acetylene and not more than 6% propylene |  |
| 3124 | 136 | Poisonous solid, self-heating, n.o.s. |  |  |  |  |
| 3124 | 136 | Toxic solid, self-heating, n.o.s. |  |  |  |  |
| 3125 | 139 | Poisonous solid, water-reactive, n.o.s. |  |  |  |  |
|  |  |  | 3139 | 140 | Oxidizing liquid, n.o.s. |  |
| 3125 | 139 | Toxic solid, water-reactive, n.o.s. | 3140 | 151 | Alkaloids, liquid, n.o.s. (poisonous) |  |
| 3126 | 136 | Self-heating solid, corrosive, organic, n.o.s. | 3140 | 151 | Alkaloid salts, liquid, n.o.s. (poisonous) |  |
| 3127 | 135 | Self-heating solid, oxidizing, n.o.s. | 3141 | 157 | Antimony compound, inorganic, liquid, n.o.s. |  |
| 3128 | 136 | Self-heating solid, poisonous, organic, n.o.s. | 3142 | 151 | Disinfectant, liquid, poisonous, n.o.s. |  |
| 3128 | 136 | Self-heating solid, toxic, organic, n.o.s. | 3142 | 151 | Disinfectant, liquid, toxic, n.o.s. |  |
| 3129 | 138 | Water-reactive liquid, corrosive, n.o.s. | 3143 | 151 | Dye, solid, poisonous, n.o.s. |  |
|  |  |  | 3143 | 151 | Dye, solid, toxic, n.o.s. |  |
| 3130 | 139 | Water-reactive liquid, poisonous, n.o.s. | 3143 | 151 | Dye intermediate, solid, poisonous, n.o.s. |  |
| 3130 | 139 | Water-reactive liquid, toxic, n.o.s. | 3143 | 151 | Dye intermediate, solid, toxic, n.o.s. |  |
| 3131 | 138 | Water-reactive solid, corrosive, n.o.s. | 3144 | 151 | Nicotine compound, liquid, n.o.s. |  |
| 3132 | 138 | Water-reactive solid, flammable, n.o.s. | 3144 | 151 | Nicotine preparation, liquid, n.o.s. |  |

Page 67

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

3145 153 Alkylphenols, liquid, n.o.s.

(including C2-C12 homologues)

3146 153 Organotin compound, solid,

n.o.s.

3147 154 Dye, solid, corrosive, n.o.s.

3147 154 Dye intermediate, solid,

corrosive, n.o.s.

3148 138 Water-reactive liquid, n.o.s.

3149 140 Hydrogen peroxide and

peroxyacetic acid mixture, with acid(s), water and not more than 5% peroxyacetic acid, stabilized

3150 115  Devices, small, hydrocarbon gas

powered, with release device

3150 115 Hydrocarbon gas refills for

small devices, with release device

3151 171  Halogenated

monomethyldiphenylmethanes, liquid

3151 171 Polyhalogenated biphenyls,

liquid

3151 171 Polyhalogenated terphenyls,

liquid

3152 171  Halogenated

monomethyldiphenylmethanes, solid

3152 171 Polyhalogenated biphenyls,

solid

3152 171 Polyhalogenated terphenyls,

solid

3153 115 Perfluoro(methyl vinyl ether)

3154 115 Perfluoro(ethyl vinyl ether)

3155 154  Pentachlorophenol

3156 122 Compressed gas, oxidizing,

n.o.s.

3157 122 Liquefied gas, oxidizing, n.o.s.

3158 120 Gas, refrigerated liquid, n.o.s.

Page 68

3159 126 Refrigerant gas R-134a

3159 126 1,1,1,2-Tetrafluoroethane

| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. |
| --- | --- | --- |
| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) |
| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) |
| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) |
| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) |

3161 115 Liquefied gas, flammable, n.o.s.

| 3162 | 123 | Liquefied gas, poisonous, n.o.s. |
| --- | --- | --- |
| 3162 | 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone A) |
| 3162 | 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone B) |
| 3162 | 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone C) |
| 3162 | 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone D) |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. |

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone A) |  | 3170 | 138 | Aluminum smelting by-products |  |
|  |  |  |  | 3171 | 154 | Battery-powered equipment (wet battery) |  |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3171 | 147 | Battery-powered equipment (with lithium ion batteries) |  |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |  |
|  |  |  |  | 3171 | 138 | Battery-powered equipment (with lithium metal batteries) |  |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |
|  |  |  |  | 3171 | 138 | Battery-powered equipment (with sodium batteries) |  |
| 3163 | 126 | Liquefied gas, n.o.s. |  |  |  |  |  |
| 3164 | 126 | Articles, pressurized, hydraulic (containing non-flammable gas) |  | 3171 | 154 | Battery-powered vehicle (wet battery) |  |
|  |  |  |  | 3171 | 147 | Battery-powered vehicle (with lithium ion batteries) |  |
| 3164 | 126 | Articles, pressurized, pneumatic (containing non-flammable gas) |  |  |  |  |  |
|  |  |  |  | 3171 | 138 | Battery-powered vehicle (with sodium batteries) |  |
| 3165 | 131 | Aircraft hydraulic power unit fuel tank |  | 3171 | 154 | Wheelchair, electric, with batteries |  |
| 3166 | 115 | Vehicle, flammable gas powered |  | 3172 | 152 | Toxins, extracted from living sources, liquid, n.o.s. |  |
| 3166 | 128 | Vehicle, flammable liquid powered |  |  |  |  |  |
|  |  |  |  | 3174 | 135 | Titanium disulfide |  |
| 3166 | 115 | Vehicle, fuel cell, flammable gas powered |  | 3174 | 135 | Titanium disulphide |  |
| 3166 | 128 | Vehicle, fuel cell, flammable liquid powered |  | 3175 | 133 | Solids containing flammable liquid, n.o.s. |  |
| 3167 | 115 | Gas sample, non-pressurized, flammable, n.o.s., not refrigerated liquid |  | 3176 | 133 | Flammable solid, organic, molten, n.o.s. |  |
|  |  |  |  | 3178 | 133 | Flammable solid, inorganic, n.o.s. |  |
| 3168 | 119 | Gas sample, non-pressurized, poisonous, flammable, n.o.s., not refrigerated liquid |  |  |  |  |  |
|  |  |  |  | 3178 | 133 | Smokeless powder for small arms |  |
| 3168 | 119 | Gas sample, non-pressurized, toxic, flammable, n.o.s., not refrigerated liquid |  | 3179 | 134 | Flammable solid, poisonous, inorganic, n.o.s. |  |
| 3169 | 123 | Gas sample, non-pressurized, poisonous, n.o.s., not refrigerated liquid |  | 3179 | 134 | Flammable solid, toxic, inorganic, n.o.s. |  |
|  |  |  |  | 3180 | 134 | Flammable solid, corrosive, inorganic, n.o.s. |  |
| 3169 | 123 | Gas sample, non-pressurized, toxic, n.o.s., not refrigerated liquid |  |  |  |  |  |
|  |  |  |  | 3181 | 133 | Metal salts of organic compounds, flammable, n.o.s. |  |
| 3170 | 138 | Aluminum dross |  |  |  |  |  |
| 3170 | 138 | Aluminum remelting by-products |  |  |  |  |  |

Page 69

| ID No. 3182 | Guid No. 170 | e Name of Material Metal hydrides, flammable, n.o.s. | ID No. 3210 | Guid No. 140 | e Name of Material Chlorates, inorganic, aqueous solution, n.o.s. |
| --- | --- | --- | --- | --- | --- |
| 3183 | 135 | Self-heating liquid, organic, n.o.s. | 3211 | 140 | Perchlorates, inorganic, aqueous solution, n.o.s. |
| 3184 | 136 | Self-heating liquid, poisonous, organic, n.o.s. | 3212 | 140 | Hypochlorites, inorganic, n.o.s. |
|  |  |  | 3213 | 140 | Bromates, inorganic, aqueous solution, n.o.s. |
| 3184 | 136 | Self-heating liquid, toxic, organic, n.o.s. |  |  |  |
|  |  |  | 3214 | 140 | Permanganates, inorganic, aqueous solution, n.o.s. |
| 3185 | 136 | Self-heating liquid, corrosive, organic, n.o.s. |  |  |  |
|  |  |  | 3215 | 140 | Persulfates, inorganic, n.o.s. |
| 3186 | 135 | Self-heating liquid, inorganic, n.o.s. | 3215 | 140 | Persulphates, inorganic, n.o.s. |
| 3187 | 136 | Self-heating liquid, poisonous, inorganic, n.o.s. | 3216 | 140 | Persulfates, inorganic, aqueous solution, n.o.s. |
| 3187 | 136 | Self-heating liquid, toxic, inorganic, n.o.s. | 3216 | 140 | Persulphates, inorganic, aqueous solution, n.o.s. |
| 3188 | 136 | Self-heating liquid, corrosive, inorganic, n.o.s. | 3218 | 140 | Nitrates, inorganic, aqueous solution, n.o.s. |
| 3189 | 135 | Metal powder, self-heating, n.o.s. | 3219 | 140 | Nitrites, inorganic, aqueous solution, n.o.s. |
| 3190 | 135 | Self-heating solid, inorganic, n.o.s. | 3220 | 126 | Pentafluoroethane |
|  |  |  | 3220 | 126 | Refrigerant gas R-125 |
| 3191 | 136 | Self-heating solid, poisonous, inorganic, n.o.s. | 3221 | 149 | Self-reactive liquid type B |
| 3191 | 136 | Self-heating solid, toxic, inorganic, n.o.s. | 3222 | 149 | Self-reactive solid type B |
|  |  |  | 3223 | 149 | Self-reactive liquid type C |
| 3192 | 136 | Self-heating solid, corrosive, inorganic, n.o.s. | 3224 | 149 | Self-reactive solid type C |
|  |  |  | 3225 | 149 | Self-reactive liquid type D |
| 3194 | 135 | Pyrophoric liquid, inorganic, n.o.s. | 3226 | 149 | Self-reactive solid type D |
| 3200 | 135 | Pyrophoric solid, inorganic, n.o.s. | 3227 | 149 | Self-reactive liquid type E |
|  |  |  | 3228 | 149 | Self-reactive solid type E |
| 3205 | 135 | Alkaline earth metal alcoholates, n.o.s. | 3229 | 149 | Self-reactive liquid type F |
| 3206 | 136 | Alkali metal alcoholates, self- heating, corrosive, n.o.s. | 3230 | 149 | Self-reactive solid type F |
|  |  |  | 3231 | 150 | Self-reactive liquid type B, temperature controlled |
| 3208 | 138 | Metallic substance, water- reactive, n.o.s. |  |  |  |
|  |  |  | 3232 | 150 | Self-reactive solid type B, temperature controlled |
| 3209 | 138 | Metallic substance, water- reactive, self-heating, n.o.s. |  |  |  |

Page 70

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

3233 150 Self-reactive liquid type C,

temperature controlled

3234 150 Self-reactive solid type C,

temperature controlled

3235 150 Self-reactive liquid type D,

temperature controlled

3236 150 Self-reactive solid type D,

temperature controlled

3237 150 Self-reactive liquid type E,

temperature controlled

3238 150 Self-reactive solid type E,

temperature controlled

3239 150 Self-reactive liquid type F,

temperature controlled

3240 150 Self-reactive solid type F,

temperature controlled

3241 133 2-Bromo-2-nitropropane-1,

3-diol

3242 149  Azodicarbonamide

3243 151 Solids containing poisonous liquid, n.o.s.

3243 151 Solids containing toxic liquid,

n.o.s.

3244 154 Solids containing corrosive liquid, n.o.s.

3245 171 Genetically modified micro- organisms

3246 156 Methanesulfonyl chloride

3245 171 Genetically modified organisms

3246 156 Methanesulphonyl chloride

3247 140 Sodium peroxoborate,

anhydrous

3248 131 Medicine, liquid, flammable,

poisonous, n.o.s.

3248 131 Medicine, liquid, flammable, toxic, n.o.s.

3249 151 Medicine, solid, poisonous,

n.o.s.

3249 151 Medicine, solid, toxic, n.o.s.

3250 153 Chloroacetic acid, molten

3251 133 Isosorbide-5-mononitrate

3252 115  Difluoromethane

3252 115 Refrigerant gas R-32

3253 154 Disodium trioxosilicate

3254 135  Tributylphosphane

3255 135 tert-Butyl hypochlorite

3256 128 Elevated temperature liquid,

flammable, n.o.s., with flash point above 37.8°C (100°F), at or above its flash point

3256 128 Elevated temperature liquid,

flammable, n.o.s., with flash point above 60°C (140°F), at or above its flash point

3257 171 Elevated temperature liquid,

n.o.s., at or above 100°C (212°F), and below its flash point

3258 171 Elevated temperature solid,

n.o.s., at or above 240°C (464°F)

3259 154 Amines, solid, corrosive, n.o.s.

3259 154 Polyamines, solid, corrosive,

n.o.s.

3260 154 Corrosive solid, acidic,

inorganic, n.o.s.

3261 154 Corrosive solid, acidic, organic,

n.o.s.

3262 154 Corrosive solid, basic,

inorganic, n.o.s.

3263 154 Corrosive solid, basic, organic,

n.o.s.

3264 154 Corrosive liquid, acidic,

inorganic, n.o.s.

3265 153 Corrosive liquid, acidic, organic,

n.o.s.

3266 154 Corrosive liquid, basic,

inorganic, n.o.s.

Page 71

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3267 | 153 | Corrosive liquid, basic, organic, n.o.s. |  | 3280 | 151 | Organoarsenic compound, liquid, n.o.s. |
|  | 3268 | 171 | Air bag inflators |  | 3281 | 151 | Metal carbonyls, liquid, n.o.s. |
|  | 3268 | 171 | Air bag modules |  | 3282 | 151 | Organometallic compound, liquid, poisonous, n.o.s. |
|  | 3268 | 171 | Safety devices |  |  |  |  |
|  | 3268 | 171 | Seat-belt pre-tensioners |  | 3282 | 151 | Organometallic compound, liquid, toxic, n.o.s. |
|  | 3269 | 128 | Polyester resin kit, liquid base material |  | 3283 | 151 | Selenium compound, solid, n.o.s. |
|  | 3270 | 133 | Nitrocellulose membrane filters |  | 3284 | 151 | Tellurium compound, n.o.s. |
|  | 3271 | 127 | Ethers, n.o.s. |  | 3285 | 151 | Vanadium compound, n.o.s. |
|  | 3272 | 127 | Esters, n.o.s. |  | 3286 | 131 | Flammable liquid, poisonous, corrosive, n.o.s. |
|  | 3273 | 131 | Nitriles, flammable, poisonous, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3286 | 131 | Flammable liquid, toxic, corrosive, n.o.s. |
|  | 3273 | 131 | Nitriles, flammable, toxic, n.o.s. |  |  |  |  |
|  | 3274 | 132 | Alcoholates solution, n.o.s., in alcohol |  | 3287 | 151 | Poisonous liquid, inorganic, n.o.s. |
|  | 3275 | 131 | Nitriles, poisonous, flammable, n.o.s. |  | 3287 | 151 | Toxic liquid, inorganic, n.o.s. |
|  |  |  |  |  | 3288 | 151 | Poisonous solid, inorganic, n.o.s. |
|  | 3275 | 131 | Nitriles, toxic, flammable, n.o.s. |  |  |  |  |
|  | 3276 | 151 | Nitriles, liquid, poisonous, n.o.s. |  | 3288 | 151 | Toxic solid, inorganic, n.o.s. |
|  | 3276 | 151 | Nitriles, liquid, toxic, n.o.s. |  | 3289 | 154 | Poisonous liquid, corrosive, inorganic, n.o.s. |
|  | 3276 | 151 | Nitriles, poisonous, liquid, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3289 | 154 | Toxic liquid, corrosive, inorganic, n.o.s. |
|  | 3276 | 151 | Nitriles, toxic, liquid, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3290 | 154 | Poisonous solid, corrosive, inorganic, n.o.s. |
|  | 3277 | 154 | Chloroformates, poisonous, corrosive, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3290 | 154 | Toxic solid, corrosive, inorganic, n.o.s. |
|  | 3277 | 154 | Chloroformates, toxic, corrosive, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3291 | 158 | (Bio)Medical waste, n.o.s. |
|  | 3278 | 151 | Organophosphorus compound, liquid, poisonous, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3291 | 158 | Clinical waste, unspecified, n.o.s. |
|  | 3278 | 151 | Organophosphorus compound, liquid, toxic, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3291 | 158 | Medical waste, n.o.s. |
|  | 3279 | 131 | Organophosphorus compound, poisonous, flammable, n.o.s. |  | 3291 | 158 | Regulated medical waste, n.o.s. |
|  |  |  |  |  | 3292 | 138 | Batteries, containing metallic sodium or sodium alloy |
|  | 3279 | 131 | Organophosphorus compound, toxic, flammable, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3292 | 138 | Batteries, containing sodium |

Page 72

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3292 | 138 | Cells, containing metallic sodium or sodium alloy |  |  | 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3292 | 138 | Cells, containing sodium |  |  |  |  |  |  |  |
|  |  |  |  |  | 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. |  |  |
| 3293 | 153 | Hydrazine, aqueous solution, with not more than 37% hydrazine |  |  |  |  |  |  |  |
|  |  |  |  |  | 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3294 | 131 | Hydrogen cyanide, solution in alcohol, with not more than 45% hydrogen cyanide |  |  |  |  |  |  |  |
|  |  |  |  |  | 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3295 | 128 | Hydrocarbons, liquid, n.o.s. |  |  |  |  |  |  |  |
| 3296 | 126 | Heptafluoropropane |  |  | 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3296 | 126 | Refrigerant gas R-227 |  |  |  |  |  |  |  |
| 3297 | 126 | Ethylene oxide and chlorotetrafluoroethane mixture, with not more than 8.8% ethylene oxide |  |  |  |  |  |  |  |
|  |  |  |  |  | 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3298 | 126 | Ethylene oxide and pentafluoroethane mixture, with not more than 7.9% ethylene oxide |  |  | 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3299 | 126 | Ethylene oxide and tetrafluoroethane mixture, with not more than 5.6% ethylene oxide |  |  |  |  |  |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3300 | 119P | Ethylene oxide and carbon dioxide mixture, with more than 87% ethylene oxide |  |  | 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3301 | 136 | Corrosive liquid, self-heating, n.o.s. |  |  | 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3302 | 152P | 2-Dimethylaminoethyl acrylate, stabilized |  |  |  |  |  |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, toxic, |  |  |
| 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. |  |  | corrosive, n.o.s. |  |  |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, toxic, |  |  |
| 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  |  | corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, toxic, |  |  |
| 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) |  |  | corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |
|  |  |  |  |  | 3304 | 125 | Compressed gas, toxic, |  |  |
| 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) |  |  | corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |

Page 73

| ID Guide Name of Material ID Guide Name of Material No. No. No. No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3304 | 125 | Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) | 3306 | 124 | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |
| 3305 | 119 | Compressed gas, poisonous, flammable, corrosive, n.o.s. | 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. |
| 3305 | 119 | Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |
| 3305 | 119 | Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |
| 3305 | 119 | Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |
| 3305 | 119 | Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. | 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) |
| 3306 | 124 | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. | 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. |
| 3306 | 124 | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) |
| 3306 | 124 | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) |
| 3306 | 124 | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) |

Page 74

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) |  |  | 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. |  |  | 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. |  |  |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  | 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  | 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. |  |  | 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. |  |  |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  | 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  | 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. |  |  | 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. |  |  |
| 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  | 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  | 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |

Page 75

| ID No. 3310 | Guid No. 124 | e Name of Material Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | ID No. 3325 | Guid No. 165 | e Name of Material Radioactive material, low specific activity (LSA-III), fissile |
| --- | --- | --- | --- | --- | --- |
| 3311 | 122 | Gas, refrigerated liquid, oxidizing, n.o.s. | 3326 | 165 | Radioactive material, surface contaminated objects (SCO-I or SCO-II), fissile |
| 3312 | 115 | Gas, refrigerated liquid, flammable, n.o.s. |  |  |  |
|  |  |  | 3327 | 165 | Radioactive material, Type A package, fissile, non-special form |
| 3313 | 135 | Organic pigments, self-heating |  |  |  |
| 3314 | 171 | Plastic molding compound | 3328 | 165 | Radioactive material, Type B(U) package, fissile |
| 3314 | 171 | Plastics moulding compound |  |  |  |
| 3315 | 151 | Chemical sample, poisonous | 3329 | 165 | Radioactive material, Type B(M) package, fissile |
| 3315 | 151 | Chemical sample, toxic | 3330 | 165 | Radioactive material, Type C package, fissile |
| 3316 | 171 | Chemical kit |  |  |  |
| 3316 | 171 | First aid kit | 3331 | 165 | Radioactive material, transported under special arrangement, fissile |
| 3317 | 113 | 2-Amino-4,6-dinitrophenol, wetted with not less than 20% water |  |  |  |
|  |  |  | 3332 | 164 | Radioactive material, Type A package, special form, non fissile or fissile-excepted |
| 3318 | 125 | Ammonia solution, with more than 50% ammonia |  |  |  |
| 3319 | 113 | Nitroglycerin mixture, desensitized, solid, n.o.s., with more than 2% but not more than 10% nitroglycerin | 3333 | 165 | Radioactive material, Type A package, special form, fissile |
|  |  |  | 3334 | 171 | Aviation regulated liquid, n.o.s. |
|  |  |  | 3334 | 171 | Self-defense spray, non- pressurized |
| 3320 | 157 | Sodium borohydride and sodium hydroxide solution, with not more than 12% sodium borohydride and not more than 40% sodium hydroxide |  |  |  |
|  |  |  | 3335 | 171 | Aviation regulated solid, n.o.s. |
|  |  |  | 3336 | 130 | Mercaptan mixture, liquid, flammable, n.o.s. |
| 3321 | 162 | Radioactive material, low specific activity (LSA-II), non fissile or fissile-excepted | 3336 | 130 | Mercaptans, liquid, flammable, n.o.s. |
| 3322 | 162 | Radioactive material, low specific activity (LSA-III), non fissile or fissile-excepted | 3337 | 126 | Refrigerant gas R-404A |
|  |  |  | 3338 | 126 | Refrigerant gas R-407A |
|  |  |  | 3339 | 126 | Refrigerant gas R-407B |
| 3323 | 163 | Radioactive material, Type C package, non fissile or fissile excepted | 3340 | 126 | Refrigerant gas R-407C |
|  |  |  | 3341 | 135 | Thiourea dioxide |
| 3324 | 165 | Radioactive material, low specific activity (LSA-II), fissile | 3342 | 135 | Xanthates |

Page 76

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3343 | 113 | Nitroglycerin mixture, desensitized, liquid, flammable, n.o.s., with not more than 30% nitroglycerin |  | 3351 | 131 | Pyrethroid pesticide, liquid, poisonous, flammable |  |  |
|  |  |  |  | 3351 | 131 | Pyrethroid pesticide, liquid, toxic, flammable |  |  |
| 3344 | 113 | Pentaerythrite tetranitrate mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN |  | 3352 | 151 | Pyrethroid pesticide, liquid, poisonous |  |  |
|  |  |  |  | 3352 | 151 | Pyrethroid pesticide, liquid, toxic |  |  |
| 3344 | 113 | Pentaerythritol tetranitrate mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN |  |  |  |  |  |  |
|  |  |  |  | 3354 | 115 | Insecticide gas, flammable, n.o.s. |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, poisonous, flammable, n.o.s. |  |  |
| 3344 | 113 | PETN mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3345 | 153 | Phenoxyacetic acid derivative pesticide, solid, poisonous |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3345 | 153 | Phenoxyacetic acid derivative pesticide, solid, toxic |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3346 | 131 | Phenoxyacetic acid derivative pesticide, liquid, flammable, poisonous |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3346 | 131 | Phenoxyacetic acid derivative pesticide, liquid, flammable, toxic |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, toxic, flammable, n.o.s. |  |  |
| 3347 | 131 | Phenoxyacetic acid derivative pesticide, liquid, poisonous, flammable |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  |
| 3347 | 131 | Phenoxyacetic acid derivative pesticide, liquid, toxic, flammable |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  |
| 3348 | 153 | Phenoxyacetic acid derivative pesticide, liquid, poisonous |  |  |  |  |  |  |
|  |  |  |  | 3355 | 119 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  |
| 3348 | 153 | Phenoxyacetic acid derivative pesticide, liquid, toxic |  |  |  |  |  |  |
| 3349 | 151 | Pyrethroid pesticide, solid, poisonous |  | 3355 | 119 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  |
| 3349 | 151 | Pyrethroid pesticide, solid, toxic |  |  |  |  |  |  |
| 3350 | 131 | Pyrethroid pesticide, liquid, flammable, poisonous |  | 3356 | 140 | Oxygen generator, chemical |  |  |
|  |  |  |  | 3356 | 140 | Oxygen generator, chemical, spent |  |  |
| 3350 | 131 | Pyrethroid pesticide, liquid, flammable, toxic |  |  |  |  |  |  |

Page 77

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3357 | 113 | Nitroglycerin mixture, desensitized, liquid, n.o.s., with not more than 30% nitroglycerin |  |  | 3368 | 113 | Trinitrobenzoic acid, wetted with not less than 10% water |
|  |  |  |  |  |  | 3369 | 113 | Sodium dinitro-o-cresolate, wetted with not less than 10% water |
|  | 3358 | 115 | Refrigerating machines, containing flammable, non- poisonous, liquefied gas |  |  | 3370 | 113 | Urea nitrate, wetted with not less than 10% water |
|  | 3358 | 115 | Refrigerating machines, containing flammable, non- toxic, liquefied gas |  |  | 3371 | 129 | 2-Methylbutanal |
|  |  |  |  |  |  | 3373 | 158 | Biological substance, category B |
|  | 3359 | 171 | Fumigated cargo transport unit |  |  | 3374 | 116 | Acetylene, solvent free |
|  | 3360 | 133 | Fibers, vegetable, dry |  |  | 3375 | 140 | Ammonium nitrate emulsion |
|  | 3360 | 133 | Fibres, vegetable, dry |  |  | 3375 | 140 | Ammonium nitrate gel |
|  | 3361 | 156 | Chlorosilanes, poisonous, corrosive, n.o.s. |  |  | 3375 | 140 | Ammonium nitrate suspension |
|  |  |  |  |  |  | 3376 | 113 | 4-Nitrophenylhydrazine, with not less than 30% water |
|  | 3361 | 156 | Chlorosilanes, toxic, corrosive, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 3377 | 140 | Sodium perborate monohydrate |
|  | 3362 | 155 | Chlorosilanes, poisonous, corrosive, flammable, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 3378 | 140 | Sodium carbonate peroxyhydrate |
|  | 3362 | 155 | Chlorosilanes, toxic, corrosive, flammable, n.o.s. |  |  |  |  |  |
|  |  |  |  |  |  | 3379 | 113 | Desensitized explosive, liquid, n.o.s. |
|  | 3363 | 171 | Dangerous goods in apparatus |  |  |  |  |  |
|  | 3363 | 171 | Dangerous goods in articles |  |  | 3380 | 113 | Desensitized explosive, solid, n.o.s. |
|  | 3363 | 171 | Dangerous goods in machinery |  |  | 3381 | 151 | Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) |
|  | 3364 | 113 | Picric acid, wetted with not less than 10% water |  |  |  |  |  |
|  | 3364 | 113 | Trinitrophenol, wetted with not less than 10% water |  |  | 3381 | 151 | Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) |
|  | 3365 | 113 | Picryl chloride, wetted with not less than 10% water |  |  | 3382 | 151 | Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) |
|  | 3365 | 113 | Trinitrochlorobenzene, wetted with not less than 10% water |  |  |  |  |  |
|  |  |  |  |  |  | 3382 | 151 | Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) |
|  | 3366 | 113 | TNT, wetted with not less than 10% water |  |  |  |  |  |
|  |  |  |  |  |  | 3383 | 131 | Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3366 | 113 | Trinitrotoluene, wetted with not less than 10% water |  |  |  |  |  |
|  |  |  |  |  |  | 3383 | 131 | Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3367 | 113 | Trinitrobenzene, wetted with not less than 10% water |  |  |  |  |  |

Page 78

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3384 | 131 | Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) |  | 3391 | 135 | Organometallic substance, solid, pyrophoric |  |
|  |  |  |  | 3392 | 135 | Organometallic substance, liquid, pyrophoric |  |
| 3384 | 131 | Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3393 | 135 | Organometallic substance, solid, pyrophoric, water- reactive |  |
| 3385 | 139 | Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  | 3394 | 135 | Organometallic substance, liquid, pyrophoric, water- reactive |  |
| 3385 | 139 | Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  | 3395 | 135 | Organometallic substance, solid, water-reactive |  |
| 3386 | 139 | Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3396 | 138 | Organometallic substance, solid, water-reactive, flammable |  |
| 3386 | 139 | Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3397 | 138 | Organometallic substance, solid, water-reactive, self- heating |  |
| 3387 | 142 | Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  | 3398 | 135 | Organometallic substance, liquid, water-reactive |  |
| 3387 | 142 | Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  | 3399 | 138 | Organometallic substance, liquid, water-reactive, flammable |  |
| 3388 | 142 | Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) |  | 3400 | 138 | Organometallic substance, solid, self-heating |  |
|  |  |  |  | 3401 | 138 | Alkali metal amalgam, solid |  |
| 3388 | 142 | Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) |  | 3402 | 138 | Alkaline earth metal amalgam, solid |  |
|  |  |  |  | 3403 | 138 | Potassium metal alloys, solid |  |
| 3389 | 154 | Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  | 3404 | 138 | Potassium sodium alloys, solid |  |
|  |  |  |  | 3405 | 141 | Barium chlorate, solution |  |
| 3389 | 154 | Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |
|  |  |  |  | 3406 | 141 | Barium perchlorate, solution |  |
|  |  |  |  | 3407 | 140 | Chlorate and magnesium chloride mixture, solution |  |
| 3390 | 154 | Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3408 | 141 | Lead perchlorate, solution |  |
|  |  |  |  | 3409 | 152 | Chloronitrobenzenes, liquid |  |
| 3390 | 154 | Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3410 | 153 | 4-Chloro-o-toluidine hydrochloride, solution |  |
|  |  |  |  | 3411 | 153 | beta-Naphthylamine, solution |  |

Page 79

| ID No. 3411 | Guide No. 153 | Name of Material Naphthylamine (beta), solution | ID No. 3436 | Guid No. 151 | e Name of Material Hexafluoroacetone hydrate, solid |
| --- | --- | --- | --- | --- | --- |
| 3412 | 153 | Formic acid, with not less than 5% but less than 10% acid |  |  |  |
|  |  |  | 3437 | 152 | Chlorocresols, solid |
| 3412 | 153 | Formic acid, with not less than 10% but not more than 85% acid | 3438 | 153 | alpha-Methylbenzyl alcohol, solid |
|  |  |  | 3438 | 153 | Methylbenzyl (alpha) alcohol, solid |
| 3413 | 157 | Potassium cyanide, solution |  |  |  |
| 3414 | 157 | Sodium cyanide, solution | 3439 | 151 | Nitriles, solid, poisonous, n.o.s. |
| 3415 | 154 | Sodium fluoride, solution | 3439 | 151 | Nitriles, solid, toxic, n.o.s. |
| 3416 | 153 | Chloroacetophenone, liquid | 3440 | 151 | Selenium compound, liquid, n.o.s. |
| 3417 | 152 | Xylyl bromide, solid |  |  |  |
| 3418 | 151 | 2,4-Toluenediamine, solution | 3441 | 153 | Chlorodinitrobenzenes, solid |
| 3418 | 151 | 2,4-Toluylenediamine, solution | 3442 | 153 | Dichloroanilines, solid |
| 3419 | 157 | Boron trifluoride acetic acid complex, solid | 3443 | 152 | Dinitrobenzenes, solid |
|  |  |  | 3444 | 151 | Nicotine hydrochloride, solid |
| 3420 | 157 | Boron trifluoride propionic acid complex, solid | 3445 | 151 | Nicotine sulfate, solid |
|  |  |  | 3445 | 151 | Nicotine sulphate, solid |
| 3421 | 154 | Potassium hydrogen difluoride, solution | 3446 | 152 | Nitrotoluenes, solid |
| 3422 | 154 | Potassium fluoride, solution | 3447 | 152 | Nitroxylenes, solid |
| 3423 | 153 | Tetramethylammonium hydroxide, solid | 3448 | 159 | Tear gas substance, solid, n.o.s. |
| 3424 | 141 | Ammonium dinitro-o-cresolate, solution | 3449 | 159 | Bromobenzyl cyanides, solid |
|  |  |  | 3450 | 151 | Diphenylchloroarsine, solid |
| 3425 | 156 | Bromoacetic acid, solid | 3451 | 153 | Toluidines, solid |
| 3426 | 153P | Acrylamide, solution | 3452 | 153 | Xylidines, solid |
| 3427 | 153 | Chlorobenzyl chlorides, solid | 3453 | 154 | Phosphoric acid, solid |
| 3428 | 156 | 3-Chloro-4-methylphenyl isocyanate, solid | 3454 | 152 | Dinitrotoluenes, solid |
| 3429 | 153 | Chlorotoluidines, liquid | 3455 | 153 | Cresols, solid |
| 3430 | 153 | Xylenols, liquid | 3456 | 157 | Nitrosylsulfuric acid, solid |
| 3431 | 152 | Nitrobenzotrifluorides, solid | 3456 | 157 | Nitrosylsulphuric acid, solid |
| 3432 | 171 | PCB, solid | 3457 | 152 | Chloronitrotoluenes, solid |
| 3432 | 171 | Polychlorinated biphenyls, solid | 3458 | 152 | Nitroanisoles, solid |
| 3434 | 153 | Nitrocresols, liquid | 3459 | 152 | Nitrobromobenzenes, solid |
|  |  |  | 3460 | 153 | N-Ethylbenzyltoluidines, solid |

Page 80

| ID Guide Name of Material No. No. |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3462 | 152 | Toxins, extracted from living sources, solid, n.o.s. | 3474 | 113 | 1-Hydroxybenzotriazole, monohydrate |  |
| 3463 | 153 | Propionic acid, with not less than 90% acid | 3475 | 127 | Ethanol and gasoline mixture, with more than 10% ethanol |  |
| 3464 | 151 | Organophosphorus compound, solid, poisonous, n.o.s. | 3475 | 127 | Ethanol and motor spirit mixture, with more than 10% ethanol |  |
| 3464 | 151 | Organophosphorus compound, solid, toxic, n.o.s. |  |  |  |  |
|  |  |  | 3475 | 127 | Ethanol and petrol mixture, with more than 10% ethanol |  |
| 3465 | 151 | Organoarsenic compound, solid, n.o.s. |  |  |  |  |
|  |  |  | 3476 | 138 | Fuel cell cartridges, containing water-reactive substances |  |
| 3466 | 151 | Metal carbonyls, solid, n.o.s. |  |  |  |  |
| 3467 | 151 | Organometallic compound, solid, poisonous, n.o.s. | 3476 | 138 | Fuel cell cartridges contained in equipment, containing water- reactive substances |  |
| 3467 | 151 | Organometallic compound, solid, toxic, n.o.s. | 3476 | 138 | Fuel cell cartridges packed with equipment, containing water- reactive substances |  |
| 3468 | 115 | Hydrogen in a metal hydride storage system |  |  |  |  |
|  |  |  | 3477 | 153 | Fuel cell cartridges, containing corrosive substances |  |
| 3468 | 115 | Hydrogen in a metal hydride storage system contained in equipment |  |  |  |  |
|  |  |  | 3477 | 153 | Fuel cell cartridges contained in equipment, containing corrosive substances |  |
| 3468 | 115 | Hydrogen in a metal hydride storage system packed with equipment |  |  |  |  |
|  |  |  | 3477 | 153 | Fuel cell cartridges packed with equipment, containing corrosive substances |  |
| 3469 | 132 | Paint, flammable, corrosive |  |  |  |  |
| 3469 | 132 | Paint related material, flammable, corrosive | 3478 | 115 | Fuel cell cartridges, containing liquefied flammable gas |  |
| 3470 | 132 | Paint, corrosive, flammable | 3478 | 115 | Fuel cell cartridges contained in equipment, containing liquefied flammable gas |  |
| 3470 | 132 | Paint related material, corrosive, flammable |  |  |  |  |
| 3471 | 154 | Hydrogendifluorides, solution, n.o.s. | 3478 | 115 | Fuel cell cartridges packed with equipment, containing liquefied flammable gas |  |
| 3472 | 153 | Crotonic acid, liquid | 3479 | 115 | Fuel cell cartridges, containing hydrogen in metal hydride |  |
| 3473 | 128 | Fuel cell cartridges, containing flammable liquids |  |  |  |  |
|  |  |  | 3479 | 115 | Fuel cell cartridges contained in equipment, containing hydrogen in metal hydride |  |
| 3473 | 128 | Fuel cell cartridges contained in equipment, containing flammable liquids |  |  |  |  |
|  |  |  | 3479 | 115 | Fuel cell cartridges packed with equipment, containing hydrogen in metal hydride |  |
| 3473 | 128 | Fuel cell cartridges packed with equipment, containing flammable liquids |  |  |  |  |

Page 81

| ID Guide Name of Material No. No. |  |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3480 | 147 | Lithium ion batteries (including lithium ion polymer batteries) |  |  | 3489 | 131 | Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |
|  | 3481 | 147 | Lithium ion batteries contained in equipment (including lithium ion polymer batteries) |  |  |  |  |  |
|  |  |  |  |  |  | 3489 | 131 | Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |
|  | 3481 | 147 | Lithium ion batteries packed with equipment (including lithium ion polymer batteries) |  |  |  |  |  |
|  |  |  |  |  |  | 3490 | 155 | Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3482 | 138 | Alkali metal dispersion, flammable |  |  |  |  |  |
|  |  |  |  |  |  | 3490 | 155 | Toxic by inhalation liquid, water- reactive, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3482 | 138 | Alkaline earth metal dispersion, flammable |  |  |  |  |  |
|  | 3483 | 131 | Motor fuel anti-knock mixture, flammable |  |  | 3491 | 155 | Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone B) |
|  | 3484 | 132 | Hydrazine aqueous solution, flammable, with more than 37% hydrazine |  |  |  |  |  |
|  |  |  |  |  |  | 3491 | 155 | Toxic by inhalation liquid, water- reactive, flammable, n.o.s. (Inhalation Hazard Zone B) |
|  | 3485 | 140 | Calcium hypochlorite, dry, corrosive, with more than 39% available chlorine (8.8% available oxygen) |  |  |  |  |  |
|  |  |  |  |  |  | 3492 | 131 | Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3485 | 140 | Calcium hypochlorite mixture, dry, corrosive, with more than 39% available chlorine (8.8% available oxygen) |  |  |  |  |  |
|  |  |  |  |  |  | 3492 | 131 | Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) |
|  | 3486 | 140 | Calcium hypochlorite mixture, dry, corrosive, with more than 10% but not more than 39% available chlorine |  |  | 3493 | 131 | Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) |
|  |  |  |  |  |  | 3493 | 131 | Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) |
|  | 3487 | 140 | Calcium hypochlorite, hydrated, corrosive, with not less than 5.5% but not more than 16% water |  |  |  |  |  |
|  |  |  |  |  |  | 3494 | 131 | Petroleum sour crude oil, flammable, poisonous |
|  | 3487 | 140 | Calcium hypochlorite, hydrated mixture, corrosive, with not less than 5.5% but not more than 16% water |  |  |  |  |  |
|  |  |  |  |  |  | 3494 | 131 | Petroleum sour crude oil, flammable, toxic |
|  |  |  |  |  |  | 3495 | 154 | Iodine |
|  | 3488 | 131 | Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 3496 | 171 | Batteries, nickel-metal hydride |
|  |  |  |  |  |  | 3497 | 133 | Krill meal |
|  | 3488 | 131 | Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 3498 | 157 | Iodine monochloride, liquid |
|  |  |  |  |  |  | 3499 | 171 | Capacitor, electric double layer |

Page 82

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3500 | 126 | Chemical under pressure, n.o.s. |  |  | 3512 | 173 | Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone B) |  |
| 3501 | 115 | Chemical under pressure, flammable, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 3512 | 173 | Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone C) |  |
| 3502 | 123 | Chemical under pressure, poisonous, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 3512 | 173 | Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone D) |  |
| 3502 | 123 | Chemical under pressure, toxic, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 3513 | 174 | Adsorbed gas, oxidizing, n.o.s. |  |
| 3503 | 125 | Chemical under pressure, corrosive, n.o.s. |  |  | 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. |  |
| 3504 | 119 | Chemical under pressure, flammable, poisonous, n.o.s. |  |  | 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) |  |
| 3504 | 119 | Chemical under pressure, flammable, toxic, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) |  |
| 3505 | 118 | Chemical under pressure, flammable, corrosive, n.o.s. |  |  |  |  |  |  |
|  |  |  |  |  | 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) |  |
| 3506 | 172 | Mercury contained in manufactured articles |  |  |  |  |  |  |
| 3507 | 166 | Uranium hexafluoride, radioactive material, excepted package, less than 0.1 kg per package, non- |  |  | 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |  |
| 3508 | 171 | fissile or fissile-excepted Capacitor, asymmetric |  |  | 3514 | 173 | Adsorbed gas, toxic, flammable, n.o.s. |  |
|  |  |  |  |  | 3514 | 173 | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) |  |
| 3509 | 171 | Packagings discarded, empty, uncleaned |  |  |  |  |  |  |
| 3510 | 174 | Adsorbed gas, flammable, n.o.s. |  |  | 3514 | 173 | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) |  |
| 3511 | 174 | Adsorbed gas, n.o.s. |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. |  |  | 3514 | 173 | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone B) |  |  | 3514 | 173 | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |  |  |
|  |  |  |  |  | 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |
|  |  |  |  |  | 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  |
| 3512 | 173 | Adsorbed gas, toxic, n.o.s. |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone A) |  |  |  |  |  |  |

Page 83

| ID Guide Name of Material ID Guide Name of Material No. No. No. No. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 3516 | 173 | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) |
| 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 3516 | 173 | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) |
| 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 3516 | 173 | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) |
| 3515 | 173 | Adsorbed gas, toxic, oxidizing, n.o.s. | 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. |
| 3515 | 173 | Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |
| 3515 | 173 | Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |
| 3515 | 173 | Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) |
| 3515 | 173 | Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. | 3517 | 173 | Adsorbed gas, toxic, flammable, corrosive, n.o.s. |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 3517 | 173 | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 3517 | 173 | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 3517 | 173 | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 3517 | 173 | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |
| 3516 | 173 | Adsorbed gas, toxic, corrosive, n.o.s. | 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. |
| 3516 | 173 | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) | 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |

Page 84

| ID Guide Name of Material No. No. |  |  |  | ID Guide Name of Material No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |  | 3528 | 128 | Machinery, internal combustion, flammable liquid powered |  |
|  |  |  |  | 3529 | 115 | Engine, fuel cell, flammable gas powered |  |
| 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |  |
|  |  |  |  | 3529 | 115 | Engine, internal combustion, flammable gas powered |  |
| 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |
|  |  |  |  | 3529 | 115 | Machinery, fuel cell, flammable gas powered |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. |  | 3529 | 115 | Machinery, internal combustion, flammable gas powered |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |  | 3530 | 171 | Engine, internal combustion |  |
|  |  |  |  | 3530 | 171 | Machinery, internal combustion |  |
|  |  |  |  | 3531 | 149P | Polymerizing substance, solid, stabilized, n.o.s. |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  |  |  |  |
|  |  |  |  | 3532 | 149P | Polymerizing substance, liquid, stabilized, n.o.s. |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  |  |  |  |
|  |  |  |  | 3533 | 150P | Polymerizing substance, solid, temperature controlled, n.o.s. |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  | 3534 | 150P | Polymerizing substance, liquid, temperature controlled, n.o.s. |  |
|  |  |  |  | 3535 | 134 | Toxic solid, flammable, inorganic, n.o.s. |  |
| 3519 | 173 | Boron trifluoride, adsorbed |  |  |  |  |  |
| 3520 | 173 | Chlorine, adsorbed |  | 3536 | 147 | Lithium batteries installed in cargo transport unit (lithium ion batteries) |  |
| 3521 | 173 | Silicon tetrafluoride, adsorbed |  |  |  |  |  |
| 3522 | 173 | Arsine, adsorbed |  | 3536 | 138 | Lithium batteries installed in cargo transport unit (lithium metal batteries) |  |
| 3523 | 173 | Germane, adsorbed |  |  |  |  |  |
| 3524 | 173 | Phosphorus pentafluoride, adsorbed |  |  |  |  |  |
|  |  |  |  | 3537 | 115 | Articles containing flammable gas, n.o.s. |  |
| 3525 | 173 | Phosphine, adsorbed |  |  |  |  |  |
|  |  |  |  | 3538 | 120 | Articles containing non- flammable, non-toxic gas, n.o.s. |  |
| 3526 | 173 | Hydrogen selenide, adsorbed |  |  |  |  |  |
| 3527 | 128P | Polyester resin kit, solid base material |  |  |  |  |  |
|  |  |  |  | 3539 | 123 | Articles containing toxic gas, n.o.s. |  |
| 3528 | 128 | Engine, fuel cell, flammable liquid powered |  |  |  |  |  |
|  |  |  |  | 3540 | 127 | Articles containing flammable liquid, n.o.s. |  |
| 3528 | 128 | Engine, internal combustion, flammable liquid powered |  |  |  |  |  |
|  |  |  |  | 3541 | 133 | Articles containing flammable solid, n.o.s. |  |
| 3528 | 128 | Machinery, fuel cell, flammable liquid powered |  |  |  |  |  |

Page 85

| ID No. 3542 | Guid No. 135 | e Name of Material Articles containing a substance liable to spontaneous combustion, n.o.s. | ID No. 3560 | Guid No. 153 | e Name of Material Tetramethylammonium hydroxide aqueous solution with not less than 25% tetramethylammonium hydroxide |
| --- | --- | --- | --- | --- | --- |
| 3543 | 138 | Articles containing a substance which in contact with water emits flammable gases, n.o.s. |  |  |  |
|  |  |  | 8000 | 171 | Consumer commodity |
| 3544 | 140 | Articles containing oxidizing substance, n.o.s. | 9035 | 123 | Gas identification set |
|  |  |  | 9191 | 143 | Chlorine dioxide, hydrate, frozen |
| 3545 | 145 | Articles containing organic peroxide, n.o.s. |  |  |  |
|  |  |  | 9202 | 168 | Carbon monoxide, refrigerated liquid (cryogenic liquid) |
| 3546 | 151 | Articles containing toxic substance, n.o.s. |  |  |  |
|  |  |  | 9206 | 137 | Methyl phosphonic dichloride |
| 3547 | 154 | Articles containing corrosive substance, n.o.s. | 9260 | 169 | Aluminum, molten |
| 3548 | 171 | Articles containing miscellaneous dangerous goods, n.o.s. | 9263 | 156 | Chloropivaloyl chloride |
|  |  |  | 9264 | 151 | 3,5-Dichloro-2,4,6- trifluoropyridine |
| 3549 | 158 | Medical waste, category A, affecting animals only, solid | 9269 | 132 | Trimethoxysilane |
| 3549 | 158 | Medical waste, category A, affecting humans, solid |  |  |  |
| 3550 | 151 | Cobalt dihydroxide powder |  |  |  |
| 3551 | 147 | Sodium ion batteries |  |  |  |
| 3552 | 147 | Sodium ion batteries contained in equipment |  |  |  |
| 3552 | 147 | Sodium ion batteries packed with equipment |  |  |  |
| 3553 | 116 | Disilane |  |  |  |
| 3554 | 172 | Gallium contained in manufactured articles |  |  |  |
| 3555 | 113 | Trifluoromethyltetrazole-sodium salt in acetone |  |  |  |
| 3556 | 147 | Vehicle, lithium ion battery powered |  |  |  |
| 3557 | 138 | Vehicle, lithium metal battery powered |  |  |  |
| 3558 | 147 | Vehicle, sodium ion battery powered |  |  |  |
| 3559 | 171 | Fire suppressant dispersing devices |  |  |  |

Page 86

##### ID

No.

Guide No.

Name of Material

ID

No.

Guide No.

Name of Material

Page 87

##### INTRODUCTION TO BLUE SECTION

For entries highlighted in green follow these steps:

##### IF THERE IS NO FIRE:

  - Go directly to Table 1 (green section)

  - Look up the ID number and name of material

  - Identify initial isolation and protective action distances

  - Also consult the appropriate Orange Guide

##### IF A FIRE IS INVOLVED:

  - Use the appropriate Orange Guide for EVACUATION distances

  - Also protect in downwind direction according to Table 1 for residual material release

Note 1: If the name in Table 1 isshown with (whenspilledinwater), these materials produce large amounts of Toxic Inhalation Hazard (TIH) (PIHin the US) gases when spilled in water. Somewater-reactivematerials arealsoTIHmaterials themselves (e.g., UN1746 (Bromine trifluoride), UN1836 (Thionyl chloride)). In these instances, two entries are provided in Table 1 for land-based and water-based spills. If a water-reactive material only has one entry in Table 1 for (when spilled in water) and the product is NOT spilled in water, Table 1 and Table 2 do not apply. You will find safe distances in the appropriate Orange Guide.

Note 2: Explosivesarenot individuallylistedbytheir namebecauseinanemergencysituation, the response will be based only on the division of the explosive, not on the individual explosive.

###### For divisions 1.1, 1.2, 1.3 and 1.5, refer to GUIDE 112. For divisions 1.4 and 1.6, refer to GUIDE 114.

Note 3: Chemical and biological warfare agents are now found in the “Criminal or Terrorist Use of Chemical, Biological and Radiological Agents” section.

Page 88

| Name of Material | Guide No. | ID No. | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acetal | 127 | 1088 |  | Adsorbed gas, flammable, n.o.s. | 174 | 3510 |  |
| Acetaldehyde | 129P | 1089 |  |  |  |  |  |
| Acetaldehyde ammonia | 171 | 1841 |  | Adsorbed gas, n.o.s. | 174 | 3511 |  |
| Acetaldehyde oxime | 129 | 2332 |  | Adsorbed gas, oxidizing, n.o.s. | 174 | 3513 |  |
| Acetic acid, glacial | 132 | 2789 |  | Adsorbed gas, poisonous, corrosive, n.o.s. | 173 | 3516 |  |
| Acetic acid, solution, more than 10% but not more than 80% acid | 153 | 2790 |  | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3516 |  |
| Acetic acid, solution, more than 80% acid | 132 | 2789 |  | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3516 |  |
| Acetic anhydride | 137 | 1715 |  |  |  |  |  |
| Acetone | 127 | 1090 |  | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3516 |  |
| Acetone cyanohydrin, stabilized | 156 | 1541 |  |  |  |  |  |
|  |  |  |  | Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3516 |  |
| Acetone oils | 127 | 1091 |  |  |  |  |  |
| Acetonitrile | 127 | 1648 |  |  |  |  |  |
| Acetyl bromide | 156 | 1716 |  | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. | 173 | 3517 |  |
| Acetyl chloride | 155 | 1717 |  | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3517 |  |
| Acetylene, dissolved | 116 | 1001 |  |  |  |  |  |
| Acetylene, solvent free | 116 | 3374 |  |  |  |  |  |
|  |  |  |  | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3517 |  |
| Acetylene tetrabromide | 159 | 2504 |  |  |  |  |  |
| Acetyl iodide | 156 | 1898 |  |  |  |  |  |
| Acetyl methyl carbinol | 127 | 2621 |  | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3517 |  |
| Acid, sludge | 153 | 1906 |  |  |  |  |  |
| Acid butyl phosphate | 153 | 1718 |  | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3517 |  |
| Acridine | 153 | 2713 |  |  |  |  |  |
| Acrolein, stabilized | 131P | 1092 |  | Adsorbed gas, poisonous, flammable, n.o.s. | 173 | 3514 |  |
| Acrolein dimer, stabilized | 129P | 2607 |  |  |  |  |  |
|  |  |  |  | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 173 | 3514 |  |
| Acrylamide, solid | 153P | 2074 |  |  |  |  |  |
| Acrylamide, solution | 153P | 3426 |  |  |  |  |  |
| Acrylic acid, stabilized | 132P | 2218 |  | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 173 | 3514 |  |
| Acrylonitrile, stabilized | 131P | 1093 |  |  |  |  |  |
| Adhesives (flammable) | 128 | 1133 |  | Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) | 173 | 3514 |  |
| Adiponitrile | 153 | 2205 |  |  |  |  |  |

Page 89

| Name of Material Guide ID Name of Material Guide ID No. No. No. No. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 173 | 3514 |  | Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 173 | 3515 |
| Adsorbed gas, poisonous, n.o.s. | 173 | 3512 |  | Adsorbed gas, toxic, corrosive, n.o.s. | 173 | 3516 |
| Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone A) | 173 | 3512 |  | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3516 |
| Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone B) | 173 | 3512 |  | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3516 |
| Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone C) | 173 | 3512 |  | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3516 |
| Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) | 173 | 3512 |  | Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3516 |
| Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. | 173 | 3518 |  | Adsorbed gas, toxic, flammable, corrosive, n.o.s. | 173 | 3517 |
| Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3518 |  | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3517 |
| Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3518 |  | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3517 |
| Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3518 |  | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3517 |
| Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3518 |  | Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3517 |
| Adsorbed gas, poisonous, oxidizing, n.o.s. | 173 | 3515 |  | Adsorbed gas, toxic, flammable, n.o.s. | 173 | 3514 |
| Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 173 | 3515 |  | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) | 173 | 3514 |
| Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 173 | 3515 |  | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 173 | 3514 |
| Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 173 | 3515 |  | Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) | 173 | 3514 |

Page 90

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 173 | 3514 |  | Air, refrigerated liquid (cryogenic liquid) | 122 | 1003 |  |
|  |  |  |  | Air bag inflators | 171 | 3268 |  |
| Adsorbed gas, toxic, n.o.s. | 173 | 3512 |  |  |  |  |  |
|  |  |  |  | Air bag modules | 171 | 3268 |  |
| Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone A) | 173 | 3512 |  |  |  |  |  |
|  |  |  |  | Aircraft hydraulic power unit fuel tank | 131 | 3165 |  |
| Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone B) | 173 | 3512 |  |  |  |  |  |
|  |  |  |  | Alcoholates solution, n.o.s., in alcohol | 132 | 3274 |  |
| Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone C) | 173 | 3512 |  |  |  |  |  |
|  |  |  |  | Alcoholic beverages | 127 | 3065 |  |
| Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone D) | 173 | 3512 |  | Alcohols, flammable, poisonous, n.o.s. | 131 | 1986 |  |
| Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. | 173 | 3518 |  | Alcohols, flammable, toxic, n.o.s. | 131 | 1986 |  |
| Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 173 | 3518 |  | Alcohols, n.o.s. | 127 | 1987 |  |
|  |  |  |  | Aldehydes, flammable, poisonous, n.o.s. | 131P | 1988 |  |
| Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 173 | 3518 |  | Aldehydes, flammable, toxic, n.o.s. | 131P | 1988 |  |
|  |  |  |  | Aldehydes, n.o.s. | 129P | 1989 |  |
| Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 173 | 3518 |  |  |  |  |  |
|  |  |  |  | Aldol | 153 | 2839 |  |
|  |  |  |  | Alkali metal alcoholates, self- heating, corrosive, n.o.s. | 136 | 3206 |  |
| Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 173 | 3518 |  |  |  |  |  |
|  |  |  |  | Alkali metal alloy, liquid, n.o.s. | 138 | 1421 |  |
| Adsorbed gas, toxic, oxidizing, n.o.s. | 173 | 3515 |  | Alkali metal amalgam, liquid | 138 | 1389 |  |
|  |  |  |  | Alkali metal amalgam, solid | 138 | 3401 |  |
| Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 173 | 3515 |  |  |  |  |  |
|  |  |  |  | Alkali metal amides | 139 | 1390 |  |
|  |  |  |  | Alkali metal dispersion | 138 | 1391 |  |
| Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 173 | 3515 |  | Alkali metal dispersion, flammable | 138 | 3482 |  |
|  |  |  |  | Alkaline earth metal alcoholates, n.o.s. | 135 | 3205 |  |
| Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 173 | 3515 |  |  |  |  |  |
|  |  |  |  | Alkaline earth metal alloy, n.o.s. | 138 | 1393 |  |
| Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 173 | 3515 |  |  |  |  |  |
|  |  |  |  | Alkaline earth metal amalgam, liquid | 138 | 1392 |  |
| Aerosols | 126 | 1950 |  | Alkaline earth metal amalgam, solid | 138 | 3402 |  |
| Air, compressed | 122 | 1002 |  |  |  |  |  |

Page 91

| Name of Material G Alkaline earth metal dispersion | uide No. 138 | ID No. 1391 | Name of Material G Alkylsulphuric acids | uide No. 156 | ID No. 2571 |
| --- | --- | --- | --- | --- | --- |
| Alkaline earth metal dispersion, flammable | 138 | 3482 | Allyl acetate | 131 | 2333 |
|  |  |  | Allyl alcohol | 131 | 1098 |
| Alkaloid salts, liquid, n.o.s. (poisonous) | 151 | 3140 | Allylamine | 131 | 2334 |
| Alkaloid salts, solid, n.o.s. (poisonous) | 151 | 1544 | Allyl bromide | 131P | 1099 |
|  |  |  | Allyl chloride | 131P | 1100 |
| Alkaloids, liquid, n.o.s. (poisonous) | 151 | 3140 | Allyl chlorocarbonate | 155 | 1722 |
| Alkaloids, solid, n.o.s. (poisonous) | 151 | 1544 | Allyl chloroformate | 155 | 1722 |
|  |  |  | Allyl ethyl ether | 131 | 2335 |
| Alkylphenols, liquid, n.o.s. (including C2-C12 homologues) | 153 | 3145 | Allyl formate | 131 | 2336 |
|  |  |  | Allyl glycidyl ether | 129 | 2219 |
| Alkylphenols, solid, n.o.s. (including C2-C12 homologues) | 153 | 2430 | Allyl iodide | 132 | 1723 |
|  |  |  | Allyl isothiocyanate, stabilized | 131 | 1545 |
| Alkyl sulfonic acids, liquid, with more than 5% free sulfuric acid | 153 | 2584 | Allyltrichlorosilane, stabilized | 155 | 1724 |
|  |  |  | alpha-Methylbenzyl alcohol, liquid | 153 | 2937 |
| Alkyl sulfonic acids, liquid, with not more than 5% free sulfuric acid | 153 | 2586 | alpha-Methylbenzyl alcohol, solid | 153 | 3438 |
|  |  |  | alpha-Methylvaleraldehyde | 130 | 2367 |
| Alkyl sulfonic acids, solid, with more than 5% free sulfuric acid | 153 | 2583 | alpha-Naphthylamine | 153 | 2077 |
|  |  |  | alpha-Pinene | 128 | 2368 |
| Alkyl sulfonic acids, solid, with not more than 5% free sulfuric acid | 153 | 2585 | Aluminum, molten | 169 | 9260 |
|  |  |  | Aluminum borohydride | 135 | 2870 |
| Alkylsulfuric acids | 156 | 2571 | Aluminum borohydride in devices | 135 | 2870 |
| Alkyl sulphonic acids, liquid, with more than 5% free sulphuric acid | 153 | 2584 |  |  |  |
|  |  |  | Aluminum bromide, anhydrous | 137 | 1725 |
|  |  |  | Aluminum bromide, solution | 154 | 2580 |
| Alkyl sulphonic acids, liquid, with not more than 5% free sulphuric acid | 153 | 2586 | Aluminum carbide | 138 | 1394 |
|  |  |  | Aluminum chloride, anhydrous | 137 | 1726 |
| Alkyl sulphonic acids, solid, with more than 5% free sulphuric acid | 153 | 2583 | Aluminum chloride, solution | 154 | 2581 |
|  |  |  | Aluminum dross | 138 | 3170 |
| Alkyl sulphonic acids, solid, with not more than 5% free sulphuric acid | 153 | 2585 | Aluminum ferrosilicon powder | 139 | 1395 |
|  |  |  | Aluminum hydride | 138 | 2463 |

Page 92

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

| Ammonia solution, with more 125 3318 than 50% ammonia |  |  |
| --- | --- | --- |
| Ammonium arsenate | 151 | 1546 |
| Ammonium bifluoride, solid | 154 | 1727 |
| Ammonium bifluoride, solution | 154 | 2817 |
| Ammonium dichromate | 141 | 1439 |
| Ammonium dinitro-o-cresolate, 141 1843 solid Ammonium dinitro-o-cresolate, 141 3424 solution |  |  |

Aluminum nitrate	140	1438

| Aluminum phosphide | 139 | 1397 |  |
| --- | --- | --- | --- |
| Aluminum phosphide pesticide | 157 | 3048 |  |
| Aluminum powder, coated | 170 | 1309 |  |
| Aluminum powder, pyrophoric | 135 | 1383 |  |
| Aluminum powder, uncoated | 138 | 1396 |  |
| Aluminum remelting by- | 138 | 3170 |  |
| products Aluminum resinate | 133 | 2715 |  |
| Aluminum silicon powder, | 138 | 1398 |  |

| Ammonium fluoride | 154 | 2505 |
| --- | --- | --- |
| Ammonium fluorosilicate | 151 | 2854 |
| Ammonium hydrogendifluoride, 154 1727 solid Ammonium hydrogendifluoride, 154 2817 solution Ammonium hydrogen sulfate 154 2506 |  |  |

uncoated

Aluminum smelting by-products 138	3170

Amines, flammable, corrosive, n.o.s.

Amines, liquid, corrosive, flammable, n.o.s.

Amines, liquid, corrosive,

132	2733

132	2734

153	2735

| n.o.s. |  | Ammonium hydrogen sulphate | 154 | 2506 |
| --- | --- | --- | --- | --- |
| Amines, solid, corrosive, n.o.s. 154 | 3259 | Ammonium hydroxide, with more than 10% but not more | 154 | 2672 |

| 2-Amino-4-chlorophenol | 151 | 2673 |
| --- | --- | --- |
| 2-Amino-5- | 153 | 2946 |

than 35% ammonia

diethylaminopentane

- Amino-4,6-dinitrophenol, wetted with not less than 20% water

113	3317

| Ammonium metavanadate | 154 | 2859 |
| --- | --- | --- |
| Ammonium nitrate, liquid (hot | 140 | 2426 |

concentrated solution)

Ammonium nitrate, with not more than 0.2% combustible substances

Ammonium nitrate based fertilizer

Ammonium nitrate based fertilizer

140	1942

140	2067

140	2071

| 2-(2-Aminoethoxy)ethanol | 154 | 3055 |
| --- | --- | --- |
| N-Aminoethylpiperazine | 153 | 2815 |
| Aminophenols | 152 | 2512 |
| Aminopyridines | 153 | 2671 |
| Ammonia, anhydrous | 125 | 1005 |
| Ammonia solution, with more | 154 | 2672 |

than 10% but not more than 35% ammonia

Ammonia solution, with more than 35% but not more than 50% ammonia

125	2073

| Ammonium nitrate emulsion | 140 | 3375 |  |
| --- | --- | --- | --- |
| Ammonium nitrate-fuel oil | 112 | — — |  |
| mixtures Ammonium nitrate gel | 140 | 3375 |  |
| Ammonium nitrate suspension | 140 | 3375 |  |
| Ammonium perchlorate | 143 | 1442 |  |

Page 93

| Name of Material G Ammonium persulfate | uide No. 140 | ID No. 1444 |  |  | Name of Material G Anisoyl chloride | uide No. 156 | ID No. 1729 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonium persulphate | 140 | 1444 |  |  | Antimony compound, inorganic, liquid, n.o.s. | 157 | 3141 |
| Ammonium picrate, wetted with not less than 10% water | 113 | 1310 |  |  |  |  |  |
|  |  |  |  |  | Antimony compound, inorganic, solid, n.o.s. | 157 | 1549 |
| Ammonium polysulfide, solution | 154 | 2818 |  |  |  |  |  |
|  |  |  |  |  | Antimony lactate | 151 | 1550 |
| Ammonium polysulphide, solution | 154 | 2818 |  |  | Antimony pentachloride, liquid | 157 | 1730 |
|  |  |  |  |  | Antimony pentachloride, solution | 157 | 1731 |
| Ammonium polyvanadate | 151 | 2861 |  |  |  |  |  |
| Ammonium silicofluoride | 151 | 2854 |  |  | Antimony pentafluoride | 157 | 1732 |
| Ammonium sulfide, solution | 132 | 2683 |  |  | Antimony potassium tartrate | 151 | 1551 |
| Ammonium sulphide, solution | 132 | 2683 |  |  | Antimony powder | 170 | 2871 |
| Ammunition, poisonous, non- explosive | 151 | 2016 |  |  | Antimony trichloride | 157 | 1733 |
|  |  |  |  |  | Antimony trichloride, liquid | 157 | 1733 |
| Ammunition, tear-producing, non-explosive | 159 | 2017 |  |  | Antimony trichloride, solid | 157 | 1733 |
| Ammunition, toxic, non- explosive | 151 | 2016 |  |  | Aqua regia | 157 | 1798 |
|  |  |  |  |  | Argon | 120 | 1006 |
| Amyl acetates | 129 | 1104 |  |  | Argon, compressed | 120 | 1006 |
| Amyl acid phosphate | 153 | 2819 |  |  | Argon, refrigerated liquid (cryogenic liquid) | 120 | 1951 |
| Amylamine | 132 | 1106 |  |  |  |  |  |
| Amyl butyrates | 130 | 2620 |  |  | Arsenic | 152 | 1558 |
| Amyl chloride | 129 | 1107 |  |  | Arsenic acid, liquid | 154 | 1553 |
| n-Amylene | 128 | 1108 |  |  | Arsenic acid, solid | 154 | 1554 |
| Amyl formates | 129 | 1109 |  |  | Arsenical dust | 152 | 1562 |
| Amyl mercaptan | 130 | 1111 |  |  | Arsenical pesticide, liquid, flammable, poisonous | 131 | 2760 |
| n-Amyl methyl ketone | 127 | 1110 |  |  | Arsenical pesticide, liquid, flammable, toxic | 131 | 2760 |
| Amyl nitrate | 128 | 1112 |  |  |  |  |  |
| Amyl nitrite | 129 | 1113 |  |  | Arsenical pesticide, liquid, poisonous | 151 | 2994 |
| Amyltrichlorosilane | 156 | 1728 |  |  |  |  |  |
| Anhydrous ammonia | 125 | 1005 |  |  | Arsenical pesticide, liquid, poisonous, flammable | 131 | 2993 |
| Aniline | 153 | 1547 |  |  | Arsenical pesticide, liquid, toxic | 151 | 2994 |
| Aniline hydrochloride | 153 | 1548 |  |  |  |  |  |
| Anisidines | 153 | 2431 |  |  | Arsenical pesticide, liquid, toxic, flammable | 131 | 2993 |
| Anisole | 128 | 2222 |  |  |  |  |  |

Page 94

| Name of Material | Guide No. | ID No. | Name of Material | Guide No. | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Arsenical pesticide, solid, poisonous | 151 | 2759 | Articles containing toxic substance, n.o.s. | 151 | 3546 |  |

| Arsenic bromide | 151 | 1555 |
| --- | --- | --- |
| Arsenic chloride | 157 | 1560 |
| Arsenic compound, liquid, | 152 | 1556 |

Arsenical pesticide, solid, toxic 151	2759

Articles, pressurized, hydraulic 126	3164

(containing non-flammable gas)

n.o.s.

Articles, pressurized, pneumatic (containing non- flammable gas)

126	3164

Arsenic compound, solid, n.o.s. 152	1557

Arsenic pentoxide	151	1559

Aryl sulfonic acids, liquid, with 153	2584

more than 5% free sulfuric acid

Arsenic trichloride	157	1560

Arsenic trioxide	151	1561

| Arsine | 119 | 2188 |
| --- | --- | --- |
| Arsine, adsorbed | 173 | 3522 |

Articles containing a substance 135	3542

liable to spontaneous combustion, n.o.s.

Aryl sulfonic acids, liquid, with not more than 5% free sulfuric acid

Aryl sulfonic acids, solid, with more than 5% free sulfuric acid

Aryl sulfonic acids, solid, with not more than 5% free sulfuric acid

153	2586

153	2583

153	2585

Articles containing a substance which in contact with water emits flammable gases, n.o.s.

Articles containing corrosive substance, n.o.s.

Articles containing flammable gas, n.o.s.

Articles containing flammable liquid, n.o.s.

Articles containing flammable solid, n.o.s.

Articles containing

miscellaneous dangerous goods, n.o.s.

Articles containing non- flammable, non-toxic gas, n.o.s.

Articles containing oxidizing substance, n.o.s.

138	3543

154	3547

115	3537

127	3540

133	3541

171	3548

120	3538

140	3544

Aryl sulphonic acids, liquid, with more than 5% free sulphuric acid

Aryl sulphonic acids, liquid, with not more than 5% free sulphuric acid

Aryl sulphonic acids, solid, with more than 5% free sulphuric acid

| Asbestos | 171 | 2212 |
| --- | --- | --- |
| Asbestos, amphibole | 171 | 2212 |
| Asbestos, chrysotile | 171 | 2590 |
| Asphalt | 130 | 1999 |
| Asphalt, cut back | 130 | 1999 |

Aryl sulphonic acids, solid, with not more than 5% free sulphuric acid

153	2584

153	2586

153	2583

153	2585

Articles containing toxic gas,	123	3539

n.o.s.

Articles containing organic peroxide, n.o.s.

145	3545

Aviation regulated liquid, n.o.s. 171	3334

Aviation regulated solid, n.o.s. 171	3335

Azodicarbonamide	149	3242

Barium	138	1400

Page 95

| Name of Material G Barium alloys, pyrophoric | uide No. 135 | ID No. 1854 | Name of Material G Battery-powered vehicle (wet battery) | uide No. 154 | ID No. 3171 |
| --- | --- | --- | --- | --- | --- |
| Barium azide, wetted with not less than 50% water | 113 | 1571 |  |  |  |
|  |  |  | Battery-powered vehicle (with lithium ion batteries) | 147 | 3171 |
| Barium bromate | 141 | 2719 |  |  |  |
| Barium chlorate, solid | 141 | 1445 | Battery-powered vehicle (with sodium batteries) | 138 | 3171 |
| Barium chlorate, solution | 141 | 3405 | Benzaldehyde | 171 | 1990 |
| Barium compound, n.o.s. | 154 | 1564 | Benzene | 130 | 1114 |
| Barium cyanide | 157 | 1565 | Benzene phosphorus dichloride | 137 | 2798 |
| Barium hypochlorite, with more than 22% available chlorine | 141 | 2741 | Benzene phosphorus thiodichloride | 137 | 2799 |
| Barium nitrate | 141 | 1446 | Benzenesulfonyl chloride | 156 | 2225 |
| Barium oxide | 157 | 1884 | Benzenesulphonyl chloride | 156 | 2225 |
| Barium perchlorate, solid | 141 | 1447 | Benzidine | 153 | 1885 |
| Barium perchlorate, solution | 141 | 3406 | Benzonitrile | 152 | 2224 |
| Barium permanganate | 141 | 1448 | Benzoquinone | 153 | 2587 |
| Barium peroxide | 141 | 1449 | Benzotrichloride | 156 | 2226 |
| Batteries, containing metallic sodium or sodium alloy | 138 | 3292 | Benzotrifluoride | 127 | 2338 |
| Batteries, containing sodium | 138 | 3292 | Benzoyl chloride | 137 | 1736 |
| Batteries, dry, containing potassium hydroxide solid | 154 | 3028 | Benzyl bromide | 156 | 1737 |
|  |  |  | Benzyl chloride | 156 | 1738 |
| Batteries, nickel-metal hydride | 171 | 3496 | Benzyl chloroformate | 137 | 1739 |
| Batteries, wet, filled with acid | 154 | 2794 | Benzyldimethylamine | 132 | 2619 |
| Batteries, wet, filled with alkali | 154 | 2795 | Benzylidene chloride | 156 | 1886 |
| Batteries, wet, non-spillable | 154 | 2800 | Benzyl iodide | 156 | 2653 |
| Battery fluid, acid | 157 | 2796 | Beryllium compound, n.o.s. | 154 | 1566 |
| Battery fluid, alkali | 154 | 2797 | Beryllium nitrate | 141 | 2464 |
| Battery-powered equipment (wet battery) | 154 | 3171 | Beryllium powder | 134 | 1567 |
|  |  |  | beta-Naphthylamine, solid | 153 | 1650 |
| Battery-powered equipment (with lithium ion batteries) | 147 | 3171 | beta-Naphthylamine, solution | 153 | 3411 |
| Battery-powered equipment (with lithium metal batteries) | 138 | 3171 | Bhusa, wet, damp or contaminated with oil | 133 | 1327 |
| Battery-powered equipment (with sodium batteries) | 138 | 3171 | Bicyclo[2.2.1]hepta-2,5-diene, stabilized | 128P | 2251 |

Page 96

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Biological substance, category B | 158 | 3373 |  |  | Boron trifluoride acetic acid complex, liquid | 157 | 1742 |  |
| (Bio)Medical waste, n.o.s. | 158 | 3291 |  |  | Boron trifluoride acetic acid complex, solid | 157 | 3419 |  |
| Bipyridilium pesticide, liquid, flammable, poisonous | 131 | 2782 |  |  |  |  |  |  |
|  |  |  |  |  | Boron trifluoride diethyl etherate | 132 | 2604 |  |
| Bipyridilium pesticide, liquid, flammable, toxic | 131 | 2782 |  |  |  |  |  |  |
|  |  |  |  |  | Boron trifluoride dimethyl etherate | 139 | 2965 |  |
| Bipyridilium pesticide, liquid, poisonous | 151 | 3016 |  |  |  |  |  |  |
|  |  |  |  |  | Boron trifluoride propionic acid complex, liquid | 157 | 1743 |  |
| Bipyridilium pesticide, liquid, poisonous, flammable | 131 | 3015 |  |  |  |  |  |  |
|  |  |  |  |  | Boron trifluoride propionic acid complex, solid | 157 | 3420 |  |
| Bipyridilium pesticide, liquid, toxic | 151 | 3016 |  |  |  |  |  |  |
|  |  |  |  |  | Bromates, inorganic, aqueous solution, n.o.s. | 140 | 3213 |  |
| Bipyridilium pesticide, liquid, toxic, flammable | 131 | 3015 |  |  |  |  |  |  |
|  |  |  |  |  | Bromates, inorganic, n.o.s. | 140 | 1450 |  |
| Bipyridilium pesticide, solid, poisonous | 151 | 2781 |  |  | Bromine | 154 | 1744 |  |
|  |  |  |  |  | Bromine, solution | 154 | 1744 |  |
| Bipyridilium pesticide, solid, toxic | 151 | 2781 |  |  |  |  |  |  |
|  |  |  |  |  | Bromine, solution (Inhalation Hazard Zone A) | 154 | 1744 |  |
| Bisulfates, aqueous solution | 154 | 2837 |  |  |  |  |  |  |
|  |  |  |  |  | Bromine, solution (Inhalation Hazard Zone B) | 154 | 1744 |  |
| Bisulfites, aqueous solution, n.o.s. | 154 | 2693 |  |  |  |  |  |  |
|  |  |  |  |  | Bromine chloride | 124 | 2901 |  |
| Bisulphates, aqueous solution | 154 | 2837 |  |  |  |  |  |  |
|  |  |  |  |  | Bromine pentafluoride | 144 | 1745 |  |
| Bisulphites, aqueous solution, n.o.s. | 154 | 2693 |  |  |  |  |  |  |
|  |  |  |  |  | Bromine trifluoride | 144 | 1746 |  |
| Blasting agent, n.o.s. | 112 | — — |  |  | Bromoacetic acid, solid | 156 | 3425 |  |
| Bleaching powder | 140 | 2208 |  |  | Bromoacetic acid, solution | 156 | 1938 |  |
| Bombs, smoke, non-explosive, with corrosive liquid, without initiating device | 153 | 2028 |  |  | Bromoacetone | 131 | 1569 |  |
|  |  |  |  |  | Bromoacetyl bromide | 156 | 2513 |  |
| Borneol | 133 | 1312 |  |  | Bromobenzene | 130 | 2514 |  |
| Boron tribromide | 157 | 2692 |  |  | Bromobenzyl cyanides, liquid | 159 | 1694 |  |
|  |  |  |  |  | Bromobenzyl cyanides, solid | 159 | 3449 |  |
| Boron trichloride | 125 | 1741 |  |  |  |  |  |  |
|  |  |  |  |  | 1-Bromobutane | 130 | 1126 |  |
| Boron trifluoride | 125 | 1008 |  |  |  |  |  |  |
|  |  |  |  |  | 2-Bromobutane | 130 | 2339 |  |
| Boron trifluoride, adsorbed | 173 | 3519 |  |  |  |  |  |  |
|  |  |  |  |  | Bromochloromethane | 160 | 1887 |  |
| Boron trifluoride, compressed | 125 | 1008 |  |  |  |  |  |  |
|  |  |  |  |  | 1-Bromo-3-chloropropane | 159 | 2688 |  |
| Boron trifluoride, dihydrate | 157 | 2851 |  |  |  |  |  |  |
|  |  |  |  |  | 2-Bromoethyl ethyl ether | 130 | 2340 |  |

Page 97

| Name of Material G Bromoform | uide No. 159 | ID No. 2515 |  | Name of Material n-Butyl formate | Guide No. 129 | ID No. 1128 |
| --- | --- | --- | --- | --- | --- | --- |
| 1-Bromo-3-methylbutane | 130 | 2341 |  | tert-Butyl hypochlorite | 135 | 3255 |
| Bromomethylpropanes | 130 | 2342 |  | N,n-Butylimidazole | 152 | 2690 |
| 2-Bromo-2-nitropropane-1,3- diol | 133 | 3241 |  | n-Butyl isocyanate | 155P | 2485 |
|  |  |  |  | tert-Butyl isocyanate | 155 | 2484 |
| 2-Bromopentane | 130 | 2343 |  | Butyl mercaptan | 130 | 2347 |
| Bromopropanes | 129 | 2344 |  | n-Butyl methacrylate, stabilized | 130P | 2227 |
| 3-Bromopropyne | 130 | 2345 |  |  |  |  |
| Bromotrifluoroethylene | 116 | 2419 |  | Butyl methyl ether | 127 | 2350 |
| Bromotrifluoromethane | 126 | 1009 |  | Butyl nitrites | 129 | 2351 |
| Brucine | 151 | 1570 |  | Butyl propionates | 130 | 1914 |
| Butadienes, stabilized | 116P | 1010 |  | Butyltoluenes | 152 | 2667 |
| Butadienes and hydrocarbon mixture, stabilized | 116P | 1010 |  | Butyltrichlorosilane | 155 | 1747 |
|  |  |  |  | 5-tert-Butyl-2,4,6-trinitro-m- xylene | 149 | 2956 |
| Butane | 115 | 1011 |  |  |  |  |
| Butane | 115 | 1075 |  | Butyl vinyl ether, stabilized | 127P | 2352 |
| Butanedione | 127 | 2346 |  | 1,4-Butynediol | 153 | 2716 |
| Butanols | 129 | 1120 |  | Butyraldehyde | 129P | 1129 |
| Butyl acetates | 129 | 1123 |  | Butyraldoxime | 129 | 2840 |
| Butyl acid phosphate | 153 | 1718 |  | Butyric acid | 153 | 2820 |
| Butyl acrylates, stabilized | 129P | 2348 |  | Butyric anhydride | 156 | 2739 |
| n-Butylamine | 132 | 1125 |  | Butyronitrile | 131 | 2411 |
| N-Butylaniline | 153 | 2738 |  | Butyryl chloride | 155 | 2353 |
| Butylbenzenes | 128 | 2709 |  | Cacodylic acid | 151 | 1572 |
| n-Butyl bromide | 130 | 1126 |  | Cadmium compound | 154 | 2570 |
| n-Butyl chloride | 130 | 1127 |  | Caesium | 138 | 1407 |
| n-Butyl chloroformate | 155 | 2743 |  | Caesium hydroxide | 157 | 2682 |
| tert-Butylcyclohexyl chloroformate | 156 | 2747 |  | Caesium hydroxide, solution | 154 | 2681 |
| Butylene | 115 | 1012 |  | Caesium nitrate | 140 | 1451 |
| Butylene | 115 | 1075 |  | Calcium | 138 | 1401 |
| 1,2-Butylene oxide, stabilized | 127P | 3022 |  | Calcium, pyrophoric | 135 | 1855 |
| Butyl ethers | 128 | 1149 |  | Calcium alloys, pyrophoric | 135 | 1855 |

Page 98

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Calcium arsenate	151	1573

| Calcium arsenate and calcium arsenite mixture, solid | 151 | 1574 |
| --- | --- | --- |
| Calcium carbide | 138 | 1402 |
| Calcium chlorate | 140 | 1452 |
| Calcium chlorate, aqueous solution Calcium chlorite | 140 140 | 2429 1453 |

| Calcium cyanamide, with more | 138 | 1403 |
| --- | --- | --- |
| than 0.1% calcium carbide Calcium cyanide | 157 | 1575 |
| Calcium dithionite | 135 | 1923 |
| Calcium hydride | 138 | 1404 |
| Calcium hydrosulfite | 135 | 1923 |
| Calcium hydrosulphite | 135 | 1923 |
| Calcium hypochlorite, dry | 140 | 1748 |
| Calcium hypochlorite, dry, | 140 | 3485 |

corrosive, with more than 39% available chlorine (8.8% available oxygen)

Calcium hypochlorite, hydrated, corrosive, with not less than 5.5% but not more than 16% water

Calcium hypochlorite, hydrated, with not less than 5.5% but not more than 16% water

140	3487

140	2880

Calcium hypochlorite, hydrated 140	3487

mixture, corrosive, with not less than 5.5% but not more than 16% water

Calcium hypochlorite, hydrated 140	2880

mixture, with not less than 5.5% but not more than 16% water

| Calcium hypochlorite mixture, 140 3485 dry, corrosive, with more than 39% available chlorine (8.8% available oxygen) Calcium hypochlorite mixture, 140 2208 dry, with more than 10% but not more than 39% available chlorine Calcium hypochlorite mixture, 140 1748 dry, with more than 39% available chlorine (8.8% available oxygen) |  |  |
| --- | --- | --- |
| Calcium manganese silicon | 138 | 2844 |
| Calcium nitrate | 140 | 1454 |
| Calcium oxide | 157 | 1910 |
| Calcium perchlorate | 140 | 1455 |
| Calcium permanganate | 140 | 1456 |
| Calcium peroxide | 140 | 1457 |
| Calcium phosphide | 139 | 1360 |
| Calcium resinate | 133 | 1313 |
| Calcium resinate, fused | 133 | 1314 |
| Calcium silicide | 138 | 1405 |
| Camphor, synthetic | 133 | 2717 |
| Camphor oil | 128 | 1130 |
| Capacitor, asymmetric | 171 | 3508 |
| Capacitor, electric double layer | 171 | 3499 |
| Caproic acid | 153 | 2829 |
| Carbamate pesticide, liquid, | 131 | 2758 |
| flammable, poisonous Carbamate pesticide, liquid, 131 2758 flammable, toxic Carbamate pesticide, liquid, 151 2992 poisonous Carbamate pesticide, liquid, 131 2991 poisonous, flammable Carbamate pesticide, liquid, 151 2992 toxic |  |  |

Calcium hypochlorite mixture, dry, corrosive, with more than 10% but not more than 39% available chlorine

140	3486

Page 99

| Name of Material Guide ID No. No. |  |  |  |  | Name of Material Guide ID No. No. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Carbamate pesticide, liquid, toxic, flammable | 131 | 2991 |  | Celluloid, in block, rods, rolls, sheets, tubes, etc., except scrap | 133 | 2000 |
|  | Carbamate pesticide, solid, poisonous | 151 | 2757 |  |  |  |  |
|  |  |  |  |  | Celluloid, scrap | 135 | 2002 |
|  | Carbamate pesticide, solid, toxic | 151 | 2757 |  | Cerium, slabs, ingots or rods | 170 | 1333 |
|  |  |  |  |  | Cerium, turnings or gritty powder | 138 | 3078 |
|  | Carbon, activated | 133 | 1362 |  |  |  |  |
|  | Carbon, animal or vegetable origin | 133 | 1361 |  | Cesium | 138 | 1407 |
|  |  |  |  |  | Cesium hydroxide | 157 | 2682 |
|  | Carbon bisulfide | 131 | 1131 |  | Cesium hydroxide, solution | 154 | 2681 |
|  | Carbon dioxide | 120 | 1013 |  | Cesium nitrate | 140 | 1451 |
|  | Carbon dioxide, compressed | 120 | 1013 |  | Charcoal | 133 | 1361 |
|  | Carbon dioxide, refrigerated liquid | 120 | 2187 |  | Chemical kit | 154 | 1760 |
|  | Carbon dioxide, solid | 120 | 1845 |  | Chemical kit | 171 | 3316 |
|  | Carbon disulfide | 131 | 1131 |  | Chemical sample, poisonous | 151 | 3315 |
|  | Carbon disulphide | 131 | 1131 |  | Chemical sample, toxic | 151 | 3315 |
|  | Carbon monoxide, compressed | 119 | 1016 |  | Chemical under pressure, corrosive, n.o.s. | 125 | 3503 |
|  | Carbon monoxide, refrigerated liquid (cryogenic liquid) | 168 | 9202 |  |  |  |  |
|  |  |  |  |  | Chemical under pressure, flammable, corrosive, n.o.s. | 118 | 3505 |
|  | Carbon tetrabromide | 151 | 2516 |  |  |  |  |
|  |  |  |  |  | Chemical under pressure, flammable, n.o.s. | 115 | 3501 |
|  | Carbon tetrachloride | 151 | 1846 |  |  |  |  |
|  | Carbonyl fluoride | 125 | 2417 |  | Chemical under pressure, flammable, poisonous, n.o.s. | 119 | 3504 |
|  | Carbonyl sulfide | 119 | 2204 |  |  |  |  |
|  |  |  |  |  | Chemical under pressure, flammable, toxic, n.o.s. | 119 | 3504 |
|  | Carbonyl sulphide | 119 | 2204 |  |  |  |  |
|  | Castor beans, meal, pomace or flake | 171 | 2969 |  | Chemical under pressure, n.o.s. | 126 | 3500 |
|  | Caustic alkali liquid, n.o.s. | 154 | 1719 |  | Chemical under pressure, poisonous, n.o.s. | 123 | 3502 |
|  | Caustic potash, solid | 154 | 1813 |  |  |  |  |
|  | Caustic potash, solution | 154 | 1814 |  | Chemical under pressure, toxic, n.o.s. | 123 | 3502 |
|  | Caustic soda, solid | 154 | 1823 |  | Chloral, anhydrous, stabilized | 153 | 2075 |
|  | Caustic soda, solution | 154 | 1824 |  | Chlorate and borate mixture | 140 | 1458 |
|  | Cells, containing metallic sodium or sodium alloy | 138 | 3292 |  | Chlorate and magnesium chloride mixture, solid | 140 | 1459 |
|  | Cells, containing sodium | 138 | 3292 |  |  |  |  |

Page 100

Name of Material

Guide		ID	Name of Material No.	No.

Guide	ID

No.	No.

| Chlorate and magnesium 140 3407 chloride mixture, solution Chlorates, inorganic, aqueous 140 3210 solution, n.o.s. Chlorates, inorganic, n.o.s. 140 1461 Chloric acid, aqueous solution, 140 2626 with not more than 10% chloric acid |  | Chlorocresols, solution 152 2669 Chlorodifluorobromomethane 126 1974 1-Chloro-1,1-difluoroethane 115 2517 Chlorodifluoromethane 126 1018 Chlorodifluoromethane and 126 1973 chloropentafluoroethane mixture Chlorodinitrobenzenes, liquid 153 1577 Chlorodinitrobenzenes, solid 153 3441 |  |  |
| --- | --- | --- | --- | --- |
| Chlorine 124 1017 |  |  |  |  |
| Chlorine, adsorbed 173 3520 |  |  |  |  |
| Chlorine dioxide, hydrate, 143 9191 frozen |  |  | 2-Chloroethanal 153 2232 |  |
|  |  |  | Chloroform 151 1888 |  |
| Chlorine pentafluoride 124 2548 |  |  | Chloroformates, poisonous, 155 2742 corrosive, flammable, n.o.s. |  |
| Chlorine trifluoride 124 1749 |  |  |  |  |
| Chlorite solution 154 1908 |  |  | Chloroformates, poisonous, 154 3277 corrosive, n.o.s. |  |
| Chlorites, inorganic, n.o.s. 143 1462 Chloroacetaldehyde 153 2232 Chloroacetic acid, molten 153 3250 Chloroacetic acid, solid 153 1751 Chloroacetic acid, solution 153 1750 Chloroacetone, stabilized 131 1695 Chloroacetonitrile 131 2668 Chloroacetophenone, liquid 153 3416 Chloroacetophenone, solid 153 1697 Chloroacetyl chloride 156 1752 Chloroanilines, liquid 152 2019 Chloroanilines, solid 152 2018 Chloroanisidines 152 2233 Chlorobenzene 130 1134 Chlorobenzotrifluorides 130 2234 Chlorobenzyl chlorides, liquid 153 2235 Chlorobenzyl chlorides, solid 153 3427 Chlorobutanes 130 1127 Chlorocresols, solid 152 3437 |  |  | Chloroformates, toxic, 155 2742 corrosive, flammable, n.o.s. |  |
|  |  |  | Chloroformates, toxic, 154 3277 corrosive, n.o.s. Chloromethyl chloroformate 157 2745 Chloromethyl ethyl ether 131 2354 3-Chloro-4-methylphenyl 156 2236 isocyanate, liquid 3-Chloro-4-methylphenyl 156 3428 isocyanate, solid Chloronitroanilines 153 2237 Chloronitrobenzenes, liquid 152 3409 Chloronitrobenzenes, solid 152 1578 Chloronitrotoluenes, liquid 152 2433 Chloronitrotoluenes, solid 152 3457 Chloropentafluoroethane 126 1020 Chlorophenolates, liquid 154 2904 Chlorophenolates, solid 154 2905 Chlorophenols, liquid 153 2021 Chlorophenols, solid 153 2020 |  |

Page 101

| Name of Material G Chlorophenyltrichlorosilane | uide No. 156 | ID No. 1753 |  |  | Name of Material G Chlorotoluenes | uide No. 129 | ID No. 2238 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chloropicrin | 154 | 1580 |  |  | 4-Chloro-o-toluidine hydrochloride, solid | 153 | 1579 |
| Chloropicrin and methyl bromide mixture | 123 | 1581 |  |  |  |  |  |
|  |  |  |  |  | 4-Chloro-o-toluidine hydrochloride, solution | 153 | 3410 |
| Chloropicrin and methyl chloride mixture | 119 | 1582 |  |  |  |  |  |
|  |  |  |  |  | Chlorotoluidines, liquid | 153 | 3429 |
| Chloropicrin mixture, n.o.s. | 154 | 1583 |  |  | Chlorotoluidines, solid | 153 | 2239 |
| Chloropivaloyl chloride | 156 | 9263 |  |  | 1-Chloro-2,2,2-trifluoroethane | 126 | 1983 |
| Chloroplatinic acid, solid | 154 | 2507 |  |  | Chlorotrifluoromethane | 126 | 1022 |
| Chloroprene, stabilized | 131P | 1991 |  |  | Chlorotrifluoromethane and trifluoromethane azeotropic mixture with approximately 60% chlorotrifluoromethane | 126 | 2599 |
| 1-Chloropropane | 129 | 1278 |  |  |  |  |  |
| 2-Chloropropane | 129 | 2356 |  |  |  |  |  |
| 3-Chloropropanol-1 | 153 | 2849 |  |  | Chromic acid, solution | 154 | 1755 |
| 2-Chloropropene | 130P | 2456 |  |  | Chromic fluoride, solid | 154 | 1756 |
| 2-Chloropropionic acid | 153 | 2511 |  |  | Chromic fluoride, solution | 154 | 1757 |
| 2-Chloropyridine | 153 | 2822 |  |  | Chromium nitrate | 141 | 2720 |
| Chlorosilanes, corrosive, flammable, n.o.s. | 155 | 2986 |  |  | Chromium oxychloride | 137 | 1758 |
|  |  |  |  |  | Chromium trioxide, anhydrous | 141 | 1463 |
| Chlorosilanes, corrosive, n.o.s. | 156 | 2987 |  |  | Chromosulfuric acid | 154 | 2240 |
| Chlorosilanes, flammable, corrosive, n.o.s. | 155 | 2985 |  |  | Chromosulphuric acid | 154 | 2240 |
| Chlorosilanes, poisonous, corrosive, flammable, n.o.s. | 155 | 3362 |  |  | Clinical waste, unspecified, n.o.s. | 158 | 3291 |
| Chlorosilanes, poisonous, corrosive, n.o.s. | 156 | 3361 |  |  | Coal gas, compressed | 119 | 1023 |
|  |  |  |  |  | Coal tar distillates, flammable | 128 | 1136 |
| Chlorosilanes, toxic, corrosive, flammable, n.o.s. | 155 | 3362 |  |  | Coating solution | 127 | 1139 |
|  |  |  |  |  | Cobalt dihydroxide powder | 151 | 3550 |
| Chlorosilanes, toxic, corrosive, n.o.s. | 156 | 3361 |  |  | Cobalt naphthenates, powder | 133 | 2001 |
| Chlorosilanes, water-reactive, flammable, corrosive, n.o.s. | 139 | 2988 |  |  | Cobalt resinate, precipitated | 133 | 1318 |
|  |  |  |  |  | Combustible liquid, n.o.s. | 128 | 1993 |
| Chlorosulfonic acid (with or without sulfur trioxide) | 137 | 1754 |  |  | Compounds, cleaning liquid (corrosive) | 154 | 1760 |
| Chlorosulphonic acid (with or without sulphur trioxide) | 137 | 1754 |  |  | Compounds, cleaning liquid (flammable) | 128 | 1993 |
| 1-Chloro-1,2,2,2- tetrafluoroethane | 126 | 1021 |  |  | Compounds, tree or weed killing, liquid (corrosive) | 154 | 1760 |

Page 102

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compounds, tree or weed killing, liquid (flammable) | 128 | 1993 |  |  | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 1953 |  |
| Compounds, tree or weed killing, liquid (toxic) | 153 | 2810 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 1953 |  |
| Compressed gas, flammable, n.o.s. | 115 | 1954 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 119 | 1953 |  |
| Compressed gas, n.o.s. | 126 | 1956 |  |  |  |  |  |  |
| Compressed gas, oxidizing, n.o.s. | 122 | 3156 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, n.o.s. | 123 | 1955 |  |
| Compressed gas, poisonous, corrosive, n.o.s. | 125 | 3304 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone A) | 123 | 1955 |  |
| Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 125 | 3304 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone B) | 123 | 1955 |  |
| Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 125 | 3304 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone C) | 123 | 1955 |  |
| Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 125 | 3304 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) | 123 | 1955 |  |
| Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 125 | 3304 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. | 124 | 3306 |  |
| Compressed gas, poisonous, flammable, corrosive, n.o.s. | 119 | 3305 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 124 | 3306 |  |
| Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 119 | 3305 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 124 | 3306 |  |
| Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 119 | 3305 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 124 | 3306 |  |
| Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 119 | 3305 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 124 | 3306 |  |
| Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 119 | 3305 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, n.o.s. | 124 | 3303 |  |
| Compressed gas, poisonous, flammable, n.o.s. | 119 | 1953 |  |  |  |  |  |  |
|  |  |  |  |  | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 124 | 3303 |  |
| Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 119 | 1953 |  |  |  |  |  |  |

Page 103

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B)

Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C)

Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D)

Compressed gas, toxic, corrosive, n.o.s.

Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A)

Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B)

Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C)

Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D)

Compressed gas, toxic, flammable, corrosive, n.o.s.

Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A)

Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B)

Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C)

Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D)

Compressed gas, toxic, flammable, n.o.s.

Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A)

Page 104

124	3303

| Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 1953 |
| --- | --- | --- |
| Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 1953 |
| Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 119 | 1953 |
| Compressed gas, toxic, n.o.s. | 123 | 1955 |
| Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone A) | 123 | 1955 |
| Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone B) | 123 | 1955 |
| Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone C) | 123 | 1955 |
| Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone D) | 123 | 1955 |
| Compressed gas, toxic, oxidizing, corrosive, n.o.s. | 124 | 3306 |
| Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 124 | 3306 |
| Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 124 | 3306 |
| Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 124 | 3306 |
| Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 124 | 3306 |
| Compressed gas, toxic, oxidizing, n.o.s. | 124 | 3303 |
| Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 124 | 3303 |
| Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 124 | 3303 |
| Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 124 | 3303 |

124	3303

124	3303

125	3304

125	3304

125	3304

125	3304

125	3304

119	3305

119	3305

119	3305

119	3305

119	3305

119	1953

119	1953

| Name of Material Guide ID No. No. |  |  | Name of Material Guide ID No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 124 | 3303 | Corrosive liquid, poisonous, n.o.s. | 154 | 2922 |  |
|  |  |  | Corrosive liquid, self-heating, n.o.s. | 136 | 3301 |  |
| Consumer commodity | 171 | 8000 |  |  |  |  |
| Copper acetoarsenite | 151 | 1585 | Corrosive liquid, toxic, n.o.s. | 154 | 2922 |  |
| Copper arsenite | 151 | 1586 | Corrosive liquid, water- reactive, n.o.s. | 138 | 3094 |  |
| Copper based pesticide, liquid, flammable, poisonous | 131 | 2776 |  |  |  |  |
|  |  |  | Corrosive solid, acidic, inorganic, n.o.s. | 154 | 3260 |  |
| Copper based pesticide, liquid, flammable, toxic | 131 | 2776 |  |  |  |  |
|  |  |  | Corrosive solid, acidic, organic, n.o.s. | 154 | 3261 |  |
| Copper based pesticide, liquid, poisonous | 151 | 3010 |  |  |  |  |
|  |  |  | Corrosive solid, basic, inorganic, n.o.s. | 154 | 3262 |  |
| Copper based pesticide, liquid, poisonous, flammable | 131 | 3009 |  |  |  |  |
|  |  |  | Corrosive solid, basic, organic, n.o.s. | 154 | 3263 |  |
| Copper based pesticide, liquid, toxic | 151 | 3010 |  |  |  |  |
|  |  |  | Corrosive solid, flammable, n.o.s. | 134 | 2921 |  |
| Copper based pesticide, liquid, toxic, flammable | 131 | 3009 |  |  |  |  |
|  |  |  | Corrosive solid, n.o.s. | 154 | 1759 |  |
| Copper based pesticide, solid, poisonous | 151 | 2775 | Corrosive solid, oxidizing, n.o.s. | 157 | 3084 |  |
| Copper based pesticide, solid, toxic | 151 | 2775 | Corrosive solid, poisonous, n.o.s. | 154 | 2923 |  |
| Copper chlorate | 140 | 2721 | Corrosive solid, self-heating, n.o.s. | 136 | 3095 |  |
| Copper chloride | 154 | 2802 |  |  |  |  |
| Copper cyanide | 151 | 1587 | Corrosive solid, toxic, n.o.s. | 154 | 2923 |  |
| Copra | 135 | 1363 | Corrosive solid, water-reactive, n.o.s. | 138 | 3096 |  |
| Corrosive liquid, acidic, inorganic, n.o.s. | 154 | 3264 | Cotton | 133 | 1365 |  |
| Corrosive liquid, acidic, organic, n.o.s. | 153 | 3265 | Cotton, wet | 133 | 1365 |  |
|  |  |  | Cotton waste, oily | 133 | 1364 |  |
| Corrosive liquid, basic, inorganic, n.o.s. | 154 | 3266 | Coumarin derivative pesticide, liquid, flammable, poisonous | 131 | 3024 |  |
| Corrosive liquid, basic, organic, n.o.s. | 153 | 3267 | Coumarin derivative pesticide, liquid, flammable, toxic | 131 | 3024 |  |
| Corrosive liquid, flammable, n.o.s. | 132 | 2920 | Coumarin derivative pesticide, liquid, poisonous | 151 | 3026 |  |
| Corrosive liquid, n.o.s. | 154 | 1760 | Coumarin derivative pesticide, liquid, poisonous, flammable | 131 | 3025 |  |
| Corrosive liquid, oxidizing, n.o.s. | 157 | 3093 |  |  |  |  |

Page 105

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Coumarin derivative pesticide, liquid, toxic

Coumarin derivative pesticide, liquid, toxic, flammable

Coumarin derivative pesticide, solid, poisonous

Coumarin derivative pesticide, solid, toxic

151	3026

| Cyclohexene | 130 | 2256 |
| --- | --- | --- |
| Cyclohexenyltrichlorosilane | 156 | 1762 |
| Cyclohexyl acetate | 130 | 2243 |
| Cyclohexylamine | 132 | 2357 |
| Cyclohexyl isocyanate | 155 | 2488 |
| Cyclohexyl mercaptan | 129 | 3054 |
| Cyclohexyltrichlorosilane | 156 | 1763 |
| Cyclooctadiene phosphines | 135 | 2940 |
| Cyclooctadienes | 130P | 2520 |
| Cyclooctatetraene | 128P | 2358 |
| Cyclopentane | 128 | 1146 |
| Cyclopentanol | 129 | 2244 |
| Cyclopentanone | 128 | 2245 |
| Cyclopentene | 128 | 2246 |
| Cyclopropane | 115 | 1027 |
| Cymenes | 130 | 2046 |
| Dangerous goods in apparatus | 171 | 3363 |
| Dangerous goods in articles | 171 | 3363 |

131	3025

151	3027

151	3027

| Cresols, liquid | 153 | 2076 |  |
| --- | --- | --- | --- |
| Cresols, solid | 153 | 3455 |  |
| Cresylic acid | 153 | 2022 |  |
| Crotonaldehyde | 131P | 1143 |  |
| Crotonaldehyde, stabilized | 131P | 1143 |  |
| Crotonic acid, liquid | 153 | 3472 |  |
| Crotonic acid, solid | 153 | 2823 |  |
| Crotonylene | 128 | 1144 |  |
| Cumene | 130 | 1918 |  |
| Cupriethylenediamine, solution | 154 | 1761 |  |

| Cyanide solution, n.o.s. | 157 | 1935 |
| --- | --- | --- |
| Cyanides, inorganic, solid, | 157 | 1588 |

| n.o.s. |  |  | Dangerous goods in machinery | 171 | 3363 |
| --- | --- | --- | --- | --- | --- |
| Cyanogen | 119 | 1026 | Decaborane | 134 | 1868 |
| Cyanogen bromide | 157 | 1889 | Decahydronaphthalene | 130 | 1147 |
| Cyanogen chloride, stabilized | 125 | 1589 | n-Decane | 128 | 2247 |
| Cyanuric chloride | 157 | 2670 | Denatured alcohol | 127 | 1987 |
| Cyclobutane | 115 | 2601 | Desensitized explosive, liquid, | 113 | 3379 |

| Cyclobutyl chloroformate | 155 | 2744 |
| --- | --- | --- |
| 1,5,9-Cyclododecatriene | 153 | 2518 |
| Cycloheptane | 128 | 2241 |
| Cycloheptatriene | 131 | 2603 |
| Cycloheptene | 128 | 2242 |
| Cyclohexane | 128 | 1145 |
| Cyclohexanethiol | 129 | 3054 |
| Cyclohexanone | 127 | 1915 |

n.o.s.

Desensitized explosive, solid, n.o.s.

113	3380

| Deuterium, compressed | 115 | 1957 |
| --- | --- | --- |
| Devices, small, hydrocarbon gas | 115 | 3150 |

Page 106

powered, with release device

Diacetone alcohol	129	1148

| Diacetyl | 127 | 2346 |
| --- | --- | --- |
| Diallylamine | 132 | 2359 |

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diallyl ether | 131P | 2360 |  |  | Dichloromethane | 160 | 1593 |  |
| 4,4'-Diaminodiphenylmethane | 153 | 2651 |  |  | 1,1-Dichloro-1-nitroethane | 153 | 2650 |  |
| Di-n-amylamine | 131 | 2841 |  |  | Dichloropentanes | 130 | 1152 |  |
| Dibenzyldichlorosilane | 156 | 2434 |  |  | Dichlorophenyl isocyanates | 156 | 2250 |  |
| Diborane | 119 | 1911 |  |  | Dichlorophenyltrichlorosilane | 156 | 1766 |  |
| Diborane mixtures | 119 | 1911 |  |  | 1,2-Dichloropropane | 130 | 1279 |  |
| 1,2-Dibromobutan-3-one | 154 | 2648 |  |  | 1,3-Dichloropropanol-2 | 153 | 2750 |  |
| Dibromochloropropanes | 159 | 2872 |  |  | Dichloropropenes | 129 | 2047 |  |
| Dibromodifluoromethane | 171 | 1941 |  |  | Dichlorosilane | 119 | 2189 |  |
| Dibromomethane | 160 | 2664 |  |  | 1,2-Dichloro-1,1,2,2- tetrafluoroethane | 126 | 1958 |  |
| Di-n-butylamine | 132 | 2248 |  |  |  |  |  |  |
|  |  |  |  |  | 3,5-Dichloro-2,4,6- trifluoropyridine | 151 | 9264 |  |
| Dibutylaminoethanol | 153 | 2873 |  |  |  |  |  |  |
| Dibutyl ethers | 128 | 1149 |  |  | Dicyclohexylamine | 153 | 2565 |  |
| Dichloroacetic acid | 153 | 1764 |  |  | Dicyclohexylammonium nitrite | 133 | 2687 |  |
| 1,3-Dichloroacetone | 153 | 2649 |  |  | Dicyclopentadiene | 130P | 2048 |  |
| Dichloroacetyl chloride | 156 | 1765 |  |  | 1,2-Di-(dimethylamino) ethane | 129 | 2372 |  |
| Dichloroanilines, liquid | 153 | 1590 |  |  | Didymium nitrate | 140 | 1465 |  |
| Dichloroanilines, solid | 153 | 3442 |  |  | Diesel fuel | 128 | 1202 |  |
| o-Dichlorobenzene | 152 | 1591 |  |  | Diesel fuel | 128 | 1993 |  |
| 2,2'-Dichlorodiethyl ether | 152 | 1916 |  |  | Diethoxymethane | 127 | 2373 |  |
| Dichlorodifluoromethane | 126 | 1028 |  |  | 3,3-Diethoxypropene | 127 | 2374 |  |
| Dichlorodifluoromethane and difluoroethane azeotropic mixture with approximately 74% dichlorodifluoromethane | 126 | 2602 |  |  | Diethylamine | 132 | 1154 |  |
|  |  |  |  |  | 2-Diethylaminoethanol | 132 | 2686 |  |
|  |  |  |  |  | 3-Diethylaminopropylamine | 132 | 2684 |  |
| Dichlorodimethyl ether, symmetrical | 131 | 2249 |  |  | N,N-Diethylaniline | 153 | 2432 |  |
| 1,1-Dichloroethane | 130 | 2362 |  |  | Diethylbenzene | 130 | 2049 |  |
| 1,2-Dichloroethylene | 130P | 1150 |  |  | Diethyl carbonate | 128 | 2366 |  |
| Dichloroethyl ether | 152 | 1916 |  |  | Diethyldichlorosilane | 155 | 1767 |  |
| Dichlorofluoromethane | 126 | 1029 |  |  | Diethylenetriamine | 154 | 2079 |  |
| Dichloroisocyanuric acid, dry | 140 | 2465 |  |  | Diethyl ether | 127 | 1155 |  |
| Dichloroisocyanuric acid salts | 140 | 2465 |  |  | N,N-Diethylethylenediamine | 132 | 2685 |  |
| Dichloroisopropyl ether | 153 | 2490 |  |  | Diethyl ketone | 127 | 1156 |  |

Page 107

| Name of Material G Diethyl sulfate | uide No. 152 | ID No. 1594 |  | Name of Material 1,3-Dimethylbutylamine | Guide No. 132 | ID No. 2379 |
| --- | --- | --- | --- | --- | --- | --- |
| Diethyl sulfide | 129 | 2375 |  | Dimethylcarbamoyl chloride | 156 | 2262 |
| Diethyl sulphate | 152 | 1594 |  | Dimethyl carbonate | 129 | 1161 |
| Diethyl sulphide | 129 | 2375 |  | Dimethylcyclohexanes | 128 | 2263 |
| Diethylthiophosphoryl chloride | 156 | 2751 |  | N,N-Dimethylcyclohexylamine | 132 | 2264 |
| Difluorochloroethanes | 115 | 2517 |  | Dimethylcyclohexylamine | 132 | 2264 |
| 1,1-Difluoroethane | 115 | 1030 |  | Dimethyldichlorosilane | 155 | 1162 |
| 1,1-Difluoroethylene | 116P | 1959 |  | Dimethyldiethoxysilane | 127 | 2380 |
| Difluoromethane | 115 | 3252 |  | Dimethyldioxanes | 127 | 2707 |
| Difluorophosphoric acid, anhydrous | 154 | 1768 |  | Dimethyl disulfide | 131 | 2381 |
|  |  |  |  | Dimethyl disulphide | 131 | 2381 |
| 2,3-Dihydropyran | 127 | 2376 |  | Dimethyl ether | 115 | 1033 |
| Diisobutylamine | 132 | 2361 |  | N,N-Dimethylformamide | 129 | 2265 |
| Diisobutylene, isomeric compounds | 128 | 2050 |  | Dimethylhydrazine, symmetrical | 131 | 2382 |
| Diisobutyl ketone | 128 | 1157 |  | Dimethylhydrazine, unsymmetrical | 131 | 1163 |
| Diisooctyl acid phosphate | 153 | 1902 |  |  |  |  |
| Diisopropylamine | 132 | 1158 |  | 2,2-Dimethylpropane | 115 | 2044 |
| Diisopropyl ether | 127 | 1159 |  | Dimethyl-N-propylamine | 132 | 2266 |
| Diketene, stabilized | 131P | 2521 |  | Dimethyl sulfate | 156 | 1595 |
| 1,1-Dimethoxyethane | 127 | 2377 |  | Dimethyl sulfide | 130 | 1164 |
| 1,2-Dimethoxyethane | 127 | 2252 |  | Dimethyl sulphate | 156 | 1595 |
| Dimethylamine, anhydrous | 118 | 1032 |  | Dimethyl sulphide | 130 | 1164 |
| Dimethylamine, aqueous solution | 132 | 1160 |  | Dimethyl thiophosphoryl chloride | 156 | 2267 |
| Dimethylamine, solution | 132 | 1160 |  | Dinitroanilines | 153 | 1596 |
| 2-Dimethylaminoacetonitrile | 131 | 2378 |  | Dinitrobenzenes, liquid | 152 | 1597 |
| 2-Dimethylaminoethanol | 132 | 2051 |  | Dinitrobenzenes, solid | 152 | 3443 |
| 2-Dimethylaminoethyl acrylate, stabilized | 152P | 3302 |  | Dinitro-o-cresol | 153 | 1598 |
|  |  |  |  | Dinitrogen tetroxide | 124 | 1067 |
| 2-Dimethylaminoethyl methacrylate, stabilized | 153P | 2522 |  | Dinitrophenol, solution | 153 | 1599 |
| N,N-Dimethylaniline | 153 | 2253 |  | Dinitrophenol, wetted with not less than 15% water | 113 | 1320 |
| 2,3-Dimethylbutane | 128 | 2457 |  |  |  |  |

Page 108

| Name of Material Guide ID No. No. |  |  | Name of Material Guide ID No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Dinitrophenolates, wetted with not less than 15% water | 113 | 1321 | Dodecyltrichlorosilane | 156 | 1771 |  |
|  |  |  | Dry ice | 120 | 1845 |  |
| Dinitroresorcinol, wetted with not less than 15% water | 113 | 1322 | Dye, liquid, corrosive, n.o.s. | 154 | 2801 |  |
| Dinitrotoluenes, liquid | 152 | 2038 | Dye, liquid, poisonous, n.o.s. | 151 | 1602 |  |
| Dinitrotoluenes, molten | 152 | 1600 | Dye, liquid, toxic, n.o.s. | 151 | 1602 |  |
| Dinitrotoluenes, solid | 152 | 3454 | Dye, solid, corrosive, n.o.s. | 154 | 3147 |  |
| Dioxane | 127 | 1165 | Dye, solid, poisonous, n.o.s. | 151 | 3143 |  |
| Dioxolane | 127 | 1166 | Dye, solid, toxic, n.o.s. | 151 | 3143 |  |
| Dipentene | 128 | 2052 | Dye intermediate, liquid, corrosive, n.o.s. | 154 | 2801 |  |
| Diphenylamine chloroarsine | 154 | 1698 | Dye intermediate, liquid, poisonous, n.o.s. | 151 | 1602 |  |
| Diphenylchloroarsine, liquid | 151 | 1699 |  |  |  |  |
| Diphenylchloroarsine, solid | 151 | 3450 | Dye intermediate, liquid, toxic, n.o.s. | 151 | 1602 |  |
| Diphenyldichlorosilane | 156 | 1769 |  | 154 | 3147 |  |
| Diphenylmethyl bromide | 153 | 1770 | Dye intermediate, solid, corrosive, n.o.s. |  |  |  |
| Dipicryl sulfide, wetted with not less than 10% water | 113 | 2852 | Dye intermediate, solid, poisonous, n.o.s. | 151 | 3143 |  |
| Dipicryl sulphide, wetted with not less than 10% water | 113 | 2852 | Dye intermediate, solid, toxic, n.o.s. | 151 | 3143 |  |
| Dipropylamine | 132 | 2383 | Elevated temperature liquid, flammable, n.o.s., with flash point above 37.8°C (100°F), at or above its flash point | 128 | 3256 |  |
| Di-n-propyl ether | 127 | 2384 |  |  |  |  |
| Dipropyl ketone | 128 | 2710 |  |  |  |  |
| Disilane | 116 | 3553 | Elevated temperature liquid, flammable, n.o.s., with flash point above 60°C (140°F), at or above its flash point | 128 | 3256 |  |
| Disinfectant, liquid, corrosive, n.o.s. | 153 | 1903 |  |  |  |  |
| Disinfectant, liquid, poisonous, n.o.s. | 151 | 3142 | Elevated temperature liquid, n.o.s., at or above 100°C (212°F), and below its flash point | 171 | 3257 |  |
| Disinfectant, liquid, toxic, n.o.s. | 151 | 3142 |  |  |  |  |
| Disinfectant, solid, poisonous, n.o.s. | 151 | 1601 | Elevated temperature solid, n.o.s., at or above 240°C (464°F) | 171 | 3258 |  |
| Disinfectant, solid, toxic, n.o.s. | 151 | 1601 | Engine, fuel cell, flammable gas powered | 115 | 3529 |  |
| Disodium trioxosilicate | 154 | 3253 |  |  |  |  |
| Dispersant gases, n.o.s. (flammable) | 115 | 1954 | Engine, fuel cell, flammable liquid powered | 128 | 3528 |  |
| Divinyl ether, stabilized | 128P | 1167 | Engine, internal combustion | 171 | 3530 |  |

Page 109

| Name of Material G Engine, internal combustion, flammable gas powered | uide No. 115 | ID No. 3529 | Name of Material G Ethylamine, aqueous solution, with not less than 50% but not more than 70% ethylamine | uide No. 132 | ID No. 2270 |
| --- | --- | --- | --- | --- | --- |
| Engine, internal combustion, flammable liquid powered | 128 | 3528 |  |  |  |
| Environmentally hazardous substance, liquid, n.o.s. | 171 | 3082 | Ethyl amyl ketone | 128 | 2271 |
|  |  |  | 2-Ethylaniline | 153 | 2273 |
| Environmentally hazardous substance, solid, n.o.s. | 171 | 3077 | N-Ethylaniline | 153 | 2272 |
|  |  |  | Ethylbenzene | 130 | 1175 |
| Epibromohydrin | 131 | 2558 | N-Ethyl-N-benzylaniline | 153 | 2274 |
| Epichlorohydrin | 131P | 2023 | N-Ethylbenzyltoluidines, liquid | 153 | 2753 |
| 1,2-Epoxy-3-ethoxypropane | 127 | 2752 | N-Ethylbenzyltoluidines, solid | 153 | 3460 |
| Esters, n.o.s. | 127 | 3272 | Ethyl borate | 129 | 1176 |
| Ethane | 115 | 1035 | Ethyl bromide | 131 | 1891 |
| Ethane, compressed | 115 | 1035 | Ethyl bromoacetate | 155 | 1603 |
| Ethane, refrigerated liquid | 115 | 1961 | 2-Ethylbutanol | 129 | 2275 |
| Ethane-propane mixture, refrigerated liquid | 115 | 1961 | 2-Ethylbutyl acetate | 130 | 1177 |
| Ethanol | 127 | 1170 | Ethyl butyl ether | 127 | 1179 |
| Ethanol and gasoline mixture, with more than 10% ethanol | 127 | 3475 | 2-Ethylbutyraldehyde | 130 | 1178 |
|  |  |  | Ethyl butyrate | 130 | 1180 |
| Ethanol and motor spirit mixture, with more than 10% ethanol | 127 | 3475 | Ethyl chloride | 115 | 1037 |
|  |  |  | Ethyl chloroacetate | 155 | 1181 |
| Ethanol and petrol mixture, with more than 10% ethanol | 127 | 3475 | Ethyl chloroformate | 155 | 1182 |
|  |  |  | Ethyl 2-chloropropionate | 129 | 2935 |
| Ethanol, solution | 127 | 1170 | Ethyl chlorothioformate | 155 | 2826 |
| Ethanolamine | 153 | 2491 | Ethyl crotonate | 130 | 1862 |
| Ethanolamine, solution | 153 | 2491 | Ethyldichloroarsine | 151 | 1892 |
| Ethers, n.o.s. | 127 | 3271 | Ethyldichlorosilane | 139 | 1183 |
| Ethyl acetate | 129 | 1173 | Ethylene | 116P | 1962 |
| Ethylacetylene, stabilized | 116P | 2452 | Ethylene, acetylene and propylene mixture, refrigerated liquid containing at least 71.5% ethylene with not more than 22.5% acetylene and not more than 6% propylene | 115 | 3138 |
| Ethyl acrylate, stabilized | 129P | 1917 |  |  |  |
| Ethyl alcohol | 127 | 1170 |  |  |  |
| Ethyl alcohol, solution | 127 | 1170 |  |  |  |
| Ethylamine | 118 | 1036 |  |  |  |
|  |  |  | Ethylene, compressed | 116P | 1962 |

Page 110

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Ethylene, refrigerated liquid (cryogenic liquid)

115	1038

Ethylene oxide and propylene oxide mixture, with not more than 30% ethylene oxide

Ethylene oxide and tetrafluoroethane mixture, with not more than 5.6% ethylene oxide

131P 2983

126	3299

| Ethylene chlorohydrin | 131 | 1135 |
| --- | --- | --- |
| Ethylenediamine | 132 | 1604 |
| Ethylene dibromide | 154 | 1605 |
| Ethylene dichloride | 131 | 1184 |
| Ethylene glycol diethyl ether | 127 | 1153 |
| Ethylene glycol monoethyl | 127 | 1171 |

ether

Ethylene glycol monoethyl ether acetate

Ethylene glycol monomethyl ether

Ethylene glycol monomethyl

129	1172

|  | Ethylene oxide with nitrogen | 119P | 1040 |
| --- | --- | --- | --- |
|  | Ethyl ether | 127 | 1155 |
|  | Ethyl fluoride | 115 | 2453 |
|  | Ethyl formate | 129 | 1190 |
|  | Ethylhexaldehyde | 129 | 1191 |
|  | 2-Ethylhexylamine | 132 | 2276 |
|  | 2-Ethylhexyl chloroformate | 156 | 2748 |

127	1188

129	1189

| ether acetate |  |  | Ethyl isobutyrate | 129 | 2385 |
| --- | --- | --- | --- | --- | --- |
| Ethyleneimine, stabilized | 131P 1185 |  | Ethyl isocyanate | 155 | 2481 |
| Ethylene oxide | 119P 1040 |  | Ethyl lactate | 129 | 1192 |
| Ethylene oxide and carbon | 115 1041 |  | Ethyl mercaptan | 129 | 2363 |
| dioxide mixture, with more than 9% but not more than 87% ethylene oxide Ethylene oxide and carbon 119P 3300 dioxide mixture, with more than 87% ethylene oxide Ethylene oxide and carbon 126 1952 dioxide mixture, with not more than 9% ethylene oxide Ethylene oxide and 126 3297 chlorotetrafluoroethane mixture, with not more than 8.8% ethylene oxide Ethylene oxide and 126 3070 dichlorodifluoromethane |  | Ethyl methacrylate, stabilized 130P 2277 Ethyl methyl ether 115 1039 Ethyl methyl ketone 127 1193 Ethyl nitrite, solution 131 1194 Ethyl orthoformate 129 2524 Ethyl oxalate 156 2525 Ethylphenyldichlorosilane 156 2435 Ethyl phosphonothioic 154 2927 dichloride, anhydrous Ethyl phosphonous dichloride, 135 2845 anhydrous |  |  |  |

mixture, with not more than 12.5% ethylene oxide

Ethylene oxide and pentafluoroethane mixture, with not more than 7.9% ethylene oxide

126	3298

Ethyl phosphorodichloridate	154	2927

| 1-Ethylpiperidine | 132 | 2386 |
| --- | --- | --- |
| Ethyl propionate | 129 | 1195 |
| Ethyl propyl ether | 127 | 2615 |
| Ethyl silicate | 129 | 1292 |
| N-Ethyltoluidines | 153 | 2754 |

Page 111

| Name of Material G Ethyltrichlorosilane | uide No. 155 | ID No. 1196 |  | Name of Material G Fibres, animal or vegetable or synthetic, n.o.s. with oil | uide No. 133 | ID No. 1373 |
| --- | --- | --- | --- | --- | --- | --- |
| Explosives, division 1.1, 1.2, 1.3 or 1.5 | 112 | — — |  |  |  |  |
|  |  |  |  | Fibres, vegetable, dry | 133 | 3360 |
| Explosives, division 1.4 or 1.6 | 114 | — — |  | Fibres impregnated with weakly nitrated nitrocellulose, n.o.s. | 133 | 1353 |
| Extracts, aromatic, liquid | 127 | 1169 |  |  |  |  |
| Extracts, flavoring, liquid | 127 | 1197 |  | Films, nitrocellulose base | 133 | 1324 |
| Extracts, flavouring, liquid | 127 | 1197 |  | Fire extinguisher charges, corrosive liquid | 154 | 1774 |
| Extracts, liquid | 127 | 1197 |  | Fire extinguishers with compressed or liquefied gas | 126 | 1044 |
| Fabrics, animal or vegetable or synthetic, n.o.s. with oil | 133 | 1373 |  |  |  |  |
|  |  |  |  | Firelighters, solid, with flammable liquid | 133 | 2623 |
| Fabrics impregnated with weakly nitrated nitrocellulose, n.o.s. | 133 | 1353 |  |  |  |  |
|  |  |  |  | Fire suppressant dispersing devices | 171 | 3559 |
| Ferric arsenate | 151 | 1606 |  | First aid kit | 171 | 3316 |
| Ferric arsenite | 151 | 1607 |  | Fish meal, stabilized | 171 | 2216 |
| Ferric chloride, anhydrous | 157 | 1773 |  | Fish meal, unstabilized | 133 | 1374 |
| Ferric chloride, solution | 154 | 2582 |  | Fish scrap, stabilized | 171 | 2216 |
| Ferric nitrate | 140 | 1466 |  | Fish scrap, unstabilized | 133 | 1374 |
| Ferrocerium | 170 | 1323 |  | Flammable liquid, corrosive, n.o.s. | 132 | 2924 |
| Ferrosilicon | 139 | 1408 |  |  |  |  |
| Ferrous arsenate | 151 | 1608 |  | Flammable liquid, n.o.s. | 128 | 1993 |
| Ferrous chloride, solid | 154 | 1759 |  | Flammable liquid, poisonous, corrosive, n.o.s. | 131 | 3286 |
| Ferrous chloride, solution | 154 | 1760 |  | Flammable liquid, poisonous, n.o.s. | 131 | 1992 |
| Ferrous metal borings, shavings, turnings or cuttings | 170 | 2793 |  |  |  |  |
|  |  |  |  | Flammable liquid, toxic, corrosive, n.o.s. | 131 | 3286 |
| Fertilizer, ammoniating solution, with free ammonia | 125 | 1043 |  | Flammable liquid, toxic, n.o.s. | 131 | 1992 |
| Fibers, animal or vegetable, burnt, wet or damp | 133 | 1372 |  | Flammable solid, corrosive, inorganic, n.o.s. | 134 | 3180 |
| Fibers, animal or vegetable or synthetic, n.o.s. with oil | 133 | 1373 |  | Flammable solid, corrosive, organic, n.o.s. | 134 | 2925 |
| Fibers, vegetable, dry | 133 | 3360 |  | Flammable solid, inorganic, n.o.s. | 133 | 3178 |
| Fibers impregnated with weakly nitrated nitrocellulose, n.o.s. | 133 | 1353 |  | Flammable solid, organic, molten, n.o.s. | 133 | 3176 |
| Fibres, animal or vegetable, burnt, wet or damp | 133 | 1372 |  |  |  |  |

Page 112

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Flammable solid, organic, n.o.s.

Flammable solid, oxidizing, n.o.s.

133	1325

140	3097

Formic acid, with not less than 153	3412

10% but not more than 85% acid

Fuel, aviation, turbine engine  128	1863

Flammable solid, poisonous, inorganic, n.o.s.

Flammable solid, poisonous, organic, n.o.s.

Flammable solid, toxic, inorganic, n.o.s.

Flammable solid, toxic, organic, n.o.s.

134	3179

134	2926

134	3179

134	2926

Fuel cell cartridges contained in equipment, containing corrosive substances

Fuel cell cartridges contained in equipment, containing flammable liquids

153	3477

128	3473

| Fluorosulfonic acid | 137 | 1777 |
| --- | --- | --- |
| Fluorosulphonic acid | 137 | 1777 |

Fluorotoluenes	130	2388

water-reactive substances

| Fuel cell cartridges contained in equipment, containing hydrogen in metal hydride | 115 | 3479 |
| --- | --- | --- |
| Fuel cell cartridges contained in equipment, containing liquefied flammable gas | 115 | 3478 |
| Fuel cell cartridges contained in equipment, containing | 138 | 3476 |

| Fluorine, compressed | 124 | 1045 |
| --- | --- | --- |
| Fluoroacetic acid | 154 | 2642 |
| Fluoroanilines | 153 | 2941 |
| Fluorobenzene | 130 | 2387 |
| Fluoroboric acid | 154 | 1775 |
| Fluorophosphoric acid, | 154 | 1776 |
| anhydrous Fluorosilicates, n.o.s. | 151 | 2856 |
| Fluorosilicic acid | 154 | 1778 |

Fuel cell cartridges, containing 153	3477

corrosive substances

Fuel cell cartridges, containing 128	3473

flammable liquids

Fuel cell cartridges, containing 115	3479

hydrogen in metal hydride

Fuel cell cartridges, containing 115	3478

liquefied flammable gas

Fuel cell cartridges, containing 138	3476

Formaldehyde, solution (corrosive)

Formaldehyde, solution, flammable

153	2209

132	1198

water-reactive substances

Fuel cell cartridges packed with equipment, containing corrosive substances

153	3477

Formalin (corrosive)	153	2209

Formalin (flammable)	132	1198

Formic acid	153	1779

Formic acid, with more than	153	1779

85% acid

Fuel cell cartridges packed with equipment, containing flammable liquids

Fuel cell cartridges packed with equipment, containing hydrogen in metal hydride

128	3473

115	3479

Formic acid, with not less than 153	3412

5% but less than 10% acid

Fuel cell cartridges packed with equipment, containing liquefied flammable gas

115	3478

Page 113

| Name of Material G Fuel cell cartridges packed with equipment, containing water-reactive substances | uide No. 138 | ID No. 3476 |  | Name of Material G Gas sample, non-pressurized, toxic, n.o.s., not refrigerated liquid | uide No. 123 | ID No. 3169 |
| --- | --- | --- | --- | --- | --- | --- |
| Fuel oil | 128 | 1993 |  | Genetically modified micro- organisms | 171 | 3245 |
| Fumaryl chloride | 156 | 1780 |  |  |  |  |
| Fumigated cargo transport unit | 171 | 3359 |  | Genetically modified organisms | 171 | 3245 |
| Furaldehydes | 153P | 1199 |  | Germane | 119 | 2192 |
| Furan | 128 | 2389 |  | Germane, adsorbed | 173 | 3523 |
| Furfuryl alcohol | 153 | 2874 |  | Glycerol alpha- monochlorohydrin | 153 | 2689 |
| Furfurylamine | 132 | 2526 |  | Glycidaldehyde | 131P | 2622 |
| Fusee (railway or highway) | 133 | 1325 |  | Guanidine nitrate | 143 | 1467 |
| Fusel oil | 127 | 1201 |  | Hafnium powder, dry | 135 | 2545 |
| Gallium | 172 | 2803 |  | Hafnium powder, wetted with not less than 25% water | 170 | 1326 |
| Gallium contained in manufactured articles | 172 | 3554 |  |  |  |  |
|  |  |  |  | Halogenated monomethyldiphenylmethanes, liquid | 171 | 3151 |
| Gas, refrigerated liquid, flammable, n.o.s. | 115 | 3312 |  |  |  |  |
| Gas, refrigerated liquid, n.o.s. | 120 | 3158 |  | Halogenated monomethyldiphenylmethanes, solid | 171 | 3152 |
| Gas, refrigerated liquid, oxidizing, n.o.s. | 122 | 3311 |  |  |  |  |
|  |  |  |  | Hay, wet, damp or contaminated with oil | 133 | 1327 |
| Gas cartridges | 115 | 2037 |  |  |  |  |
| Gas identification set | 123 | 9035 |  | Hazardous waste, liquid, n.o.s. | 171 | 3082 |
| Gas oil | 128 | 1202 |  | Hazardous waste, solid, n.o.s. | 171 | 3077 |
| Gasoline | 128 | 1203 |  | Heating oil, light | 128 | 1202 |
| Gas sample, non-pressurized, flammable, n.o.s., not refrigerated liquid | 115 | 3167 |  | Helium, compressed | 120 | 1046 |
|  |  |  |  | Helium, refrigerated liquid (cryogenic liquid) | 120 | 1963 |
| Gas sample, non-pressurized, poisonous, flammable, n.o.s., not refrigerated liquid | 119 | 3168 |  | Heptafluoropropane | 126 | 3296 |
|  |  |  |  | n-Heptaldehyde | 129 | 3056 |
| Gas sample, non-pressurized, poisonous, n.o.s., not refrigerated liquid | 123 | 3169 |  | Heptanes | 128 | 1206 |
|  |  |  |  | n-Heptene | 128 | 2278 |
| Gas sample, non-pressurized, toxic, flammable, n.o.s., not refrigerated liquid | 119 | 3168 |  | Hexachloroacetone | 153 | 2661 |
|  |  |  |  | Hexachlorobenzene | 152 | 2729 |
|  |  |  |  | Hexachlorobutadiene | 151 | 2279 |
|  |  |  |  | Hexachlorocyclopentadiene | 151 | 2646 |

Page 114

| Name of Material Guide No. |  | ID No. | Name of Material Guide No. |  |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hexachlorophene | 151 156 130 151 123 125 151 151 126 154 126 126 130 153 153 156 132 133 128 153 129 128 156 132 132 153 | 2875 1781 2458 1611 1612 2420 2552 3436 2193 1782 1858 1858 1207 2280 1783 2281 2493 1328 1208 2829 2282 2370 1784 2029 3484 2030 |  | Hydrazine, aqueous solution, with not more than 37% hydrazine Hydriodic acid Hydrobromic acid Hydrocarbon gas mixture, compressed, n.o.s. Hydrocarbon gas mixture, liquefied, n.o.s. Hydrocarbon gas refills for small devices, with release device Hydrocarbons, liquid, n.o.s. Hydrochloric acid Hydrocyanic acid, aqueous solution, with less than 5% hydrogen cyanide | 153 | 3293 |  |
| Hexadecyltrichlorosilane |  |  |  |  |  |  |  |
| Hexadiene |  |  |  |  | 154 | 1787 |  |
| Hexaethyl tetraphosphate |  |  |  |  | 154 | 1788 |  |
| Hexaethyl tetraphosphate and compressed gas mixture |  |  |  |  | 115 | 1964 |  |
| Hexafluoroacetone |  |  |  |  | 115 | 1965 |  |
| Hexafluoroacetone hydrate, |  |  |  |  |  |  |  |
| liquid |  |  |  |  | 115 | 3150 |  |
| Hexafluoroacetone hydrate, |  |  |  |  |  |  |  |
| solid |  |  |  |  |  |  |  |
| Hexafluoroethane |  |  |  |  | 128 | 3295 |  |
| Hexafluorophosphoric acid |  |  |  |  | 157 | 1789 |  |
| Hexafluoropropylene |  |  |  |  | 154 | 1613 |  |
| Hexafluoropropylene, |  |  |  |  |  |  |  |
| compressed Hexaldehyde |  |  |  |  |  |  |  |
|  |  |  |  | Hydrocyanic acid, aqueous solution, with not more than 20% hydrogen cyanide | 154 | 1613 |  |
| Hexamethylenediamine, solid Hexamethylenediamine, |  |  |  |  |  |  |  |
|  |  |  |  | Hydrofluoric acid Hydrofluoric acid and sulfuric acid mixture Hydrofluoric acid and sulphuric acid mixture Hydrofluorosilicic acid Hydrogen, compressed Hydrogen in a metal hydride storage system Hydrogen in a metal hydride storage system contained in equipment Hydrogen in a metal hydride storage system packed with equipment Hydrogen, refrigerated liquid (cryogenic liquid) Hydrogen and methane mixture, compressed | 157 | 1790 |  |
| solution |  |  |  |  | 157 | 1786 |  |
| Hexamethylene diisocyanate Hexamethyleneimine |  |  |  |  | 157 | 1786 |  |
| Hexamethylenetetramine |  |  |  |  | 154 | 1778 |  |
| Hexanes |  |  |  |  | 115 | 1049 |  |
| Hexanoic acid |  |  |  |  | 115 | 3468 |  |
| Hexanols |  |  |  |  |  |  |  |
| 1-Hexene |  |  |  |  | 115 | 3468 |  |
| Hexyltrichlorosilane |  |  |  |  |  |  |  |
| Hydrazine, anhydrous |  |  |  |  | 115 | 3468 |  |
| Hydrazine aqueous solution, |  |  |  |  |  |  |  |
| flammable, with more than |  |  |  |  |  |  |  |
| 37% hydrazine |  |  |  |  | 115 | 1966 |  |
| Hydrazine, aqueous solution, |  |  |  |  |  |  |  |
| with more than 37% hydrazine |  |  |  |  | 115 | 2034 |  |
|  |  |  |  | Hydrogen bromide, anhydrous | 125 | 1048 |  |

Page 115

| Name of Material G Hydrogen chloride, anhydrous | uide No. 125 | ID No. 1050 | Name of Material G 1-Hydroxybenzotriazole, monohydrate | uide No. 113 | ID No. 3474 |
| --- | --- | --- | --- | --- | --- |
| Hydrogen chloride, refrigerated liquid | 125 | 2186 |  |  |  |
|  |  |  | Hydroxylamine sulfate | 154 | 2865 |
| Hydrogen cyanide, aqueous solution, with not more than 20% hydrogen cyanide | 154 | 1613 | Hydroxylamine sulphate | 154 | 2865 |
|  |  |  | Hypochlorite solution | 154 | 1791 |
| Hydrogen cyanide, solution in alcohol, with not more than 45% hydrogen cyanide | 131 | 3294 | Hypochlorites, inorganic, n.o.s. | 140 | 3212 |
|  |  |  | 3,3'-Iminodipropylamine | 153 | 2269 |
| Hydrogen cyanide, stabilized | 117P | 1051 | Infectious substance, affecting animals only | 158 | 2900 |
| Hydrogen cyanide, stabilized (absorbed) | 152 | 1614 | Infectious substance, affecting humans | 158 | 2814 |
| Hydrogendifluorides, solid, n.o.s. | 154 | 1740 | Insecticide gas, flammable, n.o.s. | 115 | 3354 |
| Hydrogendifluorides, solution, n.o.s. | 154 | 3471 | Insecticide gas, n.o.s. | 126 | 1968 |
| Hydrogen fluoride, anhydrous | 125 | 1052 | Insecticide gas, poisonous, flammable, n.o.s. | 119 | 3355 |
| Hydrogen iodide, anhydrous | 125 | 2197 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 119 | 3355 |
| Hydrogen peroxide, aqueous solution, stabilized, with more than 60% hydrogen peroxide | 143 | 2015 |  |  |  |
|  |  |  | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 3355 |
| Hydrogen peroxide, aqueous solution, with not less than 8% but less than 20% hydrogen peroxide | 140 | 2984 |  |  |  |
|  |  |  | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 3355 |
| Hydrogen peroxide, aqueous solution, with not less than 20% but not more than 60% hydrogen peroxide (stabilized as necessary) | 140 | 2014 | Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 119 | 3355 |
|  |  |  | Insecticide gas, poisonous, n.o.s. | 123 | 1967 |
| Hydrogen peroxide, stabilized | 143 | 2015 |  |  |  |
| Hydrogen peroxide and peroxyacetic acid mixture, with acid(s), water and not more than 5% peroxyacetic acid, stabilized | 140 | 3149 | Insecticide gas, toxic, flammable, n.o.s. | 119 | 3355 |
|  |  |  | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) | 119 | 3355 |
| Hydrogen selenide, adsorbed | 173 | 3526 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 3355 |
| Hydrogen selenide, anhydrous | 117 | 2202 |  |  |  |
| Hydrogen sulfide | 117 | 1053 | Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 3355 |
| Hydrogen sulphide | 117 | 1053 |  |  |  |

Page 116

| Name of Material Guide No. |  | ID No. |  | Name of Material Guide No. |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 119 123 154 157 157 144 129 129 129 135 136 135 115 115 129 129 129P 129 130 132 115 115 129 130 155P 130P 129 130 132 131 | 3355 1967 3495 3498 1792 2495 2390 2391 2392 1376 1994 1376 1075 1969 1212 1213 2527 1212 2045 1214 1055 1075 2393 2528 2486 2283 2394 2045 2529 2284 |  |  |  |  |  |
| Insecticide gas, toxic, |  |  |  | Isobutyryl chloride | 155 | 2395 |  |
| flammable, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |
|  |  |  |  | Isocyanate solution, flammable, poisonous, n.o.s. | 155 | 2478 |  |
| Insecticide gas, toxic, n.o.s. Iodine |  |  |  |  |  |  |  |
|  |  |  |  | Isocyanate solution, flammable, toxic, n.o.s. | 155 | 2478 |  |
| Iodine monochloride, liquid |  |  |  | Isocyanate solution, poisonous, flammable, n.o.s. Isocyanate solution, poisonous, n.o.s. Isocyanate solution, toxic, flammable, n.o.s. Isocyanate solution, toxic, n.o.s. | 155 | 3080 |  |
| Iodine monochloride, solid Iodine pentafluoride |  |  |  |  | 156 | 2206 |  |
| 2-Iodobutane |  |  |  |  | 155 | 3080 |  |
| Iodomethylpropanes |  |  |  |  |  |  |  |
| Iodopropanes |  |  |  |  | 156 | 2206 |  |
| Iron oxide, spent |  |  |  |  |  |  |  |
|  |  |  |  | Isocyanates, flammable, poisonous, n.o.s. | 155 | 2478 |  |
| Iron pentacarbonyl |  |  |  |  |  |  |  |
| Iron sponge, spent |  |  |  | Isocyanates, flammable, toxic, n.o.s. | 155 | 2478 |  |
| Isobutane |  |  |  |  |  |  |  |
|  |  |  | Isocyanates, poisonous, 155 3080 flammable, n.o.s. Isocyanates, poisonous, n.o.s. 156 2206 Isocyanates, toxic, flammable, 155 3080 n.o.s. Isocyanates, toxic, n.o.s. 156 2206 |  |  |  |  |
| Isobutane |  |  |  |  |  |  |  |
| Isobutanol |  |  |  |  |  |  |  |
| Isobutyl acetate |  |  |  |  |  |  |  |
| Isobutyl acrylate, stabilized |  |  |  |  |  |  |  |
| Isobutyl alcohol |  |  |  |  |  |  |  |
|  |  |  |  | Isocyanatobenzotrifluorides | 155 | 2285 |  |
| Isobutyl aldehyde |  |  |  |  |  |  |  |
| Isobutylamine |  |  |  | Isoheptenes | 128 | 2287 |  |
| Isobutylene |  |  |  | Isohexenes | 128 | 2288 |  |
| Isobutylene |  |  |  | Isooctane | 128 | 1262 |  |
| Isobutyl formate |  |  |  | Isooctenes | 128 | 1216 |  |
| Isobutyl isobutyrate |  |  |  | Isopentane | 128 | 1265 |  |
| Isobutyl isocyanate |  |  |  | Isopentenes | 128 | 2371 |  |
| Isobutyl methacrylate, |  |  |  | Isophoronediamine | 153 | 2289 |  |
| stabilized |  |  |  | Isophorone diisocyanate | 156 | 2290 |  |
| Isobutyl propionate |  |  |  | Isoprene, stabilized | 130P | 1218 |  |
| Isobutyraldehyde |  |  |  | Isopropanol | 129 | 1219 |  |
| Isobutyric acid |  |  |  | Isopropenyl acetate | 129P | 2403 |  |
| Isobutyronitrile |  |  |  | Isopropenylbenzene | 128 | 2303 |  |

Page 117

|  | Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Isopropyl acetate | 129 | 1220 |  |  | Lead sulphate, with more than 3% free acid | 154 | 1794 |
|  | Isopropyl acid phosphate | 153 | 1793 |  |  |  |  |  |
|  | Isopropyl alcohol | 129 | 1219 |  |  | Life-saving appliances, not self-inflating | 171 | 3072 |
|  | Isopropylamine | 132 | 1221 |  |  | Life-saving appliances, self- inflating | 171 | 2990 |
|  | Isopropylbenzene | 130 | 1918 |  |  |  |  |  |
|  | Isopropyl butyrate | 129 | 2405 |  |  | Lighter refills containing flammable gas | 115 | 1057 |
|  | Isopropyl chloroacetate | 127 | 2947 |  |  | Lighters containing flammable gas | 115 | 1057 |
|  | Isopropyl chloroformate | 155 | 2407 |  |  |  |  |  |
|  | Isopropyl 2-chloropropionate | 129 | 2934 |  |  | Lighters, non-pressurized, containing flammable liquid | 128 | 1057 |
|  | Isopropyl isobutyrate | 127 | 2406 |  |  |  |  |  |
|  |  |  |  |  |  | Liquefied gas, flammable, n.o.s. | 115 | 3161 |
|  | Isopropyl isocyanate | 155P | 2483 |  |  |  |  |  |
|  | Isopropyl nitrate | 130 | 1222 |  |  | Liquefied gas, n.o.s. | 126 | 3163 |
|  | Isopropyl propionate | 129 | 2409 |  |  | Liquefied gas, oxidizing, n.o.s. | 122 | 3157 |
|  | Isosorbide dinitrate mixture | 133 | 2907 |  |  | Liquefied gas, poisonous, corrosive, n.o.s. | 125 | 3308 |
|  | Isosorbide-5-mononitrate | 133 | 3251 |  |  |  |  |  |
|  | Kerosene | 128 | 1223 |  |  | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 125 | 3308 |
|  | Ketones, liquid, n.o.s. | 127 | 1224 |  |  |  |  |  |
|  | Krill meal | 133 | 3497 |  |  | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 125 | 3308 |
|  | Krypton, compressed | 120 | 1056 |  |  |  |  |  |
|  | Krypton, refrigerated liquid (cryogenic liquid) | 120 | 1970 |  |  | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 125 | 3308 |
|  | Lead acetate | 151 | 1616 |  |  |  |  |  |
|  | Lead arsenates | 151 | 1617 |  |  | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 125 | 3308 |
|  | Lead arsenites | 151 | 1618 |  |  |  |  |  |
|  | Lead compound, soluble, n.o.s. | 151 | 2291 |  |  | Liquefied gas, poisonous, flammable, corrosive, n.o.s. | 119 | 3309 |
|  | Lead cyanide | 151 | 1620 |  |  |  |  |  |
|  |  |  |  |  |  | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 119 | 3309 |
|  | Lead dioxide | 140 | 1872 |  |  |  |  |  |
|  | Lead nitrate | 141 | 1469 |  |  |  |  |  |
|  | Lead perchlorate, solid | 141 | 1470 |  |  | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 119 | 3309 |
|  | Lead perchlorate, solution | 141 | 3408 |  |  |  |  |  |
|  | Lead phosphite, dibasic | 133 | 2989 |  |  | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 119 | 3309 |
|  | Lead sulfate, with more than 3% free acid | 154 | 1794 |  |  |  |  |  |

Page 118

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 119 | 3309 |  |  | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 124 | 3310 |  |
| Liquefied gas, poisonous, flammable, n.o.s. | 119 | 3160 |  |  | Liquefied gas, poisonous, oxidizing, n.o.s. | 124 | 3307 |  |
| Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 119 | 3160 |  |  | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 124 | 3307 |  |
| Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 3160 |  |  | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 124 | 3307 |  |
| Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 3160 |  |  | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 124 | 3307 |  |
| Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 119 | 3160 |  |  | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 124 | 3307 |  |
| Liquefied gas, poisonous, n.o.s. | 123 | 3162 |  |  | Liquefied gas, toxic, corrosive, n.o.s. | 125 | 3308 |  |
| Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone A) | 123 | 3162 |  |  | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) | 125 | 3308 |  |
| Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone B) | 123 | 3162 |  |  | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) | 125 | 3308 |  |
| Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone C) | 123 | 3162 |  |  | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) | 125 | 3308 |  |
| Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone D) | 123 | 3162 |  |  | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) | 125 | 3308 |  |
| Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. | 124 | 3310 |  |  | Liquefied gas, toxic, flammable, corrosive, n.o.s. | 119 | 3309 |  |
| Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 124 | 3310 |  |  | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 119 | 3309 |  |
| Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 124 | 3310 |  |  | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 119 | 3309 |  |
| Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 124 | 3310 |  |  | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) | 119 | 3309 |  |

Page 119

| Name of Material G Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | uide No. 119 | ID No. 3309 | Name of Material G Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) | uide No. 124 | ID No. 3307 |
| --- | --- | --- | --- | --- | --- |
| Liquefied gas, toxic, flammable, n.o.s. | 119 | 3160 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 124 | 3307 |
| Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) | 119 | 3160 |  |  |  |
|  |  |  | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) | 124 | 3307 |
| Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 119 | 3160 |  |  |  |
|  |  |  | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 124 | 3307 |
| Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) | 119 | 3160 |  |  |  |
|  |  |  | Liquefied gases, non- flammable, charged with nitrogen, carbon dioxide or air | 120 | 1058 |
| Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 119 | 3160 |  |  |  |
|  |  |  | Liquefied natural gas (cryogenic liquid) | 115 | 1972 |
| Liquefied gas, toxic, n.o.s. | 123 | 3162 |  |  |  |
| Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone A) | 123 | 3162 | Liquefied petroleum gas | 115 | 1075 |
|  |  |  | Lithium | 138 | 1415 |
| Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone B) | 123 | 3162 | Lithium aluminum hydride | 138 | 1410 |
|  |  |  | Lithium aluminum hydride, ethereal | 138 | 1411 |
| Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone C) | 123 | 3162 |  |  |  |
|  |  |  | Lithium batteries installed in cargo transport unit (lithium ion batteries) | 147 | 3536 |
| Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone D) | 123 | 3162 |  |  |  |
| Liquefied gas, toxic, oxidizing, corrosive, n.o.s. | 124 | 3310 | Lithium batteries installed in cargo transport unit (lithium metal batteries) | 138 | 3536 |
| Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 124 | 3310 |  |  |  |
|  |  |  | Lithium borohydride | 138 | 1413 |
|  |  |  | Lithium ferrosilicon | 139 | 2830 |
| Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 124 | 3310 | Lithium hydride | 138 | 1414 |
|  |  |  | Lithium hydride, fused solid | 138 | 2805 |
| Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 124 | 3310 | Lithium hydroxide | 154 | 2680 |
|  |  |  | Lithium hydroxide, solution | 154 | 2679 |
| Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 124 | 3310 | Lithium hypochlorite, dry | 140 | 1471 |
|  |  |  | Lithium hypochlorite mixture | 140 | 1471 |
| Liquefied gas, toxic, oxidizing, n.o.s. | 124 | 3307 | Lithium ion batteries (including lithium ion polymer batteries) | 147 | 3480 |

Page 120

| Name of Material | Guide No. | ID No. | Name of Material |  | Guide No. | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lithium ion batteries contained 147 |  | 3481 |  | Magnesium alloys, with more than 50% magnesium, in pellets, turnings or ribbons Magnesium alloys powder | 138 | 1869 |  |
| in equipment (including |  |  |  |  |  |  |  |
| lithium ion polymer |  |  |  |  |  |  |  |
| batteries) |  |  |  |  | 138 | 1418 |  |
| Lithium ion batteries packed with equipment (including | 147 | 3481 |  |  |  |  |  |
|  |  |  |  | Magnesium aluminum phosphide | 139 | 1419 |  |
| lithium ion polymer |  |  |  |  |  |  |  |
|  |  |  |  | Magnesium arsenate | 151 | 1622 |  |
| batteries) |  |  |  |  |  |  |  |
| Lithium metal batteries (including lithium alloy batteries) | 138 | 3090 |  | Magnesium bromate Magnesium chlorate | 140 140 | 1473 2723 |  |
| Lithium metal batteries | 138 | 3091 |  | Magnesium diamide | 135 | 2004 |  |
| contained in equipment (including lithium alloy |  |  |  |  |  |  |  |
|  |  |  |  | Magnesium fluorosilicate | 151 | 2853 |  |
| batteries) |  |  |  | Magnesium granules, coated | 138 | 2950 |  |
| Lithium metal batteries packed 138 with equipment (including lithium alloy batteries) |  | 3091 |  | Magnesium hydride Magnesium nitrate | 138 140 | 2010 1474 |  |
| Lithium nitrate | 140 | 2722 |  | Magnesium perchlorate | 140 | 1475 |  |
| Lithium nitride | 139 | 2806 |  | Magnesium peroxide | 140 | 1476 |  |
| Lithium peroxide | 143 | 1472 |  | Magnesium phosphide | 139 | 2011 |  |
| Lithium silicon | 138 | 1417 |  | Magnesium powder | 138 | 1418 |  |
| LNG (cryogenic liquid) | 115 | 1972 |  | Magnesium silicide | 138 | 2624 |  |
| London purple | 151 | 1621 |  | Magnetized material | 171 | 2807 |  |
| LPG | 115 | 1075 |  | Maleic anhydride | 156 | 2215 |  |
| Machinery, fuel cell, flammable 115 gas powered Machinery, fuel cell, flammable 128 liquid powered |  | 3529 3528 |  | Maleic anhydride, molten Malononitrile Maneb | 156 153 135 | 2215 2647 2210 |  |
| Machinery, internal combustion 171 |  | 3530 |  | Maneb, stabilized | 135 | 2968 |  |
| Machinery, internal | 115 | 3529 |  | Maneb preparation, stabilized Maneb preparation, with not less than 60% maneb | 135 | 2968 |  |
| combustion, flammable gas powered |  |  |  |  | 135 | 2210 |  |
| Machinery, internal combustion, flammable | 128 | 3528 |  | Manganese nitrate | 140 | 2724 |  |
| liquid powered |  |  |  | Manganese resinate | 133 | 1330 |  |
| Magnesium | 138 | 1869 |  | Matches, fusee | 133 | 2254 |  |
| Magnesium, in pellets, turnings 138 or ribbons |  | 1869 |  | Matches, safety Matches, "strike anywhere" | 133 133 | 1944 1331 |  |
|  |  |  |  | Matches, wax "vesta" | 133 | 1945 |  |

Page 121

| Name of Material G Medical waste, category A, affecting animals only, solid | uide No. 158 | ID No. 3549 | Name of Material G Mercurous nitrate | uide No. 141 | ID No. 1627 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Mercury | 172 | 2809 |
| Medical waste, category A, affecting humans, solid | 158 | 3549 | Mercury acetate | 151 | 1629 |
| Medical waste, n.o.s. | 158 | 3291 | Mercury ammonium chloride | 151 | 1630 |
| Medicine, liquid, flammable, poisonous, n.o.s. | 131 | 3248 | Mercury based pesticide, liquid, flammable, poisonous | 131 | 2778 |
| Medicine, liquid, flammable, toxic, n.o.s. | 131 | 3248 | Mercury based pesticide, liquid, flammable, toxic | 131 | 2778 |
| Medicine, liquid, poisonous, n.o.s. | 151 | 1851 | Mercury based pesticide, liquid, poisonous | 151 | 3012 |
| Medicine, liquid, toxic, n.o.s. | 151 | 1851 | Mercury based pesticide, liquid, poisonous, flammable | 131 | 3011 |
| Medicine, solid, poisonous, n.o.s. | 151 | 3249 | Mercury based pesticide, liquid, toxic | 151 | 3012 |
| Medicine, solid, toxic, n.o.s. | 151 | 3249 | Mercury based pesticide, liquid, toxic, flammable | 131 | 3011 |
| Mercaptan mixture, liquid, flammable, n.o.s. | 130 | 3336 |  |  |  |
|  |  |  | Mercury based pesticide, solid, poisonous | 151 | 2777 |
| Mercaptan mixture, liquid, flammable, poisonous, n.o.s. | 131 | 1228 |  |  |  |
|  |  |  | Mercury based pesticide, solid, toxic | 151 | 2777 |
| Mercaptan mixture, liquid, flammable, toxic, n.o.s. | 131 | 1228 |  |  |  |
|  |  |  | Mercury benzoate | 154 | 1631 |
| Mercaptan mixture, liquid, poisonous, flammable, n.o.s. | 131 | 3071 | Mercury bromides | 154 | 1634 |
| Mercaptan mixture, liquid, toxic, flammable, n.o.s. | 131 | 3071 | Mercury compound, liquid, n.o.s. | 151 | 2024 |
| Mercaptans, liquid, flammable, n.o.s. | 130 | 3336 | Mercury compound, solid, n.o.s. | 151 | 2025 |
| Mercaptans, liquid, flammable, poisonous, n.o.s. | 131 | 1228 | Mercury contained in manufactured articles | 172 | 3506 |
| Mercaptans, liquid, flammable, toxic, n.o.s. | 131 | 1228 | Mercury cyanide | 154 | 1636 |
|  |  |  | Mercury gluconate | 151 | 1637 |
| Mercaptans, liquid, poisonous, flammable, n.o.s. | 131 | 3071 | Mercury iodide | 151 | 1638 |
| Mercaptans, liquid, toxic, flammable, n.o.s. | 131 | 3071 | Mercury nucleate | 151 | 1639 |
|  |  |  | Mercury oleate | 151 | 1640 |
| Mercuric arsenate | 151 | 1623 | Mercury oxide | 151 | 1641 |
| Mercuric chloride | 154 | 1624 | Mercury oxycyanide, desensitized | 151 | 1642 |
| Mercuric nitrate | 141 | 1625 |  |  |  |
| Mercuric potassium cyanide | 157 | 1626 | Mercury potassium iodide | 151 | 1643 |

Page 122

| Name of Material Guide No. | ID No. | Name of Material Guide No. |  |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Mercury salicylate 151 | 1644 |  | 4-Methoxy-4-methylpentan- 2-one 1-Methoxy-2-propanol Methyl acetate Methylacetylene and propadiene mixture, stabilized Methyl acrylate, stabilized Methylal Methyl alcohol Methylallyl chloride | 128 129 129 116P 129P 127 131 130P | 2293 3092 1231 1060 1919 1234 1230 2554 |  |
| Mercury sulfate 151 | 1645 |  |  |  |  |  |
| Mercury sulphate 151 | 1645 |  |  |  |  |  |
| Mercury thiocyanate 151 | 1646 |  |  |  |  |  |
| Mesityl oxide 129 | 1229 |  |  |  |  |  |
| Metal carbonyls, liquid, n.o.s. 151 | 3281 |  |  |  |  |  |
| Metal carbonyls, solid, n.o.s. 151 | 3466 |  |  |  |  |  |
| Metal catalyst, dry 135 | 2881 |  |  |  |  |  |
| Metal catalyst, wetted 170 | 1378 |  |  |  |  |  |
| Metaldehyde 133 | 1332 |  |  |  |  |  |
| Metal hydrides, flammable, 170 n.o.s. | 3182 |  | Methylamine, anhydrous | 118 | 1061 |  |
|  |  |  | Methylamine, aqueous solution Methylamyl acetate Methyl amyl ketone N-Methylaniline Methylbenzyl (alpha) alcohol, liquid Methylbenzyl (alpha) alcohol, solid | 132 130 127 153 153 153 | 1235 1233 1110 2294 2937 3438 |  |
| Metal hydrides, water-reactive, 138 n.o.s. | 1409 |  |  |  |  |  |
| Metallic substance, water- 138 reactive, n.o.s. | 3208 |  |  |  |  |  |
| Metallic substance, water- 138 reactive, self-heating, n.o.s. | 3209 |  |  |  |  |  |
| Metal powder, flammable, 170 n.o.s. | 3089 |  |  |  |  |  |
| Metal powder, self-heating, 135 n.o.s. | 3189 |  | Methyl bromide | 123 | 1062 |  |
|  |  |  | Methyl bromide and ethylene dibromide mixture, liquid | 151 | 1647 |  |
| Metal salts of organic 133 compounds, flammable, n.o.s. | 3181 |  |  |  |  |  |
|  |  |  | Methyl bromoacetate 2-Methylbutanal 3-Methylbutan-2-one 2-Methyl-1-butene 2-Methyl-2-butene 3-Methyl-1-butene N-Methylbutylamine Methyl tert-butyl ether Methyl butyrate Methyl chloride Methyl chloride and methylene chloride mixture | 153 129 127 128 128 128 132 127 129 115 115 | 2643 3371 2397 2459 2460 2561 2945 2398 1237 1063 1912 |  |
| Methacrylaldehyde, stabilized 131P | 2396 |  |  |  |  |  |
| Methacrylic acid, stabilized 153P | 2531 |  |  |  |  |  |
| Methacrylonitrile, stabilized 131P | 3079 |  |  |  |  |  |
| Methallyl alcohol 129 | 2614 |  |  |  |  |  |
| Methane, compressed 115 | 1971 |  |  |  |  |  |
| Methane, refrigerated liquid 115 (cryogenic liquid) | 1972 |  |  |  |  |  |
| Methanesulfonyl chloride 156 | 3246 |  |  |  |  |  |
| Methanesulphonyl chloride 156 | 3246 |  |  |  |  |  |
| Methanol 131 | 1230 |  |  |  |  |  |
| Methoxymethyl isocyanate 155 | 2605 |  |  |  |  |  |

Page 123

Name of Material

Guide		ID	Name of Material No.	No.

Guide	ID

No.	No.

|  | Methyl chloroacetate 131 2295 |  | Methyl methacrylate monomer, 129P 1247 stabilized 4-Methylmorpholine 132 2535 N-Methylmorpholine 132 2535 Methyl nitrite 116 2455 Methyl orthosilicate 155 2606 Methylpentadiene 128 2461 2-Methylpentan-2-ol 129 2560 |  |
| --- | --- | --- | --- | --- |
|  | Methyl chloroformate 155 1238 |  |  |  |
|  | Methyl chloromethyl ether 131 1239 |  |  |  |
|  | Methyl 2-chloropropionate 129 2933 |  |  |  |
|  | Methylchlorosilane 119 2534 |  |  |  |
|  | Methylcyclohexane 128 2296 Methylcyclohexanols 129 2617 Methylcyclohexanone 128 2297 |  |  |  |
|  | Methylcyclopentane 128 2298 Methyl dichloroacetate 156 2299 |  |  | Methylphenyldichlorosilane 156 2437 |
|  |  |  |  | Methyl phosphonic dichloride 137 9206 |
|  | Methyldichloroarsine 152 1556 |  |  | Methyl phosphonous dichloride 135 2845 |
|  | Methyldichlorosilane 139 1242 |  |  | 1-Methylpiperidine 132 2399 |
|  | Methylene chloride 160 1593 Methyl ethyl ether 115 1039 Methyl ethyl ketone 127 1193 2-Methyl-5-ethylpyridine 153 2300 Methyl fluoride 115 2454 Methyl formate 129 1243 2-Methylfuran 128 2301 |  | Methyl propionate 129 1248 Methyl propyl ether 127 2612 Methyl propyl ketone 127 1249 Methyltetrahydrofuran 127 2536 Methyl trichloroacetate 156 2533 Methyltrichlorosilane 155 1250 Methyl valeraldehyde (alpha) 130 2367 Methyl vinyl ketone, stabilized 131P 1251 Molten sulfur 133 2448 Molten sulphur 133 2448 Molybdenum pentachloride 156 2508 Monoethanolamine 153 2491 Mononitrotoluidines 153 2660 Morpholine 132 2054 Motor fuel anti-knock mixture 152 1649 Motor fuel anti-knock mixture, 131 3483 flammable Motor spirit 128 1203 Muriatic acid 157 1789 Musk xylene 149 2956 Naphthalene, crude 133 1334 |  |
|  | 2-Methyl-2-heptanethiol 131 3023 |  |  |  |
|  | 5-Methylhexan-2-one 127 2302 |  |  |  |
|  | Methylhydrazine 131 1244 |  |  |  |
|  | Methyl iodide 151 2644 |  |  |  |
|  | Methyl isobutyl carbinol 129 2053 Methyl isobutyl ketone 127 1245 |  |  |  |
|  | Methyl isocyanate 155P 2480 |  |  |  |
|  | Methyl isopropenyl ketone, 127P 1246 stabilized |  |  |  |
|  | Methyl isothiocyanate 131 2477 |  |  |  |
|  | Methyl isovalerate 130 2400 Methyl magnesium bromide in 138 1928 ethyl ether |  |  |  |
|  | Methyl mercaptan 117 1064 |  |  |  |

Page 124

| Name of Material Guide No. | ID No. | Name of Material Guide No. |  |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Naphthalene, molten 133 | 2304 1334 2077 1650 3411 1651 1652 1971 1972 1208 1065 1913 1259 2881 1653 2725 2726 1654 3144 1655 1656 3444 1656 3144 1655 1657 3445 1658 |  | Nicotine sulphate, solid Nicotine sulphate, solution Nicotine tartrate Nitrates, inorganic, aqueous solution, n.o.s. Nitrates, inorganic, n.o.s. Nitrating acid mixture with more than 50% nitric acid Nitrating acid mixture with not more than 50% nitric acid Nitrating acid mixture, spent, with more than 50% nitric acid Nitrating acid mixture, spent, with not more than 50% nitric acid Nitric acid, other than red fuming, with more than 65% nitric acid Nitric acid, other than red fuming, with not more than 65% nitric acid | 151 151 151 140 140 157 157 157 157 157 157 | 3445 1658 1659 3218 1477 1796 1796 1826 1826 2031 2031 |  |
| Naphthalene, refined 133 |  |  |  |  |  |  |
| Naphthylamine (alpha) 153 |  |  |  |  |  |  |
| Naphthylamine (beta), solid 153 |  |  |  |  |  |  |
| Naphthylamine (beta), solution 153 |  |  |  |  |  |  |
| Naphthylthiourea 153 |  |  |  |  |  |  |
| Naphthylurea 153 |  |  |  |  |  |  |
| Natural gas, compressed 115 |  |  |  |  |  |  |
| Natural gas, refrigerated liquid 115 |  |  |  |  |  |  |
| (cryogenic liquid) |  |  |  |  |  |  |
| Neohexane 128 |  |  |  |  |  |  |
| Neon, compressed 120 |  |  |  |  |  |  |
| Neon, refrigerated liquid 120 |  |  |  |  |  |  |
| (cryogenic liquid) |  |  |  |  |  |  |
| Nickel carbonyl 131 |  |  |  |  |  |  |
| Nickel catalyst, dry 135 |  |  |  |  |  |  |
| Nickel cyanide 151 |  |  |  |  |  |  |
| Nickel nitrate 140 |  |  |  |  |  |  |
| Nickel nitrite 140 |  |  |  |  |  |  |
| Nicotine 151 |  |  | Nitric acid, red fuming | 157 | 2032 |  |
| Nicotine compound, liquid, 151 |  |  | Nitric oxide, compressed | 124 | 1660 |  |
| n.o.s. Nicotine compound, solid, 151 |  |  |  |  |  |  |
|  |  |  | Nitric oxide and dinitrogen tetroxide mixture | 124 | 1975 |  |
| n.o.s. Nicotine hydrochloride, liquid 151 |  |  |  |  |  |  |
|  |  |  | Nitric oxide and nitrogen dioxide mixture | 124 | 1975 |  |
| Nicotine hydrochloride, solid 151 |  | Nitriles, flammable, poisonous, 131 3273 n.o.s. Nitriles, flammable, toxic, 131 3273 n.o.s. |  |  |  |  |
| Nicotine hydrochloride, 151 |  |  |  |  |  |  |
| solution |  |  |  |  |  |  |
| Nicotine preparation, liquid, 151 n.o.s. |  |  |  |  |  |  |
|  |  |  | Nitriles, liquid, poisonous, n.o.s. | 151 | 3276 |  |
| Nicotine preparation, solid, 151 |  |  |  |  |  |  |
|  |  |  | Nitriles, liquid, toxic, n.o.s. | 151 | 3276 |  |
| n.o.s. |  |  |  |  |  |  |
| Nicotine salicylate 151 |  |  | Nitriles, poisonous, flammable, 131 3275 n.o.s. |  |  |  |
| Nicotine sulfate, solid 151 Nicotine sulfate, solution 151 |  |  |  |  |  |  |
|  |  |  | Nitriles, poisonous, liquid, n.o.s. | 151 | 3276 |  |

Page 125

| Name of Material G Nitriles, solid, poisonous, n.o.s. | uide No. 151 | ID No. 3439 |  | Name of Material G Nitrogen, refrigerated liquid (cryogenic liquid) | uide No. 120 | ID No. 1977 |
| --- | --- | --- | --- | --- | --- | --- |
| Nitriles, solid, toxic, n.o.s. | 151 | 3439 |  | Nitrogen dioxide | 124 | 1067 |
| Nitriles, toxic, flammable, n.o.s. | 131 | 3275 |  | Nitrogen trifluoride | 122 | 2451 |
|  |  |  |  | Nitrogen trioxide | 124 | 2421 |
|  |  |  |  | Nitroglycerin, solution in alcohol, with more than 1% but not more than 5% nitroglycerin |  |  |
| Nitriles, toxic, liquid, n.o.s. | 151 | 3276 |  |  | 127 | 3064 |
| Nitrites, inorganic, aqueous solution, n.o.s. | 140 | 3219 |  |  |  |  |
| Nitrites, inorganic, n.o.s. | 140 | 2627 |  |  |  |  |
| Nitroanilines | 153 | 1661 |  | Nitroglycerin, solution in alcohol, with not more than 1% nitroglycerin | 127 | 1204 |
| Nitroanisoles, liquid | 152 | 2730 |  |  |  |  |
| Nitroanisoles, solid | 152 | 3458 |  | Nitroglycerin mixture, desensitized, liquid, flammable, n.o.s., with not more than 30% nitroglycerin | 113 | 3343 |
| Nitrobenzene | 152 | 1662 |  |  |  |  |
| Nitrobenzenesulfonic acid | 153 | 2305 |  |  |  |  |
| Nitrobenzenesulphonic acid | 153 | 2305 |  | Nitroglycerin mixture, desensitized, liquid, n.o.s., with not more than 30% nitroglycerin | 113 | 3357 |
| Nitrobenzotrifluorides, liquid | 152 | 2306 |  |  |  |  |
| Nitrobenzotrifluorides, solid | 152 | 3431 |  | Nitroglycerin mixture, desensitized, solid, n.o.s., with more than 2% but not more than 10% nitroglycerin | 113 | 3319 |
| Nitrobromobenzenes, liquid | 152 | 2732 |  |  |  |  |
| Nitrobromobenzenes, solid | 152 | 3459 |  |  |  |  |
| Nitrocellulose membrane filters | 133 | 3270 |  | Nitroguanidine, wetted with not less than 20% water | 113 | 1336 |
| Nitrocellulose mixture, with or without pigment | 133 | 2557 |  |  |  |  |
|  |  |  |  | Nitrohydrochloric acid | 157 | 1798 |
| Nitrocellulose mixture, with or without plasticizer | 133 | 2557 |  | Nitromethane | 129 | 1261 |
| Nitrocellulose solution, flammable | 127 | 2059 |  | Nitronaphthalene | 133 | 2538 |
|  |  |  |  | Nitrophenols | 153 | 1663 |
| Nitrocellulose with alcohol, not less than 25% alcohol | 113 | 2556 |  | 4-Nitrophenylhydrazine, with not less than 30% water | 113 | 3376 |
| Nitrocellulose with water, not less than 25% water | 113 | 2555 |  | Nitropropanes | 129 | 2608 |
|  |  |  |  | p-Nitrosodimethylaniline | 135 | 1369 |
| 3-Nitro-4-chlorobenzotrifluoride | 152 | 2307 |  | Nitrostarch, wetted with not less than 20% water | 113 | 1337 |
| Nitrocresols, liquid | 153 | 3434 |  |  |  |  |
| Nitrocresols, solid | 153 | 2446 |  | Nitrosyl chloride | 125 | 1069 |
| Nitroethane | 129 | 2842 |  | Nitrosylsulfuric acid, liquid | 157 | 2308 |
| Nitrogen, compressed | 120 | 1066 |  | Nitrosylsulfuric acid, solid | 157 | 3456 |

Page 126

| Name of Material Guide No. | ID No. |  | Name of Material Guide No. |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nitrosylsulphuric acid, liquid 157 | 2308 3456 1664 3446 2660 1070 1070 2201 1665 3447 1920 1799 2251 1800 2309 2422 1976 2424 1262 1191 1801 1071 3101 3111 3102 3112 3103 3113 | Organic peroxide type C, solid 146 3104 Organic peroxide type C, 148 3114 solid, temperature controlled Organic peroxide type D, liquid 145 3105 Organic peroxide type D, 148 3115 liquid, temperature controlled Organic peroxide type D, solid 145 3106 Organic peroxide type D, 148 3116 solid, temperature controlled Organic peroxide type E, liquid 145 3107 Organic peroxide type E, 148 3117 liquid, temperature controlled Organic peroxide type E, solid 145 3108 Organic peroxide type E, solid, 148 3118 temperature controlled Organic peroxide type F, liquid 145 3109 Organic peroxide type F, 148 3119 liquid, temperature controlled Organic peroxide type F, solid 145 3110 Organic peroxide type F, solid, 148 3120 temperature controlled |  |  |  |  |
| Nitrosylsulphuric acid, solid 157 |  |  |  |  |  |  |
| Nitrotoluenes, liquid 152 |  |  |  |  |  |  |
| Nitrotoluenes, solid 152 |  |  |  |  |  |  |
| Nitrotoluidines (mono) 153 |  |  |  |  |  |  |
| Nitrous oxide 122 |  |  |  |  |  |  |
| Nitrous oxide, compressed 122 |  |  |  |  |  |  |
| Nitrous oxide, refrigerated 122 |  |  |  |  |  |  |
| liquid |  |  |  |  |  |  |
| Nitroxylenes, liquid 152 |  |  |  |  |  |  |
| Nitroxylenes, solid 152 |  |  |  |  |  |  |
| Nonanes 128 |  |  |  |  |  |  |
| Nonyltrichlorosilane 156 |  |  |  |  |  |  |
| 2,5-Norbornadiene, stabilized 128P |  |  |  |  |  |  |
| Octadecyltrichlorosilane 156 |  |  |  |  |  |  |
| Octadiene 128P |  |  |  |  |  |  |
| Octafluorobut-2-ene 126 |  |  |  |  |  |  |
| Octafluorocyclobutane 126 |  |  |  |  |  |  |
| Octafluoropropane 126 |  |  |  |  |  |  |
| Octanes 128 |  |  |  |  |  |  |
| Octyl aldehydes 129 |  |  |  |  |  |  |
|  |  |  | Organic phosphate compound | 123 | 1955 |  |
|  |  |  | mixed with compressed gas |  |  |  |
| Octyltrichlorosilane 156 |  |  |  |  |  |  |
| Oil gas, compressed 119 |  |  | Organic phosphate mixed with compressed gas | 123 | 1955 |  |
| Organic peroxide type B, liquid 146 Organic peroxide type B, 148 liquid, temperature |  |  |  |  |  |  |
|  |  |  | Organic phosphorus compound mixed with compressed gas | 123 | 1955 |  |
|  |  |  | Organic pigments, self-heating | 135 | 3313 |  |
| controlled |  |  |  |  |  |  |
| Organic peroxide type B, solid 146 |  |  | Organoarsenic compound, liquid, n.o.s. | 151 | 3280 |  |
| Organic peroxide type B, solid, 148 |  |  |  |  |  |  |
|  |  |  | Organoarsenic compound, solid, n.o.s. Organochlorine pesticide, liquid, flammable, poisonous Organochlorine pesticide, liquid, flammable, toxic | 151 131 131 | 3465 2762 2762 |  |
| temperature controlled |  |  |  |  |  |  |
| Organic peroxide type C, liquid 146 |  |  |  |  |  |  |
| Organic peroxide type C, 148 |  |  |  |  |  |  |
| liquid, temperature |  |  |  |  |  |  |
| controlled |  |  |  |  |  |  |

Page 127

| Name of Material G Organochlorine pesticide, liquid, poisonous | uide No. 151 | ID No. 2996 | Name of Material G Organometallic substance, solid, water-reactive, flammable | uide No. 138 | ID No. 3396 |
| --- | --- | --- | --- | --- | --- |
| Organochlorine pesticide, liquid, poisonous, flammable | 131 | 2995 |  |  |  |
|  |  |  | Organometallic substance, solid, water-reactive, self- heating | 138 | 3397 |
| Organochlorine pesticide, liquid, toxic | 151 | 2996 |  |  |  |
| Organochlorine pesticide, liquid, toxic, flammable | 131 | 2995 | Organophosphorus compound, liquid, poisonous, n.o.s. | 151 | 3278 |
| Organochlorine pesticide, solid, poisonous | 151 | 2761 | Organophosphorus compound, liquid, toxic, n.o.s. | 151 | 3278 |
| Organochlorine pesticide, solid, toxic | 151 | 2761 | Organophosphorus compound, poisonous, flammable, n.o.s. | 131 | 3279 |
| Organometallic compound, liquid, poisonous, n.o.s. | 151 | 3282 | Organophosphorus compound, solid, poisonous, n.o.s. | 151 | 3464 |
| Organometallic compound, liquid, toxic, n.o.s. | 151 | 3282 | Organophosphorus compound, solid, toxic, n.o.s. | 151 | 3464 |
| Organometallic compound, solid, poisonous, n.o.s. | 151 | 3467 | Organophosphorus compound, toxic, flammable, n.o.s. | 131 | 3279 |
| Organometallic compound, solid, toxic, n.o.s. | 151 | 3467 | Organophosphorus pesticide, liquid, flammable, poisonous | 131 | 2784 |
| Organometallic substance, liquid, pyrophoric | 135 | 3392 | Organophosphorus pesticide, liquid, flammable, toxic | 131 | 2784 |
| Organometallic substance, liquid, pyrophoric, water- reactive | 135 | 3394 | Organophosphorus pesticide, liquid, poisonous | 152 | 3018 |
|  |  |  | Organophosphorus pesticide, liquid, poisonous, flammable | 131 | 3017 |
| Organometallic substance, liquid, water-reactive | 135 | 3398 |  |  |  |
|  |  |  | Organophosphorus pesticide, liquid, toxic | 152 | 3018 |
| Organometallic substance, liquid, water-reactive, flammable | 138 | 3399 |  |  |  |
|  |  |  | Organophosphorus pesticide, liquid, toxic, flammable | 131 | 3017 |
| Organometallic substance, solid, pyrophoric | 135 | 3391 | Organophosphorus pesticide, solid, poisonous | 152 | 2783 |
| Organometallic substance, solid, pyrophoric, water- reactive | 135 | 3393 | Organophosphorus pesticide, solid, toxic | 152 | 2783 |
|  |  |  | Organotin compound, liquid, n.o.s. | 153 | 2788 |
| Organometallic substance, solid, self-heating | 138 | 3400 |  |  |  |
|  |  |  | Organotin compound, solid, n.o.s. | 153 | 3146 |
| Organometallic substance, solid, water-reactive | 135 | 3395 |  |  |  |
|  |  |  | Organotin pesticide, liquid, flammable, poisonous | 131 | 2787 |

Page 128

| Name of Material Guide ID No. No. |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Organotin pesticide, liquid, flammable, toxic | 131 | 2787 |  | Oxygen, refrigerated liquid (cryogenic liquid) | 122 | 1073 |  |
| Organotin pesticide, liquid, poisonous | 153 | 3020 |  | Oxygen difluoride, compressed | 124 | 2190 |  |
|  |  |  |  | Oxygen generator, chemical | 140 | 3356 |  |
| Organotin pesticide, liquid, poisonous, flammable | 131 | 3019 |  | Oxygen generator, chemical, spent | 140 | 3356 |  |
| Organotin pesticide, liquid, toxic | 153 | 3020 |  | Packagings discarded, empty, uncleaned | 171 | 3509 |  |
| Organotin pesticide, liquid, toxic, flammable | 131 | 3019 |  | Paint (corrosive) | 153 | 3066 |  |
| Organotin pesticide, solid, poisonous | 153 | 2786 |  | Paint, corrosive, flammable | 132 | 3470 |  |
|  |  |  |  | Paint (flammable) | 128 | 1263 |  |
| Organotin pesticide, solid, toxic | 153 | 2786 |  | Paint, flammable, corrosive | 132 | 3469 |  |
| Osmium tetroxide | 154 | 2471 |  | Paint related material (corrosive) | 153 | 3066 |  |
| Other regulated substances, liquid, n.o.s. | 171 | 3082 |  | Paint related material, corrosive, flammable | 132 | 3470 |  |
| Other regulated substances, solid, n.o.s. | 171 | 3077 |  | Paint related material (flammable) | 128 | 1263 |  |
| Oxidizing liquid, corrosive, n.o.s. | 140 | 3098 |  | Paint related material, flammable, corrosive | 132 | 3469 |  |
| Oxidizing liquid, n.o.s. | 140 | 3139 |  | Paper, unsaturated oil treated | 133 | 1379 |  |
| Oxidizing liquid, poisonous, n.o.s. | 142 | 3099 |  | Paraformaldehyde | 133 | 2213 |  |
| Oxidizing liquid, toxic, n.o.s. | 142 | 3099 |  | Paraldehyde | 129 | 1264 |  |
|  |  |  |  | Parathion and compressed gas mixture | 123 | 1967 |  |
| Oxidizing solid, corrosive, n.o.s. | 140 | 3085 |  |  |  |  |  |
|  |  |  |  | PCB, liquid | 171 | 2315 |  |
| Oxidizing solid, flammable, n.o.s. | 140 | 3137 |  |  |  |  |  |
|  |  |  |  | PCB, solid | 171 | 3432 |  |
| Oxidizing solid, n.o.s. | 140 | 1479 |  | Pentaborane | 135 | 1380 |  |
| Oxidizing solid, poisonous, n.o.s. | 141 | 3087 |  | Pentachloroethane | 151 | 1669 |  |
|  |  |  |  | Pentachlorophenol | 154 | 3155 |  |
| Oxidizing solid, self-heating, n.o.s. | 135 | 3100 |  | Pentaerythrite tetranitrate mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN | 113 | 3344 |  |
| Oxidizing solid, toxic, n.o.s. | 141 | 3087 |  |  |  |  |  |
| Oxidizing solid, water-reactive, n.o.s. | 144 | 3121 |  |  |  |  |  |
|  |  |  |  | Pentaerythritol tetranitrate mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN | 113 | 3344 |  |
| Oxygen, compressed | 122 | 1072 |  |  |  |  |  |

Page 129

| Name of Material G Pentafluoroethane | uide No. 126 | ID No. 3220 |  |  | Name of Material G Pesticide, liquid, poisonous, flammable, n.o.s. | uide No. 131 | ID No. 2903 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pentamethylheptane | 128 | 2286 |  |  |  |  |  |
| Pentane-2,4-dione | 131 | 2310 |  |  | Pesticide, liquid, poisonous, n.o.s. | 151 | 2902 |
| Pentanes | 128 | 1265 |  |  | Pesticide, liquid, toxic, flammable, n.o.s. | 131 | 2903 |
| Pentanols | 129 | 1105 |  |  |  |  |  |
| 1-Pentene | 128 | 1108 |  |  | Pesticide, liquid, toxic, n.o.s. | 151 | 2902 |
| 1-Pentol | 153P | 2705 |  |  | Pesticide, solid, poisonous, n.o.s. | 151 | 2588 |
| Perchlorates, inorganic, aqueous solution, n.o.s. | 140 | 3211 |  |  | Pesticide, solid, toxic, n.o.s. | 151 | 2588 |
| Perchlorates, inorganic, n.o.s. | 140 | 1481 |  |  | PETN mixture, desensitized, solid, n.o.s., with more than 10% but not more than 20% PETN | 113 | 3344 |
| Perchloric acid, with more than 50% but not more than 72% acid | 143 | 1873 |  |  |  |  |  |
|  |  |  |  |  | Petrol | 128 | 1203 |
| Perchloric acid, with not more than 50% acid | 157 | 1802 |  |  | Petroleum crude oil | 128 | 1267 |
| Perchloroethylene | 160 | 1897 |  |  | Petroleum distillates, n.o.s. | 128 | 1268 |
| Perchloromethyl mercaptan | 157 | 1670 |  |  | Petroleum gases, liquefied | 115 | 1075 |
| Perchloryl fluoride | 124 | 3083 |  |  | Petroleum oil | 128 | 1270 |
| Perfluoro(ethyl vinyl ether) | 115 | 3154 |  |  | Petroleum products, n.o.s. | 128 | 1268 |
| Perfluoro(methyl vinyl ether) | 115 | 3153 |  |  | Petroleum sour crude oil, flammable, poisonous | 131 | 3494 |
| Perfumery products, with flammable solvents | 127 | 1266 |  |  | Petroleum sour crude oil, flammable, toxic | 131 | 3494 |
| Permanganates, inorganic, aqueous solution, n.o.s. | 140 | 3214 |  |  | Phenacyl bromide | 153 | 2645 |
| Permanganates, inorganic, n.o.s. | 140 | 1482 |  |  | Phenetidines | 153 | 2311 |
|  |  |  |  |  | Phenol, molten | 153 | 2312 |
| Peroxides, inorganic, n.o.s. | 140 | 1483 |  |  | Phenol, solid | 153 | 1671 |
| Persulfates, inorganic, aqueous solution, n.o.s. | 140 | 3216 |  |  | Phenol solution | 153 | 2821 |
| Persulfates, inorganic, n.o.s. | 140 | 3215 |  |  | Phenolates, liquid | 154 | 2904 |
| Persulphates, inorganic, aqueous solution, n.o.s. | 140 | 3216 |  |  | Phenolates, solid | 154 | 2905 |
|  |  |  |  |  | Phenolsulfonic acid, liquid | 153 | 1803 |
| Persulphates, inorganic, n.o.s. | 140 | 3215 |  |  | Phenolsulphonic acid, liquid | 153 | 1803 |
| Pesticide, liquid, flammable, poisonous, n.o.s. | 131 | 3021 |  |  | Phenoxyacetic acid derivative pesticide, liquid, flammable, poisonous | 131 | 3346 |
| Pesticide, liquid, flammable, toxic, n.o.s. | 131 | 3021 |  |  |  |  |  |

Page 130

| Name of Material Guide ID No. No. |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phenoxyacetic acid derivative pesticide, liquid, flammable, toxic | 131 | 3346 |  | Phosgene | 125 | 1076 |  |
|  |  |  |  | 9-Phosphabicyclononanes | 135 | 2940 |  |
| Phenoxyacetic acid derivative pesticide, liquid, poisonous | 153 | 3348 |  | Phosphine | 119 | 2199 |  |
|  |  |  |  | Phosphine, adsorbed | 173 | 3525 |  |
| Phenoxyacetic acid derivative pesticide, liquid, poisonous, flammable | 131 | 3347 |  | Phosphoric acid, solid | 154 | 3453 |  |
|  |  |  |  | Phosphoric acid, solution | 154 | 1805 |  |
| Phenoxyacetic acid derivative pesticide, liquid, toxic | 153 | 3348 |  | Phosphorous acid | 154 | 2834 |  |
|  |  |  |  | Phosphorus, amorphous | 133 | 1338 |  |
| Phenoxyacetic acid derivative pesticide, liquid, toxic, flammable | 131 | 3347 |  | Phosphorus, white, dry or under water or in solution | 136 | 1381 |  |
| Phenoxyacetic acid derivative pesticide, solid, poisonous | 153 | 3345 |  | Phosphorus, white, molten | 136 | 2447 |  |
|  |  |  |  | Phosphorus, yellow, dry or under water or in solution | 136 | 1381 |  |
| Phenoxyacetic acid derivative pesticide, solid, toxic | 153 | 3345 |  |  |  |  |  |
|  |  |  |  | Phosphorus heptasulfide, free from yellow and white phosphorus | 139 | 1339 |  |
| Phenylacetonitrile, liquid | 152 | 2470 |  |  |  |  |  |
| Phenylacetyl chloride | 156 | 2577 |  | Phosphorus heptasulphide, free from yellow and white phosphorus | 139 | 1339 |  |
| Phenylcarbylamine chloride | 151 | 1672 |  |  |  |  |  |
| Phenyl chloroformate | 156 | 2746 |  |  |  |  |  |
| Phenylenediamines | 153 | 1673 |  | Phosphorus oxybromide, molten | 137 | 2576 |  |
| Phenylhydrazine | 153 | 2572 |  | Phosphorus oxybromide, solid | 137 | 1939 |  |
| Phenyl isocyanate | 155 | 2487 |  | Phosphorus oxychloride | 137 | 1810 |  |
| Phenyl mercaptan | 131 | 2337 |  | Phosphorus pentabromide | 137 | 2691 |  |
| Phenylmercuric acetate | 151 | 1674 |  | Phosphorus pentachloride | 137 | 1806 |  |
| Phenylmercuric compound, n.o.s. | 151 | 2026 |  | Phosphorus pentafluoride | 125 | 2198 |  |
|  |  |  |  | Phosphorus pentafluoride, adsorbed | 173 | 3524 |  |
| Phenylmercuric hydroxide | 151 | 1894 |  |  |  |  |  |
| Phenylmercuric nitrate | 151 | 1895 |  | Phosphorus pentafluoride, compressed | 125 | 2198 |  |
| Phenylphosphorus dichloride | 137 | 2798 |  |  |  |  |  |
| Phenylphosphorus thiodichloride | 137 | 2799 |  | Phosphorus pentasulfide, free from yellow and white phosphorus | 139 | 1340 |  |
| Phenyltrichlorosilane | 156 | 1804 |  |  |  |  |  |
|  |  |  |  | Phosphorus pentasulphide, free from yellow and white phosphorus | 139 | 1340 |  |
| Phenyl urea pesticides, liquid, poisonous | 151 | 3002 |  |  |  |  |  |
| Phenyl urea pesticides, liquid, toxic | 151 | 3002 |  | Phosphorus pentoxide | 137 | 1807 |  |

Page 131

|  | Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Phosphorus sesquisulfide, free from yellow and white phosphorus | 139 | 1341 |  |  | Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) | 154 | 3389 |
|  | Phosphorus sesquisulphide, free from yellow and white phosphorus | 139 | 1341 |  |  | Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) | 154 | 3390 |
|  | Phosphorus tribromide | 137 | 1808 |  |  | Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 131 | 3488 |
|  | Phosphorus trichloride | 137 | 1809 |  |  |  |  |  |
|  | Phosphorus trioxide | 157 | 2578 |  |  | Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 131 | 3489 |
|  | Phosphorus trisulfide, free from yellow and white | 139 | 1343 |  |  |  |  |  |
|  | phosphorus Phosphorus trisulphide, free from yellow and white phosphorus | 139 | 1343 |  |  | Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) | 131 | 3383 |
|  |  |  |  |  |  | Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) | 131 | 3384 |
|  | Phthalic anhydride | 156 | 2214 |  |  |  |  |  |
|  | Picolines | 129 | 2313 |  |  |  |  |  |
|  |  |  |  |  |  | Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) | 151 | 3381 |
|  | Picric acid, wetted with not less than 10% water | 113 | 3364 |  |  |  |  |  |
|  | Picric acid, wetted with not less than 30% water | 113 | 1344 |  |  | Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) | 151 | 3382 |
|  | Picrite, wetted with not less than 20% water | 113 | 1336 |  |  |  |  |  |
|  |  |  |  |  |  | Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 142 | 3387 |
|  | Picryl chloride, wetted with not less than 10% water | 113 | 3365 |  |  |  |  |  |
|  |  |  |  |  |  | Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 142 | 3388 |
|  | Pinene (alpha) | 128 | 2368 |  |  |  |  |  |
|  | Pine oil | 129 | 1272 |  |  |  |  |  |
|  | Piperazine | 153 | 2579 |  |  | Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone A) | 155 | 3490 |
|  | Piperidine | 132 | 2401 |  |  |  |  |  |
|  | Plastic molding compound | 171 | 3314 |  |  |  |  |  |
|  |  |  |  |  |  | Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone B) | 155 | 3491 |
|  | Plastics moulding compound | 171 | 3314 |  |  |  |  |  |
|  | Plastics, nitrocellulose-based, self-heating, n.o.s. | 135 | 2006 |  |  |  |  |  |
|  | Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) | 131 | 3492 |  |  | Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) | 139 | 3385 |
|  | Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) | 131 | 3493 |  |  | Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) | 139 | 3386 |

Page 132

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

Poisonous liquid, corrosive, inorganic, n.o.s.

Poisonous liquid, corrosive, organic, n.o.s.

Poisonous liquid, flammable, organic, n.o.s.

Poisonous liquid, inorganic, n.o.s.

Poisonous liquid, organic, n.o.s.

Poisonous liquid, oxidizing, n.o.s.

Poisonous liquid, water- reactive, n.o.s.

154	3289

154	2927

131	2929

151	3287

153	2810

142	3122

139	3123

Polychlorinated biphenyls, solid

Polyester resin kit, liquid base material

Polyester resin kit, solid base material

Polyhalogenated biphenyls, liquid

Polyhalogenated biphenyls, solid

Polyhalogenated terphenyls, liquid

Polyhalogenated terphenyls, solid

171	3432

128	3269

128P 3527

171	3151

171	3152

171	3151

171	3152

Poisonous solid, corrosive, inorganic, n.o.s.

Poisonous solid, corrosive, organic, n.o.s.

Poisonous solid, flammable, organic, n.o.s.

154	3290

154	2928

134	2930

Polymeric beads, expandable 171	2211

Polymerizing substance, liquid, 149P 3532

stabilized, n.o.s.

Polymerizing substance, liquid, 150P 3534

temperature controlled, n.o.s.

Poisonous solid, inorganic, n.o.s.

Poisonous solid, organic, n.o.s.

Poisonous solid, oxidizing,

151	3288

154	2811

141	3086

Polymerizing substance, solid, stabilized, n.o.s.

Polymerizing substance, solid, temperature controlled, n.o.s.

149P 3531

150P 3533

n.o.s.

Poisonous solid, self-heating, n.o.s.

Poisonous solid, water- reactive, n.o.s.

Polyamines, flammable, corrosive, n.o.s.

Polyamines, liquid, corrosive, flammable, n.o.s.

Polyamines, liquid, corrosive, n.o.s.

Polyamines, solid, corrosive, n.o.s.

Polychlorinated biphenyls, liquid

136	3124

139	3125

132	2733

132	2734

153	2735

154	3259

171	2315

Potassium	138	2257

Potassium metal alloys, liquid 138	1420

Potassium metal alloys, solid  138	3403

Potassium arsenate	151	1677

Potassium arsenite	154	1678

Potassium borohydride	138	1870

Potassium bromate	140	1484

Potassium chlorate	140	1485

Potassium chlorate, aqueous	140	2427

solution

Potassium cuprocyanide	157	1679

Potassium cyanide, solid	157	1680

Potassium cyanide, solution	157	3413

Page 133

| Name of Material G Potassium dithionite | uide No. 135 | ID No. 1929 |  |  | Name of Material G Potassium sulfide, with less than 30% water of crystallization | uide No. 135 | ID No. 1382 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Potassium fluoride, solid | 154 | 1812 |  |  |  |  |  |
| Potassium fluoride, solution | 154 | 3422 |  |  | Potassium sulphide, anhydrous | 135 | 1382 |
| Potassium fluoroacetate | 151 | 2628 |  |  | Potassium sulphide, hydrated, with not less than 30% water of crystallization | 153 | 1847 |
| Potassium fluorosilicate | 151 | 2655 |  |  |  |  |  |
| Potassium hydrogen difluoride, solid | 154 | 1811 |  |  | Potassium sulphide, with less than 30% water of crystallization | 135 | 1382 |
| Potassium hydrogen difluoride, solution | 154 | 3421 |  |  |  |  |  |
|  |  |  |  |  | Potassium superoxide | 143 | 2466 |
| Potassium hydrogen sulfate | 154 | 2509 |  |  | Printing ink, flammable | 129 | 1210 |
| Potassium hydrogen sulphate | 154 | 2509 |  |  | Printing ink related material, flammable | 129 | 1210 |
| Potassium hydrosulfite | 135 | 1929 |  |  |  |  |  |
| Potassium hydrosulphite | 135 | 1929 |  |  | Propadiene, stabilized | 116P | 2200 |
| Potassium hydroxide, solid | 154 | 1813 |  |  | Propane | 115 | 1075 |
| Potassium hydroxide, solution | 154 | 1814 |  |  | Propane | 115 | 1978 |
| Potassium metavanadate | 151 | 2864 |  |  | Propanethiols | 130 | 2402 |
| Potassium monoxide | 154 | 2033 |  |  | n-Propanol | 129 | 1274 |
| Potassium nitrate | 140 | 1486 |  |  | Propionaldehyde | 129P | 1275 |
| Potassium nitrate and sodium nitrite mixture | 140 | 1487 |  |  | Propionic acid | 153 | 1848 |
|  |  |  |  |  | Propionic acid, with not less than 10% and less than 90% acid | 153 | 1848 |
| Potassium nitrite | 140 | 1488 |  |  |  |  |  |
| Potassium perchlorate | 140 | 1489 |  |  |  |  |  |
| Potassium permanganate | 140 | 1490 |  |  | Propionic acid, with not less than 90% acid | 153 | 3463 |
| Potassium peroxide | 144 | 1491 |  |  | Propionic anhydride | 156 | 2496 |
| Potassium persulfate | 140 | 1492 |  |  | Propionitrile | 131 | 2404 |
| Potassium persulphate | 140 | 1492 |  |  | Propionyl chloride | 155 | 1815 |
| Potassium phosphide | 139 | 2012 |  |  | n-Propyl acetate | 129 | 1276 |
| Potassium sodium alloys, liquid | 138 | 1422 |  |  | Propyl alcohol, normal | 129 | 1274 |
| Potassium sodium alloys, solid | 138 | 3404 |  |  | Propylamine | 132 | 1277 |
| Potassium sulfide, anhydrous | 135 | 1382 |  |  | n-Propylbenzene | 128 | 2364 |
| Potassium sulfide, hydrated, with not less than 30% water of crystallization | 153 | 1847 |  |  | Propyl chloride | 129 | 1278 |
|  |  |  |  |  | n-Propyl chloroformate | 155 | 2740 |
|  |  |  |  |  | Propylene | 115 | 1075 |

Page 134

| Name of Material | Guide | ID | Name of Material | Guide | ID |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | No. | No. |  | No. | No. |  |
| Propylene | 115 | 1077 | Pyrosulfuryl chloride | 137 | 1817 |  |
| Propylene chlorohydrin | 131 | 2611 | Pyrosulphuryl chloride | 137 | 1817 |  |
| 1,2-Propylenediamine | 132 | 2258 | Pyrrolidine | 132 | 1922 |  |
| Propyleneimine, stabilized | 131P | 1921 | Quinoline | 154 | 2656 |  |

Propylene oxide	127P 1280

Radioactive material, excepted 161	2911

| Propylene tetramer | 128 | 2850 |
| --- | --- | --- |
| Propyl formates | 129 | 1281 |
| n-Propyl isocyanate | 155P | 2482 |
| n-Propyl nitrate | 128 | 1865 |
| Propyltrichlorosilane 155 1816 |  |  |
| Pyrethroid pesticide, liquid, flammable, poisonous | 131 | 3350 |
| Pyrethroid pesticide, liquid, | 131 | 3350 |

package, articles

flammable, toxic Pyrethroid pesticide, liquid,

151	3352

Radioactive material, excepted package, articles manufactured from depleted uranium

Radioactive material, excepted package, articles manufactured from natural thorium

Radioactive material, excepted package, articles manufactured from natural uranium

161	2909

161	2909

161	2909

poisonous

Pyrethroid pesticide, liquid, poisonous, flammable

Pyrethroid pesticide, liquid, toxic

Pyrethroid pesticide, liquid, toxic, flammable

131	3351

151	3352

131	3351

Radioactive material, excepted 161	2908

package, empty packaging

Radioactive material, excepted 161	2911

package, instruments

Radioactive material, excepted 161	2910

package, limited quantity of material

Pyrethroid pesticide, solid, poisonous

Pyrethroid pesticide, solid, toxic

151	3349

151	3349

Radioactive material, low specific activity (LSA-I), non fissile or fissile-excepted

Radioactive material, low specific activity (LSA-II),

162	2912

165	3324

Pyridine	129	1282

Pyrophoric alloy, n.o.s.	135	1383

fissile

Radioactive material, low

162	3321

Pyrophoric liquid, inorganic, n.o.s.

Pyrophoric liquid, organic, n.o.s.

135	3194

135	2845

specific activity (LSA-II), non fissile or fissile-excepted

Radioactive material, low specific activity (LSA-III), fissile

165	3325

Pyrophoric metal, n.o.s.	135	1383

Radioactive material, low

162	3322

Pyrophoric solid, inorganic, n.o.s.

Pyrophoric solid, organic, n.o.s.

135	3200

135	2846

specific activity (LSA-III), non fissile or fissile- excepted

Page 135

| Name of Material G Radioactive material, surface contaminated objects (SCO-I or SCO-II), fissile | uide No. 165 | ID No. 3326 |  | Name of Material G Radioactive material, uranium hexafluoride, fissile | uide No. 166 | ID No. 2977 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Radioactive material, uranium hexafluoride, non fissile or fissile-excepted | 166 | 2978 |
| Radioactive material, surface contaminated objects (SCO-I, SCO-II or SCO-III), non fissile or fissile- excepted | 162 | 2913 |  |  |  |  |
|  |  |  |  | Rags, oily | 133 | 1856 |
|  |  |  |  | Receptacles, small, containing gas | 115 | 2037 |
| Radioactive material, transported under special arrangement, fissile | 165 | 3331 |  |  |  |  |
|  |  |  |  | Red phosphorus | 133 | 1338 |
| Radioactive material, transported under special arrangement, non fissile or fissile-excepted | 163 | 2919 |  | Refrigerant gas, n.o.s. | 126 | 1078 |
|  |  |  |  | Refrigerant gases, n.o.s. (flammable) | 115 | 1954 |
|  |  |  |  | Refrigerant gas R-12 | 126 | 1028 |
| Radioactive material, Type A package, fissile, non-special form | 165 | 3327 |  | Refrigerant gas R-12B1 | 126 | 1974 |
|  |  |  |  | Refrigerant gas R-12B2 | 171 | 1941 |
| Radioactive material, Type A package, non-special form, non fissile or fissile- excepted | 163 | 2915 |  | Refrigerant gas R-13 | 126 | 1022 |
|  |  |  |  | Refrigerant gas R-13B1 | 126 | 1009 |
|  |  |  |  | Refrigerant gas R-14 | 126 | 1982 |
| Radioactive material, Type A package, special form, fissile | 165 | 3333 |  | Refrigerant gas R-21 | 126 | 1029 |
|  |  |  |  | Refrigerant gas R-22 | 126 | 1018 |
| Radioactive material, Type A package, special form, non fissile or fissile-excepted | 164 | 3332 |  | Refrigerant gas R-23 | 126 | 1984 |
|  |  |  |  | Refrigerant gas R-32 | 115 | 3252 |
| Radioactive material, Type B(M) package, fissile | 165 | 3329 |  | Refrigerant gas R-40 | 115 | 1063 |
|  |  |  |  | Refrigerant gas R-41 | 115 | 2454 |
| Radioactive material, Type B(M) package, non fissile or fissile-excepted | 163 | 2917 |  | Refrigerant gas R-114 | 126 | 1958 |
|  |  |  |  | Refrigerant gas R-115 | 126 | 1020 |
| Radioactive material, Type B(U) package, fissile | 165 | 3328 |  | Refrigerant gas R-116 | 126 | 2193 |
| Radioactive material, Type B(U) package, non fissile or fissile-excepted | 163 | 2916 |  | Refrigerant gas R-124 | 126 | 1021 |
|  |  |  |  | Refrigerant gas R-125 | 126 | 3220 |
|  |  |  |  | Refrigerant gas R-133a | 126 | 1983 |
| Radioactive material, Type C package, fissile | 165 | 3330 |  | Refrigerant gas R-134a | 126 | 3159 |
| Radioactive material, Type C package, non fissile or fissile excepted | 163 | 3323 |  | Refrigerant gas R-142b | 115 | 2517 |
|  |  |  |  | Refrigerant gas R-143a | 115 | 2035 |
|  |  |  |  | Refrigerant gas R-152a | 115 | 1030 |

Page 136

| Name of Material | Guide No. | ID No. | Name of Material | Guide No. | ID No. |
| --- | --- | --- | --- | --- | --- |
| Refrigerant gas R-161 | 115 | 2453 | Rubber shoddy, powdered or granulated | 133 | 1345 |

| Rubber solution | 127 | 1287 |
| --- | --- | --- |
| Rubidium | 138 | 1423 |
| Rubidium hydroxide, solid | 154 | 2678 |
| Rubidium hydroxide, solution | 154 | 2677 |
| Safety devices | 171 | 3268 |
| Seat-belt pre-tensioners | 171 | 3268 |
| Seed cake, with more than | 135 | 1386 |
| 1.5% oil and not more than 11% moisture Seed cake, with not more than 135 2217 1.5% oil and not more than |  |  |

| Refrigerant gas R-218 | 126 | 2424 |
| --- | --- | --- |
| Refrigerant gas R-227 | 126 | 3296 |
| Refrigerant gas R-404A | 126 | 3337 |
| Refrigerant gas R-407A | 126 | 3338 |
| Refrigerant gas R-407B | 126 | 3339 |
| Refrigerant gas R-407C | 126 | 3340 |
| Refrigerant gas R-500 | 126 | 2602 |

Refrigerant gas R-502	126	1973

Refrigerant gas R-503	126	2599

Refrigerant gas R-1113	119P 1082

| Refrigerant gas R-1132a | 116P | 1959 | 11% moisture |  |  |
| --- | --- | --- | --- | --- | --- |
| Refrigerant gas R-1216 | 126 | 1858 | Selenates | 151 | 2630 |
| Refrigerant gas R-1318 | 126 | 2422 | Selenic acid | 154 | 1905 |
| Refrigerant gas RC-318 | 126 | 1976 | Selenites | 151 | 2630 |
| Refrigerating machines, containing ammonia solutions (UN2672) | 126 | 2857 | Selenium compound, liquid, n.o.s. | 151 | 3440 |

Refrigerating machines, containing flammable, non- poisonous, liquefied gas

Refrigerating machines, containing flammable, non- toxic, liquefied gas

Refrigerating machines, containing non-flammable, non-poisonous gases

115	3358

115	3358

126	2857

Selenium compound, solid, n.o.s.

151	3283

| Refrigerating machines, | 126 | 2857 | Self-heating liquid, corrosive, | 136 | 3188 |
| --- | --- | --- | --- | --- | --- |
| containing non-flammable, non-toxic gases |  |  | inorganic, n.o.s. |  |  |

Regulated medical waste, n.o.s.

158	3291

Self-heating liquid, corrosive, organic, n.o.s.

| Selenium disulfide | 153 | 2657 |
| --- | --- | --- |
| Selenium disulphide | 153 | 2657 |
| Selenium hexafluoride | 125 | 2194 |
| Selenium oxychloride | 157 | 2879 |
| Self-defense spray, non- | 171 | 3334 |
| pressurized |  |  |

Self-heating liquid, inorganic,

136	3185

135	3186

Resin solution	128	1866

Resorcinol	153	2876

Rosin oil	127	1286

Rubber scrap, powdered or	133	1345

granulated

n.o.s.

Self-heating liquid, organic, n.o.s.

Self-heating liquid, poisonous, inorganic, n.o.s.

135	3183

136	3187

Page 137

| Name of Material G Self-heating liquid, poisonous, organic, n.o.s. | uide No. 136 | ID No. 3184 | Name of Material G Self-reactive solid type B | uide No. 149 | ID No. 3222 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Self-reactive solid type B, temperature controlled | 150 | 3232 |
| Self-heating liquid, toxic, inorganic, n.o.s. | 136 | 3187 |  |  |  |
|  |  |  | Self-reactive solid type C | 149 | 3224 |
| Self-heating liquid, toxic, organic, n.o.s. | 136 | 3184 | Self-reactive solid type C, temperature controlled | 150 | 3234 |
| Self-heating solid, corrosive, inorganic, n.o.s. | 136 | 3192 | Self-reactive solid type D | 149 | 3226 |
| Self-heating solid, corrosive, organic, n.o.s. | 136 | 3126 | Self-reactive solid type D, temperature controlled | 150 | 3236 |
| Self-heating solid, inorganic, n.o.s. | 135 | 3190 | Self-reactive solid type E | 149 | 3228 |
|  |  |  | Self-reactive solid type E, temperature controlled | 150 | 3238 |
| Self-heating solid, organic, n.o.s. | 135 | 3088 |  |  |  |
|  |  |  | Self-reactive solid type F | 149 | 3230 |
| Self-heating solid, oxidizing, n.o.s. | 135 | 3127 | Self-reactive solid type F, temperature controlled | 150 | 3240 |
| Self-heating solid, poisonous, inorganic, n.o.s. | 136 | 3191 | Shale oil | 128 | 1288 |
| Self-heating solid, poisonous, organic, n.o.s. | 136 | 3128 | Silane | 116 | 2203 |
|  |  |  | Silicon powder, amorphous | 170 | 1346 |
| Self-heating solid, toxic, inorganic, n.o.s. | 136 | 3191 | Silicon tetrachloride | 157 | 1818 |
|  |  |  | Silicon tetrafluoride | 125 | 1859 |
| Self-heating solid, toxic, organic, n.o.s. | 136 | 3128 | Silicon tetrafluoride, adsorbed | 173 | 3521 |
| Self-reactive liquid type B | 149 | 3221 | Silicon tetrafluoride, compressed | 125 | 1859 |
| Self-reactive liquid type B, temperature controlled | 150 | 3231 | Silver arsenite | 151 | 1683 |
| Self-reactive liquid type C | 149 | 3223 | Silver cyanide | 151 | 1684 |
| Self-reactive liquid type C, temperature controlled | 150 | 3233 | Silver nitrate | 140 | 1493 |
|  |  |  | Silver picrate, wetted with not less than 30% water | 113 | 1347 |
| Self-reactive liquid type D | 149 | 3225 |  |  |  |
| Self-reactive liquid type D, temperature controlled | 150 | 3235 | Sludge acid | 153 | 1906 |
|  |  |  | Smokeless powder for small arms | 133 | 3178 |
| Self-reactive liquid type E | 149 | 3227 |  |  |  |
| Self-reactive liquid type E, temperature controlled | 150 | 3237 | Soda lime, with more than 4% sodium hydroxide | 154 | 1907 |
| Self-reactive liquid type F | 149 | 3229 | Sodium | 138 | 1428 |
| Self-reactive liquid type F, temperature controlled | 150 | 3239 | Sodium aluminate, solid | 154 | 2812 |
|  |  |  | Sodium aluminate, solution | 154 | 1819 |

Page 138

| Name of Material Guide No. |  | ID No. | Name of Material Guide No. |  |  | ID No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sodium aluminum hydride Sodium ammonium vanadate Sodium arsanilate Sodium arsenate Sodium arsenite, aqueous solution Sodium arsenite, solid Sodium azide Sodium bisulfate, solution Sodium bisulphate, solution Sodium borohydride Sodium borohydride and sodium hydroxide solution, with not more than 12% sodium borohydride and not more than 40% sodium hydroxide Sodium bromate Sodium cacodylate Sodium carbonate peroxyhydrate Sodium chlorate Sodium chlorate, aqueous solution Sodium chlorite Sodium chloroacetate Sodium cuprocyanide, solid Sodium cuprocyanide, solution Sodium cyanide, solid Sodium cyanide, solution Sodium dichloroisocyanurate Sodium dichloro-s- triazinetrione Sodium dinitro-o-cresolate, wetted with not less than 10% water | 138 154 154 151 154 151 153 154 154 138 157 140 152 140 140 140 143 151 157 157 157 157 140 140 113 | 2835 2863 2473 1685 1686 2027 1687 2837 2837 1426 3320 1494 1688 3378 1495 2428 1496 2659 2316 2317 1689 3414 2465 2465 3369 |  | Sodium dinitro-o-cresolate, wetted with not less than 15% water | 113 | 1348 |  |
|  |  |  |  | Sodium dithionite | 135 | 1384 |  |
|  |  |  | Sodium fluoride, solid 154 1690 Sodium fluoride, solution 154 3415 Sodium fluoroacetate 151 2629 Sodium fluorosilicate 154 2674 Sodium hydride 138 1427 Sodium hydrogendifluoride 154 2439 Sodium hydrosulfide, hydrated, 154 2949 with not less than 25% water of crystallization Sodium hydrosulfide, with 135 2318 less than 25% water of crystallization Sodium hydrosulfide, with 154 2949 not less than 25% water of crystallization |  |  |  |  |
|  |  |  |  | Sodium hydrosulfite | 135 | 1384 |  |
|  |  |  |  | Sodium hydrosulphide, hydrated, with not less than 25% water of crystallization Sodium hydrosulphide, with less than 25% water of crystallization Sodium hydrosulphide, with not less than 25% water of crystallization | 154 135 154 | 2949 2318 2949 |  |
|  |  |  |  | Sodium hydrosulphite | 135 | 1384 |  |
|  |  |  |  | Sodium hydroxide, solid Sodium hydroxide, solution Sodium hypochlorite Sodium ion batteries Sodium ion batteries contained in equipment Sodium ion batteries packed with equipment Sodium methylate, dry | 154 154 154 147 147 147 138 | 1823 1824 1791 3551 3552 3552 1431 |  |

Page 139

| Name of Material G Sodium methylate, solution in alcohol | uide No. 132 | ID No. 1289 |  | Name of Material G Solids containing toxic liquid, n.o.s. | uide No. 151 | ID No. 3243 |
| --- | --- | --- | --- | --- | --- | --- |
| Sodium monoxide | 157 | 1825 |  | Stannic chloride, anhydrous | 137 | 1827 |
| Sodium nitrate | 140 | 1498 |  | Stannic chloride, pentahydrate | 154 | 2440 |
| Sodium nitrate and potassium nitrate mixture | 140 | 1499 |  | Stannic phosphides | 139 | 1433 |
|  |  |  |  | Stibine | 119 | 2676 |
| Sodium nitrite | 141 | 1500 |  | Straw, wet, damp or contaminated with oil | 133 | 1327 |
| Sodium pentachlorophenate | 154 | 2567 |  |  |  |  |
| Sodium perborate monohydrate | 140 | 3377 |  | Strontium arsenite | 151 | 1691 |
| Sodium perchlorate | 140 | 1502 |  | Strontium chlorate | 143 | 1506 |
| Sodium permanganate | 140 | 1503 |  | Strontium nitrate | 140 | 1507 |
| Sodium peroxide | 144 | 1504 |  | Strontium perchlorate | 140 | 1508 |
| Sodium peroxoborate, anhydrous | 140 | 3247 |  | Strontium peroxide | 143 | 1509 |
|  |  |  |  | Strontium phosphide | 139 | 2013 |
| Sodium persulfate | 140 | 1505 |  | Strychnine | 151 | 1692 |
| Sodium persulphate | 140 | 1505 |  | Strychnine salts | 151 | 1692 |
| Sodium phosphide | 139 | 1432 |  | Styrene monomer, stabilized | 128P | 2055 |
| Sodium picramate, wetted with not less than 20% water | 113 | 1349 |  | Substituted nitrophenol pesticide, liquid, flammable, poisonous | 131 | 2780 |
| Sodium sulfide, anhydrous | 135 | 1385 |  |  |  |  |
| Sodium sulfide, hydrated, with not less than 30% water | 153 | 1849 |  | Substituted nitrophenol pesticide, liquid, flammable, toxic | 131 | 2780 |
| Sodium sulfide, with less than 30% water of crystallization | 135 | 1385 |  |  |  |  |
|  |  |  |  | Substituted nitrophenol pesticide, liquid, poisonous | 153 | 3014 |
| Sodium sulphide, anhydrous | 135 | 1385 |  |  |  |  |
| Sodium sulphide, hydrated, with not less than 30% water | 153 | 1849 |  | Substituted nitrophenol pesticide, liquid, poisonous, flammable | 131 | 3013 |
| Sodium sulphide, with less than 30% water of crystallization | 135 | 1385 |  | Substituted nitrophenol pesticide, liquid, toxic | 153 | 3014 |
| Sodium superoxide | 143 | 2547 |  | Substituted nitrophenol pesticide, liquid, toxic, flammable | 131 | 3013 |
| Solids containing corrosive liquid, n.o.s. | 154 | 3244 |  |  |  |  |
|  |  |  |  | Substituted nitrophenol pesticide, solid, poisonous | 153 | 2779 |
| Solids containing flammable liquid, n.o.s. | 133 | 3175 |  |  |  |  |
|  |  |  |  | Substituted nitrophenol pesticide, solid, toxic | 153 | 2779 |
| Solids containing poisonous liquid, n.o.s. | 151 | 3243 |  |  |  |  |
|  |  |  |  | Sulfamic acid | 154 | 2967 |

Page 140

Name of Material

Guide		ID	Name of Material No.	No.

Guide	ID

No.	No.

| Sulfur 133 1350 Sulfur, molten 133 2448 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | Sulphur tetrafluoride 125 2418 |  |
|  |  |  | Sulphur trioxide, stabilized 137 1829 |  |
| Sulfur chlorides 137 1828 Sulfur dioxide 125 1079 |  |  | Sulphuryl chloride 137 1834 |  |
|  |  |  | Sulphuryl fluoride 123 2191 |  |
| Sulfur hexafluoride 126 1080 Sulfuric acid 137 1830 Sulfuric acid, fuming 137 1831 Sulfuric acid, spent 137 1832 Sulfuric acid, with more than 137 1830 51% acid Sulfuric acid, with not more 157 2796 than 51% acid Sulfuric acid and hydrofluoric 157 1786 acid mixture Sulfurous acid 154 1833 |  |  | Tars, liquid 130 1999 Tear gas candles 159 1700 Tear gas devices 159 1693 Tear gas grenades 159 1700 Tear gas substance, liquid, 159 1693 n.o.s. Tear gas substance, solid, 159 3448 n.o.s. Tellurium compound, n.o.s. 151 3284 |  |
|  |  |  | Tellurium hexafluoride 125 2195 |  |
|  |  |  | Terpene hydrocarbons, n.o.s. 128 2319 |  |
| Sulfur tetrafluoride 125 2418 |  | Terpinolene 128 2541 Tetrabromoethane 159 2504 1,1,2,2-Tetrachloroethane 151 1702 Tetrachloroethylene 160 1897 Tetraethyl dithiopyrophosphate 153 1704 Tetraethylenepentamine 153 2320 Tetraethyl silicate 129 1292 1,1,1,2-Tetrafluoroethane 126 3159 Tetrafluoroethylene, stabilized 116P 1081 Tetrafluoromethane 126 1982 1,2,3,6-Tetrahydrobenzaldehyde 129 2498 Tetrahydrofuran 127 2056 Tetrahydrofurfurylamine 129 2943 Tetrahydrophthalic anhydrides 156 2698 1,2,3,6-Tetrahydropyridine 129 2410 Tetrahydrothiophene 130 2412 |  |  |
| Sulfur trioxide, stabilized 137 1829 |  |  |  |  |
| Sulfuryl chloride 137 1834 |  |  |  |  |
| Sulfuryl fluoride 123 2191 |  |  |  |  |
| Sulphamic acid 154 2967 Sulphur 133 1350 Sulphur, molten 133 2448 Sulphur chlorides 137 1828 Sulphur dioxide 125 1079 Sulphur hexafluoride 126 1080 Sulphuric acid 137 1830 Sulphuric acid, fuming 137 1831 Sulphuric acid, spent 137 1832 Sulphuric acid, with more than 137 1830 51% acid Sulphuric acid, with not more 157 2796 than 51% acid Sulphuric acid and hydrofluoric 157 1786 acid mixture Sulphurous acid 154 1833 |  |  |  |  |

Page 141

| Name of Material G Tetramethylammonium hydroxide aqueous solution with more than 2.5% but less than 25% tetramethylammonium hydroxide | uide No. 153 | ID No. 1835 |  | Name of Material G Thioglycol | uide No. 153 | ID No. 2966 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Thioglycolic acid | 153 | 1940 |
|  |  |  |  | Thiolactic acid | 153 | 2936 |
|  |  |  |  | Thionyl chloride | 137 | 1836 |
| Tetramethylammonium hydroxide aqueous solution with not less than 25% tetramethylammonium hydroxide | 153 | 3560 |  | Thiophene | 130 | 2414 |
|  |  |  |  | Thiophosgene | 156 | 2474 |
|  |  |  |  | Thiophosphoryl chloride | 157 | 1837 |
| Tetramethylammonium hydroxide, solid | 153 | 3423 |  | Thiourea dioxide | 135 | 3341 |
|  |  |  |  | Tinctures, medicinal | 127 | 1293 |
| Tetramethylammonium hydroxide, solution | 153 | 1835 |  | Tin tetrachloride | 137 | 1827 |
|  |  |  |  | Titanium disulfide | 135 | 3174 |
| Tetramethylsilane | 130 | 2749 |  | Titanium disulphide | 135 | 3174 |
| Tetranitromethane | 143 | 1510 |  | Titanium hydride | 170 | 1871 |
| Tetrapropyl orthotitanate | 128 | 2413 |  | Titanium powder, dry | 135 | 2546 |
| Textile waste, wet | 133 | 1857 |  | Titanium powder, wetted with not less than 25% water | 170 | 1352 |
| Thallium chlorate | 141 | 2573 |  |  |  |  |
| Thallium compound, n.o.s. | 151 | 1707 |  | Titanium sponge granules | 170 | 2878 |
| Thallium nitrate | 141 | 2727 |  | Titanium sponge powders | 170 | 2878 |
| 4-Thiapentanal | 152 | 2785 |  | Titanium tetrachloride | 137 | 1838 |
| Thioacetic acid | 129 | 2436 |  | Titanium trichloride, pyrophoric | 135 | 2441 |
| Thiocarbamate pesticide, liquid, flammable, poisonous | 131 | 2772 |  | Titanium trichloride mixture | 157 | 2869 |
|  |  |  |  | Titanium trichloride mixture, pyrophoric | 135 | 2441 |
| Thiocarbamate pesticide, liquid, flammable, toxic | 131 | 2772 |  |  |  |  |
|  |  |  |  | TNT, wetted with not less than 10% water | 113 | 3366 |
| Thiocarbamate pesticide, liquid, poisonous | 151 | 3006 |  |  |  |  |
|  |  |  |  | TNT, wetted with not less than 30% water | 113 | 1356 |
| Thiocarbamate pesticide, liquid, poisonous, flammable | 131 | 3005 |  |  |  |  |
|  |  |  |  | Toluene | 130 | 1294 |
| Thiocarbamate pesticide, liquid, toxic | 151 | 3006 |  | 2,4-Toluenediamine, solid | 151 | 1709 |
| Thiocarbamate pesticide, liquid, toxic, flammable | 131 | 3005 |  | 2,4-Toluenediamine, solution | 151 | 3418 |
|  |  |  |  | Toluene diisocyanate | 156 | 2078 |
| Thiocarbamate pesticide, solid, poisonous | 151 | 2771 |  | Toluidines, liquid | 153 | 1708 |
| Thiocarbamate pesticide, solid, toxic | 151 | 2771 |  | Toluidines, solid | 153 | 3451 |
|  |  |  |  | 2,4-Toluylenediamine, solid | 151 | 1709 |

Page 142

| Name of Material Guide ID No. No. |  |  |  | Name of Material Guide ID No. No. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4-Toluylenediamine, solution | 151 | 3418 |  |  | Toxic by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone B) | 155 | 3491 |  |
| Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) | 131 | 3492 |  |  |  |  |  |  |
| Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) | 131 | 3493 |  |  | Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) | 139 | 3385 |  |
| Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) | 154 | 3389 |  |  | Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) | 139 | 3386 |  |
| Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) | 154 | 3390 |  |  | Toxic liquid, corrosive, inorganic, n.o.s. | 154 | 3289 |  |
|  |  |  |  |  | Toxic liquid, corrosive, | 154 | 2927 |  |
| Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 131 | 3488 |  |  | organic, n.o.s. Toxic liquid, flammable, organic, n.o.s. | 131 | 2929 |  |
| Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 131 | 3489 |  |  | Toxic liquid, inorganic, n.o.s. | 151 | 3287 |  |
|  |  |  |  |  | Toxic liquid, organic, n.o.s. | 153 | 2810 |  |
| Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) | 131 | 3383 |  |  | Toxic liquid, oxidizing, n.o.s. | 142 | 3122 |  |
|  |  |  |  |  | Toxic liquid, water-reactive, n.o.s. | 139 | 3123 |  |
| Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) | 131 | 3384 |  |  | Toxic solid, corrosive, inorganic, n.o.s. | 154 | 3290 |  |
|  |  |  |  |  | Toxic solid, corrosive, organic, n.o.s. | 154 | 2928 |  |
| Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) | 151 | 3381 |  |  |  |  |  |  |
|  |  |  |  |  | Toxic solid, flammable, inorganic, n.o.s. | 134 | 3535 |  |
| Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) | 151 | 3382 |  |  |  |  |  |  |
|  |  |  |  |  | Toxic solid, flammable, organic, n.o.s. | 134 | 2930 |  |
| Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 142 | 3387 |  |  | Toxic solid, inorganic, n.o.s. | 151 | 3288 |  |
|  |  |  |  |  | Toxic solid, organic, n.o.s. | 154 | 2811 |  |
| Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 142 | 3388 |  |  | Toxic solid, oxidizing, n.o.s. | 141 | 3086 |  |
|  |  |  |  |  | Toxic solid, self-heating, n.o.s. | 136 | 3124 |  |
|  |  |  |  |  | Toxic solid, water-reactive, n.o.s. | 139 | 3125 |  |
| Toxic by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone A) | 155 | 3490 |  |  |  |  |  |  |
|  |  |  |  |  | Toxins, extracted from living sources, liquid, n.o.s. | 152 | 3172 |  |
|  |  |  |  |  | Toxins, extracted from living sources, solid, n.o.s. | 152 | 3462 |  |

Page 143

| Name of Material Guide ID No. No. |  |  | Name of Material Guide ID No. No. |  |  |
| --- | --- | --- | --- | --- | --- |
| Triallylamine | 132 | 2610 | 1,1,1-Trifluoroethane | 115 | 2035 |
| Triallyl borate | 156 | 2609 | Trifluoromethane | 126 | 1984 |
| Triazine pesticide, liquid, flammable, poisonous | 131 | 2764 | Trifluoromethane, refrigerated liquid | 120 | 3136 |
| Triazine pesticide, liquid, | 131 | 2764 | 2-Trifluoromethylaniline | 153 | 2942 |

| flammable, toxic Triazine pesticide, liquid, 151 2998 poisonous Triazine pesticide, liquid, 131 2997 poisonous, flammable Triazine pesticide, liquid, toxic 151 2998 Triazine pesticide, liquid, toxic, 131 2997 flammable Triazine pesticide, solid, 151 2763 poisonous |  |  |  |
| --- | --- | --- | --- |
| Triazine pesticide, solid, toxic | 151 | 2763 |  |
| Tributylamine | 153 | 2542 |  |
| Tributylphosphane | 135 | 3254 |  |
| Trichloroacetic acid | 153 | 1839 |  |
| Trichloroacetic acid, solution | 153 | 2564 |  |
| Trichloroacetyl chloride | 156 | 2442 |  |
| Trichlorobenzenes, liquid | 153 | 2321 |  |
| Trichlorobutene | 152 | 2322 |  |
| 1,1,1-Trichloroethane | 160 | 2831 |  |
| Trichloroethylene | 160 | 1710 |  |
| Trichloroisocyanuric acid, dry | 140 | 2468 |  |
| Trichlorosilane | 139 | 1295 |  |
| Tricresyl phosphate | 151 | 2574 |  |
| Triethylamine | 132 | 1296 |  |
| Triethylenetetramine | 153 | 2259 |  |
| Triethyl phosphite | 130 | 2323 |  |
| Trifluoroacetic acid | 154 | 2699 |  |
| Trifluoroacetyl chloride | 125 | 3057 |  |
| Trifluorochloroethylene, | 119P | 1082 |  |
| stabilized |  |  |  |

Trifluoromethylaniline	153	2948

| Trimethoxysilane | 132 | 9269 |
| --- | --- | --- |
| Trimethylacetyl chloride | 131 | 2438 |

Trifluoromethyltetrazole- sodium salt in acetone

113	3555

| Triisobutylene | 128 | 2324 |
| --- | --- | --- |
| Triisopropyl borate | 129 | 2616 |

| Trimethylamine, anhydrous | 118 | 1083 |
| --- | --- | --- |
| Trimethylamine, aqueous solution | 132 | 1297 |
| 1,3,5-Trimethylbenzene | 129 | 2325 |
| Trimethyl borate | 129 | 2416 |
| Trimethylchlorosilane | 155 | 1298 |
| Trimethylcyclohexylamine | 153 | 2326 |
| Trimethylhexamethylenediamines | 153 | 2327 |
| Trimethylhexamethylene diisocyanate Trimethyl phosphite | 156 130 | 2328 2329 |
| Trinitrobenzene, wetted with | 113 | 3367 |
| not less than 10% water |  |  |
| Trinitrobenzene, wetted with | 113 | 1354 |
| not less than 30% water |  |  |
| Trinitrobenzoic acid, wetted | 113 | 3368 |

Page 144

with not less than 10% water

Trinitrobenzoic acid, wetted with not less than 30% water

Trinitrochlorobenzene, wetted with not less than 10% water

Trinitrophenol, wetted with not less than 10% water

Trinitrophenol, wetted with not less than 30% water

113	1355

113	3365

113	3364

113	1344

Name of Material

Guide		ID	Name of Material No.	No.

Guide	ID

No.	No.

| Trinitrotoluene, wetted with not 113 3366 less than 10% water Trinitrotoluene, wetted with not 113 1356 less than 30% water Tripropylamine 132 2260 Tripropylene 128 2057 Tris-(1-aziridinyl)phosphine 152 2501 oxide, solution Tungsten hexafluoride 125 2196 Turpentine 128 1299 Turpentine substitute 128 1300 Undecane 128 2330 |  | Vehicle, flammable gas 115 3166 powered Vehicle, flammable liquid 128 3166 powered Vehicle, fuel cell, flammable 115 3166 gas powered Vehicle, fuel cell, flammable 128 3166 liquid powered Vehicle, lithium ion battery 147 3556 powered Vehicle, lithium metal battery 138 3557 powered Vehicle, sodium ion battery 147 3558 powered Vinyl acetate, stabilized 129P 1301 Vinyl bromide, stabilized 116P 1085 Vinyl butyrate, stabilized 129P 2838 Vinyl chloride, stabilized 116P 1086 Vinyl chloroacetate 155 2589 Vinyl ethyl ether, stabilized 127P 1302 Vinyl fluoride, stabilized 116P 1860 Vinylidene chloride, stabilized 130P 1303 Vinyl isobutyl ether, stabilized 127P 1304 Vinyl methyl ether, stabilized 116P 1087 Vinylpyridines, stabilized 131P 3073 Vinyltoluenes, stabilized 130P 2618 Vinyltrichlorosilane 155P 1305 Water-reactive liquid, 138 3129 corrosive, n.o.s. Water-reactive liquid, n.o.s. 138 3148 Water-reactive liquid, 139 3130 poisonous, n.o.s. Water-reactive liquid, toxic, 139 3130 n.o.s. Water-reactive solid, corrosive, 138 3131 n.o.s. |  |
| --- | --- | --- | --- |
| Uranium hexafluoride, 166 3507 radioactive material, excepted package, less than 0.1 kg per package, non- fissile or fissile-excepted |  |  |  |
| Uranium hexafluoride, 166 2977 radioactive material, fissile |  |  |  |
| Uranium hexafluoride, 166 2978 radioactive material, non fissile or fissile-excepted |  |  |  |
| Urea hydrogen peroxide 140 1511 Urea nitrate, wetted with not 113 3370 less than 10% water Urea nitrate, wetted with not 113 1357 less than 20% water Valeraldehyde 129 2058 Valeryl chloride 132 2502 Vanadium compound, n.o.s. 151 3285 Vanadium oxytrichloride 137 2443 Vanadium pentoxide 151 2862 Vanadium tetrachloride 137 2444 Vanadium trichloride 157 2475 Vanadyl sulfate 151 2931 Vanadyl sulphate 151 2931 |  |  |  |

Page 145

| Name of Material G Water-reactive solid, flammable, n.o.s. | uide No. 138 | ID No. 3132 | Name of Material G Zinc arsenite | uide No. 151 | ID No. 1712 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Zinc ashes | 138 | 1435 |
| Water-reactive solid, n.o.s. | 138 | 2813 | Zinc bromate | 140 | 2469 |
| Water-reactive solid, oxidizing, n.o.s. | 138 | 3133 | Zinc chlorate | 140 | 1513 |
| Water-reactive solid, poisonous, n.o.s. | 139 | 3134 | Zinc chloride, anhydrous | 154 | 2331 |
|  |  |  | Zinc chloride, solution | 154 | 1840 |
| Water-reactive solid, self- heating, n.o.s. | 138 | 3135 | Zinc cyanide | 151 | 1713 |
| Water-reactive solid, toxic, n.o.s. | 139 | 3134 | Zinc dithionite | 171 | 1931 |
|  |  |  | Zinc dross | 138 | 1435 |
| Wheelchair, electric, with batteries | 154 | 3171 | Zinc dust | 138 | 1436 |
|  |  |  | Zinc fluorosilicate | 151 | 2855 |
| White phosphorus, dry or under water or in solution | 136 | 1381 | Zinc hydrosulfite | 171 | 1931 |
| White phosphorus, molten | 136 | 2447 | Zinc hydrosulphite | 171 | 1931 |
| Wood preservatives, liquid | 129 | 1306 | Zinc nitrate | 140 | 1514 |
| Wool waste, wet | 133 | 1387 | Zinc permanganate | 140 | 1515 |
| Xanthates | 135 | 3342 | Zinc peroxide | 143 | 1516 |
| Xenon | 120 | 2036 | Zinc phosphide | 139 | 1714 |
| Xenon, compressed | 120 | 2036 | Zinc powder | 138 | 1436 |
| Xenon, refrigerated liquid (cryogenic liquid) | 120 | 2591 | Zinc residue | 138 | 1435 |
|  |  |  | Zinc resinate | 133 | 2714 |
| Xylenes | 130 | 1307 | Zinc silicofluoride | 151 | 2855 |
| Xylenols, liquid | 153 | 3430 | Zinc skimmings | 138 | 1435 |
| Xylenols, solid | 153 | 2261 | Zirconium, dry, coiled wire, finished metal sheets, strip | 170 | 2858 |
| Xylidines, liquid | 153 | 1711 |  |  |  |
| Xylidines, solid | 153 | 3452 | Zirconium, dry, finished sheets, strip or coiled wire | 135 | 2009 |
| Xylyl bromide, liquid | 152 | 1701 | Zirconium hydride | 138 | 1437 |
| Xylyl bromide, solid | 152 | 3417 | Zirconium nitrate | 140 | 2728 |
| Yellow phosphorus, dry or under water or in solution | 136 | 1381 | Zirconium picramate, wetted with not less than 20% water | 113 | 1517 |
| Zinc ammonium nitrite | 140 | 1512 | Zirconium powder, dry | 135 | 2008 |
| Zinc arsenate | 151 | 1712 | Zirconium powder, wetted with not less than 25% water | 170 | 1358 |
| Zinc arsenate and zinc arsenite mixture | 151 | 1712 |  |  |  |
|  |  |  | Zirconium scrap | 135 | 1932 |

Page 146

###### Name of Material	Guide No.

ID

###### No.

Name of Material	Guide No.

ID

###### No.

| Zirconium suspended in a flammable liquid | 170 | 1308 |
| --- | --- | --- |
| Zirconium suspended in a liquid (flammable) | 170 | 1308 |
| Zirconium tetrachloride | 137 | 2503 |

Page 147

SUGGESTED OPERATIONS SHOULD ONLY BE PERFORMED BY ADEQUATELY TRAINED AND EQUIPPED PERSONNEL

HOWTO USE THE ORANGE GUIDES

117

GUIDE Gases - Toxic - Flammable

(Extreme Hazard)

117

Gases - Toxic - Flammable  GUIDE

(Extreme Hazard)

POTENTIAL HAZARDS

HEALTH

- TOXIC; Extremely Hazardous.

- May be fatal if inhaled or absorbed through skin.

- Initial odor may be irritating or foul and may deaden your sense of smell.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- These materials are extremely flammable.

- May formexplosive mixtures with air.

- May be ignited by heat, sparks or flames.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff may create fire or explosion hazard.

- Cylinders exposed to fire may vent and release toxic and flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NORISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

EMERGENCY RESPONSE

FIRE

- DONOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

- Dry chemical, CO2, water spray or regular foam.

Large Fire

- Water spray, fog or regular foam.

- If it can be done safely, move undamaged containers away fromthe area around the fire.

- Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

- Fight fire frommaximumdistance or use unmanned master stream devices or monitor nozzles.

- Cool containers with flooding quantities of water until well after fire is out.

- Do not direct water at source of leak or safety devices; icing may occur.

- Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

- ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

- ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

- All equipment used when handling the product must be grounded.

- Do not touch or walk through spilled material.

- Stop leak if you can do it without risk.

- Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

- Do not direct water at spill or source of leak.

- If possible, turn leaking containers so that gas escapes rather than liquid.

- Prevent entry into waterways, sewers, basements or confined areas.

- Isolate area until gas has dispersed.

- Consider igniting spill or leak to eliminate toxic gas concerns.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

- In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

- In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" Section.

Page 164

ERG 2024

ERG 2024

Page 165

##### GUIDE NUMBER AND TITLE

- The guide title identifies the general hazards associated with the materials in this Guide.

##### POTENTIAL HAZARDS

- Emergency responders should consult this section first!

- Describesthematerial hazardintermsof FIREOREXPLOSIONandHEALTH effects upon exposure.

- The primary potential hazard is listed first.

- Allows the responders to make decisions to protect the emergency response team, and the surrounding population.

Page 148

ERG 2024

#### SUGGESTED OPERATIONS SHOULD ONLY BE PERFORMED BY ADEQUATELY TRAINED AND EQUIPPED PERSONNEL

##### PUBLIC SAFETY

  - This section is divided into three subsections:

› General Information: describesinitial precautionarymeasurestobetaken by those first on the scene.

› PROTECTIVE CLOTHING: provides general guidance on personal protective equipment requirements including respiratory protection. The protective clothing information is general and correct selection issituation dependent, after considering the physical and chemical properties of the material, weather conditions, spill versus fire, topography, etc.

› EVACUATION: suggestsprotectivedistancesfor immediateprecautionary measures definedfor small and large spills, including suggested guidance for conditions where fire is present or likely (potential fragmentation hazard).

− The term“isolate” indicates azone of no entry that applies tothe public and first responders who are not equipped, trained, and prepared to mitigate the incident.

− The term “evacuate” aims to protect as many people as possible by removing persons from inside a zone safely. If removal is too risky, sheltering-in-place can also be considered in this zone.

  - Materials highlightedingreenintheyellowand bluesections direct thereader to consult Table 1, detailing specific response distances for toxic inhalation hazard materials and water-reactive materials (green section).

##### EMERGENCY RESPONSE

  - This section is divided into three subsections:

› FIRE: provides extinguishing procedures for Small Fire, Large Fire, and/ or Fire Involving Tanks or Car/Trailer Loads

› SPILL ORLEAK: includes general recommendations, and may describe the response procedure for Small Spill and Large Spill

› FIRST AID: provides specific first aid guidance to use for a product or a guide in addition tothe general first aid guidance for hazardous materials/ dangerous goods incidents. General first aid guidance is found in the “General First Aid” section situated immediately after the “How to use the Orange Guides” section.

If a Canadian flag appears inthis section, and the incident is located in Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product.

##### ERG 2024

Page 149

# GENERAL FIRST AID

- Call 911 or emergency medical service.

- Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and avoid contamination.

- Move victimto fresh air if it can be done safely.

- Administer oxygen if breathing is difficult.

- If victimis not breathing:

  - DO NOT perform mouth-to-mouth resuscitation; the victim may have ingested or inhaled the substance.

  - If equipped and pulse detected, wash face and mouth, then give artificial respiration using a proper respiratory medical device (bag-valve mask, pocket mask equipped with a one-way valve or other device).

  - If nopulsedetectedor norespiratorymedical deviceavailable, providecontinuous compressions. Conduct apulsecheck every twominutes or monitor for any signs of spontaneous respirations.

- Remove and isolate contaminated clothing and shoes.

- For minor skin contact, avoid spreading material on unaffected skin.

- In case of contact with substance, remove immediately by flushing skin or eyes with running water for at least 20 minutes.

- For severe burns, immediate medical attention is required.

- Effectsof exposure(inhalation, ingestion, or skincontact) tosubstancemaybedelayed.

- Keep victimcalm and warm.

- Keep victim under observation.

- For further assistance, contact your local Poison Control Center.

- Note: Basic Life Support (BLS) and Advanced Life Support (ALS) should be done by trained professionals.

Page 150

ERG 2024

NOTES

ERG 2024

Page 151

# GUIDE 111

#### Mixed Load/Unidentified Cargo

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May explode from heat, shock, friction or contamination.

- May react violently or explosively on contact with air, water or foam.

- May be ignited by heat, sparks or flames.

- Vapors may travel to source of ignition and flash back.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

HEALTH

- Inhalation, ingestion or contact with substance may cause severe injury, infection, disease or death.

- High concentration of gas may cause asphyxiation without warning.

- Contact may cause burns to skin and eyes.

- Fire or contact with water may produce irritating, toxic and/or corrosive gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 152	ERG 2024

FIRE

#### Mixed Load/Unidentified Cargo

EMERGENCY RESPONSE

# GUIDE

## 111

CAUTION: Material may react with extinguishing agent. Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not get water inside containers.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Prevent entry into waterways, sewers, basements or confined areas.

Small Spill

  - Pick up with sand or other non-combustible absorbent material and place into containers for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

FIRST AID

Refer to the “General First Aid” section.

ERG 2024	Page 153

GUIDE	Explosives* - Division 1.1, 1.2, 1.3 or 1.5

## 112

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- MAY EXPLODE AND THROWFRAGMENTS 1600 METERS (1 MILE) OR MORE IF FIRE REACHES CARGO.

HEALTH

- Fire may produce irritating, corrosive and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Move people out of line of sight of the scene and away from windows.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area immediately for at least 500 meters (1/3 mile) in all directions.

Large Spill

- Consider initial evacuation for 800 meters (1/2 mile) in all directions. Fire

- If rail car or trailer is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, initiate evacuation including emergency responders for 1600 meters (1 mile) in all directions.

* For information on "Compatibility Group" letters, refer to the Glossary section.

Page 154	ERG 2024

Explosives* - Division 1.1, 1.2, 1.3 or 1.5	GUIDE

## 112

##### EMERGENCY RESPONSE

FIRE

CARGO Fire

- DO NOT fight fire when fire reaches cargo! Cargo may EXPLODE!

- Stop all traffic and clear the area for at least 1600 meters (1 mile) in all directions and let burn.

- Do not move cargo or vehicle if cargo has been exposed to heat. TIRE or VEHICLE Fire

- Use plenty of water - FLOOD it! If water is not available, use CO2, dry chemical or dirt.

- If possible, and WITHOUT RISK, use unmanned master stream devices or monitor nozzles from maximum distance to prevent fire fromspreading to cargo area.

- Pay special attention to tire fires as re-ignition may occur. Stand by, at a safe distance, with extinguisher ready for possible re-ignition.

SPILL OR LEAK

- ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

- All equipment used when handling the product must be grounded.

- Do not touch or walk through spilled material.

- DO NOT OPERATE RADIO TRANSMITTERS WITHIN 100 METERS (330 FEET) OF ELECTRIC DETONATORS.

- DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 155

# GUIDE 113

#### Flammable Materials

(Wet/Desensitized  Explosive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Flammable/combustible material.

- May be ignited by heat, sparks or flames.

- DRIED OUT material may explode if exposed to heat, flame, friction or shock; treat as an explosive (GUIDE 112).

- Keep material wet with water or treat as an explosive (GUIDE 112).

- Runoff to sewer may create fire or explosion hazard.

HEALTH

- Some are toxic and may be fatal if inhaled, ingested or absorbed through skin. Specifically, Dinitrophenol, wetted (UN1320); Dinitrophenolates, wetted (UN1321), Sodium dinitro-o-cresolate, wetted (UN1348); and Barium azide, wetted (UN1571) are known to be toxic.

- Contact may cause burns to skin and eyes.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff from fire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area immediately for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial evacuation for 500 meters (1/3 mile) in all directions. Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 156	ERG 2024

FIRE

CARGO Fire

#### Flammable Materials (Wet/Desensitized  Explosive)

EMERGENCY RESPONSE

# GUIDE

## 113

  - DO NOT fight fire when fire reaches cargo! Cargo may EXPLODE!

  - Stop all traffic and clear the area for at least 1600 meters (1 mile) in all directions and let burn.

  - Do not move cargo or vehicle if cargo has been exposed to heat. TIRE or VEHICLE Fire

  - Use plenty of water - FLOOD it! If water is not available, use CO2, dry chemical or dirt.

  - If possible, and WITHOUT RISK, use unmanned master stream devices or monitor nozzles from maximum distance to prevent fire fromspreading to cargo area.

  - Pay special attention to tire fires as re-ignition may occur. Stand by, at a safe distance, with extinguisher ready for possible re-ignition.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

Small Spill

  - Flush area with large amounts of water.

Large Spill

  - Wet down with water and dike for later disposal.

  - KEEP “WETTED” PRODUCT WET BY SLOWLY ADDING FLOODING QUANTITIES OF WATER.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 157

GUIDE	Explosives* - Division 1.4 or 1.6

## 114

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- MAY EXPLODE AND THROWFRAGMENTS 800 METERS (1/2 MILE) OR MORE IF FIRE REACHES CARGO.

HEALTH

- Fire may produce irritating, corrosive and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Move people out of line of sight of the scene and away from windows.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area immediately for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial evacuation for 250 meters (800 feet) in all directions. Fire

- If rail car or trailer is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also initiate evacuation including emergency responders for 800 meters (1/2 mile) in all directions.

- If fire threatens cargo area containing packages bearing the 1.4S label or packages containing material classified as 1.4S, consider isolating at least 15 meters (50 feet) in all directions.

* For information on "Compatibility Group" letters, refer to the Glossary section.

Page 158	ERG 2024

FIRE

CARGO Fire

#### Explosives* - Division 1.4 or 1.6

EMERGENCY RESPONSE

# GUIDE

## 114

- DO NOT fight fire when fire reaches cargo! Cargo may EXPLODE!

- Stop all traffic and clear the area for at least 800 meters (1/2 mile) in all directions and let burn.

- Do not move cargo or vehicle if cargo has been exposed to heat. TIRE or VEHICLE Fire

- Use plenty of water - FLOOD it! If water is not available, use CO2, dry chemical or dirt.

- If possible, and WITHOUT RISK, use unmanned master stream devices or monitor nozzles from maximum distance to prevent fire fromspreading to cargo area.

- Pay special attention to tire fires as re-ignition may occur. Stand by, at a safe distance, with extinguisher ready for possible re-ignition.

CLASS 1.4S Fire

- Packages bearing the 1.4S label or packages containing material classified as 1.4S are designed or packaged in such a manner that when involved in a fire, they may burn vigorously with localized detonations and projection of fragments.

- Effects are usually confined to immediate vicinity of packages.

- Fight fire with normal precautions froma reasonable distance.

SPILL OR LEAK

- ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

- All equipment used when handling the product must be grounded.

- Do not touch or walk through spilled material.

- DO NOT OPERATE RADIO TRANSMITTERS WITHIN 100 METERS (330 FEET) OF ELECTRIC DETONATORS.

- DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 159

# GUIDE 115

#### Gases - Flammable

(Including Refrigerated Liquids)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- EXTREMELY FLAMMABLE.

- Will be easily ignited by heat, sparks or flames.

- Will formexplosive mixtures with air.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

CAUTION: Hydrogen (UN1049), Deuterium (UN1957), Hydrogen, refrigerated liquid (UN1966), Methane (UN1971) and Hydrogen and Methane mixture, compressed (UN2034) are lighter than air and will rise. Hydrogen and Deuterium fires are difficult to detect since they burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

- Vapors may travel to source of ignition and flash back.

- Cylinders exposed to fire may vent and release flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

CAUTION: When LNG – Liquefied natural gas (UN1972) is released on or near water, product may vaporize explosively.

HEALTH

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Some may be irritating if inhaled at high concentrations.

- Contact with gas, liquefied gas or cryogenic liquids may cause burns, severe injury and/or frostbite.

- Fire may produce irritating and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

- Always wear thermal protective clothing when handling refrigerated/cryogenic liquids.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 800 meters (1/2 mile).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

- In fires involving Liquefied Petroleum Gases (LPG) (UN1075), Butane (UN1011), Butylene (UN1012), Isobutylene (UN1055), Propylene (UN1077), Isobutane (UN1969), and Propane (UN1978), also refer to the "BLEVE – Safety Precautions" section.

Page 160	ERG 2024

FIRE

#### Gases - Flammable (Including Refrigerated Liquids)

EMERGENCY RESPONSE

# GUIDE

## 115

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

CAUTION: Hydrogen (UN1049), Deuterium (UN1957), Hydrogen, refrigerated liquid (UN1966) and Hydrogen and Methane mixture, compressed (UN2034) will burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray or fog.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

CAUTION: For LNG - Liquefied natural gas (UN1972) pool fires, DO NOT USE water. Use dry chemical or high-expansion foam.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

CAUTION: For LNG - Liquefied natural gas (UN1972), DO NOT apply water, regular or alcohol-resistant foam directly on spill. Use a high-expansion foam if available to reduce vapors.

  - Prevent spreading of vapors through sewers, ventilation systems and confined areas.

  - Isolate area until gas has dispersed.

CAUTION: When in contact with refrigerated/cryogenic liquids, many materials become brittle and are likely to break without warning.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Clothing frozen to the skin should be thawed before being removed.

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 161

# GUIDE 116

#### Gases - Flammable (Unstable)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- EXTREMELY FLAMMABLE.

- Will be easily ignited by heat, sparks or flames.

- Will formexplosive mixtures with air. Acetylene (UN1001, UN3374) may react explosively even in the absence of air.

- Disilane (UN3553) and Silane (UN2203) will ignite spontaneously in air and may re-ignite.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Cylinders exposed to fire may vent and release flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

HEALTH

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Some may be toxic if inhaled at high concentrations.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire may produce irritating and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 800 meters (1/2 mile).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 162	ERG 2024

FIRE

#### Gases - Flammable (Unstable)

EMERGENCY RESPONSE

# GUIDE

## 116

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray or fog.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Stop leak if you can do it without risk.

  - Do not touch or walk through spilled material.

  - Do not direct water at spill or source of leak.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 163

# GUIDE 117

HEALTH

#### Gases - Toxic - Flammable (Extreme Hazard)

##### POTENTIAL HAZARDS

- TOXIC; Extremely Hazardous.

- May be fatal if inhaled or absorbed through skin.

- Initial odor may be irritating or foul and may deaden your sense of smell.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- These materials are extremely flammable.

- May formexplosive mixtures with air.

- May be ignited by heat, sparks or flames.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff may create fire or explosion hazard.

- Cylinders exposed to fire may vent and release toxic and flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 164	ERG 2024

FIRE

#### Gases - Toxic - Flammable

(Extreme Hazard)

EMERGENCY RESPONSE

# GUIDE

## 117

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Isolate area until gas has dispersed.

  - Consider igniting spill or leak to eliminate toxic gas concerns.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 165

# GUIDE 118

#### Gases - Flammable - Corrosive

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- EXTREMELY FLAMMABLE.

- May be ignited by heat, sparks or flames.

- May formexplosive mixtures with air.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Some of these materials may react violently with water.

- Cylinders exposed to fire may vent and release flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

HEALTH

- May cause toxic effects if inhaled.

- Vapors are extremely irritating.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 166	ERG 2024

FIRE

#### Gases - Flammable - Corrosive

EMERGENCY RESPONSE

# GUIDE

## 118

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 167

# GUIDE 119

HEALTH

#### Gases - Toxic - Flammable

##### POTENTIAL HAZARDS

- TOXIC; may be fatal if inhaled or absorbed through skin. Some may cause severe skin burns and eye damage.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Flammable; may be ignited by heat, sparks or flames.

- May formexplosive mixtures with air. Ethylene oxide (UN1040) may react explosively even in the absence of air.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Some of these materials may react violently with water.

- Cylinders exposed to fire may vent and release toxic and flammable gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

- Runoff may create fire or explosion hazard.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 168	ERG 2024

FIRE

#### Gases - Toxic - Flammable

EMERGENCY RESPONSE

# GUIDE

## 119

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - FOR CHLOROSILANES, DO NOT USE WATER; use alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Do not direct water at spill or source of leak.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - FOR CHLOROSILANES, use alcohol-resistant foam to reduce vapors.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 169

# GUIDE 120

HEALTH

#### Gases - Inert

(Including Refrigerated Liquids)

##### POTENTIAL HAZARDS

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Contact with gas, liquefied gas or cryogenic liquids may cause burns, severe injury and/or frostbite.

FIRE OR EXPLOSION

- Non-flammable gases.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

- Always wear thermal protective clothing when handling refrigerated/cryogenic liquids or solids.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 170	ERG 2024

FIRE

#### Gases - Inert (Including Refrigerated Liquids)

EMERGENCY RESPONSE

# GUIDE

## 120

  - Use extinguishing agent suitable for type of surrounding fire.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Allow substance to evaporate.

  - Ventilate the area.

CAUTION: When in contact with refrigerated/cryogenic liquids, many materials become brittle and are likely to break without warning.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Clothing frozen to the skin should be thawed before being removed.

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

ERG 2024	Page 171

# GUIDE 121

Intentionally left blank

There are no materials that refer to this guide.

Page 172	ERG 2024

# GUIDE

## 121

Intentionally left blank

There are no materials that refer to this guide.

ERG 2024	Page 173

# GUIDE 122

#### Gases - Oxidizing

(Including Refrigerated Liquids)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Substance does not burn but will support combustion.

- Some may react explosively with fuels.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Runoff may create fire or explosion hazard.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

HEALTH

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Contact with gas, liquefied gas or cryogenic liquids may cause burns, severe injury and/or frostbite.

- Fire may produce irritating and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

- Always wear thermal protective clothing when handling refrigerated/cryogenic liquids.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 500 meters (1/3 mile).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 174	ERG 2024

FIRE

#### Gases - Oxidizing (Including Refrigerated Liquids)

EMERGENCY RESPONSE

# GUIDE

## 122

  - Use extinguishing agent suitable for type of surrounding fire.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Do not direct water at spill or source of leak.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Allow substance to evaporate.

  - Isolate area until gas has dispersed.

CAUTION: When in contact with refrigerated/cryogenic liquids, many materials become brittle and are likely to break without warning.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Clothing frozen to the skin should be thawed before being removed.

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 175

# GUIDE 123

HEALTH

#### Gases - Toxic

##### POTENTIAL HAZARDS

- TOXIC; may be fatal if inhaled or absorbed through skin.

- Vapors may be irritating and/or corrosive.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Some may burn but none ignite readily.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Cylinders exposed to fire may vent and release toxic and/or corrosive gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 176	ERG 2024

FIRE

Small Fire

  - Dry chemical or CO2.

Large Fire

#### Gases - Toxic

EMERGENCY RESPONSE

# GUIDE

## 123

  - Water spray, fog or regular foam.

  - Do not get water inside containers.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 177

# GUIDE 124

HEALTH

#### Gases - Toxic and/or Corrosive - Oxidizing

##### POTENTIAL HAZARDS

- TOXIC and/or CORROSIVE; may be fatal if inhaled or absorbed through skin.

- Fire will produce irritating, corrosive and/or toxic gases.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Substance does not burn but will support combustion.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- These are strong oxidizers and will react vigorously or explosively with many materials including fuels.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Some will react violently with air, moist air and/or water.

- Cylinders exposed to fire may vent and release toxic and/or corrosive gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 178	ERG 2024

FIRE

#### Gases - Toxic and/or Corrosive -

Oxidizing

EMERGENCY RESPONSE

# GUIDE

## 124

CAUTION: These materials do not burn but will support combustion. Some will react violently with water.

Small Fire

  - Contain fire and let burn. If fire must be fought, water spray or fog is recommended.

  - Water only; no dry chemical, CO2 or Halon®.

  - Do not get water inside containers.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Isolate area until gas has dispersed.

  - Ventilate the area.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Clothing frozen to the skin should be thawed before being removed.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 179

# GUIDE 125

HEALTH

#### Gases - Toxic and/or Corrosive

##### POTENTIAL HAZARDS

- TOXIC and/or CORROSIVE; may be fatal if inhaled, ingested or absorbed through skin.

- Vapors are extremely irritating and corrosive.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Some may burn but none ignite readily.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Some of these materials may react violently with water.

- Cylinders exposed to fire may vent and release toxic and/or corrosive gas through pressure relief devices.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

- For UN1005: Anhydrous ammonia, at high concentrations in confined spaces, presents a flammability risk if a source of ignition is introduced.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 180	ERG 2024

FIRE

Small Fire

  - Dry chemical or CO2.

Large Fire

#### Gases - Toxic and/or Corrosive

EMERGENCY RESPONSE

# GUIDE

## 125

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not get water inside containers.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Do not direct water at spill or source of leak.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

  - In case of skin contact with hydrogen fluoride, anhydrous (UN1052), if calciumgluconate gel is available, rinse 5 minutes, then apply gel. Otherwise, continue rinsing until medical treatment is available.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 181

# GUIDE 126

#### Gases - Compressed or Liquefied (Including Refrigerant Gases)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Some may burn but none ignite readily.

- Containers may explode when heated.

- Ruptured cylinders may rocket.

CAUTION: Aerosols (UN1950) may contain a flammable propellant.

HEALTH

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Contact with gas or liquefied gas may cause burns, severe injury and/or frostbite.

- Fire may produce irritating, corrosive and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 500 meters (1/3 mile).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 182	ERG 2024

FIRE

#### Gases - Compressed or Liquefied (Including Refrigerant Gases)

EMERGENCY RESPONSE

# GUIDE

## 126

  - Use extinguishing agent suitable for type of surrounding fire.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - Some of these materials, if spilled, may evaporate leaving a flammable residue.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Do not direct water at spill or source of leak.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Allow substance to evaporate.

  - Ventilate the area.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

ERG 2024	Page 183

# GUIDE 127

#### Flammable Liquids (Water-Miscible)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

CAUTION: Ethanol (UN1170) can burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

HEALTH

- Inhalation or contact with material may irritate or burn skin and eyes.

- Fire may produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 300 meters (1000 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 184	ERG 2024

FIRE

#### Flammable Liquids (Water-Miscible)

EMERGENCY RESPONSE

# GUIDE

## 127

CAUTION: The majority of these products have a very lowflash point. Use of water spray when fighting fire may be inefficient.

CAUTION: For fire involving UN1170, UN1987 or UN3475, alcohol-resistant foamshould be used.

CAUTION: Ethanol (UN1170) can burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

Small Fire

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - Avoid aiming straight or solid streams directly onto the product.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - Use clean, non-sparking tools to collect absorbed material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Water spray may reduce vapor, but may not prevent ignition in closed spaces.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 185

# GUIDE 128

#### Flammable Liquids (Water-Immiscible)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

- Substance may be transported hot.

- For hybrid vehicles, GUIDE 147 (lithium ion or sodium ion batteries) or GUIDE 138 (sodium batteries) should also be consulted.

- If molten aluminum is involved, refer to GUIDE 169.

HEALTH

CAUTION: Petroleum crude oil (UN1267) may contain TOXIC hydrogen sulphide gas.

- Inhalation or contact with material may irritate or burn skin and eyes.

- Fire may produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 300 meters (1000 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 186	ERG 2024

FIRE

#### Flammable Liquids (Water-Immiscible)

EMERGENCY RESPONSE

# GUIDE

## 128

CAUTION: The majority of these products have a very lowflash point. Use of water spray when fighting fire may be inefficient.

CAUTION: For mixtures containing alcohol or polar solvent, alcohol-resistant foam may be more effective.

Small Fire

  - Dry chemical, CO2, water spray or regular foam. If regular foam is ineffective or unavailable, use alcohol-resistant foam.

Large Fire

  - Water spray, fog or regular foam. If regular foam is ineffective or unavailable, use alcohol-resistant foam.

  - Avoid aiming straight or solid streams directly onto the product.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - For petroleum crude oil, do not spray water directly into a breached tank car. This can lead to a dangerous boil over.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - Use clean, non-sparking tools to collect absorbed material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Water spray may reduce vapor, but may not prevent ignition in closed spaces.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

  - In case of burns, immediately cool affected skin for as long as possible with cold water.Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 187

# GUIDE 129

#### Flammable Liquids

(Water-Miscible/Noxious)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

HEALTH

- May cause toxic effects if inhaled or absorbed through skin.

- Inhalation or contact with material may irritate or burn skin and eyes.

- Fire will produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 300 meters (1000 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 188	ERG 2024

FIRE

#### Flammable Liquids (Water-Miscible/Noxious)

EMERGENCY RESPONSE

# GUIDE

## 129

CAUTION: The majority of these products have a very lowflash point. Use of water spray when fighting fire may be inefficient.

Small Fire

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

  - Do not use dry chemical extinguishers to control fires involving nitromethane (UN1261) or nitroethane (UN2842).

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - Avoid aiming straight or solid streams directly onto the product.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - Use clean, non-sparking tools to collect absorbed material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Water spray may reduce vapor, but may not prevent ignition in closed spaces.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 189

# GUIDE 130

#### Flammable Liquids

(Water-Immiscible/Noxious)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

HEALTH

- May cause toxic effects if inhaled or absorbed through skin.

- Inhalation or contact with material may irritate or burn skin and eyes.

- Fire will produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 300 meters (1000 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 190	ERG 2024

FIRE

#### Flammable Liquids (Water-Immiscible/Noxious)

EMERGENCY RESPONSE

# GUIDE

## 130

CAUTION: The majority of these products have a very lowflash point. Use of water spray when fighting fire may be inefficient.

Small Fire

  - Dry chemical, CO2, water spray or regular foam. If regular foam is ineffective or unavailable, use alcohol-resistant foam.

Large Fire

  - Water spray, fog or regular foam. If regular foam is ineffective or unavailable, use alcohol-resistant foam.

  - Avoid aiming straight or solid streams directly onto the product.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - Use clean, non-sparking tools to collect absorbed material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Water spray may reduce vapor, but may not prevent ignition in closed spaces.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

ERG 2024	Page 191

# GUIDE 131

HEALTH

#### Flammable Liquids - Toxic

##### POTENTIAL HAZARDS

- TOXIC; may be fatal if inhaled, ingested or absorbed through skin.

- Inhalation or contact with some of these materials will irritate or burn skin and eyes.

- Methyl chloroacetate (UN2295) is an eye irritant/lachrymator (causes flow of tears).

- Fire will produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

CAUTION: Methanol (UN1230) will burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion and poison hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 192	ERG 2024

FIRE

#### Flammable Liquids - Toxic

EMERGENCY RESPONSE

# GUIDE

## 131

CAUTION: The majority of these products have a very lowflash point. Use of water spray when fighting fire may be inefficient.

CAUTION: Methanol (UN1230) will burn with an invisible flame. Use an alternate method of detection (thermal camera, broom handle, etc.)

Small Fire

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

  - Avoid aiming straight or solid streams directly onto the product.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

Small Spill

  - Absorb with earth, sand or other non-combustible material and transfer to containers for later disposal.

  - Use clean, non-sparking tools to collect absorbed material.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

Large Spill

- Dike far ahead of liquid spill for later disposal.

- Water spray may reduce vapor, but may not prevent ignition in closed spaces.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 193

# GUIDE 132

#### Flammable Liquids - Corrosive

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Flammable/combustible material.

- May be ignited by heat, sparks or flames.

- Vapors may formexplosive mixtures with air.

- Vapors may travel to source of ignition and flash back.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapor explosion hazard indoors, outdoors or in sewers.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff to sewer may create fire or explosion hazard.

- Containers may explode when heated.

- Many liquids will float on water.

HEALTH

- May cause toxic effects if inhaled or ingested.

- Contact with substance may cause severe burns to skin and eyes.

- Fire will produce irritating, corrosive and/or toxic gases.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 194	ERG 2024

FIRE

#### Flammable Liquids - Corrosive

EMERGENCY RESPONSE

# GUIDE

## 132

  - Some of these materials may react violently with water. Small Fire

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

  - Do not get water inside containers.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - A vapor-suppressing foam may be used to reduce vapors.

  - Absorb with earth, sand or other non-combustible material.

  - For hydrazine, absorb with DRY sand or inert absorbent (vermiculite or absorbent pads).

  - Use clean, non-sparking tools to collect absorbed material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Water spray may reduce vapor, but may not prevent ignition in closed spaces.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 195

# GUIDE 133

#### Flammable Solids

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Flammable/combustible material.

- May be ignited by friction, heat, sparks or flames.

- Some may burn rapidly with flare-burning effect.

- Powders, dusts, shavings, borings, turnings or cuttings may explode or burn with explosive violence.

- Substance may be transported in a molten format a temperature that may be above its flash point.

- May re-ignite after fire is extinguished.

HEALTH

- Fire may produce irritating and/or toxic gases.

- Contact may cause burns to skin and eyes.

- Contact with molten substance may cause severe burns to skin and eyes.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 196	ERG 2024

FIRE

Small Fire

#### Flammable Solids

EMERGENCY RESPONSE

# GUIDE

## 133

  - Dry chemical, CO2, sand, earth, water spray or regular foam.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Metal Pigments or Pastes (e.g. “Aluminum Paste”)

  - Aluminum Paste fires should be treated as a combustible metal fire. Use DRY sand, graphite powder, dry sodium chloride-based extinguishers or class D extinguishers. Also, see GUIDE 170.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Cool containers with flooding quantities of water until well after fire is out.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

Small Dry Spill

  - With clean shovel, place material into clean, dry container and cover loosely; move containers fromspill area.

Large Spill

  - Wet down with water and dike for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Removal of solidified molten material from skin requires medical assistance.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 197

# GUIDE 134

#### Flammable Solids - Toxic and/or Corrosive

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Flammable/combustible material.

- May be ignited by heat, sparks or flames.

- When heated, vapors may formexplosive mixtures with air: indoors, outdoors and sewers explosion hazards.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated.

HEALTH

- TOXIC and/or CORROSIVE; inhalation, ingestion or skin contact with material may cause severe injury or death.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 198	ERG 2024

FIRE

Small Fire

#### Flammable Solids - Toxic and/or

Corrosive

EMERGENCY RESPONSE

# GUIDE

## 134

  - Dry chemical, CO2, water spray or alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Avoid aiming straight or solid streams directly onto the product.

  - Do not get water inside containers.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Stop leak if you can do it without risk.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 199

GUIDE	Substances - Spontaneously Combustible

## 135

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Flammable/combustible material.

- May ignite on contact with moist air or moisture.

- May burn rapidly with flare-burning effect.

- Some react vigorously or explosively on contact with water.

- Some may decompose explosively when heated or involved in a fire.

- May re-ignite after fire is extinguished.

- Runoff may create fire or explosion hazard.

- Containers may explode when heated.

HEALTH

- Fire will produce irritating, corrosive and/or toxic gases.

- Inhalation of decomposition products may cause severe injury or death.

- Contact with substance may cause severe burns to skin and eyes.

- Runoff fromfire control or dilution water may cause environmental contamination.

CAUTION: Pentaborane (UN1380) is highly toxic and may be fatal if inhaled, ingested or absorbed through skin.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 200	ERG 2024

Substances - Spontaneously Combustible	GUIDE

## 135

##### EMERGENCY RESPONSE

FIRE

  - DO NOT USE WATER, CO2 OR FOAM ON MATERIAL ITSELF.

  - Some of these materials may react violently with water.

CAUTION: For Xanthates, UN3342 and for Dithionite (Hydrosulfite/Hydrosulphite) UN1384, UN1923 and UN1929, USE FLOODING AMOUNTS OF WATER for SMALL AND LARGE fires to stop the reaction. Smothering will not work for these materials, they do not need air to burn.

Small Fire

  - Dry chemical, soda ash, lime or DRY sand, EXCEPT for UN1384, UN1923, UN1929 and UN3342. Large Fire

  - DRY sand, dry chemical, soda ash or lime EXCEPT for UN1384, UN1923, UN1929 and UN3342, or withdraw fromarea and let fire burn.

CAUTION: UN3342 when flooded with water will continue to evolve flammable Carbon disulfide/Carbon disulphide vapors.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers or in contact with substance.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

Small Spill

CAUTION: For spills of Xanthates, UN3342 and for Dithionite (Hydrosulfite/Hydrosulphite), UN1384, UN1923 and UN1929, dissolve in 5 parts water and collect for proper disposal.

CAUTION: UN3342 when flooded with water will continue to evolve flammable Carbon disulfide/Carbon disulphide vapors.

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 201

# GUIDE 136

#### Substances - Spontaneously Combustible - Toxic and/or Corrosive (Air-Reactive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Extremely flammable; will ignite itself if exposed to air.

- Burns rapidly, releasing dense, white, irritating fumes.

- Substance may be transported in a molten form.

- May re-ignite after fire is extinguished.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated.

HEALTH

- Fire will produce irritating, corrosive and/or toxic gases.

- TOXIC and/or CORROSIVE; ingestion of substance or inhalation of decomposition products will cause severe injury or death.

- Contact with substance may cause severe burns to skin and eyes.

- Some effects may be experienced due to skin absorption.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

- For Phosphorus (UN1381): Special aluminized protective clothing should be worn when direct contact with the substance is possible.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 202	ERG 2024

FIRE

Small Fire

#### Substances - Spontaneously Combustible - Toxic and/or Corrosive (Air-Reactive)

EMERGENCY RESPONSE

# GUIDE

## 136

  - Water spray, wet sand or wet earth.

Large Fire

  - Water spray or fog.

  - Do not scatter spilled material with high-pressure water streams.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

Small Spill

  - Cover with water, sand or earth. Shovel into metal container and keep material under water.

Large Spill

  - Dike for later disposal and cover with wet sand or earth.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with substance, keep exposed skin areas immersed in water or covered with wet bandages until medical attention is received.

  - Removal of solidified molten material from skin requires medical assistance.

  - Remove and isolate contaminated clothing and shoes at the site and place in metal container filled with water. Fire hazard if allowed to dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 203

# GUIDE 137

HEALTH

#### Substances - Water-Reactive - Corrosive

##### POTENTIAL HAZARDS

- CORROSIVE and/or TOXIC; inhalation, ingestion or contact (skin, eyes) with vapors, dusts or substance may cause severe injury, burns or death.

- Fire will produce irritating, corrosive and/or toxic gases.

- Reaction with water may generate much heat that will increase the concentration of fumes in the air.

- Contact with molten substance may cause severe burns to skin and eyes.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- EXCEPT FOR ACETIC ANHYDRIDE (UN1715), THAT IS FLAMMABLE, some of these materials may burn, but none ignite readily.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Substance will react with water (some violently), releasing corrosive and/or toxic gases and runoff.

- Flammable/toxic gases may accumulate in confined areas (basement, tanks, hopper/tank cars, etc.).

- Contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated or if contaminated with water.

- Substance may be transported in a molten form.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 204	ERG 2024

Substances - Water-Reactive - Corrosive	GUIDE

## 137

##### EMERGENCY RESPONSE

FIRE

  - When material is not involved in fire, do not use water on material itself. Small Fire

  - Dry chemical or CO2.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Large Fire

  - Flood fire area with large quantities of water, while knocking down vapors with water fog. If insufficient water supply, responders should withdraw.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not get water inside containers.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors; do not put water directly on leak, spill area or inside container.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

  - Removal of solidified molten material from skin requires medical assistance.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 205

# GUIDE 138

#### Substances - Water-Reactive (Emitting Flammable Gases)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Produce flammable gases on contact with water.

- May ignite on contact with water or moist air.

- Some react vigorously or explosively on contact with water.

- May be ignited by heat, sparks or flames.

- May re-ignite after fire is extinguished.

- Some are transported in highly flammable liquids.

- Runoff may create fire or explosion hazard.

HEALTH

- Inhalation or contact with vapors, substance or decomposition products may cause severe injury or death.

- May produce corrosive solutions on contact with water.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial downwind evacuation for at least 300 meters (1000 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 206	ERG 2024

FIRE

#### Substances - Water-Reactive (Emitting Flammable Gases)

EMERGENCY RESPONSE

# GUIDE

## 138

  - DO NOT USE WATER OR FOAM.

Small Fire

  - Dry chemical, soda ash, lime or sand.

Large Fire

  - DRY sand, dry chemical, soda ash or lime or withdraw fromarea and let fire burn.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Metals or Powders (Aluminum, Lithium, Magnesium, etc.)

  - Use dry chemical, DRY sand, sodium chloride powder, graphite powder or class D extinguishers; in addition, for Lithiumyou may use Lith-X® powder or copper powder. Also, see GUIDE 170.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - DO NOT GET WATER on spilled substance or inside containers. Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Dike for later disposal; do not apply water unless directed to do so.

Powder Spill

  - Cover powder spill with plastic sheet or tarp to minimize spreading and keep powder dry.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with substance, wipe from skin immediately; flush skin or eyes with running water for at least 20 minutes.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 207

# GUIDE 139

#### Substances - Water-Reactive

(Emitting Flammable And Toxic Gases)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Produce flammable and toxic gases on contact with water.

- May ignite on contact with water or moist air.

- Some react vigorously or explosively on contact with water.

- May be ignited by heat, sparks or flames.

- May re-ignite after fire is extinguished.

- Some are transported in highly flammable liquids.

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Highly toxic: contact with water produces toxic gas, may be fatal if inhaled.

- Inhalation or contact with vapors, substance or decomposition products may cause severe injury or death.

- May produce corrosive solutions on contact with water.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 208	ERG 2024

FIRE

#### Substances - Water-Reactive (Emitting Flammable And Toxic Gases)

EMERGENCY RESPONSE

# GUIDE

## 139

  - DO NOT USE WATER OR FOAM. (FOAM MAY BE USED FOR CHLOROSILANES, SEE BELOW)

Small Fire

  - Dry chemical, soda ash, lime or sand.

Large Fire

  - DRY sand, dry chemical, soda ash or lime or withdraw fromarea and let fire burn.

  - FOR CHLOROSILANES, DO NOT USE WATER; use alcohol-resistant foam; DO NOT USE dry chemicals, soda ash or lime on chlorosilane fires (large or small) as they may release large quantities of hydrogen gas that may explode.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not get water inside containers.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - DO NOT GET WATER on spilled substance or inside containers.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - FOR CHLOROSILANES, use alcohol-resistant foam to reduce vapors.

Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Dike for later disposal; do not apply water unless directed to do so.

Powder Spill

  - Cover powder spill with plastic sheet or tarp to minimize spreading and keep powder dry.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with substance, wipe from skin immediately; flush skin or eyes with running water for at least 20 minutes.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 209

# GUIDE 140

#### Oxidizers

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

CAUTION: Ammonium nitrate products may explode if involved in fire or contaminated with hydrocarbons (fuels), organic matter, other contaminants or when hot molten and contained. Treat as an explosive (GUIDE 112).

- These substances will accelerate burning when involved in a fire.

- Some may decompose explosively when heated or involved in a fire.

- May explode from heat or contamination.

- Some will react explosively with hydrocarbons (fuels).

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Inhalation, ingestion or contact (skin, eyes) with vapors or substance may cause severe injury, burns or death.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

- If ammonium nitrate products are in a tank, rail car or truck and involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, initiate evacuation including emergency responders for 1600 meters (1 mile) in all directions.

Page 210	ERG 2024

FIRE

Small Fire

#### Oxidizers

EMERGENCY RESPONSE

# GUIDE

## 140

  - Use water. Do not use dry chemicals or foams. CO2 or Halon® may provide limited control.

Large Fire

  - Flood fire area with water froma distance.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - For ammonium nitrate products: Do not fight cargo fire. Withdraw, evacuate and isolate area for at least 1600 meters (1 mile). Treat as an explosive (GUIDE 112). Do not enter area for 24 hours or until expert advice has been provided.

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Do not get water inside containers.

Small Dry Spill

  - With clean shovel, place material into clean, dry container and cover loosely; move containers fromspill area.

Small Liquid Spill

  - Use a non-combustible material like vermiculite or sand to soak up the product and place into a container for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 211

# GUIDE 141

#### Oxidizers - Toxic

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- These substances will accelerate burning when involved in a fire.

- May explode from heat or contamination.

- Some may burn rapidly.

- Some will react explosively with hydrocarbons (fuels).

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Toxic by ingestion.

- Inhalation of dust is toxic.

- Fire may produce irritating, corrosive and/or toxic gases.

- Contact with substance may cause severe burns to skin and eyes.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 212	ERG 2024

FIRE

Small Fire

#### Oxidizers - Toxic

EMERGENCY RESPONSE

# GUIDE

## 141

  - Use water. Do not use dry chemicals or foams. CO2 or Halon® may provide limited control.

Large Fire

  - Flood fire area with water froma distance.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

Small Dry Spill

  - With clean shovel, place material into clean, dry container and cover loosely; move containers fromspill area.

Large Spill

  - Dike far ahead of spill for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 213

# GUIDE 142

#### Oxidizers - Toxic (Liquid)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- These substances will accelerate burning when involved in a fire.

- May explode from heat or contamination.

- Some will react explosively with hydrocarbons (fuels).

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- TOXIC; inhalation, ingestion or contact (skin, eyes) with vapors or substance may cause severe injury, burns or death.

- Fire may produce irritating, corrosive and/or toxic gases.

- Toxic/flammable fumes may accumulate in confined areas (basement, tanks, tank cars, etc.).

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 214	ERG 2024

FIRE

Small Fire

#### Oxidizers - Toxic (Liquid)

EMERGENCY RESPONSE

# GUIDE

## 142

  - Use water. Do not use dry chemicals or foams. CO2 or Halon® may provide limited control.

Large Fire

  - Flood fire area with water froma distance.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift.

  - Do not get water inside containers.

Small Liquid Spill

  - Use a non-combustible material like vermiculite or sand to soak up the product and place into a container for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 215

# GUIDE 143

#### Oxidizers (Unstable)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May explode fromfriction, heat or contamination.

- These substances will accelerate burning when involved in a fire.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- Some will react explosively with hydrocarbons (fuels).

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- TOXIC; inhalation, ingestion or contact (skin, eyes) with vapors, dusts or substance may cause severe injury, burns or death.

- Fire may produce irritating and/or toxic gases.

- Toxic fumes or dust may accumulate in confined areas (basement, tanks, hopper/tank cars, etc.).

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 216	ERG 2024

FIRE

Small Fire

#### Oxidizers (Unstable)

EMERGENCY RESPONSE

# GUIDE

## 143

  - Use water. Do not use dry chemicals or foams. CO2 or Halon® may provide limited control.

Large Fire

  - Flood fire area with water froma distance.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not get water inside containers: a violent reaction may occur.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Cool containers with flooding quantities of water until well after fire is out.

  - Dike runoff fromfire control for later disposal.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Use water spray to reduce vapors or divert vapor cloud drift.

  - Prevent entry into waterways, sewers, basements or confined areas.

Small Spill

  - Flush area with large amounts of water.

Large Spill

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 217

# GUIDE 144

#### Oxidizers (Water-Reactive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- React vigorously and/or explosively with water.

- Produce toxic and/or corrosive substances on contact with water.

- Flammable/toxic gases may accumulate in tanks and hopper cars.

- Some may produce flammable hydrogen gas upon contact with metals.

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- TOXIC; inhalation or contact with vapor, substance, or decomposition products may cause severe injury or death.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 218	ERG 2024

FIRE

#### Oxidizers (Water-Reactive)

EMERGENCY RESPONSE

# GUIDE

## 144

  - DO NOT USE WATER OR FOAM.

Small Fire

  - Dry chemical, soda ash or lime.

Large Fire

  - DRY sand, dry chemical, soda ash or lime or withdraw fromarea and let fire burn.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - DO NOT GET WATER on spilled substance or inside containers. Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

Large Spill

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 219

# GUIDE 145

#### Organic Peroxides

(Heat and Contamination Sensitive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May explode from heat or contamination.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- May be ignited by heat, sparks or flames.

- May burn rapidly with flare-burning effect.

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Fire may produce irritating, corrosive and/or toxic gases.

- Ingestion or contact (skin, eyes) with substance may cause severe injury or burns.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial evacuation for at least 250 meters (800 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 220	ERG 2024

FIRE

Small Fire

#### Organic Peroxides (Heat and Contamination Sensitive)

EMERGENCY RESPONSE

# GUIDE

## 145

  - Water spray or fog is preferred; if water not available use dry chemical, CO2 or regular foam.

Large Fire

  - Flood fire area with water froma distance.

  - Use water spray or fog; avoid aiming straight or solid streams directly onto the product.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Keep substance wet using water spray.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with inert, damp, non-combustible material using clean, non-sparking tools and place into loosely covered plastic containers for later disposal.

Large Spill

  - Wet down with water and dike for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

ERG 2024	Page 221

# GUIDE 146

#### Organic Peroxides

(Heat, Contamination and Friction Sensitive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May explode from heat, shock, friction or contamination.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- May be ignited by heat, sparks or flames.

- May burn rapidly with flare-burning effect.

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Fire may produce irritating, corrosive and/or toxic gases.

- Ingestion or contact (skin, eyes) with substance may cause severe injury or burns.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial evacuation for at least 250 meters (800 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 222	ERG 2024

FIRE

#### Organic Peroxides (Heat, Contamination and Friction Sensitive)

EMERGENCY RESPONSE

# GUIDE

## 146

Small Fire

  - Water spray or fog is preferred; if water not available use dry chemical, CO2 or regular foam.

Large Fire

  - Flood fire area with water froma distance.

  - Use water spray or fog; avoid aiming straight or solid streams directly onto the product.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Keep substance wet using water spray.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with inert, damp, non-combustible material using clean, non-sparking tools and place into loosely covered plastic containers for later disposal.

Large Spill

  - Wet down with water and dike for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 223

# GUIDE 147

#### Lithium Ion and Sodium Ion Batteries

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Lithiumion and sodium ion batteries contain flammable liquid electrolyte that may vent, ignite and produce sparks when subjected to high temperatures (> 150°C (302°F)), when damaged or abused (e.g., mechanical damage or electrical overcharging).

- May burn rapidly with flare-burning effect.

- May ignite other batteries in close proximity.

HEALTH

- Contact with battery electrolyte may be irritating to skin, eyes and mucous membranes.

- Fire will produce irritating, corrosive and/or toxic gases.

- Burning batteries may produce toxic hydrogen fluoride gas (see GUIDE 125).

- Fumes may cause dizziness or asphyxiation.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Spill

- Increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If rail car or trailer is involved in a fire, ISOLATE for 500 meters (1/3 mile) in all directions; also initiate evacuation including emergency responders for 500 meters (1/3 mile) in all directions.

Page 224	ERG 2024

FIRE

#### Lithium Ion and Sodium Ion Batteries

EMERGENCY RESPONSE

# GUIDE

## 147

  - A lithium ion or sodium ion battery fire may reignite at any point after the initial fire is extinguished, up to weeks later.

  - Use thermal imaging, if available, to continuously monitor the battery.

  - Reignition can be accompanied by off-gassing of white smoke or electrical arcs or sparks that reignite with visible flames or fire.

CAUTION: The use of salt water for firefighting is not recommended since it may increase production of hydrogen and hydrogen fluoride gas.

Vehicle Fire

  - If battery is not connected to a vehicle, see “Small Fire or Fire Involving Small Battery” below.

  - Check manufacturer’s specific emergency response guide before attempting to disable vehicle.

  - Turn off the ignition and disconnect the 12-volt battery if it can be done safely.

  - Never cut the high voltage (HV) or medium voltage (MV) cabling.

  - Never touch damaged or submerged HV or MV cables or components.

  - If available, use large amount of water to extinguish or suppress a high-voltage battery fire. Using small amount of water could release toxic gases.

  - If possible, spray water directly onto battery.

  - DO NOT pierce, cut, pry, or dismantle any of the vehicle's structure to access the battery. Contact with a high voltage component may cause an electric shock.

Small Fire or Fire Involving Small Battery (e.g., personal electronic devices, e-bike, etc.)

  - Water spray only (large amounts); do not use dry chemical, CO2 or Halon®.

Large Fire or Fire Involving Large Battery or Multiple Small Batteries

  - Allow battery fire to burn itself out and protect surroundings.

  - Safely remove undamaged containers fromarea.

  - Apply water spray to neighboring batteries to reduce the spread of the hazard.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Absorb with earth, sand or other non-combustible material.

  - Leaking batteries and contaminated absorbent material should be placed in metal containers.

FIRST AID

Refer to the “General First Aid” section.

ERG 2024	Page 225

# GUIDE 148

#### Organic Peroxides (Heat and Contamination Sensitive/Temperature Controlled)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May explode from heat, contamination or loss of temperature control.

- These materials are particularly sensitive to temperature rises. Above a given "Control Temperature" they may decompose violently and catch fire.

- May ignite combustibles (wood, paper, oil, clothing, etc.).

- May ignite spontaneously if exposed to air.

- May be ignited by heat, sparks or flames.

- May burn rapidly with flare-burning effect.

- Containers may explode when heated.

- Runoff may create fire or explosion hazard.

HEALTH

- Fire may produce irritating, corrosive and/or toxic gases.

- Ingestion or contact (skin, eyes) with substance may cause severe injury or burns.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial evacuation for at least 250 meters (800 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 226	ERG 2024

FIRE

#### Organic Peroxides (Heat and Contamination Sensitive/Temperature  Controlled)

EMERGENCY RESPONSE

# GUIDE

## 148

  - The temperature of the substance must be maintained at or belowthe “Control Temperature” at all times.

Small Fire

  - Water spray or fog is preferred; if water not available use dry chemical, CO2 or regular foam.

Large Fire

  - Flood fire area with water froma distance.

  - Use water spray or fog; avoid aiming straight or solid streams directly onto the product.

  - Do not move cargo or vehicle if cargo has been exposed to heat.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - BEWARE OF POSSIBLE CONTAINER EXPLOSION.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - DO NOT allow the substance to warm up. Use a coolant agent such as dry ice or ice (wear thermal protective gloves). If this is not possible or none can be obtained, evacuate the area immediately.

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with inert, damp, non-combustible material using clean, non-sparking tools and place into loosely covered plastic containers for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Contaminated clothing may be a fire risk when dry.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 227

# GUIDE 149

#### Substances (Self-Reactive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Self-decomposition, self-polymerization, or self-ignition may be triggered by heat, chemical reaction, friction or impact.

- May be ignited by heat, sparks or flames.

- Some may decompose explosively when heated or involved in a fire.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- May burn violently. Decomposition or polymerization may be self-accelerating and produce large amounts of gases.

- Vapors or dust may formexplosive mixtures with air.

HEALTH

- Inhalation or contact with vapors, substance or decomposition products may cause severe injury or death.

- May produce irritating, toxic and/or corrosive gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial evacuation for at least 250 meters (800 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 228	ERG 2024

FIRE

Small Fire

#### Substances (Self-Reactive)

EMERGENCY RESPONSE

# GUIDE

## 149

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Flood fire area with water froma distance.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - BEWARE OF POSSIBLE CONTAINER EXPLOSION.

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with inert, damp, non-combustible material using clean, non-sparking tools and place into loosely covered plastic containers for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 229

# GUIDE 150

#### Substances (Self-Reactive/ Temperature Controlled)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Self-decomposition, self-polymerization, or self-ignition may be triggered by heat, chemical reaction, friction or impact.

- Self-accelerating decomposition may occur if the specific "control temperature" is not maintained.

- These materials are particularly sensitive to temperature rises. Above a given "Control Temperature" they may decompose or polymerize violently and catch fire.

- May be ignited by heat, sparks or flames.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Some may decompose explosively when heated or involved in a fire.

- May burn violently. Decomposition or polymerization may be self-accelerating and produce large amounts of gases.

- Vapors or dust may formexplosive mixtures with air.

HEALTH

- Inhalation or contact with vapors, substance or decomposition products may cause severe injury or death.

- May produce irritating, toxic and/or corrosive gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial evacuation for at least 250 meters (800 feet) in all directions.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 230	ERG 2024

FIRE

#### Substances (Self-Reactive/ Temperature  Controlled)

EMERGENCY RESPONSE

# GUIDE

## 150

  - The temperature of the substance must be maintained at or belowthe “Control Temperature” at all times.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Flood fire area with water froma distance.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - BEWARE OF POSSIBLE CONTAINER EXPLOSION.

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - DO NOT allow the substance to warm up. Use a coolant agent such as dry ice or ice (wear thermal protective gloves). If this is not possible or none can be obtained, evacuate the area immediately.

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with inert, damp, non-combustible material using clean, non-sparking tools and place into loosely covered plastic containers for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 231

# GUIDE 151

HEALTH

#### Substances - Toxic (Non-Combustible)

##### POTENTIAL HAZARDS

- Highly toxic, may be fatal if inhaled, ingested or absorbed through skin.

- Avoid any skin contact.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

FIRE OR EXPLOSION

- Non-combustible, substance itself does not burn but may decompose upon heating to produce corrosive and/or toxic fumes.

- Containers may explode when heated.

- Runoff may pollute waterways.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 232	ERG 2024

Substances - Toxic (Non-Combustible)	GUIDE

## 151

##### EMERGENCY RESPONSE

FIRE

Small Fire

  - Dry chemical, CO2 or water spray.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

  - Avoid aiming straight or solid streams directly onto the product.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Cover with plastic sheet to prevent spreading.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - DO NOT GET WATER INSIDE CONTAINERS.

  - For solids, prevent dust cloud and avoid inhalation of dust.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 233

# GUIDE 152

HEALTH

#### Substances - Toxic (Combustible)

##### POTENTIAL HAZARDS

- Highly toxic, may be fatal if inhaled, ingested or absorbed through skin.

- Contact with molten substance may cause severe burns to skin and eyes.

- Avoid any skin contact.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

FIRE OR EXPLOSION

- Combustible material: may burn but does not ignite readily.

- Containers may explode when heated.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Runoff may pollute waterways.

- Substance may be transported in a molten form.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 234	ERG 2024

FIRE

Small Fire

#### Substances - Toxic (Combustible)

EMERGENCY RESPONSE

# GUIDE

## 152

  - Dry chemical, CO2 or water spray.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

  - Avoid aiming straight or solid streams directly onto the product.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Cover with plastic sheet to prevent spreading.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - DO NOT GET WATER INSIDE CONTAINERS.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Removal of solidified molten material from skin requires medical assistance.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 235

# GUIDE 153

HEALTH

#### Substances - Toxic and/or Corrosive (Combustible)

##### POTENTIAL HAZARDS

- TOXIC and/or CORROSIVE; inhalation, ingestion or skin contact with material may cause severe injury or death.

- Methyl bromoacetate (UN2643) is an eye irritant/lachrymator (causes flow of tears).

- Contact with molten substance may cause severe burns to skin and eyes.

- Avoid any skin contact.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

FIRE OR EXPLOSION

- Combustible material: may burn but does not ignite readily.

- When heated, vapors may formexplosive mixtures with air: indoors, outdoors and sewers explosion hazards.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated.

- Runoff may pollute waterways.

- Substance may be transported in a molten form.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 236	ERG 2024

FIRE

Small Fire

#### Substances - Toxic and/or Corrosive

(Combustible)

EMERGENCY RESPONSE

# GUIDE

## 153

  - Dry chemical, CO2 or water spray.

Large Fire

  - Dry chemical, CO2, alcohol-resistant foam or water spray.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - DO NOT GET WATER INSIDE CONTAINERS.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

  - Removal of solidified molten material from skin requires medical assistance.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 237

# GUIDE 154

HEALTH

#### Substances - Toxic and/or Corrosive (Non-Combustible)

##### POTENTIAL HAZARDS

- TOXIC and/or CORROSIVE; inhalation, ingestion or skin contact with material may cause severe injury or death.

- Contact with molten substance may cause severe burns to skin and eyes.

- Avoid any skin contact.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

FIRE OR EXPLOSION

- Non-combustible, substance itself does not burn but may decompose upon heating to produce corrosive and/or toxic fumes.

- Some are oxidizers and may ignite combustibles (wood, paper, oil, clothing, etc.).

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated.

- For electric vehicles or equipment, GUIDE 147 (lithium ion or sodium ion batteries) or GUIDE 138 (sodium batteries) should also be consulted.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 238	ERG 2024

FIRE

Small Fire

#### Substances - Toxic and/or Corrosive

(Non-Combustible)

EMERGENCY RESPONSE

# GUIDE

## 154

  - Dry chemical, CO2 or water spray.

Large Fire

  - Dry chemical, CO2, alcohol-resistant foam or water spray.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Absorb or cover with dry earth, sand or other non-combustible material and transfer to containers.

  - DO NOT GET WATER INSIDE CONTAINERS.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 239

# GUIDE 155

#### Substances - Toxic and/or Corrosive (Flammable/Water-Sensitive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- HIGHLY FLAMMABLE: Will be easily ignited by heat, sparks or flames.

- Vapors formexplosive mixtures with air: indoors, outdoors and sewers explosion hazards.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapors may travel to source of ignition and flash back.

- Those substances designated with a (P) may polymerize explosively when heated or involved in a fire.

- Substance will react with water (some violently) releasing flammable, toxic or corrosive gases and runoff.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated or if contaminated with water.

HEALTH

- TOXIC and/or CORROSIVE; inhalation, ingestion or contact (skin, eyes) with vapors, dusts or substance may cause severe injury, burns or death.

- Bromoacetates and chloroacetates are extremely irritating/lachrymators (cause eye irritation and flowof tears).

- Reaction with water or moist air may release toxic, corrosive or flammable gases.

- Reaction with water may generate much heat that will increase the concentration of fumes in the air.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 240	ERG 2024

FIRE

#### Substances - Toxic and/or Corrosive (Flammable/Water-Sensitive)

EMERGENCY RESPONSE

# GUIDE

## 155

  - Note: Most foams will react with the material and release corrosive/toxic gases. CAUTION: For Acetyl chloride (UN1717), use CO2 or dry chemical only. Small Fire

  - CO2, dry chemical, dry sand, alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - FOR CHLOROSILANES, DO NOT USE WATER; use alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Avoid aiming straight or solid streams directly onto the product.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - A vapor-suppressing foam may be used to reduce vapors.

  - FOR CHLOROSILANES, use alcohol-resistant foam to reduce vapors.

  - DO NOT GET WATER on spilled substance or inside containers.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Prevent entry into waterways, sewers, basements or confined areas.

Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 241

# GUIDE 156

#### Substances - Toxic and/or Corrosive (Combustible/Water-Sensitive)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Combustible material: may burn but does not ignite readily.

- Substance will react with water (some violently) releasing flammable, toxic or corrosive gases and runoff.

- When heated, vapors may formexplosive mixtures with air: indoors, outdoors and sewers explosion hazards.

- Most vapors are heavier than air. They will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Vapors may travel to source of ignition and flash back.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated or if contaminated with water.

HEALTH

- TOXIC and/or CORROSIVE; inhalation, ingestion or contact (skin, eyes) with vapors, dusts or substance may cause severe injury, burns or death.

- Contact with molten substance may cause severe burns to skin and eyes.

- Reaction with water or moist air may release toxic, corrosive or flammable gases.

- Reaction with water may generate much heat that will increase the concentration of fumes in the air.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 242	ERG 2024

FIRE

#### Substances - Toxic and/or Corrosive (Combustible/Water-Sensitive)

EMERGENCY RESPONSE

# GUIDE

## 156

  - Note: Most foams will react with the material and release corrosive/toxic gases. CAUTION: For Acetyl bromide (UN1716), use CO2 or dry chemical only. Small Fire

  - CO2, dry chemical, dry sand, alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - FOR CHLOROSILANES, DO NOT USE WATER; use alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Avoid aiming straight or solid streams directly onto the product.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - A vapor-suppressing foam may be used to reduce vapors.

  - FOR CHLOROSILANES, use alcohol-resistant foam to reduce vapors.

  - DO NOT GET WATER on spilled substance or inside containers.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Prevent entry into waterways, sewers, basements or confined areas.

Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

  - Removal of solidified molten material from skin requires medical assistance.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 243

# GUIDE 157

HEALTH

#### Substances - Toxic and/or Corrosive (Non-Combustible/Water-Sensitive)

##### POTENTIAL HAZARDS

- TOXIC and/or CORROSIVE; inhalation, ingestion or contact (skin, eyes) with vapors, dusts or substance may cause severe injury, burns or death.

- Reaction with water or moist air may release toxic, corrosive or flammable gases.

- Reaction with water may generate much heat that will increase the concentration of fumes in the air.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may be corrosive and/or toxic and cause environmental contamination.

FIRE OR EXPLOSION

- Non-combustible, substance itself does not burn but may decompose upon heating to produce corrosive and/or toxic fumes.

- UN1802, UN2032, UN3084, UN3093, UN1796 (above 50%), UN1826 (above 50%), and UN2031 (above

65%) may act as oxidizers. Also consult GUIDE 140.

- Vapors may accumulate in confined areas (basement, tanks, hopper/tank cars, etc.).

- Substance may react with water (some violently), releasing corrosive and/or toxic gases and runoff.

- Corrosives in contact with metals may evolve flammable hydrogen gas.

- Containers may explode when heated or if contaminated with water.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 244	ERG 2024

FIRE

#### Substances - Toxic and/or Corrosive (Non-Combustible/Water-Sensitive)

EMERGENCY RESPONSE

# GUIDE

## 157

  - Note: Some foams will react with the material and release corrosive/toxic gases.

Small Fire

  - CO2 (except for Cyanides), dry chemical, dry sand, alcohol-resistant foam.

Large Fire

  - Water spray, fog or alcohol-resistant foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Avoid aiming straight or solid streams directly onto the product.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - A vapor-suppressing foam may be used to reduce vapors.

  - DO NOT GET WATER INSIDE CONTAINERS.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Prevent entry into waterways, sewers, basements or confined areas.

Small Spill

  - Cover with DRY earth, DRY sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - Use clean, non-sparking tools to collect material and place it into loosely covered plastic containers for later disposal.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - For corrosives, in case of contact, immediately flush skin or eyes with running water for at least 30 minutes. Additional flushing may be required.

  - In case of skin contact with Hydrofluoric acid (UN1790), if calciumgluconate gel is available, rinse 5 minutes, then apply gel. Otherwise, continue rinsing until medical treatment is available.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 245

# GUIDE 158

HEALTH

#### Infectious Substances

##### POTENTIAL HAZARDS

- Inhalation or contact with substance may cause infection, disease or death.

- Category A Infectious Substances (UN2814, UN2900 or UN3549) are more hazardous, or are in a more hazardous form, than infectious substances shipped as Category B Biological Substances (UN3373) or clinical waste/medical waste (UN3291).

- Runoff fromfire control or dilution water may cause environmental contamination.

- Damaged packages containing solid CO2 as a refrigerant may produce water or frost fromcondensation of air. Do not touch this liquid as it could be contaminated by the contents of the parcel.

- Contact with solid CO2 may cause burns, severe injury and/or frostbite.

FIRE OR EXPLOSION

- Some of these materials may burn, but none ignite readily.

- Some may be transported in flammable liquids.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Consult the shipping paper to identify the substance involved.

PROTECTIVE CLOTHING

- Use judgement based on the amount of material present and the possible routes of exposure to select protective clothing.

- Wear appropriate respiratory protection, such as fit-tested N95 respirator (at minimum), powered air purifying respirator (PAPR), or positive pressure self-contained breathing apparatus (SCBA).

- Wear full coverage body protection (e.g., Tyvek suit), faceshield, and disposable fluid-resistant gloves (e.g., latex or nitrile).

- Wear appropriate footwear; disposable shoe covers can be worn to protect against contamination.

- Puncture- and cut-resistant gloves should be worn over fluid-resistant gloves if sharp objects (e.g., broken glass, needles) are present.

- Wear insulated gloves (e.g. cryo gloves) over fluid-resistant gloves when handling dry ice (UN1845).

- Decontaminate protective clothing and personal protective equipment after use and before cleaning or disposal with a compatible chemical disinfectant (e.g., 10% solution of bleach, equivalent to 0.5% sodium hypochlorite) or through a validated decontamination technology (e.g., autoclave) or process.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Page 246	ERG 2024

FIRE

Small Fire

#### Infectious Substances

EMERGENCY RESPONSE

# GUIDE

## 158

  - Dry chemical, soda ash, lime or sand.

Large Fire

  - Use extinguishing agent suitable for type of surrounding fire.

  - Do not scatter spilled material with high-pressure water streams.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Absorb with earth, sand or other non-combustible material.

  - Cover damaged package or spilled material with absorbent material such as paper towel, towel or rag to absorb any liquids, and, beginning from outside edge, pour liquid bleach or other chemical disinfectant to saturate. Keep wet with liquid bleach or other disinfectant.

  - DO NOT CLEAN-UP OR DISPOSE OF, EXCEPT UNDER SUPERVISION OF A SPECIALIST.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Move victimto an isolated area if it can be done safely.

CAUTION: Victim may be a source of contamination.

  - In case of contact with substance, immediately flush eyes with running water and wash skin thoroughly with soap and water. Take caution not to break the skin.

  - Additional decontamination may also be necessary.

  - Effects of exposure (inhalation, ingestion, injection/inoculation or skin contact) to substance may be delayed. Victim should consult medical professional for information regarding symptoms and treatment.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 247

# GUIDE 159

HEALTH

#### Substances (Irritating)

##### POTENTIAL HAZARDS

- Inhalation of vapors or dust is extremely irritating.

- May cause burning of eyes and lachrymation (flow of tears).

- May cause coughing, difficult breathing and nausea.

- Brief exposure effects last only a few minutes.

- Exposure in an enclosed area may be very harmful.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Some of these materials may burn, but none ignite readily.

- Containers may explode when heated.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 248	ERG 2024

FIRE

Small Fire

#### Substances (Irritating)

EMERGENCY RESPONSE

# GUIDE

## 159

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Do not get water inside containers.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

Small Spill

  - Pick up with sand or other non-combustible absorbent material and place into containers for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 249

# GUIDE 160

HEALTH

#### Halogenated Solvents

##### POTENTIAL HAZARDS

- Toxic by ingestion.

- Vapors may cause dizziness or asphyxiation, especially when in closed or confined areas.

- Exposure in an enclosed area may be very harmful.

- Contact may irritate or burn skin and eyes.

- Fire may produce irritating and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Some of these materials may burn, but none ignite readily.

- Most vapors are heavier than air.

- Air/vapor mixtures may explode when ignited.

- Container may explode in heat of fire.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 250	ERG 2024

FIRE

Small Fire

#### Halogenated Solvents

EMERGENCY RESPONSE

# GUIDE

## 160

  - Dry chemical, CO2 or water spray.

Large Fire

  - Dry chemical, CO2, alcohol-resistant foam or water spray.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Stop leak if you can do it without risk.

Small Liquid Spill

  - Pick up with sand, earth or other non-combustible absorbent material.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Wash skin with soap and water.

ERG 2024	Page 251

# GUIDE 161

HEALTH

#### Radioactive Materials (Low Level Radiation)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Very low levels of contained radioactive materials and low radiation levels outside packages result in low risks to people. Damaged packages may release measurable amounts of radioactive material, but the resulting risks are expected to be low.

- Some radioactive materials cannot be detected by commonly available instruments.

- Packages do not have RADIOACTIVE I, II, or III labels. Some may have EMPTY labels or may have the word “Radioactive” in the package marking.

FIRE OR EXPLOSION

- Some of these materials may burn, but most do not ignite readily.

- Many have cardboard outer packaging; content (physically large or small) can be of many different physical forms.

- Radioactivity does not change flammability or other properties of materials.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Detain or isolate uninjured persons or equipment suspected to be contaminated; delay decontamination and cleanup until instructions are received from Radiation Authority.

PROTECTIVE CLOTHING

- Positive pressure self-contained breathing apparatus (SCBA) and structural firefighters’ protective clothing will provide adequate protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 252	ERG 2024

FIRE

#### Radioactive Materials (Low Level Radiation)

EMERGENCY RESPONSE

# GUIDE

## 161

  - Presence of radioactive material will not influence the fire control processes and should not influence selection of techniques.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not move damaged packages; move undamaged packages out of fire zone.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog (flooding amounts).

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - Cover liquid spill with sand, earth or other non-combustible absorbent material.

  - Cover powder spill with plastic sheet or tarp to minimize spreading.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - Do not delay care and transport of a seriously injured person.

  - Injured persons contaminated by contact with released material are not a serious hazard to health care personnel, equipment or facilities.

ERG 2024	Page 253

# GUIDE 162

HEALTH

#### Radioactive  Materials

(Low to Moderate Level Radiation)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Undamaged packages are safe. Contents of damaged packages may cause higher external radiation exposure, or both external and internal radiation exposure if contents are released.

- Low radiation hazard when material is inside container. If material is released frompackage or bulk container, hazard will vary fromlowto moderate. Level of hazard will depend on the type and amount of radioactivity, the kind of material it is in, and/or the surfaces it is on.

- Some material may be released frompackages during accidents of moderate severity but risks to people are not great.

- Released radioactive materials or contaminated objects usually will be visible if packaging fails.

- Some exclusive use shipments of bulk and packaged materials will not have RADIOACTIVE labels. Placards, markings and shipping papers provide identification.

- Some packages may have a RADIOACTIVE label and a second hazard label. The second hazard is usually greater than the radiation hazard; so follow this GUIDE as well as the response GUIDE for the second hazard class label.

- Some radioactive materials cannot be detected by commonly available instruments.

- Runoff fromcontrol of cargo fire may cause low-level pollution.

FIRE OR EXPLOSION

- Some of these materials may burn, but most do not ignite readily.

- Uraniumand Thoriummetal cuttings may ignite spontaneously if exposed to air (see GUIDE 136).

- Nitrates are oxidizers and may ignite other combustibles (see GUIDE 141).

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Detain or isolate uninjured persons or equipment suspected to be contaminated; delay decontamination and cleanup until instructions are received fromRadiation Authority.

PROTECTIVE CLOTHING

- Positive pressure self-contained breathing apparatus (SCBA) and structural firefighters’ protective clothing will provide adequate protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 254	ERG 2024

FIRE

#### Radioactive Materials (Low to Moderate Level Radiation)

EMERGENCY RESPONSE

# GUIDE

## 162

  - Presence of radioactive material will not influence the fire control processes and should not influence selection of techniques.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not move damaged packages; move undamaged packages out of fire zone.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog (flooding amounts).

  - Dike runoff fromfire control for later disposal.

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - Cover liquid spill with sand, earth or other non-combustible absorbent material.

  - Dike to collect large liquid spills.

  - Cover powder spill with plastic sheet or tarp to minimize spreading.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - Do not delay care and transport of a seriously injured person.

  - In case of contact with substance, wipe from skin immediately; flush skin or eyes with running water for at least 20 minutes.

  - Injured persons contaminated by contact with released material are not a serious hazard to health care personnel, equipment or facilities.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 255

# GUIDE 163

HEALTH

#### Radioactive  Materials

(Low to High Level Radiation)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Undamaged packages are safe. Contents of damaged packages may cause higher external radiation exposure, or both external and internal radiation exposure if contents are released.

- Type A packages (cartons, boxes, drums, articles, etc.) identified as Type A by marking on packages or by shipping papers contain non-life-endangering amounts. Partial releases might be expected if Type A packages are damaged in moderately severe accidents.

- Type B packages, and the rarely occurring Type C packages (large and small, usually metal), contain the most hazardous amounts. They can be identified by package markings or by shipping papers.

Life-threatening conditions may exist only if contents are released or package shielding fails. Because of design, evaluation and testing of packages, these conditions would be expected only for accidents of utmost severity.

- The rarely occurring "Special Arrangement" shipments may be of Type A, Type B or Type C packages. Package type will be marked on packages, and shipment details will be on shipping papers.

- Radioactive White-I labels indicate radiation levels outside single, isolated, undamaged packages are very low (less than 0.005 mSv/h (0.5 mrem/h)).

- Radioactive Yellow-II and Yellow-III labeled packages have higher radiation levels. The transport index (TI) on the label identifies the maximum radiation level in mrem/h one meter froma single, isolated, undamaged package.

- Some radioactive materials cannot be detected by commonly available instruments.

- Water fromcargo fire control may cause pollution.

FIRE OR EXPLOSION

- Some of these materials may burn, but most do not ignite readily.

- Radioactivity does not change flammability or other properties of materials.

- Type B packages are designed and evaluated to withstand total engulfment in flames at temperatures of 800°C (1475°F) for a period of 30 minutes.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.	• Keep unauthorized personnel away.

- Detain or isolate uninjured persons or equipment suspected to be contaminated; delay decontamination and cleanup until instructions are received from Radiation Authority.

PROTECTIVE CLOTHING

- Positive pressure self-contained breathing apparatus (SCBA) and structural firefighters’ protective clothing will provide adequate protection against internal radiation exposure, but not external radiation exposure.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 256	ERG 2024

FIRE

#### Radioactive Materials (Low to High Level Radiation)

EMERGENCY RESPONSE

# GUIDE

## 163

  - Presence of radioactive material will not influence the fire control processes and should not influence selection of techniques.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not move damaged packages; move undamaged packages out of fire zone.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog (flooding amounts).

  - Dike runoff fromfire control for later disposal.

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - Damp surfaces on undamaged or slightly damaged packages are seldom an indication of packaging failure. Most packaging for liquid content have inner containers and/or inner absorbent materials.

  - Cover liquid spill with sand, earth or other non-combustible absorbent material.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - Do not delay care and transport of a seriously injured person.

  - Injured persons contaminated by contact with released material are not a serious hazard to health care personnel, equipment or facilities.

ERG 2024	Page 257

# GUIDE 164

HEALTH

#### Radioactive Materials (Special Form/ Low to High Level External Radiation)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Undamaged packages are safe; contents of damaged packages may cause external radiation exposure, and much higher external exposure if contents (source capsules) are released.

- Contamination and internal radiation hazards are not expected, but not impossible.

- Type A packages (cartons, boxes, drums, articles, etc.) identified as Type A by marking on packages or by shipping papers contain non-life-endangering amounts. Radioactive sources may be released if Type A packages are damaged in moderately severe accidents.

- Type B packages, and the rarely occurring Type C packages, (large and small, usually metal) contain the most hazardous amounts. They can be identified by package markings or by shipping papers.

Life-threatening conditions may exist only if contents are released or package shielding fails. Because of design, evaluation and testing of packages, these conditions would be expected only for accidents of utmost severity.

- Radioactive White-I labels indicate radiation levels outside single, isolated, undamaged packages are very low (less than 0.005 mSv/h (0.5 mrem/h)).

- Radioactive Yellow-II and Yellow-III labeled packages have higher radiation levels. The transport index (TI) on the label identifies the maximum radiation level in mrem/h one meter froma single, isolated, undamaged package.

- Radiation fromthe package contents, usually in durable metal capsules, can be detected by most radiation instruments.

- Water fromcargo fire control is not expected to cause pollution.

FIRE OR EXPLOSION

- Packagings can burn completely without risk of content loss fromsealed source capsule.

- Radioactivity does not change flammability or other properties of materials.

- Radioactive source capsules and Type B packages are designed and evaluated to withstand total engulfment in flames at temperatures of 800°C (1475°F) for a period of 30 minutes.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.	• Keep unauthorized personnel away.

- Delay final cleanup until instructions or advice is received from Radiation Authority.

PROTECTIVE CLOTHING

- Positive pressure self-contained breathing apparatus (SCBA) and structural firefighters’ protective clothing will provide adequate protection against internal radiation exposure, but not external radiation exposure.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 258	ERG 2024

FIRE

#### Radioactive Materials (Special Form/ Low to High Level External Radiation)

EMERGENCY RESPONSE

# GUIDE

## 164

  - Presence of radioactive material will not influence the fire control processes and should not influence selection of techniques.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not move damaged packages; move undamaged packages out of fire zone.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog (flooding amounts).

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - Damp surfaces on undamaged or slightly damaged packages are seldom an indication of packaging failure. Contents are seldom liquid. Content is usually a metal capsule, easily seen if released from package.

  - If source capsule is identified as being out of package, DO NOT TOUCH. Stay away and await advice from Radiation Authority.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - Do not delay care and transport of a seriously injured person.

  - Persons exposed to special formsources are not likely to be contaminated with radioactive material.

  - Injured persons contaminated by contact with released material are not a serious hazard to health care personnel, equipment or facilities.

ERG 2024	Page 259

# GUIDE 165

HEALTH

#### Radioactive  Materials

(Fissile/Low to High Level Radiation)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Undamaged packages aresafe. Contents of damaged packages may cause higher external radiation exposure, or both external and internal radiation exposure if contents are released.

- Type AF or IF packages, identified by package markings, do not contain life-threatening amounts of material. External radiation levels are lowand packages are designed, evaluated and tested tocontrol releases and to prevent afission chain reaction under severe transport conditions.

- Type B(U)F, B(M)F and CF packages (identified by markings on packages or shipping papers) contain potentially life-endangering amounts. Because of design, evaluation and testing of packages, fission chain reactions are prevented and releases are not expected to be life-endangering for all accidents except those of utmost severity.

- The rarely occurring "Special Arrangement" shipments may be of Type AF, BF or CF packages. Package type will be marked on packages, and shipment details will be on shipping papers.

- The transport index (TI) shown on labels or ashipping paper might not indicate the radiation level at one meter fromasingle, isolated, undamaged package; instead, it might relate tocontrols needed during transport because of the fissile properties of the materials. Alternatively, the fissile nature of the contents is indicated by acriticality safety index (CSI) on aspecial FISSILE label or on the shipping paper.

- Some radioactive materials cannot be detected by commonly available instruments.

- Water fromcargo fire control is not expected tocause pollution.

FIRE OR EXPLOSION

- These materials are seldomflammable. Packages are designed to withstand fires without damage to contents.

- Radioactivity does not change flammability or other properties of materials.

- Type AF, IF, B(U)F, B(M)F and CF packages are designed and evaluated to withstand total engulfment in flames at temperatures of 800°C (1475°F) for a period of 30 minutes.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Detain or isolate uninjured persons or equipment suspected to be contaminated; delay decontamination and cleanup until instructions are received fromRadiation Authority.

PROTECTIVE CLOTHING

- Positive pressure self-contained breathing apparatus (SCBA) and structural firefighters’ protective clothing will provide adequate protection against internal radiation exposure, but not external radiation exposure.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 260	ERG 2024

FIRE

#### Radioactive Materials (Fissile/Low to High Level Radiation)

EMERGENCY RESPONSE

# GUIDE

## 165

  - Presence of radioactive material will not influence the fire control processes and should not influence selection of techniques.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Do not move damaged packages; move undamaged packages out of fire zone.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog (flooding amounts).

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - Damp surfaces on undamaged or slightly damaged packages are seldom an indication of packaging failure. Most packaging for liquid content have inner containers and/or inner absorbent materials.

Liquid Spill

  - Package contents are seldom liquid. If any radioactive contamination resulting froma liquid release is present, it probably will be low-level.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - Do not delay care and transport of a seriously injured person.

  - Injured persons contaminated by contact with released material are not a serious hazard to health care personnel, equipment or facilities.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 261

# GUIDE 166

HEALTH

#### Radioactive Materials - Corrosive

(Uranium  Hexafluoride/Water-Reactive)

##### POTENTIAL HAZARDS

- Radiation presents minimal risk to transport workers, emergency response personnel and the public during transportation accidents. Packaging durability increases as potential hazard of radioactive content increases.

- Low radiation hazard to people. Chemical hazard greatly exceeds radiation hazard.

- Substance reacts with water and water vapor in air to form toxic and corrosive hydrogen fluoride gas, hydrofluoric acid, and an extremely irritating and corrosive, white-colored, water-soluble residue.

- Toxic; may be fatal if inhaled, ingested, or absorbed through skin.

- Direct contact with substance and gas may cause burns to skin, eyes, or respiratory tract.

- Runoff fromcontrol of cargo fire may cause low-level pollution.

FIRE OR EXPLOSION

- Substance does not burn.

- The material may react violently with fuels.

- Product will decompose to produce toxic and/or corrosive fumes.

- Containers in protective overpacks (horizontal cylindrical shape with short legs for tie-downs), are identified with AF, B(U)F or H(U) on shipping papers or by markings on the overpacks. They are designed and evaluated to withstand severe conditions including total engulfment in flames at temperatures of 800°C (1475°F) for a period of 30 minutes.

- Bare filled cylinders, identified with UN2978 as part of the marking (may also be marked H(U) or H(M)), may rupture in heat of engulfing fire; bare empty (except for residue) cylinders will not rupture in fires.

- Radioactivity does not change flammability or other properties of materials.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Priorities for rescue, life-saving, first aid, fire control and other hazards are higher than the priority for measuring radiation levels.

- Radiation Authority must be notified of accident conditions. Radiation Authority is usually responsible for decisions about radiological consequences and closure of emergencies.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Detain or isolate uninjured persons or equipment suspected to be contaminated; delay decontamination and cleanup until instructions are received from Radiation Authority.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 25 meters (75 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- When a large quantity of this material is involved in a major fire, consider an initial evacuation distance of 300 meters (1000 feet) in all directions.

Page 262	ERG 2024

FIRE

#### Radioactive Materials - Corrosive (Uranium  Hexafluoride/Water-Reactive)

EMERGENCY RESPONSE

# GUIDE

## 166

  - DO NOT USE WATER OR FOAM ON MATERIAL ITSELF.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Dry chemical, CO2, or withdraw fromarea and let fire burn.

  - Only use water if the package is intact.

  - DO NOT GET WATER on spilled substance or inside containers.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

SPILL OR LEAK

  - Do not touch damaged packages or spilled material.

  - DO NOT GET WATER on spilled substance or inside containers.

  - Without fire or smoke, leak will be evident by visible and irritating vapors and residue forming at the point of release.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Residue buildup may self-seal small leaks.

  - Dike far ahead of spill to collect runoff water.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Medical problems take priority over radiological concerns.

  - Use first aid treatment according to the nature of the injury.

  - In case of skin contact with hydrogen fluoride gas and/or Hydrofluoric acid, if calciumgluconate gel is available, rinse 5 minutes, then apply gel. Otherwise, continue rinsing until medical treatment is available.

  - Do not delay care and transport of a seriously injured person.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 263

# GUIDE 167

Intentionally left blank

There are no materials that refer to this guide.

Page 264	ERG 2024

# GUIDE

## 167

Intentionally left blank

There are no materials that refer to this guide.

ERG 2024	Page 265

# GUIDE 168

HEALTH

#### Carbon Monoxide (Refrigerated Liquid)

##### POTENTIAL HAZARDS

- TOXIC; Extremely Hazardous.

- Inhalation extremely dangerous; may be fatal.

- Contact with gas, liquefied gas or cryogenic liquids may cause burns, severe injury and/or frostbite.

- Odorless, will not be detected by sense of smell.

FIRE OR EXPLOSION

- EXTREMELY FLAMMABLE.

CAUTION: Flame can be invisible. Use an alternate method of detection (thermal camera, broom handle, etc.)

- May be ignited by heat, sparks or flames.

- Containers may explode when heated.

- Vapor explosion and poison hazard indoors, outdoors or in sewers.

- Vapors from liquefied gas are initially heavier than air and spread along ground.

- Vapors may travel to source of ignition and flash back.

- Runoff may create fire or explosion hazard.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

- Always wear thermal protective clothing when handling refrigerated/cryogenic liquids.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 266	ERG 2024

FIRE

#### Carbon Monoxide (Refrigerated Liquid)

EMERGENCY RESPONSE

# GUIDE

## 168

CAUTION: Flame can be invisible. Use an alternate method of detection (thermal camera, broom handle, etc.)

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

  - Dry chemical, CO2 or water spray.

Large Fire

  - Water spray, fog or regular foam.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices; icing may occur.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - If possible, turn leaking containers so that gas escapes rather than liquid.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of contact with liquefied gas, only medical personnel should attempt thawing frosted parts.

ERG 2024	Page 267

# GUIDE 169

#### Aluminum (Molten)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Substance is transported in molten format a temperature above 705°C (1300°F).

- Violent reaction with water; contact may cause an explosion or may produce a flammable gas.

- Will ignite combustible materials (wood, paper, oil, debris, etc.).

- Contact with nitrates or other oxidizers may cause an explosion.

- Contact with containers or other materials, including cold, wet or dirty tools, may cause an explosion.

- Contact with concrete will cause spalling and small pops.

HEALTH

- Contact causes severe burns to skin and eyes.

- Fire may produce irritating and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear flame-retardant structural firefighters’ protective clothing, including faceshield, helmet and gloves, as this will provide limited thermal protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Page 268	ERG 2024

FIRE

#### Aluminum (Molten)

EMERGENCY RESPONSE

# GUIDE

## 169

  - Do not use water, except in life-threatening situations and then only in a fine spray.

  - Do not use halogenated extinguishing agents or foam.

  - Move combustibles out of path of advancing pool if you can do so without risk.

  - Extinguish fires started by molten material by using appropriate method for the burning material; keep water, halogenated extinguishing agents and foam away fromthe molten material.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Do not attempt to stop leak, due to danger of explosion.

  - Keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - Substance is very fluid, spreads quickly, and may splash. Do not try to stop it with shovels or other objects.

  - Dike far ahead of spill; use dry sand to contain the flow of material.

  - Where possible allow molten material to solidify naturally.

  - Avoid contact even after material solidifies. Molten, heated and cold aluminum look alike; do not touch unless you know it is cold.

  - Clean up under the supervision of an expert after material has solidified.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - Removal of solidified molten material from skin requires medical assistance.

ERG 2024	Page 269

# GUIDE 170

#### Metals (Powders, Dusts, Shavings, Borings, Turnings, or Cuttings, etc.)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- May react violently or explosively on contact with water.

- Some are transported in flammable liquids.

- May be ignited by friction, heat, sparks or flames.

- Some of these materials will burn with intense heat.

- Dusts or fumes may formexplosive mixtures in air.

- Containers may explode when heated.

- May re-ignite after fire is extinguished.

HEALTH

- Oxides from metallic fires are a severe health hazard.

- Inhalation or contact with substance or decomposition products may cause severe injury or death.

- Fire may produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Large Spill

- Consider initial downwind evacuation for at least 50 meters (160 feet).

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 270	ERG 2024

FIRE

#### Metals (Powders, Dusts, Shavings, Borings,

Turnings, or Cuttings, etc.)

EMERGENCY RESPONSE

# GUIDE

## 170

  - DO NOT USE WATER, FOAM OR CO2.

  - Dousing metallic fires with water will generate hydrogen gas, an extremely dangerous explosion hazard, particularly if fire is in a confined environment (i.e., building, cargo hold, etc.).

  - Use DRY sand, graphite powder, dry sodium chloride-based extinguishers, or class D extinguishers.

  - Confining and smothering metal fires is preferable rather than applying water.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

Fire Involving Tanks, Rail Tank Cars or Highway Tanks

  - If impossible to extinguish, protect surroundings and allow fire to burn itself out.

SPILL OR LEAK

  - ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 271

# GUIDE 171

#### Substances (Low to Moderate Hazard)

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Some may burn but none ignite readily.

- Containers may explode when heated.

- Some may be transported hot.

- For UN3508, Capacitor, asymmetric, be aware of possible short circuiting as this product is transported in a charged state.

- Polymeric beads, expandable (UN2211) may evolve flammable vapours.

HEALTH

- Inhalation of material may be harmful.

- Contact may cause burns to skin and eyes.

- Inhalation of Asbestos dust may have a damaging effect on the lungs.

- Fire may produce irritating, corrosive and/or toxic gases.

- Some liquids produce vapors that may cause dizziness or asphyxiation.

- Runoff fromfire control or dilution water may cause environmental contamination.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area in all directions for at least 50 meters (150 feet) for liquids and at least 25 meters (75 feet) for solids.

Spill

- For highlighted materials: see Table 1 - Initial Isolation and Protective Action Distances.

- For non-highlighted materials: increase the immediate precautionary measure distance, in the downwind direction, as necessary.

Fire

- If tank, rail tank car or highway tank is involved in a fire, ISOLATE for 800 meters (1/2 mile) in all directions; also, consider initial evacuation for 800 meters (1/2 mile) in all directions.

Page 272	ERG 2024

FIRE

#### Substances (Low to Moderate Hazard)

EMERGENCY RESPONSE

# GUIDE

## 171

CAUTION: Fire involving Safety devices (UN3268) and Fire suppressant dispersing devices (UN3559) may have a delayed activation and a risk of hazardous projectiles. Extinguish the fire at a safe distance.

Small Fire

  - Dry chemical, CO2, water spray or regular foam.

Large Fire

  - Water spray, fog or regular foam.

  - Do not scatter spilled material with high-pressure water streams.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Dike runoff fromfire control for later disposal.

Fire Involving Tanks

  - Cool containers with flooding quantities of water until well after fire is out.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Prevent dust cloud.

  - For Asbestos, avoid inhalation of dust. Cover spill with plastic sheet or tarp to minimize spreading. Do not clean up or dispose of, except under supervision of a specialist.

Small Dry Spill

  - With clean shovel, place material into clean, dry container and cover loosely; move containers fromspill area.

Small Spill

  - Pick up with sand or other non-combustible absorbent material and place into containers for later disposal.

Large Spill

  - Dike far ahead of liquid spill for later disposal.

  - Cover powder spill with plastic sheet or tarp to minimize spreading.

  - Prevent entry into waterways, sewers, basements or confined areas.

FIRST AID

Refer to the “General First Aid” section.

ERG 2024	Page 273

# GUIDE 172

HEALTH

#### Gallium and Mercury

##### POTENTIAL HAZARDS

- Inhalation of vapors or contact with substance will result in contamination and potential harmful effects.

- Fire will produce irritating, corrosive and/or toxic gases.

FIRE OR EXPLOSION

- Non-combustible, substance itself does not burn but may react upon heating to produce corrosive and/or toxic fumes.

- Runoff may pollute waterways.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Stay upwind, uphill and/or upstream.

- Keep unauthorized personnel away.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 50 meters (150 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 100 meters (330 feet).

Fire

- When any large container is involved in a fire, consider initial evacuation for 500 meters (1/3 mile) in all directions.

Page 274	ERG 2024

FIRE

#### Gallium and Mercury

EMERGENCY RESPONSE

# GUIDE

## 172

  - Use extinguishing agent suitable for type of surrounding fire.

  - Do not direct water at the heated metal.

SPILL OR LEAK

  - Do not touch or walk through spilled material.

  - Do not touch damaged containers or spilled material unless wearing appropriate protective clothing.

  - Stop leak if you can do it without risk.

  - Prevent entry into waterways, sewers, basements or confined areas.

  - Do not use steel or aluminum tools or equipment.

  - Cover with earth, sand or other non-combustible material followed with plastic sheet to minimize spreading or contact with rain.

  - For mercury, use a mercury spill kit.

  - Mercury spill areas may be subsequently treated with calciumsulphide/calcium sulfide or with sodium thiosulphate/sodium thiosulfate wash to neutralize any residual mercury.

FIRST AID

Refer to the “General First Aid” section.

ERG 2024	Page 275

# GUIDE 173

HEALTH

#### Adsorbed Gases - Toxic*

##### POTENTIAL HAZARDS

- TOXIC; may be fatal if inhaled or absorbed through skin.

- Vapors may be irritating.

- Contact with gas may cause burns and injury.

- Fire will produce irritating, corrosive and/or toxic gases.

- Runoff fromfire control or dilution water may cause environmental contamination.

FIRE OR EXPLOSION

- Some gases may burn or be ignited by heat, sparks or flames.

- May formexplosive mixtures with air.

- Oxidizers may ignite combustibles (wood, paper, oil, clothing, etc.) but NOT readily due to low transportation pressures.

- Vapors may travel to source of ignition and flash back.

- Some of these materials may react violently with water.

- Cylinders exposed to fire may vent and release toxic and flammable gas through pressure relief devices.

- Runoff may create fire hazard.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Wear chemical protective clothing that is specifically recommended by the manufacturer when there is NO RISK OF FIRE.

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Spill

- See Table 1 - Initial Isolation and Protective Action Distances.

Fire

- If several small packages (inside a railcar or trailer) are involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

* Some substances may also be flammable, corrosive and/or oxidizing

Page 276	ERG 2024

Adsorbed Gases - Toxic*	GUIDE

## 173

##### EMERGENCY RESPONSE

FIRE

- DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

Small Fire

- Dry chemical, CO2, water spray or alcohol-resistant foam.

- For UN3515, UN3518, UN3520, use water only; no dry chemical, CO2 or Halon®.

Large Fire

- Water spray, fog or alcohol-resistant foam.

- Do not get water inside containers.

- If it can be done safely, move undamaged containers away fromthe area around the fire.

- Damaged cylinders should be handled only by specialists.

Fire Involving Several Small Packages (inside a railcar or trailer)

- Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

- Cool containers with flooding quantities of water until well after fire is out.

- Do not direct water at source of leak or safety devices.

- Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

- ALWAYS stay away fromtanks in direct contact with flames.

SPILL OR LEAK

- Some gases may be flammable. ELIMINATE all ignition sources (no smoking, flares, sparks or flames) fromimmediate area.

- For flammable gases, all equipment used when handling the product must be grounded.

- For oxidizing substances, keep combustibles (wood, paper, oil, etc.) away fromspilled material.

- Do not touch or walk through spilled material.

- Stop leak if you can do it without risk.

- Do not direct water at spill or source of leak.

- Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

- Prevent entry into waterways, sewers, basements or confined areas.

- Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

- In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 277

# GUIDE 174

#### Adsorbed Gases - Flammable or Oxidizing

##### POTENTIAL HAZARDS

FIRE OR EXPLOSION

- Some gases will be ignited by heat, sparks or flames.

- Substance does not burn but will support combustion.

- Vapors may travel to source of ignition and flash back.

- Cylinders exposed to fire may vent and release flammable gas through pressure relief devices.

- Containers may explode when exposed to prolonged direct flame impingement.

HEALTH

- Vapors may cause dizziness or asphyxiation without warning, especially when in closed or confined areas.

- Some may be irritating if inhaled at high concentrations.

- Contact with gas may cause burns and injury.

- Fire may produce irritating and/or toxic gases.

##### PUBLIC SAFETY

- CALL 911. Then call emergency response telephone number on shipping paper. If shipping paper not available or no answer, refer to appropriate telephone number listed on the inside back cover.

- Keep unauthorized personnel away.

- Stay upwind, uphill and/or upstream.

- Many gases are heavier than air and will spread along the ground and collect in low or confined areas (sewers, basements, tanks, etc.).

- Ventilate closed spaces before entering, but only if properly trained and equipped.

PROTECTIVE CLOTHING

- Wear positive pressure self-contained breathing apparatus (SCBA).

- Structural firefighters' protective clothing provides thermal protection but only limited chemical protection.

EVACUATION

Immediate precautionary measure

- Isolate spill or leak area for at least 100 meters (330 feet) in all directions.

Large Spill

- Consider initial downwind evacuation for at least 800 meters (1/2 mile).

Fire

- If several small packages (inside a railcar or trailer) are involved in a fire, ISOLATE for 1600 meters (1 mile) in all directions; also, consider initial evacuation for 1600 meters (1 mile) in all directions.

Page 278	ERG 2024

Adsorbed Gases - Flammable or Oxidizing	GUIDE

## 174

##### EMERGENCY RESPONSE

FIRE

  - DO NOT EXTINGUISH A LEAKING GAS FIRE UNLESS LEAK CAN BE STOPPED.

  - Use extinguishing agent suitable for type of surrounding fire.

Small Fire

  - Dry chemical or CO2.

Large Fire

  - Water spray or fog.

  - If it can be done safely, move undamaged containers away fromthe area around the fire.

  - Damaged cylinders should be handled only by specialists.

Fire Involving Several Small Packages (inside a railcar or trailer)

  - Fight fire from maximum distance or use unmanned master stream devices or monitor nozzles.

  - Cool containers with flooding quantities of water until well after fire is out.

  - Do not direct water at source of leak or safety devices.

  - Withdraw immediately in case of rising sound fromventing safety devices or discoloration of tank.

  - ALWAYS stay away fromtanks in direct contact with flames.

  - For massive fire, use unmanned master stream devices or monitor nozzles; if this is impossible, withdraw fromarea and let fire burn.

SPILL OR LEAK

  - For flammable gases, ELIMINATE all ignition sources (no smoking, flares, sparks or flames) from immediate area.

  - For oxidizing substances, keep combustibles (wood, paper, oil, etc.) away fromspilled material.

  - All equipment used when handling the product must be grounded.

  - Do not touch or walk through spilled material.

  - Stop leak if you can do it without risk.

  - Use water spray to reduce vapors or divert vapor cloud drift. Avoid allowing water runoff to contact spilled material.

  - Do not direct water at spill or source of leak.

  - Prevent spreading of vapors through sewers, ventilation systems and confined areas.

  - Ventilate the area.

  - Isolate area until gas has dispersed.

FIRST AID

Refer to the “General First Aid” section. Specific First Aid:

  - In case of burns, immediately cool affected skin for as long as possible with cold water. Do not remove clothing if adhering to skin.

In Canada, an Emergency Response Assistance Plan (ERAP) may be required for this product. Please consult the shipping paper and/or the "ERAP" section.

ERG 2024	Page 279

##### INTRODUCTION TO GREEN TABLES

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

This table suggests distances useful to protect people from vapors/gases resulting from spills involving:

  - materials that are considered toxic by inhalation (TIH) (PIH in the US)

  - materials which produce toxic gases upon contact with water

This table provides first responders with initial guidance until technically qualified emergency response personnel are available. For each material, first responders will find distances for the following zones:

  - The Initial Isolation Zone defines an area surrounding the incident in which people may be exposed to dangerous (upwind) and life-threatening (downwind) concentrations of material.

  - The Protective Action Zone defines an area downwind fromthe incident in which people may become incapacitated and unable to take protective action and/or incur serious or irreversible health effects. Table 1 provides specific guidance for small and large spills occurring day or night.

Adjusting distances for a specific incident involves many interdependent variables. These adjustments should only be made by technically qualified personnel. For this reason, no precise guidance can be provided in this document to aid in adjusting the table distances; however, general guidance follows.

###### Factors that May Change the Protective Action Distances Fire

In the orange section, under EVACUATION – Fire, the evacuation distance required to protect against fragmentation hazard of a large container is clearly indicated. If involved in a fire, the toxic hazard may be less dangerous than the fire or explosion hazard.

Inthese cases, the fire hazarddistance shouldbe usedas an isolation distance and Table 1 should be used to protect downwind for residual material release.

###### Worst-case scenario: terrorism, sabotage or catastrophic accident

Initial isolation and protective action distances are derived from historical data on transportation incidents and the use of statistical models. For worst-case scenarios involving the instantaneous release of the entire contents of a package (e.g., as a result of terrorism, sabotage or catastrophic accident), the distances may increase substantially.

For such events, doubling the initial isolation and protective action distances is appropriate in absence of other information.

###### When more than one large package is leaking

If more than one rail tank car, highway tank, tank or large cylinder, containing TIH materials is leaking, large spill distances may need to be increased.

Page 280

###### Other factors that can increase the protective action distance:

- If amaterial has aprotective actiondistance of 11.0+km(7.0+miles), the actual distance can be larger in certain atmospheric conditions.

- If the material’s vapor plume is channeled in a valley or between many tall buildings, protectiveactiondistances may belarger thanshownduetoless mixing of the plume with the atmosphere.

- If there is a daytime spill in a region with known strong temperature inversions or snow cover, or it occurs near sunset, this may require an increase of the protective action distance because airborne contaminants mix and disperse more slowly and may travel much farther downwind.

› In such cases, the nighttime protective action distances may be more appropriate.

- If the temperature of the liquid spill or the outdoor temperature exceeds 30°C (86°F), the protective action distance may be larger.

###### Water-reactive materials

Materials that react with water to produce large amounts of toxic gases are included in Table

1. Some of these materials have 2 entries in Table 1. They are identified by (when spilled on land) since they are TIH products and (when spilled in water) because they produce additional toxic gases when spilled in water.

Choose the larger protective action distance if:

- it is not clear whether the spill is on land or in water

- the spill occurs both on land and in water

##### TABLE 2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

This table lists materials which produce large amounts of Toxic Inhalation Hazard gases (TIH) when spilled in water as well as the TIH gases that are produced.

NOTE: The produced TIH gases indicated in Table 2 are for information purposes only. In Table 1, the initial isolation and protective action distances have already taken into consideration the produced TIH gas.

When a water-reactive TIH-producing material is spilled into a river or stream, the source of the toxic gas may flow downstream for a great distance.

Page 281

##### TABLE 3 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES FOR LARGE SPILLS FOR DIFFERENT QUANTITIES OF SIX COMMON TIH (PIH IN THE US) GASES

This table lists materials that may be more commonly encountered. These materials are:

- UN1005 - Ammonia, anhydrous

- UN1017 - Chlorine

- UN1040 - Ethylene oxide and UN1040 - Ethylene oxide with nitrogen

- UN1050 - Hydrogen chloride, anhydrous and UN2186 - Hydrogen chloride, refrigerated liquid

- UN1052 - Hydrogen fluoride, anhydrous

- UN1079 - Sulfur dioxide/Sulphur dioxide

This table provides initial isolation and protective action distances for large spills (more than 208 liters or 55 US gallons):

- involving different container types (therefore different volume capacities)

- for daytime and nighttime situations

- for different wind speeds (low, moderate and high)

Page 282

##### PROTECTIVE ACTIONS

Protective actions are the steps taken to preserve the health and safety of emergency responders and the public during an incident involving releases of hazardous materials/ dangerous goods.

Table 1 - Initial Isolation and Protective Action Distances (green section) predicts the size of the area that could be affected by a cloud of toxic gas. People in this area should be evacuated and/or sheltered-in-place inside buildings.

Isolatehazardarea anddenyentrymeans tokeep everybody away fromthe areaif they are not directly involved in emergency response operations. Unprotected emergency responders should not be allowed to enter the isolation zone.

This "isolation" task is done to establish control over the area of operations. This is the first step for any protective actions that may follow.

Evacuate means to move all people froma threatened area to a safer place. To performan evacuation, there must be enough time for people to be warned, get ready, and leave an area. If there is enough time, evacuation is the best protective action.

Begin evacuating people nearby and those who are outdoors in direct view of the scene. When additional help arrives, expand the area to be evacuated downwind and crosswind to at least the extent recommended in this guidebook.

Even after people move to the distances recommended, they may not be completely safe from harm. They should not be permitted to gather at such distances. Send evacuees to a definite place, by a specific route, far enough away so they will not have to relocate again if the wind shifts.

Shelter-in-place means people should seek shelter inside a building and remain inside until the danger passes. It is vital for first responders to maintain communications with sheltered-in-place people so that they are advised about changing conditions.

Sheltering-in-place is used either when:

- evacuating the public would cause greater risk than staying where they are

- an evacuation cannot be safely performed Direct the people inside to:

- close all doors and windows

- shut off all ventilating, heating and cooling systems

- stay far from windows to avoid shattered glass and projectile metal fragments in the event of a fire and/or explosion

- seal cracks around doors, windows and vents with duct tape or wet cloths

- tune in to local media, and remain inside until told it is safe to leave by first responders or emergency response authorities

- breathe through a wet cloth until an all clear has been communicated

Vehicles can offer some protection for a short period if the windows are closed and the ventilation systems are shut off. Vehicles are not nearly as effective as buildings for in-place protection.

Page 283

##### PROTECTIVE ACTION DECISION FACTORS TO CONSIDER

The choice of protective actions for a given situation depends on a number of factors. For some cases, evacuation may be the best option; in others, sheltering-in-place may be the best course. Sometimes, these two actions may be used in combination. In any emergency, officials needtoquickly givethe public instructions. The public will needcontinuinginformation and instructions while being evacuated or sheltered-in-place.

Proper evaluation of the factors listed belowwill determine the effectiveness of evacuation or in-placeprotection(shelter-in-place). Theimportanceof thesefactors canvary withemergency conditions. Inspecific emergencies, other factors may need to be identified and considered as well. This list indicates what kind of information may be needed to make the initial decision.

###### The hazardous materials/dangerous goods:

- degree of health hazard

- chemical and physical properties

- amount involved

- containment/control of release

- rate of vapor movement

###### The population threatened:

- location

- number of people

- time available to evacuate or shelter-in-place

- ability to control evacuation or shelter-in-place

- building types and availability

- special institutions or populations, e.g., nursing homes, hospitals, prisons

###### The weather conditions:

- effect on vapor and cloud movement

- potential for change

- effect on evacuation or shelter-in-place

NOTE: Every hazardous materials/dangerous goods incident is different. Each will have special problems and concerns. Actions to protect the public must be carefully selected. This section can help with initial decisions on how to protect the public. Officials must continue to gather information and monitor the situation until the threat is removed.

Page 284

The following table can help to decide if evacuation or sheltering-in-place is the best option:

| Consider Evacuation: | Consider Sheltering-in-place: |
| --- | --- |
| Vapors are flammable. | Vapors are toxic, and people are likely to be exposed by evacuating. |
| Buildings cannot be closed tightly. | Buildings can be quickly sealed by closing all windows and ventilation systems, if applicable. |
| The vapors are continuously generated and will hug the ground, or it will take a long time for the vapors to clear the area. | The vapors will quickly rise in the air column or rapidly dissipate. |
| For anyone outdoors. | For anyone already indoors. |
| There are few people to evacuate. | There are too many people to evacuate for current available resources. |
| The threat seems stable but long- lasting. | Circumstances are changing too quickly to evacuate safely. |

Page 285

##### BACKGROUND ON TABLE 1 – INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Initial isolation and protective action distances in this guidebook were determined for small and large spills occurring during day or night. The overall analysis, statistical in nature, was conducted using:

- state-of-the-art emission rate and dispersion models

- statistical release data from the U.S. Department of Transportation (DOT) Hazardous Materials Information System (HMIS) database

- meteorological observations from more than 120 locations in the United States, Canada, and Mexico

- the most current toxicological exposure guidelines

For eachchemical, thousandsof hypothetical releasesweremodeledtoaccount for thestatistical variance in both release amount and atmospheric conditions. Based on this statistical sample, they selected the 90th percentile protective action distance for each chemical and category to appear in the table. A brief description of the analysis is provided below.

Adetailed report outlining the methodology and data used to generate the initial isolation and protective action distances may be obtained from the U.S. DOT, Pipeline and Hazardous Materials Safety Administration (PHMSA).

##### DESCRIPTION OF THE ANALYSIS

Release amounts and emission rates into the atmosphere were statistically modeled based on:

- data from the U.S. DOT HMIS database

- container types and sizes authorized for transport as specified in 49 CFR§172.101 and Part 173

- physical properties of the individual materials

- atmospheric data from a historical database

For liquefied gases, which can flash toformboth avapor/aerosol mixture and an evaporating pool, the emission model calculated one or both of:

- the release of vapor due to evaporation of pools on the ground

- direct release of vapors from the container

The emission model also calculated the emission of toxic vapor by-products generated from spilling water-reactive materials in water.

Small spills involve 208 liters (55 US gallons) or less.

Large spills involve greater quantities.

Page 286

Downwinddispersionof thevapor was estimatedfor eachcasemodeled. Usingadatabase containing hourly meteorological data from 120 American, Canadian, and Mexican cities, the atmospheric parameters affecting the dispersion and the emission rate were selected.

The dispersion calculation accounted for both the:

- time-dependent emission rate from the source

- density of the vapor plume (i.e., heavy gas effects)

Since atmospheric mixing is less effective at dispersing vapor plumes during nighttime, day and night were separated in the analysis.

In the table:

- day refers to time periods after sunrise and before sunset

- night includes all hours between sunset and sunrise

Toxicological short-termexposure guidelines for the materials were applied to determine the downwind distance to which people may:

- become incapacitated and unable to take protective action

- incur serious health effects after a single, or rare, exposure

When available, toxicological exposure guidelines were chosen from AEGL-2 or ERPG-2 emergency response guidelines. AEGL-2 values were the first choice.

For materials without AEGL-2 or ERPG-2 values, emergency response guidelines were estimated based on lethal concentration limits derived from animal-based-studies. This approach was recommended by an independent panel of toxicological experts fromindustry and academia.

Page 287

##### HOWTO USE TABLE 1 – INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

- The responder should already have:

  - identified the material by its IDnumber and name (if you cannot find an IDnumber, use the Name of Material index in the blue section to find that number);

  - confirmed that the material is highlighted in green in the yellow or blue section. If not, Table 1 doesn’t apply;

  - found the three-digit guide for the material, in order to consult emergency actions it recommends along with this table; and

###### noted the wind direction

- Look in Table 1 (green section) for the ID number and name of the material involved. Some IDnumbers have more than one shipping name listed. Look for the specific name of the material. If you do not know the shipping name and Table 1 lists more than one name for the same ID number, use the entry with the largest distances.

- Determine if the incident involves a SMALL or LARGE spill and if it is DAY or NIGHT. ASMALL SPILLconsists of areleaseof 208 liters (55 USgallons) or less. This generally corresponds toaspill fromasinglesmall package(for example, adrum), asmall cylinder, or a small leak froma large package. ALARGESPILL consists of arelease of more than 208 liters (55 US gallons). This usually involves a spill from a large package, or multiple spills from many small packages. DAY is any time after sunrise and before sunset. NIGHT is any time between sunset and sunrise.

- Look up the INITIAL ISOLATION DISTANCE.

Thisdistancedefinestheradiusof azone(initial isolation zone) surrounding the spill in ALL DIRECTIONS. In this zone, protective clothing andrespiratory protectionis required. Evacuate the general public in a direction perpendicular to wind direction (crosswind) and away from the spill.

- LookupthePROTECTIVEACTIONDISTANCE.

Spill

Initial Isolation Zone

Initial Isolation Distance

For a given material, spill size, and whether day or night, Table 1 gives the downwind distance—in kilometers and miles—from the spill or leak source, for which you should consider protective actions. For practical purposes, the protective action zone (i.e., the area in which people are at risk of harmful exposure) is a square. Its length and width are the same as the downwind distance shown in Table 1. Protective actions are the steps you take to preserve the health and safety of emergency responders and

Page 288

the public. People in this area should be evacuated and/or sheltered-in-place. For more information, consult the “Protective Actions” section.

- Initiate protective actions beginning with those closest to the spill site and working away in a downwind direction. When a water-reactive TIH (PIH in the US) producing material is spilled into a river or stream, the source of the toxic gas may move with the current or stretch from the spill point downstream for a large distance.

In the figure below, the spill is located at the center of the small black circle. The larger circle represents the initial isolation zone around the spill. The square (the protective action zone) is the area in which you should take protective actions.

Wind Direction

##### UPWIND	DOWNWIND

Crosswind Direction(s)

Protective Action Zone

1/2 Downwind Distance

Initial Isolation Zone

Spill

1/2 Downwind Distance

Downwind Distance

Note 1: For factors that may change the protective action distances, see the "Introduction to Green Tables" section.

Note 2: When a product in Table 1 has the mention (when spilled in water), you can refer to Table 2 for the list of gases produced when these materials are spilled in water. The TIH gases indicated in Table 2 are for information purposes only.

Note 3: For the instantaneous release of the entire contents of a package (e.g., as a result of terrorism, sabotage or catastrophic failure), the distances should be doubled.

For more information on the material, safety precautions and mitigation procedures, call the emergency response telephone number listed on the shipping paper or the appropriate response agency as soon as possible.

Page 289

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1005 1005 | 125 125 | Ammonia, anhydrous Anhydrous ammonia | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | Refer to Table 3 |  |  |  |  |  |
| 1008 1008 | 125 125 | Boron trifluoride Boron trifluoride, compressed | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.5 mi) | 400 m | (1250 ft) | 2.4 km | (1.5 mi) | 4.7 km | (2.9 mi) |
| 1016 | 119 | Carbon monoxide, compressed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 200 m | (600 ft) | 1.2 km | (0.7 mi) | 3.9 km | (2.4 mi) |
| 1017 | 124 | Chlorine | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 1.5 km | (0.9 mi) | Refer to Table 3 |  |  |  |  |  |
| 1026 | 119 | Cyanogen | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 60 m (200 ft) |  | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) |
| 1040 1040 | 119P 119P | Ethylene oxide Ethylene oxide with nitrogen | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | Refer to Table 3 |  |  |  |  |  |
| 1045 | 124 | Fluorine, compressed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 100 m (300 ft) |  | 0.5 km | (0.3 mi) | 2.3 km | (1.4 mi) |
| 1048 | 125 | Hydrogen bromide, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.7 mi) | 3.2 km | (2.0 mi) |
| 1050 | 125 | Hydrogen chloride, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | Refer to Table 3 |  |  |  |  |  |
| 1051 | 117P | Hydrogen cyanide, stabilized | 60 m | (200 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.4 mi) | 200 m | (600 ft) | 0.7 km | (0.5 mi) | 1.8 km | (1.1 mi) |
| 1052 | 125 | Hydrogen fluoride, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | Refer to Table 3 |  |  |  |  |  |
| 1053 1053 | 117 117 | Hydrogen sulfide Hydrogen sulphide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 400 m | (1250 ft) | 2.4 km | (1.5 mi) | 6.3 km | (4.0 mi) |
| 1061 | 118 | Methylamine, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 200 m | (600 ft) | 0.6 km | (0.4 mi) | 2.1 km | (1.3 mi) |
| 1062 | 123 | Methyl bromide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 150 m | (500 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) |
| 1064 | 117 | Methyl mercaptan | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 200 m | (600 ft) | 1.3 km | (0.8 mi) | 3.9 km | (2.4 mi) |
| 1067 1067 | 124 124 | Dinitrogen tetroxide Nitrogen dioxide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 400 m | (1250 ft) | 1.4 km | (0.9 mi) | 3.3 km | (2.1 mi) |

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 290

| 1069 | 125 | Nitrosyl chloride | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.0 km | (0.7 mi) | 800 m | (2500 ft) | 4.3 km | (2.7 mi) | 9.6 km | (6.0 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1076 | 125 | Phosgene | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.5 km | (1.6 mi) | 500 m | (1500 ft) | 3.0 km | (1.9 mi) | 9.5 km | (5.9 mi) |
| 1079 1079 | 125 125 | Sulfur dioxide Sulphur dioxide | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.6 km | (1.6 mi) | Refer to Table 3 |  |  |  |  |  |
| 1082 1082 | 119P 119P | Refrigerant gas R-1113 Trifluorochloroethylene, stabilized | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.4 km | (0.3 mi) | 0.7 km | (0.5 mi) |
| 1092 | 131P | Acrolein, stabilized | 100 m | (300 ft) | 1.3 km | (0.8 mi) | 3.5 km | (2.2 mi) | 600 m | (2000 ft) | 6.8 km | (4.2 mi) | 11.1 km | (6.9 mi) |
| 1093 | 131P | Acrylonitrile, stabilized | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 1.3 km | (0.8 mi) | 2.3 km | (1.5 mi) |
| 1098 | 131 | Allyl alcohol | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.8 km | (0.5 mi) | 1.2 km | (0.8 mi) |
| 1135 | 131 | Ethylene chlorohydrin | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 1143 1143 | 131P 131P | Crotonaldehyde Crotonaldehyde, stabilized | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.7 km | (0.5 mi) |
| 1162 | 155 | Dimethyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.2 km | (0.8 mi) |
| 1163 | 131 | Dimethylhydrazine, unsymmetrical | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.3 mi) | 100 m | (300 ft) | 1.0 km | (0.7 mi) | 1.8 km | (1.1 mi) |
| 1182 | 155 | Ethyl chloroformate | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.9 km | (0.6 mi) |
| 1183 | 139 | Ethyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (0.9 mi) |
| 1185 | 131P | Ethyleneimine, stabilized | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.3 mi) | 200 m | (600 ft) | 1.0 km | (0.6 mi) | 1.8 km | (1.1 mi) |
| 1196 | 155 | Ethyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 150 m | (500 ft) | 1.8 km | (1.2 mi) | 3.7 km | (2.3 mi) |
| 1238 | 155 | Methyl chloroformate | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.5 km | (0.4 mi) | 150 m | (500 ft) | 1.2 km | (0.7 mi) | 2.2 km | (1.4 mi) |
| 1239 | 131 | Methyl chloromethyl ether | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |

Page 291

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 292

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1242 | 139 | Methyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.4 mi) | 1.7 km | (1.1 mi) |
| 1244 | 131 | Methylhydrazine | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.6 km | (0.4 mi) | 150 m | (500 ft) | 1.5 km | (0.9 mi) | 2.2 km | (1.4 mi) |
| 1250 | 155 | Methyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.9 km | (1.2 mi) |
| 1251 | 131P | Methyl vinyl ketone, stabilized | 100 m | (300 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 800 m | (2500 ft) | 1.7 km | (1.1 mi) | 2.8 km | (1.8 mi) |
| 1259 | 131 | Nickel carbonyl | 100 m | (300 ft) | 1.4 km | (0.9 mi) | 5.2 km | (3.3 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 1295 | 139 | Trichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (0.9 mi) |
| 1298 | 155 | Trimethylchlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.0 km | (0.6 mi) |
| 1305 | 155P | Vinyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.3 km | (0.8 mi) |
| 1340 1340 | 139 139 | Phosphorus pentasulfide, free fromyellow and white phosphorus (when spilled in water) Phosphorus pentasulphide, free fromyellow and white phosphorus (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.0 km | (0.6 mi) |
| 1360 | 139 | Calcium phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 200 m | (600 ft) | 0.8 km | (0.5 mi) | 2.7 km | (1.7 mi) |

| 1380 | 135 | Pentaborane | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 2.0 km | (1.3 mi) | 300 m | (1000 ft) | 3.0 km | (1.9 mi) | 6.5 km | (4.1 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1384 1384 1384 | 135 135 135 | Sodium dithionite (when spilled in water) Sodium hydrosulfite (when spilled in water) Sodium hydrosulphite (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 2.1 km | (1.3 mi) |
| 1390 | 139 | Alkali metal amides (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.8 km | (1.1 mi) |
| 1397 | 139 | Aluminum phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.7 km | (0.4 mi) | 400 m | (1250 ft) | 1.6 km | (1.0 mi) | 4.7 km | (2.9 mi) |
| 1419 | 139 | Magnesium aluminum phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 400 m | (1250 ft) | 1.4 km | (0.9 mi) | 4.1 km | (2.6 mi) |
| 1432 | 139 | Sodium phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.0 km | (0.6 mi) | 3.0 km | (1.9 mi) |
| 1510 | 143 | Tetranitromethane | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 0.7 km | (0.4 mi) |
| 1541 156 Acetone cyanohydrin, stabilized (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.3 mi) |
| 1556 | 152 | Methyldichloroarsine | 150 m | (500 ft) | 1.4 km | (0.9 mi) | 2.2 km | (1.4 mi) | 300 m | (1000 ft) | 4.0 km | (2.5 mi) | 5.8 km | (3.6 mi) |
| 1560 1560 | 157 157 | Arsenic chloride Arsenic trichloride | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 1.0 km | (0.7 mi) | 1.5 km | (1.0 mi) |
| 1569 | 131 | Bromoacetone | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.2 km | (0.8 mi) | 150 m | (500 ft) | 1.8 km | (1.1 mi) | 3.3 km | (2.1 mi) |
| 1580 | 154 | Chloropicrin | 60 m | (200 ft) | 0.5 km | (0.4 mi) | 1.2 km | (0.8 mi) | 200 m | (600 ft) | 2.4 km | (1.5 mi) | 3.7 km | (2.3 mi) |
| 1581 | 123 | Chloropicrin and methyl bromide mixture | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 300 m | (1000 ft) | 2.1 km | (1.3 mi) | 5.9 km | (3.7 mi) |
| 1582 | 119 | Chloropicrin and methyl chloride mixture | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 2.1 km | (1.3 mi) |

Page 293

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 294

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1583 | 154 | Chloropicrin mixture, n.o.s. | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 300 m | (1000 ft) | 2.1 km | (1.3 mi) | 5.9 km | (3.7 mi) |
| 1589 | 125 | Cyanogen chloride, stabilized | 300 m (1000 ft) |  | 1.9 km | (1.2 mi) | 6.6 km | (4.1 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 1595 1595 | 156 156 | Dimethyl sulfate Dimethyl sulphate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.4 mi) |
| 1605 | 154 | Ethylene dibromide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) |
| 1612 | 123 | Hexaethyl tetraphosphate and compressed gas mixture | 100 m | (300 ft) | 0.8 km | (0.5 mi) | 2.7 km | (1.7 mi) | 400 m | (1250 ft) | 3.5 km | (2.2 mi) | 8.1 km | (5.1 mi) |
| 1613 1613 | 154 154 | Hydrocyanic acid, aqueous solution, with not more than 20% hydrogen cyanide Hydrogen cyanide, aqueous solution, with not more than 20% hydrogen cyanide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 1.1 km | (0.7 mi) |
| 1614 | 152 | Hydrogen cyanide, stabilized (absorbed) | 60 m | (200 ft) | 0.2 km | (0.1 mi) | 0.6 km | (0.4 mi) | 150 m | (500 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) |
| 1647 | 151 | Methyl bromide and ethylene dibromide mixture, liquid | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 150 m | (500 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) |
| 1660 | 124 | Nitric oxide, compressed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km | (1.4 mi) |
| 1670 | 157 | Perchloromethyl mercaptan | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.2 mi) | 100 m | (300 ft) | 0.8 km | (0.5 mi) | 1.3 km | (0.8 mi) |
| 1672 | 151 | Phenylcarbylamine chloride | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.7 km | (0.4 mi) |
| 1680 | 157 | Potassium cyanide, solid (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.4 mi) |

| 1689 | 157 | Sodium cyanide, solid (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.2 km | (0.2 mi) | 0.9 km | (0.6 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1695 | 131 | Chloroacetone, stabilized | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.4 km | (0.3 mi) | 0.6 km | (0.4 mi) |
| 1716 | 156 | Acetyl bromide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.7 km | (0.4 mi) |
| 1717 | 155 | Acetyl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.7 km | (0.4 mi) | 2.0 km | (1.2 mi) |
| 1722 1722 | 155 155 | Allyl chlorocarbonate Allyl chloroformate | 100 m | (300 ft) | 0.3 km | (0.2 mi) | 0.8 km | (0.5 mi) | 400 m | (1250 ft) | 1.5 km | (0.9 mi) | 2.4 km | (1.5 mi) |
| 1724 | 155 | Allyltrichlorosilane, stabilized (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.2 km | (0.8 mi) |
| 1725 | 137 | Aluminum bromide, anhydrous (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) |
| 1726 | 137 | Aluminum chloride, anhydrous (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.5 km | (1.0 mi) |
| 1728 | 156 | Amyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.2 km | (0.7 mi) |
| 1732 | 157 | Antimony pentafluoride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 0.8 km | (0.5 mi) | 3.0 km | (1.9 mi) |
| 1741 | 125 | Boron trichloride (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 1.3 km | (0.8 mi) |
| 1741 | 125 | Boron trichloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 2.8 km | (1.7 mi) |
| 1744 1744 1744 | 154 154 154 | Bromine Bromine, solution Bromine, solution (Inhalation Hazard Zone A) | 60 m | (200 ft) | 0.8 km | (0.5 mi) | 2.4 km | (1.5 mi) | 400 m (1250 ft) |  | 4.2 km | (2.6 mi) | 7.6 km | (4.7 mi) |

Page 295

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 296

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1744 | 154 | Bromine, solution (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 0.5 km | (0.3 mi) |
| 1745 | 144 | Bromine pentafluoride (when spilled on land) | 100 m | (300 ft) | 0.9 km | (0.5 mi) | 2.7 km | (1.7 mi) | 500 m | (1500 ft) | 5.7 km | (3.6 mi) | 10.8 km | (6.7 mi) |
| 1745 | 144 | Bromine pentafluoride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 3.0 km | (1.9 mi) |
| 1746 | 144 | Bromine trifluoride (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.5 km | (0.3 mi) |
| 1746 | 144 | Bromine trifluoride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 0.8 km | (0.5 mi) | 2.8 km | (1.8 mi) |
| 1747 | 155 | Butyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.2 km | (0.7 mi) |
| 1749 | 124 | Chlorine trifluoride | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |
| 1752 | 156 | Chloroacetyl chloride (when spilled on land) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 1.2 km | (0.8 mi) | 1.9 km | (1.2 mi) |
| 1752 | 156 | Chloroacetyl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) |
| 1753 | 156 | Chlorophenyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.4 mi) |

| 1754 1754 | 137 137 | Chlorosulfonic acid (with or without sulfur trioxide) (when spilled on land) Chlorosulphonic acid (with or without sulphur trioxide) (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.3 km | (0.2 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1754 1754 | 137 137 | Chlorosulfonic acid (with or without sulfur trioxide) (when spilled in water) Chlorosulphonic acid (with or without sulphur trioxide) (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.7 km | (1.1 mi) |
| 1758 | 137 | Chromium oxychloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) |
| 1762 | 156 | Cyclohexenyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.8 km | (0.5 mi) |
| 1763 | 156 | Cyclohexyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.8 km | (0.5 mi) |
| 1765 | 156 | Dichloroacetyl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) |
| 1766 | 156 | Dichlorophenyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.4 km | (0.9 mi) |
| 1767 | 155 | Diethyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.6 km | (0.4 mi) |
| 1769 | 156 | Diphenyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.7 km | (0.5 mi) |
| 1771 | 156 | Dodecyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.8 km | (0.5 mi) |

Page 297

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 298

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1777 1777 | 137 137 | Fluorosulfonic acid (when spilled in water) Fluorosulphonic acid (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) |
| 1781 | 156 | Hexadecyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) |
| 1784 | 156 | Hexyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.9 km | (0.6 mi) |
| 1799 | 156 | Nonyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.0 km | (0.6 mi) |
| 1800 | 156 | Octadecyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.9 km | (0.6 mi) |
| 1801 | 156 | Octyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.0 km | (0.6 mi) |
| 1804 | 156 | Phenyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.0 km | (0.6 mi) |
| 1806 | 137 | Phosphorus pentachloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.9 km | (0.6 mi) |
| 1808 | 137 | Phosphorus tribromide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) |
| 1809 | 137 | Phosphorus trichloride (when spilled on land) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 1.1 km | (0.7 mi) | 2.0 km | (1.3 mi) |

| 1809 | 137 | Phosphorus trichloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.8 km | (1.1 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1810 | 137 | Phosphorus oxychloride (when spilled on land) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 1.1 km | (0.7 mi) | 1.8 km | (1.2 mi) |
| 1810 | 137 | Phosphorus oxychloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) |
| 1815 | 155 | Propionyl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) |
| 1816 | 155 | Propyltrichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.3 km | (0.8 mi) |
| 1818 | 157 | Silicon tetrachloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 2.0 km | (1.3 mi) |
| 1828 1828 | 137 137 | Sulfur chlorides (when spilled on land) Sulphur chlorides (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.3 mi) |
| 1828 1828 | 137 137 | Sulfur chlorides (when spilled in water) Sulphur chlorides (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.4 mi) |
| 1829 1829 | 137 137 | Sulfur trioxide, stabilized Sulphur trioxide, stabilized | 60 m | (200 ft) | 0.4 km | (0.2 mi) | 1.0 km | (0.6 mi) | 300 m | (1000 ft) | 2.9 km | (1.8 mi) | 6.3 km | (4.0 mi) |
| 1831 1831 | 137 137 | Sulfuric acid, fuming Sulphuric acid, fuming | 60 m | (200 ft) | 0.4 km | (0.2 mi) | 1.0 km | (0.6 mi) | 300 m | (1000 ft) | 2.9 km | (1.8 mi) | 6.3 km | (4.0 mi) |
| 1834 1834 | 137 137 | Sulfuryl chloride (when spilled on land) Sulphuryl chloride (when spilled on land) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.4 km | (0.3 mi) | 60 m | (200 ft) | 0.8 km | (0.5 mi) | 1.5 km | (0.9 mi) |

Page 299

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 300

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1834 1834 | 137 137 | Sulfuryl chloride (when spilled in water) Sulphuryl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) |
| 1836 | 137 | Thionyl chloride (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.5 km | (0.4 mi) |
| 1836 | 137 | Thionyl chloride (when spilled in water) | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 2.9 km | (1.8 mi) | 600 m | (2000 ft) | 7.6 km | (4.7 mi) | 11.0+ km (7.0+ mi) |  |
| 1838 | 137 | Titanium tetrachloride (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 0.5 km | (0.3 mi) |
| 1838 | 137 | Titanium tetrachloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.2 km | (0.7 mi) |
| 1859 1859 | 125 125 | Silicon tetrafluoride Silicon tetrafluoride, compressed | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.8 km | (0.5 mi) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 1.8 km | (1.2 mi) |
| 1892 | 151 | Ethyldichloroarsine | 150 m | (500 ft) | 1.5 km | (1.0 mi) | 2.2 km | (1.4 mi) | 400 m | (1250 ft) | 5.1 km | (3.2 mi) | 6.4 km | (4.0 mi) |
| 1898 | 156 | Acetyl iodide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.9 km | (0.6 mi) |
| 1911 1911 | 119 119 | Diborane Diborane mixtures | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 1.2 km | (0.7 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 4.6 km | (2.9 mi) |

| 1923 1923 1923 | 135 135 135 | Calcium dithionite (when spilled in water) Calcium hydrosulfite (when spilled in water) Calcium hydrosulphite (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 60 m | (200 ft) | 0.5 km | (0.4 mi) | 2.1 km | (1.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1929 1929 1929 | 135 135 135 | Potassium dithionite (when spilled in water) Potassium hydrosulfite (when spilled in water) Potassium hydrosulphite (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.9 km | (1.2 mi) |
| 1931 1931 1931 | 171 171 171 | Zinc dithionite (when spilled in water) Zinc hydrosulfite (when spilled in water) Zinc hydrosulphite (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.9 km | (1.2 mi) |
| 1953 1953 | 119 119 | Compressed gas, poisonous, flammable, n.o.s. Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 1953 | 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 1953 1953 | 119 119 | Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) Compressed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |

Page 301

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 302

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 1953 1953 | 119 119 | Compressed gas, toxic, flammable, n.o.s. Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 1953 | 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 1953 1953 | 119 119 | Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) Compressed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 1955 1955 | 123 123 | Compressed gas, poisonous, n.o.s. Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 1955 | 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |

| 1955 1955 | 123 123 | Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone C) Compressed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1955 1955 | 123 123 | Compressed gas, toxic, n.o.s. Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 1955 | 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |
| 1955 1955 | 123 123 | Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone C) Compressed gas, toxic, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 1955 1955 1955 | 123 123 123 | Organic phosphate compound mixed with compressed gas Organic phosphate mixed with compressed gas Organic phosphorus compound mixed with compressed gas | 100 m | (300 ft) | 1.0 km | (0.7 mi) | 3.4 km | (2.1 mi) | 500 m | (1500 ft) | 4.4 km | (2.7 mi) | 9.6 km | (6.0 mi) |
| 1967 1967 1967 | 123 123 123 | Insecticide gas, poisonous, n.o.s. Insecticide gas, toxic, n.o.s. Parathion and compressed gas mixture | 100 m | (300 ft) | 1.0 km | (0.7 mi) | 3.4 km | (2.1 mi) | 500 m | (1500 ft) | 4.4 km | (2.7 mi) | 9.6 km | (6.0 mi) |
| 1975 1975 | 124 124 | Nitric oxide and dinitrogen tetroxide mixture Nitric oxide and nitrogen dioxide mixture | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km | (1.4 mi) |
| 1994 | 136 | Iron pentacarbonyl | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 2.1 km | (1.3 mi) | 400 m | (1250 ft) | 5.2 km | (3.2 mi) | 7.8 km | (4.8 mi) |

Page 303

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 2004 | 135 | Magnesium diamide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.8 km | (1.1 mi) |
| 2011 | 139 | Magnesium phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 400 m | (1250 ft) | 1.4 km | (0.9 mi) | 3.9 km | (2.4 mi) |
| 2012 | 139 | Potassium phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 200 m | (600 ft) | 0.9 km | (0.6 mi) | 2.8 km | (1.8 mi) |
| 2013 | 139 | Strontium phosphide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 200 m | (600 ft) | 0.8 km | (0.5 mi) | 2.7 km | (1.7 mi) |
| 2032 | 157 | Nitric acid, red fuming | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 150 m | (500 ft) | 0.3 km | (0.2 mi) | 0.5 km | (0.3 mi) |
| 2186 | 125 | Hydrogen chloride, refrigerated liquid | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | Refer to Table 3 |  |  |  |  |  |
| 2188 | 119 | Arsine | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 2189 | 119 | Dichlorosilane | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 2190 | 124 | Oxygen difluoride, compressed | 300 m (1000 ft) |  | 1.8 km | (1.1 mi) | 7.2 km | (4.5 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 2191 2191 | 123 123 | Sulfuryl fluoride Sulphuryl fluoride | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 400 m | (1250 ft) | 2.2 km | (1.4 mi) | 5.0 km | (3.1 mi) |
| 2192 | 119 | Germane | 150 m | (500 ft) | 0.9 km | (0.5 mi) | 3.3 km | (2.1 mi) | 600 m | (2000 ft) | 3.6 km | (2.3 mi) | 7.4 km | (4.6 mi) |
| 2194 | 125 | Selenium hexafluoride | 200 m | (600 ft) | 1.1 km | (0.7 mi) | 3.4 km | (2.1 mi) | 600 m | (2000 ft) | 3.9 km | (2.4 mi) | 7.6 km | (4.8 mi) |
| 2195 | 125 | Tellurium hexafluoride | 1000 m | (3000 ft) | 5.9 km | (3.7 mi) | 11.1 km | (6.9 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 2196 | 125 | Tungsten hexafluoride | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.8 km | (0.5 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.8 km | (1.7 mi) |
| 2197 | 125 | Hydrogen iodide, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 304

| 2198 2198 | 125 125 | Phosphorus pentafluoride Phosphorus pentafluoride, compressed | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.0 km | (0.7 mi) | 200 m | (600 ft) | 1.1 km (0.7 mi) 3.5 km (2.2 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2199 | 119 | Phosphine | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) | 400 m | (1250 ft) | 1.3 km (0.8 mi) 3.7 km (2.3 mi) |
| 2202 | 117 | Hydrogen selenide, anhydrous | 300 m | (1000 ft) | 1.7 km | (1.1 mi) | 6.0 km | (3.7 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) 11.0+ km (7.0+ mi) |
| 2204 2204 | 119 119 | Carbonyl sulfide Carbonyl sulphide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 300 m | (1000 ft) | 1.5 km (1.0 mi) 3.6 km (2.3 mi) |
| 2232 2232 | 153 153 | Chloroacetaldehyde 2-Chloroethanal | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km (0.4 mi) 1.1 km (0.7 mi) |
| 2285 | 155 | Isocyanatobenzotrifluorides | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km (0.3 mi) 0.6 km (0.4 mi) |
| 2308 2308 | 157 157 | Nitrosylsulfuric acid, liquid (when spilled in water) Nitrosylsulphuric acid, liquid (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 300 m | (1000 ft) | 0.8 km (0.5 mi) 2.3 km (1.4 mi) |
| 2334 | 131 | Allylamine | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.4 mi) | 150 m | (500 ft) | 1.6 km (1.0 mi) 2.5 km (1.6 mi) |
| 2337 | 131 | Phenyl mercaptan | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km (0.2 mi) 0.4 km (0.2 mi) |
| 2353 | 155 | Butyryl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km (0.1 mi) 0.5 km (0.3 mi) |
| 2382 | 131 | Dimethylhydrazine, symmetrical | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.8 km (0.5 mi) 1.3 km (0.8 mi) |
| 2395 | 155 | Isobutyryl chloride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km (0.1 mi) 0.3 km (0.2 mi) |
| 2407 | 155 | Isopropyl chloroformate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km (0.4 mi) 0.9 km (0.6 mi) |
| 2417 | 125 | Carbonyl fluoride | 100 m | (300 ft) | 0.7 km | (0.5 mi) | 2.5 km | (1.6 mi) | 600 m | (2000 ft) | 3.8 km (2.4 mi) 8.2 km (5.1 mi) |
| 2418 2418 | 125 125 | Sulfur tetrafluoride Sulphur tetrafluoride | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.4 km | (1.5 mi) | 400 m | (1250 ft) | 2.4 km (1.5 mi) 5.9 km (3.7 mi) |
| 2420 | 125 | Hexafluoroacetone | 100 m | (300 ft) | 0.7 km | (0.4 mi) | 2.7 km | (1.7 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) 11.0+ km (7.0+ mi) |

Page 305

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 306

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 2421 | 124 | Nitrogen trioxide | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 1.2 km | (0.8 mi) | 200 m | (600 ft) | 1.4 km | (0.9 mi) | 4.3 km | (2.7 mi) |
| 2434 | 156 | Dibenzyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) |
| 2435 | 156 | Ethylphenyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.6 km | (0.4 mi) |
| 2437 | 156 | Methylphenyldichlorosilane (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.8 km | (0.5 mi) |
| 2438 | 131 | Trimethylacetyl chloride | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.0 km | (0.7 mi) | 200 m | (600 ft) | 2.3 km | (1.5 mi) | 3.3 km | (2.1 mi) |
| 2442 | 156 | Trichloroacetyl chloride | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km | (0.5 mi) | 1.1 km | (0.7 mi) |
| 2474 | 156 | Thiophosgene | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.8 km | (1.1 mi) | 200 m | (600 ft) | 2.3 km | (1.4 mi) | 4.2 km | (2.6 mi) |
| 2477 | 131 | Methyl isothiocyanate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.3 mi) |
| 2478 | 155 | Isocyanate solution, flammable, poisonous, n.o.s. Isocyanate solution, flammable, toxic, n.o.s. Isocyanates, flammable, poisonous, n.o.s. Isocyanates, flammable, toxic, n.o.s. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2478 | 155 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2478 | 155 |  | 60 m | (200 ft) | 0.8 km | (0.5 mi) | 1.8 km | (1.2 mi) | 400 m | (1250 ft) | 4.7 km | (3.0 mi) | 7.0 km | (4.4 mi) |
| 2478 | 155 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2480 | 155P | Methyl isocyanate | 150 m | (500 ft) | 1.7 km | (1.1 mi) | 5.2 km | (3.3 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 2481 | 155 | Ethyl isocyanate | 150 m | (500 ft) | 2.0 km | (1.3 mi) | 5.3 km | (3.3 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 2482 | 155P | n-Propyl isocyanate | 100 m | (300 ft) | 1.3 km | (0.8 mi) | 2.8 km | (1.8 mi) | 600 m | (2000 ft) | 7.8 km | (4.8 mi) | 10.7 km | (6.6 mi) |

| 2483 | 155P | Isopropyl isocyanate | 150 m | (500 ft) | 1.5 km | (1.0 mi) | 3.3 km | (2.1 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2484 | 155 | tert-Butyl isocyanate | 60 m | (200 ft) | 0.8 km | (0.5 mi) | 1.8 km | (1.2 mi) | 400 m | (1250 ft) | 4.7 km | (3.0 mi) | 7.0 km | (4.4 mi) |
| 2485 | 155P | n-Butyl isocyanate | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.2 km | (0.8 mi) | 300 m | (1000 ft) | 2.9 km | (1.8 mi) | 4.2 km | (2.6 mi) |
| 2486 | 155P | Isobutyl isocyanate | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.3 km | (0.8 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 4.8 km | (3.0 mi) |
| 2487 | 155 | Phenyl isocyanate | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 1.5 km | (0.9 mi) | 400 m | (1250 ft) | 4.2 km | (2.6 mi) | 5.4 km | (3.4 mi) |
| 2488 | 155 | Cyclohexyl isocyanate | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.3 mi) | 100 m | (300 ft) | 1.1 km | (0.7 mi) | 1.4 km | (0.9 mi) |
| 2495 | 144 | Iodine pentafluoride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 3.2 km | (2.0 mi) |
| 2521 | 131P | Diketene, stabilized | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km | (0.4 mi) | 1.0 km | (0.6 mi) |
| 2534 | 119 | Methylchlorosilane | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 1.8 km | (1.1 mi) |
| 2548 | 124 | Chlorine pentafluoride | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |  |
| 2605 | 155 | Methoxymethyl isocyanate | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km | (0.4 mi) | 0.9 km | (0.6 mi) |
| 2606 | 155 | Methyl orthosilicate | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km | (0.5 mi) | 1.1 km | (0.7 mi) |
| 2644 | 151 | Methyl iodide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 100 m | (300 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.4 mi) |
| 2646 | 151 | Hexachlorocyclopentadiene | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.3 km | (0.2 mi) |
| 2668 | 131 | Chloroacetonitrile | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.2 mi) |
| 2676 | 119 | Stibine | 60 m | (200 ft) | 0.3 km | (0.2 mi) | 1.6 km | (1.0 mi) | 200 m | (600 ft) | 1.3 km | (0.8 mi) | 4.1 km | (2.6 mi) |
| 2691 | 137 | Phosphorus pentabromide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) |
| 2692 | 157 | Boron tribromide (when spilled on land) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.4 km | (0.3 mi) |
| 2692 | 157 | Boron tribromide (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.4 km | (0.9 mi) |
| 2740 | 155 | n-Propyl chloroformate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.0 km | (0.7 mi) |

Page 307

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 308

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 2742 2742 | 155 155 | Chloroformates, poisonous, corrosive, flammable, n.o.s. Chloroformates, toxic, corrosive, flammable, n.o.s. | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.7 km | (0.5 mi) |
| 2743 | 155 | n-Butyl chloroformate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.4 km | (0.3 mi) |
| 2806 | 139 | Lithiumnitride (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 60 m | (200 ft) | 0.4 km | (0.3 mi) | 1.6 km | (1.0 mi) |
| 2826 | 155 | Ethyl chlorothioformate | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.2 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.7 km | (0.5 mi) |
| 2845 | 135 | Ethyl phosphonous dichloride, anhydrous | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 100 m | (300 ft) | 1.4 km | (0.9 mi) | 2.3 km | (1.4 mi) |
| 2845 | 135 | Methyl phosphonous dichloride | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.2 km | (0.7 mi) | 200 m | (600 ft) | 2.6 km | (1.6 mi) | 4.2 km | (2.6 mi) |
| 2901 | 124 | Bromine chloride | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 1.8 km | (1.1 mi) | 1000 m (3000 ft) |  | 5.7 km | (3.5 mi) | 11.0+ km (7.0+ mi) |  |
| 2927 | 154 | Ethyl phosphonothioic dichloride, anhydrous | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) |
| 2927 | 154 | Ethyl phosphorodichloridate | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.3 km | (0.2 mi) |
| 2965 | 139 | Boron trifluoride dimethyl etherate (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 100 m | (300 ft) | 0.9 km | (0.6 mi) | 2.8 km | (1.7 mi) |

| 2977 166 Radioactive material, uranium hexafluoride, fissile (when spilled in water) 2977 166 Uranium hexafluoride, radioactive material, fissile (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.6 km (1.0 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2978 166 Radioactive material, uranium hexafluoride, non fissile or fissile-excepted (when spilled in water) 2978 166 Uranium hexafluoride, radioactive material, non fissile or fissile-excepted (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 30 m | (100 ft) | 0.4 km | (0.2 mi) | 1.6 km (1.0 mi) |
| 2985 155 Chlorosilanes, flammable, corrosive, n.o.s. (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km (1.0 mi) |
| 2986 155 Chlorosilanes, corrosive, flammable, n.o.s. (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km (1.0 mi) |
| 2987 156 Chlorosilanes, corrosive, n.o.s. (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km (1.0 mi) |
| 2988 139 Chlorosilanes, water-reactive, flammable, corrosive, n.o.s. (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km (1.0 mi) |
| 3023 | 131 | 2-Methyl-2-heptanethiol | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.8 km (0.5 mi) |
| 3048 157 Aluminum phosphide pesticide (when spilled in water) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.7 km | (0.4 mi) | 400 m | (1250 ft) | 1.6 km | (1.0 mi) | 4.5 km (2.8 mi) |
| 3057 | 125 | Trifluoroacetyl chloride | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.9 km | (0.6 mi) | 800 m | (2500 ft) | 4.9 km | (3.1 mi) | 11.0+ km (7.0+ mi) |

Page 309

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 310

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3079 | 131P | Methacrylonitrile, stabilized | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 150 m | (500 ft) | 1.7 km | (1.1 mi) | 2.8 km | (1.7 mi) |
| 3083 | 124 | Perchloryl fluoride | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 1000 m (3000 ft) |  | 5.5 km | (3.4 mi) | 10.9 km | (6.8 mi) |
| 3160 3160 | 119 119 | Liquefied gas, poisonous, flammable, n.o.s. Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3160 | 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 3160 3160 | 119 119 | Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3160 3160 | 119 119 | Liquefied gas, toxic, flammable, n.o.s. Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3160 | 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |

| 3160 3160 | 119 119 | Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3162 3162 | 123 123 | Liquefied gas, poisonous, n.o.s. Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3162 | 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |
| 3162 3162 | 123 123 | Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, poisonous, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3162 3162 | 123 123 | Liquefied gas, toxic, n.o.s. Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3162 | 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |
| 3162 3162 | 123 123 | Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, toxic, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3246 3246 | 156 156 | Methanesulfonyl chloride Methanesulphonyl chloride | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.7 km | (0.5 mi) | 1.0 km | (0.6 mi) |
| 3275 3275 | 131 131 | Nitriles, poisonous, flammable, n.o.s. Nitriles, toxic, flammable, n.o.s. | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 150 m | (500 ft) | 1.7 km | (1.1 mi) | 2.8 km | (1.7 mi) |

Page 311

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 312

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3276 3276 3276 3276 | 151 151 151 151 | Nitriles, liquid, poisonous, n.o.s. Nitriles, liquid, toxic, n.o.s. Nitriles, poisonous, liquid, n.o.s. Nitriles, toxic, liquid, n.o.s. | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 150 m | (500 ft) | 1.7 km | (1.1 mi) | 2.8 km | (1.7 mi) |
| 3278 3278 | 151 151 | Organophosphorus compound, liquid, poisonous, n.o.s. Organophosphorus compound, liquid, toxic, n.o.s. | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.2 km | (0.7 mi) | 200 m | (600 ft) | 2.6 km | (1.6 mi) | 4.2 km | (2.6 mi) |
| 3279 3279 | 131 131 | Organophosphorus compound, poisonous, flammable, n.o.s. Organophosphorus compound, toxic, flammable, n.o.s. | 30 m | (100 ft) | 0.4 km | (0.3 mi) | 1.2 km | (0.7 mi) | 200 m | (600 ft) | 2.6 km | (1.6 mi) | 4.2 km | (2.6 mi) |
| 3280 | 151 | Organoarsenic compound, liquid, n.o.s. | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.7 km | (0.5 mi) | 150 m | (500 ft) | 1.7 km | (1.1 mi) | 3.6 km | (2.2 mi) |
| 3281 | 151 | Metal carbonyls, liquid, n.o.s. | 100 m | (300 ft) | 1.4 km | (0.9 mi) | 5.2 km | (3.3 mi) | 1000 m (3000 ft) |  | 11.0+ km (7.0+ mi) |  | 11.0+ km (7.0+ mi) |  |
| 3294 | 131 | Hydrogen cyanide, solution in alcohol, with not more than 45% hydrogen cyanide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 150 m | (500 ft) | 0.7 km | (0.5 mi) | 2.0 km | (1.2 mi) |
| 3300 | 119P | Ethylene oxide and carbon dioxide mixture, with more than 87% ethylene oxide | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.2 mi) | 150 m | (500 ft) | 0.7 km | (0.5 mi) | 2.0 km | (1.2 mi) |

| 3303 3303 | 124 124 | Compressed gas, poisonous, oxidizing, n.o.s. Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3303 | 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 500 m | (1500 ft) | 3.5 km | (2.2 mi) | 9.9 km | (6.2 mi) |
| 3303 3303 | 124 124 | Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) Compressed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km | (1.4 mi) |
| 3303 3303 | 124 124 | Compressed gas, toxic, oxidizing, n.o.s. Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |  |
| 3303 | 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 500 m | (1500 ft) | 3.5 km | (2.2 mi) | 9.9 km | (6.2 mi) |
| 3303 3303 | 124 124 | Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) Compressed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km | (1.4 mi) |

Page 313

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 314

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3304 3304 | 125 125 | Compressed gas, poisonous, corrosive, n.o.s. Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 200 m | (600 ft) | 1.1 km | (0.7 mi) | 3.4 km | (2.1 mi) | 600 m | (2000 ft) | 3.9 km | (2.4 mi) | 7.6 km | (4.8 mi) |
| 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.7 km | (2.3 mi) |
| 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.2 km | (2.0 mi) |
| 3304 | 125 | Compressed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| 3304 3304 | 125 125 | Compressed gas, toxic, corrosive, n.o.s. Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) | 200 m | (600 ft) | 1.1 km | (0.7 mi) | 3.4 km | (2.1 mi) | 600 m | (2000 ft) | 3.9 km | (2.4 mi) | 7.6 km | (4.8 mi) |
| 3304 | 125 | Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.7 km | (2.3 mi) |
| 3304 | 125 | Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.2 km | (2.0 mi) |

| 3304 | 125 | Compressed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3305 119 Compressed gas, poisonous, flammable, corrosive, n.o.s. 3305 119 Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m | (3000 ft) | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3305 119 Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 3305 119 Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) 3305 119 Compressed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3305 3305 | 119 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m | (3000 ft) | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3305 | 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 3305 3305 | 119 119 | Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) Compressed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |

Page 315

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 316

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3306 124 Compressed gas, poisonous, oxidizing, corrosive, n.o.s. 3306 124 Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 1000 m (3000 ft) |  | 5.5 km | (3.4 mi) | 11.0+ km (7.0+ mi) |  |
| 3306 124 Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 10.9 km | (6.8 mi) |
| 3306 124 Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.2 km | (2.0 mi) |
| 3306 124 Compressed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| 3306 3306 | 124 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 1000 m (3000 ft) |  | 5.5 km | (3.4 mi) | 11.0+ km (7.0+ mi) |  |
| 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 10.9 km | (6.8 mi) |
| 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.2 km | (2.0 mi) |

| 3306 | 124 | Compressed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km (1.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3307 3307 | 124 124 | Liquefied gas, poisonous, oxidizing, n.o.s. Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |
| 3307 | 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 500 m | (1500 ft) | 2.8 km | (1.8 mi) | 10.9 km (6.8 mi) |
| 3307 3307 | 124 124 | Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km (1.4 mi) |
| 3307 124 Liquefied gas, toxic, oxidizing, n.o.s. 3307 124 Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) |  |  | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 800 m | (2500 ft) | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |
| 3307 | 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 500 m | (1500 ft) | 2.8 km | (1.8 mi) | 10.9 km (6.8 mi) |
| 3307 3307 | 124 124 | Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.6 km | (0.4 mi) | 100 m | (300 ft) | 0.6 km | (0.4 mi) | 2.2 km (1.4 mi) |

Page 317

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 318

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3308 3308 | 125 125 | Liquefied gas, poisonous, corrosive, n.o.s. Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) | 200 m | (600 ft) | 1.1 km | (0.7 mi) | 3.4 km | (2.1 mi) | 600 m | (2000 ft) | 3.9 km | (2.4 mi) | 7.6 km | (4.8 mi) |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.7 km | (2.3 mi) |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.2 km | (2.0 mi) |
| 3308 | 125 | Liquefied gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| 3308 125 Liquefied gas, toxic, corrosive, n.o.s. 3308 125 Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) |  |  | 200 m | (600 ft) | 1.1 km | (0.7 mi) | 3.4 km | (2.1 mi) | 600 m | (2000 ft) | 3.9 km | (2.4 mi) | 7.6 km | (4.8 mi) |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.7 km | (2.3 mi) |
| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.2 km | (2.0 mi) |

| 3308 | 125 | Liquefied gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3309 3309 | 119 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m | (3000 ft) | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3309 | 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.6 km | (0.4 mi) | 300 m | (1000 ft) | 2.5 km | (1.6 mi) | 3.1 km | (1.9 mi) |
| 3309 3309 | 119 119 | Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3309 3309 | 119 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m | (3000 ft) | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3309 | 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.3 km | (0.2 mi) | 0.6 km | (0.4 mi) | 300 m | (1000 ft) | 2.5 km | (1.6 mi) | 3.1 km | (1.9 mi) |
| 3309 3309 | 119 119 | Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) Liquefied gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |

Page 319

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 320

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3310 3310 | 124 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 1000 m (3000 ft) |  | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |  |
| 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 800 m | (2500 ft) | 4.5 km | (2.8 mi) | 10.9 km | (6.8 mi) |
| 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.2 km | (2.0 mi) |
| 3310 | 124 | Liquefied gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| 3310 3310 | 124 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.5 km | (0.3 mi) | 2.5 km | (1.6 mi) | 1000 m (3000 ft) |  | 5.1 km | (3.2 mi) | 11.0+ km (7.0+ mi) |  |
| 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 1.1 km | (0.7 mi) | 800 m | (2500 ft) | 4.5 km | (2.8 mi) | 10.9 km | (6.8 mi) |
| 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.5 km | (0.3 mi) | 300 m | (1000 ft) | 1.6 km | (1.0 mi) | 3.2 km | (2.0 mi) |

| 3310 | 124 | Liquefied gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3318 125 Ammonia solution, with more than 50% ammonia |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 150 m | (500 ft) | 0.8 km | (0.5 mi) | 2.0 km | (1.3 mi) |
| 3355 119 Insecticide gas, poisonous, flammable, n.o.s. 3355 119 Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3355 119 Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 3355 119 Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |
| 3355 119 Insecticide gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 0.6 km | (0.4 mi) | 1.6 km | (1.0 mi) |
| 3355 119 Insecticide gas, toxic, flammable, n.o.s. 3355 119 Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 3.9 km | (2.4 mi) | 1000 m (3000 ft) |  | 6.2 km | (3.9 mi) | 10.5 km | (6.5 mi) |
| 3355 119 Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.2 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.1 km | (1.9 mi) |
| 3355 119 Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 1.0 km | (0.6 mi) | 2.7 km | (1.7 mi) |

Page 321

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 322

|  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3355 | 119 Insecticide gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.3 km | (0.2 mi) | 150 m | (500 ft) | 0.6 km | (0.4 mi) | 1.6 km | (1.0 mi) |
| 3361 156 Chlorosilanes, poisonous, corrosive, n.o.s. (when spilled in water) 3361 156 Chlorosilanes, toxic, corrosive, n.o.s. (when spilled in water) |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km | (1.0 mi) |
| 3362 3362 | 155 Chlorosilanes, poisonous, corrosive, flammable, n.o.s. (when spilled in water) 155 Chlorosilanes, toxic, corrosive, flammable, n.o.s. (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.6 km | (1.0 mi) |
| 3381 3381 | 151 Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) 151 Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone A) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.8 km | (1.1 mi) | 200 m | (600 ft) | 2.3 km | (1.4 mi) | 4.2 km | (2.6 mi) |
| 3382 3382 | 151 Poisonous by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) 151 Toxic by inhalation liquid, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.8 km | (0.5 mi) |

| 3383 131 Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) 3383 131 Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3384 131 Poisonous by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) 3384 131 Toxic by inhalation liquid, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.9 km | (0.6 mi) |
| 3385 139 Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) 3385 139 Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone A) |  |  | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 1.8 km | (1.1 mi) | 200 m | (600 ft) | 2.3 km | (1.4 mi) | 4.2 km | (2.6 mi) |
| 3386 139 Poisonous by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) 3386 139 Toxic by inhalation liquid, water-reactive, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.8 km | (0.5 mi) |
| 3387 3387 | 142 142 | Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone A) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |

Page 323

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 324

|  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3388 3388 | 142 Poisonous by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) 142 Toxic by inhalation liquid, oxidizing, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 150 m | (500 ft) | 0.3 km | (0.2 mi) | 0.5 km | (0.3 mi) |
| 3389 3389 | 154 Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) 154 Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone A) | 100 m | (300 ft) | 0.3 km | (0.2 mi) | 0.7 km | (0.5 mi) | 800 m | (2500 ft) | 1.7 km | (1.1 mi) | 2.8 km | (1.8 mi) |
| 3390 3390 | 154 Poisonous by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) 154 Toxic by inhalation liquid, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.6 km | (0.4 mi) |
| 3456 3456 | 157 Nitrosylsulfuric acid, solid (when spilled in water) 157 Nitrosylsulphuric acid, solid (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.4 km | (0.3 mi) | 200 m | (600 ft) | 0.7 km | (0.4 mi) | 2.3 km | (1.5 mi) |

| 3488 131 Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) 3488 131 Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3489 131 Poisonous by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) 3489 131 Toxic by inhalation liquid, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.9 km | (0.6 mi) |
| 3490 155 Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone A) 3490 155 Toxic by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone A) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |
| 3491 155 Poisonous by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone B) 3491 155 Toxic by inhalation liquid, water-reactive, flammable, n.o.s. (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.9 km | (0.6 mi) |

Page 325

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 326

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3492 131 Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) 3492 131 Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone A) |  |  | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 1.5 km | (1.0 mi) | 300 m | (1000 ft) | 3.4 km | (2.1 mi) | 5.7 km | (3.6 mi) |
| 3493 131 Poisonous by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) 3493 131 Toxic by inhalation liquid, corrosive, flammable, n.o.s. (Inhalation Hazard Zone B) |  |  | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.3 km | (0.2 mi) | 60 m | (200 ft) | 0.6 km | (0.4 mi) | 0.9 km | (0.6 mi) |
| 3494 3494 | 131 131 | Petroleum sour crude oil, flammable, poisonous Petroleum sour crude oil, flammable, toxic | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.8 km | (0.5 mi) |
| 3507 | 166 | Uranium hexafluoride, radioactive material, excepted package, less than 0.1 kg per package, non-fissile or fissile-excepted (when spilled in water) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |

| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (Inhalation Hazard Zone A) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (Inhalation Hazard Zone B) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (Inhalation Hazard Zone C) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, poisonous, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 | Adsorbed gas, toxic, n.o.s. Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, toxic, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3512 | 173 |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3512 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3514 | 173 | Adsorbed gas, poisonous, flammable, n.o.s. Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, poisonous, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3514 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3514 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3514 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3514 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Page 327

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 328

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3514 173 Adsorbed gas, toxic, flammable, n.o.s. 3514 173 Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone A) 3514 173 Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone B) 3514 173 Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone C) 3514 173 Adsorbed gas, toxic, flammable, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3515 | 173 | Adsorbed gas, poisonous, oxidizing, n.o.s. Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, poisonous, oxidizing, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3515 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3515 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3515 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3515 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3515 173 Adsorbed gas, toxic, oxidizing, n.o.s. 3515 173 Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone A) 3515 173 Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone B) 3515 173 Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone C) 3515 173 Adsorbed gas, toxic, oxidizing, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3516 | 173 | Adsorbed gas, poisonous, corrosive, n.o.s. Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, poisonous, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3516 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3516 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3516 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3516 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Page 329

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 330

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3516 173 Adsorbed gas, toxic, corrosive, n.o.s. 3516 173 Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone A) 3516 173 Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone B) 3516 173 Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone C) 3516 173 Adsorbed gas, toxic, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3517 | 173 | Adsorbed gas, poisonous, flammable, corrosive, n.o.s. Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, poisonous, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3517 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3517 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3517 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3517 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3517 173 Adsorbed gas, toxic, flammable, corrosive, n.o.s. 3517 173 Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone A) 3517 173 Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone B) 3517 173 Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone C) 3517 173 Adsorbed gas, toxic, flammable, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3518 | 173 | Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, poisonous, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Page 331

"+" means distance can be larger in certain atmospheric conditions	TABLE 1

TABLE 1 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

Page 332

|  |  |  | SMALL SPILLS (Froma small package or small leak froma large package) |  |  |  |  |  | LARGE SPILLS (Froma large package or from many small packages) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID No. | Guide No. | Name of Material | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |
|  |  |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |  |  | DAY Kilometers (Miles) |  | NIGHT Kilometers (Miles) |  |
| 3518 | 173 | Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone A) Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone B) Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone C) Adsorbed gas, toxic, oxidizing, corrosive, n.o.s. (Inhalation Hazard Zone D) |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3518 | 173 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3519 | 173 | Boron trifluoride, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3520 | 173 | Chlorine, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3521 | 173 | Silicon tetrafluoride, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3522 | 173 | Arsine, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3523 | 173 | Germane, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3524 | 173 | Phosphorus pentafluoride, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3525 | 173 | Phosphine, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |
| 3526 | 173 | Hydrogen selenide, adsorbed | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) |

| 30 m | (100 ft) | 0.3 km | (0.2 mi) | 1.1 km | (0.7 mi) | 300 m | (1000 ft) | 1.4 km | (0.9 mi) | 3.7 km | (2.3 mi) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.5 km | (0.3 mi) |
| 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.2 km | (0.1 mi) | 200 m | (600 ft) | 1.2 km | (0.7 mi) | 3.9 km | (2.4 mi) |
| 30 m | (100 ft) | 0.2 km | (0.1 mi) | 0.2 km | (0.1 mi) | 60 m | (200 ft) | 0.5 km | (0.3 mi) | 0.6 km | (0.4 mi) |
| 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.3 km | (0.2 mi) |
| 30 m | (100 ft) | 0.1 km | (0.1 mi) | 0.1 km | (0.1 mi) | 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.3 km | (0.2 mi) |
| 30 m | (100 ft) | 0.2 km | (0.2 mi) | 0.7 km | (0.4 mi) | 150 m | (500 ft) | 1.4 km | (0.9 mi) | 2.4 km | (1.5 mi) |

3539 123	Articles containing toxic gas,

n.o.s.

9191 143	Chlorine dioxide, hydrate, frozen

(when spilled in water)

9202 168	Carbon monoxide, refrigerated

liquid (cryogenic liquid) 9206 137	Methyl phosphonic dichloride

9263 156	Chloropivaloyl chloride

9264 151	3,5-Dichloro-2,4,6-

trifluoropyridine 9269 132	Trimethoxysilane

Page 333

See Next Page for Table 2 - Water-Reactive Materials Which Produce Toxic Gases

"+" means distance can be larger in certain atmospheric conditions

TABLE 1

##### HOWTO USE TABLE 2 – WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

Table 2 lists materials which produce large amounts of Toxic Inhalation Hazard (TIH) (PIH in the US) gases when spilled in water, and identifies the TIH gases produced.

The materials are listed by order of ID number.

These water-reactive materials are easily identified in Table 1 as their names are immediately followed by (when spilled in water).

Note 1: The TIH gases indicated in Table 2 are for information purposes only. In Table 1, the initial isolation and protective action distances have already taken into consideration the TIH gases produced.

For example: Table 2 indicates that UN1689 sodium cyanide, when spilled in water, will generate hydrogen cyanide gas (HCN). In Table 1, you must refer to the distances for sodiumcyanide and not the distances for hydrogen cyanide gas.

Note 2: Some water-reactive materials are also TIHmaterials themselves (e.g., UN1746 (Bromine trifluoride), UN1836 (Thionyl chloride)). Inthese instances, two entries are provided in Table 1 for land-based and water-based spills. If a water-reactive material only has one entry in Table 1 for (when spilled in water), and the product is NOT spilled in water, Tables 1 and 2 do NOT apply. Refer only to the appropriate Orange Guide.

Note 3: Materials classified as a Division 4.3 are substances that, on contact with water, are liable to become spontaneously FLAMMABLE or give off FLAMMABLE or sometimes TOXIC gases in dangerous quantities. For the purpose of this table, water-reactive materials are materials that generate substantial quantities of TOXIC gases rapidly after a spill into water; therefore, a material classified as a Division 4.3 will not always be included in Table 2.

Page 334

| ID No. | Mate Guide No. | rials Which Produce Large Amounts of Toxic-by-Inhalation (PIH in the US) Gas(es) When Spilled in Water Name of Material | (TIH) TIH Gas(es) Produced |  |
| --- | --- | --- | --- | --- |
| 1162 | 155 | Dimethyldichlorosilane | HCl |  |
| 1183 | 139 | Ethyldichlorosilane | HCl |  |
| 1196 | 155 | Ethyltrichlorosilane | HCl |  |
| 1242 | 139 | Methyldichlorosilane | HCl |  |
| 1250 | 155 | Methyltrichlorosilane | HCl |  |
| 1295 | 139 | Trichlorosilane | HCl |  |
| 1298 | 155 | Trimethylchlorosilane | HCl |  |
| 1305 | 155P | Vinyltrichlorosilane | HCl |  |
| 1340 | 139 | Phosphorus pentasulfide, free fromyellow and white phosphorus | H2S |  |
| 1340 | 139 | Phosphorus pentasulphide, free fromyellow and white phosphorus | H2S |  |
| 1360 | 139 | Calcium phosphide | PH3 | E 2 |
| 1384 | 135 | Sodium dithionite | H2S | SO2 |
| 1384 | 135 | Sodium hydrosulfite | H2S | SO2 |
| 1384 | 135 | Sodium hydrosulphite | H2S | SO2 |
| 1390 | 139 | Alkali metal amides | NH3 |  |
| 1397 | 139 | Aluminum phosphide | PH3 |  |
| 1419 | 139 | Magnesium aluminum phosphide | PH3 |  |
| i 1432 | 139 | Sodium phosphide | PH3 |  |
| 1541 | 156 | Acetone cyanohydrin, stabilized | HCN |  |
| 1680 | 157 | Potassium cyanide, solid | HCN |  |
| 1689 | 157 | Sodium cyanide, solid | HCN |  |
| 1716 | 156 | Acetyl bromide | HBr |  |
| 1717 | 155 | Acetyl chloride | HCl |  |
| 1724 Chemica Br2 Cl2 HBr HCl HCN | 155 l Symbo Bromi Chlori Hydrog Hydrog Hydrog | Allyltrichlorosilane, stabilized ls for TIH (PIH in the US) Gases: ne HF Hydrogen fluoride NO2 N ne HI Hydrogen iodide PH3 P en bromide H2S Hydrogen sulfide SO2 S en chloride H2S Hydrogen sulphide SO2 S en cyanide NH3 Ammonia | HCl trogen hosph ulfur d ulphur | dioxide ine ioxide dioxide |

TABLE2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

TABL

Use this list only when material is spilled in water.

Page 335

| 1725 | 137 | Aluminum bromide, anhydrous | HBr |  |
| --- | --- | --- | --- | --- |
| 1726 | 137 | Aluminum chloride, anhydrous | HCl |  |
| 1728 | 156 | Amyltrichlorosilane | HCl |  |
| 1732 | 157 | Antimony pentafluoride | HF |  |
| 1741 | 125 | Boron trichloride | HCl |  |
| 1745 | 144 | Bromine pentafluoride | HF | Br2 |
| 1746 | 144 | Bromine trifluoride | HF | Br2 |
| 1747 | 155 | Butyltrichlorosilane | HCl |  |
| 1752 | 156 | Chloroacetyl chloride | HCl |  |
| 1753 | 156 | Chlorophenyltrichlorosilane | HCl |  |
| 1754 | 137 | Chlorosulfonic acid (with or without sulfur trioxide) | HCl |  |
| 1754 | 137 | Chlorosulphonic acid (with or without sulphur trioxide) | HCl |  |
| 1758 | 137 | Chromium oxychloride | HCl |  |
| 1762 | 156 | Cyclohexenyltrichlorosilane | HCl |  |
| 1763 | 156 | Cyclohexyltrichlorosilane | HCl |  |
| 1765 | 156 | Dichloroacetyl chloride | HCl |  |
| 1766 | 156 | Dichlorophenyltrichlorosilane | HCl |  |
| 1767 | 155 | Diethyldichlorosilane | HCl |  |
| 1769 | 156 | Diphenyldichlorosilane | HCl |  |
| 1771 | 156 | Dodecyltrichlorosilane | HCl |  |
| 1777 | 137 | Fluorosulfonic acid | HF |  |
| 1777 | 137 | Fluorosulphonic acid | HF |  |
| 1781 | 156 | Hexadecyltrichlorosilane | HCl |  |
| 1784 | 156 | Hexyltrichlorosilane | HCl |  |

Chemical Symbols for TIH (PIH in the US) Gases:

Br2   Bromine

Cl2   Chlorine

HBr Hydrogen bromide HCl Hydrogen chloride HCN  Hydrogen cyanide

HF	Hydrogen fluoride HI	Hydrogen iodide H2S	Hydrogen sulfide H2S	Hydrogen sulphide NH3	Ammonia

NO2	Nitrogen dioxide PH3	Phosphine

SO2	Sulfur dioxide SO2	Sulphur dioxide

TABLE2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

Materials Which Produce Large Amounts of Toxic-by-Inhalation (TIH) (PIH in the US) Gas(es) When Spilled in Water

ID	Guide Name of Material No.	No.

TIH Gas(es) Produced

Page 336

Use this list only when material is spilled in water.

| ID No. 1799 | Guid No. 156 | Materials Which Produce Large Amounts of To (PIH in the US) Gas(es) When Spill e Name of Material Nonyltrichlorosilane | xic-by-Inhalation (TIH) ed in Water TIH Prod HCl | Gas( uce | es) d |
| --- | --- | --- | --- | --- | --- |
| 1800 | 156 | Octadecyltrichlorosilane | HCl |  |  |
| 1801 | 156 | Octyltrichlorosilane | HCl |  |  |
| 1804 | 156 | Phenyltrichlorosilane | HCl |  |  |
| 1806 | 137 | Phosphorus pentachloride | HCl |  |  |
| 1808 | 137 | Phosphorus tribromide | HBr |  |  |
| 1809 | 137 | Phosphorus trichloride | HCl |  |  |
| 1810 | 137 | Phosphorus oxychloride | HCl |  |  |
| 1815 | 155 | Propionyl chloride | HCl |  |  |
| 1816 | 155 | Propyltrichlorosilane | HCl |  |  |
| 1818 | 157 | Silicon tetrachloride | HCl |  | E 2 |
| 1828 | 137 | Sulfur chlorides | HCl | SO2 | H2S |
| 1828 | 137 | Sulphur chlorides | HCl | SO2 | H2S |
| 1834 | 137 | Sulfuryl chloride | HCl |  |  |
| 1834 | 137 | Sulphuryl chloride | HCl |  |  |
| 1836 | 137 | Thionyl chloride | HCl | SO2 |  |
| 1838 | 137 | Titanium tetrachloride | HCl |  |  |
| r 1898 | 156 | Acetyl iodide | HI |  |  |
| 1923 | 135 | Calcium dithionite | H2S | SO2 |  |
| 1923 | 135 | Calcium hydrosulfite | H2S | SO2 |  |
| 1923 | 135 | Calcium hydrosulphite | H2S | SO2 |  |
| 1929 | 135 | Potassium dithionite | H2S | SO2 |  |
| 1929 | 135 | Potassium hydrosulfite | H2S | SO2 |  |
| 1929 Chemica Br2 Cl2 HBr HCl HCN | 135 l Sym Brom Chlo Hydr Hydr Hydr | Potassium hydrosulphite bols for TIH (PIH in the US) Gases: ine HF Hydrogen fluoride rine HI Hydrogen iodide ogen bromide H2S Hydrogen sulfide ogen chloride H2S Hydrogen sulphi ogen cyanide NH3 Ammonia | H2S NO2 Nitrogen PH3 Phosph SO2 Sulfur d de SO2 Sulphu | SO2 dio ine ioxide diox | xide ide |

TABLE2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

TABL

Use this list only when material is spilled in water.

Page 337

| 1931 | 171 | Zinc dithionite | H2S | SO2 |
| --- | --- | --- | --- | --- |
| 1931 | 171 | Zinc hydrosulfite | H2S | SO2 |
| 1931 | 171 | Zinc hydrosulphite | H2S | SO2 |
| 2004 | 135 | Magnesium diamide | NH3 |  |
| 2011 | 139 | Magnesium phosphide | PH3 |  |
| 2012 | 139 | Potassium phosphide | PH3 |  |
| 2013 | 139 | Strontium phosphide | PH3 |  |
| 2308 | 157 | Nitrosylsulfuric acid, liquid | NO2 |  |
| 2308 | 157 | Nitrosylsulphuric acid, liquid | NO2 |  |
| 2353 | 155 | Butyryl chloride | HCl |  |
| 2395 | 155 | Isobutyryl chloride | HCl |  |
| 2434 | 156 | Dibenzyldichlorosilane | HCl |  |
| 2435 | 156 | Ethylphenyldichlorosilane | HCl |  |
| 2437 | 156 | Methylphenyldichlorosilane | HCl |  |
| 2495 | 144 | Iodine pentafluoride | HF |  |
| 2691 | 137 | Phosphorus pentabromide | HBr |  |
| 2692 | 157 | Boron tribromide | HBr |  |
| 2806 | 139 | Lithiumnitride | NH3 |  |
| 2965 | 139 | Boron trifluoride dimethyl etherate | HF |  |
| 2977 | 166 | Radioactive material, uranium hexafluoride, fissile | HF |  |
| 2977 | 166 | Uranium hexafluoride, radioactive material, fissile | HF |  |
| 2978 | 166 | Radioactive material, uranium hexafluoride, non fissile or fissile-excepted | HF |  |

Chemical Symbols for TIH (PIH in the US) Gases:

Br2   Bromine

Cl2   Chlorine

HBr Hydrogen bromide HCl Hydrogen chloride HCN  Hydrogen cyanide

HF	Hydrogen fluoride HI	Hydrogen iodide H2S	Hydrogen sulfide H2S	Hydrogen sulphide NH3	Ammonia

NO2	Nitrogen dioxide PH3	Phosphine

SO2	Sulfur dioxide SO2	Sulphur dioxide

TABLE2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

Materials Which Produce Large Amounts of Toxic-by-Inhalation (TIH) (PIH in the US) Gas(es) When Spilled in Water

ID	Guide Name of Material No.	No.

TIH Gas(es) Produced

Page 338

Use this list only when material is spilled in water.

| Materials Which Produce Large Amounts of Toxic-by-Inhalation (TIH) (PIH in the US) Gas(es) When Spilled in Water |  |  |  |  |
| --- | --- | --- | --- | --- |
| ID Guide Name of Material TIH Gas(es) No. No. Produced |  |  |  |  |
| 2978 | 166 | Uranium hexafluoride, radioactive material, non fissile or fissile-excepted | HF |  |
| 2985 | 155 | Chlorosilanes, flammable, corrosive, n.o.s. | HCl |  |
| 2986 | 155 | Chlorosilanes, corrosive, flammable, n.o.s. | HCl |  |
| 2987 | 156 | Chlorosilanes, corrosive, n.o.s. | HCl |  |
| 2988 | 139 | Chlorosilanes, water-reactive, flammable, corrosive, n.o.s. | HCl |  |
| 3048 | 157 | Aluminum phosphide pesticide | PH3 |  |
| 3361 | 156 | Chlorosilanes, poisonous, corrosive, n.o.s. | HCl |  |
| 3361 | 156 | Chlorosilanes, toxic, corrosive, n.o.s. | HCl |  |
| 3362 | 155 | Chlorosilanes, poisonous, corrosive, flammable, n.o.s. | HCl |  |
| 3362 | 155 | Chlorosilanes, toxic, corrosive, flammable, n.o.s. | HCl | E 2 |
| 3456 | 157 | Nitrosylsulfuric acid, solid | NO2 | BL |
| 3456 | 157 | Nitrosylsulphuric acid, solid | NO2 | TA |
| 3507 | 166 | Uranium hexafluoride, radioactive material, excepted package, less than 0.1 kg per package, non-fissile or fissile-excepted | HF |  |
| 9191 | 143 | Chlorine dioxide, hydrate, frozen | Cl2 |  |

Chemical Symbols for TIH (PIH in the US) Gases:

Br2   Bromine

Cl2   Chlorine

HBr Hydrogen bromide HCl Hydrogen chloride HCN  Hydrogen cyanide

HF	Hydrogen fluoride HI	Hydrogen iodide H2S	Hydrogen sulfide H2S	Hydrogen sulphide NH3	Ammonia

NO2	Nitrogen dioxide PH3	Phosphine

SO2	Sulfur dioxide SO2	Sulphur dioxide

TABLE2 - WATER-REACTIVE MATERIALS WHICH PRODUCE TOXIC GASES

Use this list only when material is spilled in water.

Page 339

##### HOWTO USE TABLE 3 – INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES FOR LARGE SPILLS FOR DIFFERENT QUANTITIES OF

SIX COMMON TIH (PIH IN THE US) GASES

Table 3 lists Toxic Inhalation Hazard (TIH) materials that may be more commonly encountered.

The selected materials are:

- UN1005 - Ammonia, anhydrous

- UN1017 - Chlorine

- UN1040 - Ethylene oxide and UN1040 – Ethylene oxide with nitrogen

- UN1050 - Hydrogen chloride, anhydrous and UN2186 - Hydrogen chloride, refrigerated liquid

- UN1052 - Hydrogen fluoride, anhydrous

- UN1079 - Sulfur dioxide/Sulphur dioxide

The materials are presented in numerical order of IDnumber and provide Initial Isolation and Protective Action Distances FOR LARGE SPILLS (more than 208 liters or 55 US gallons) involving different container types (therefore different volume capacities, see below) for day time and night time situations and different wind speeds.

- Rail tank car: 80 000 kg (176 370 lbs.)

- Highway tank truck or trailer: 20 000 – 25 000 kg (44 092 – 55 116 lbs.)

- Agricultural nurse tank: 3785 L (1000 gallons)

- Small cylinder: 72 L (19 gallons)

- Ton cylinder: 757 - 1135 L (200 - 300 gallons)

###### Estimating Wind Speed from Environmental Clues

| mph | km/h | Wind Description | Specifications |
| --- | --- | --- | --- |
| < 6 | < 10 | Low wind | Wind felt on face; leaves rustle; ordinary vane moved by wind |
| 6 - 12 | 10 - 20 | Moderate wind | Raises dust, loose paper; small branches are moved |
| > 12 | > 20 | High wind | Large branches in motion; whistling heard in telephone wires; umbrellas used with difficulty |

(Data taken fromthe Beaufort Wind Scale has been reworked in order to create 3 categories of wind speed: Low, Moderate and High)

Page 340

"+" means distance can be larger in certain atmospheric conditions

| TABLE 3 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES FOR LARGE SPILLS FOR DIFFERENT QUANTITIES OF SIX COMMON TIH (PIH IN THE US) GASES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DAY |  |  |  |  |  | NIGHT |  |  |  |  |  |
|  |  |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  |
| TRANSPORT CONTAINER | UN1005 Ammonia, anhydrous / Anhydrous ammonia: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 300 | (1000) | 1.6 | (1.0) | 1.2 | (0.8) | 1.0 | (0.6) | 4.1 | (2.6) | 2.1 | (1.3) | 1.3 | (0.8) |
| Highway tank truck or trailer | 150 | (500) | 0.8 | (0.5) | 0.5 | (0.3) | 0.4 | (0.3) | 1.8 | (1.1) | 0.7 | (0.4) | 0.6 | (0.4) |
| Agricultural nurse tank | 60 | (200) | 0.5 | (0.3) | 0.3 | (0.2) | 0.3 | (0.2) | 1.4 | (0.9) | 0.3 | (0.2) | 0.3 | (0.2) |
| Multiple small cylinders | 30 | (100) | 0.3 | (0.2) | 0.2 | (0.1) | 0.1 | (0.1) | 0.7 | (0.5) | 0.3 | (0.2) | 0.2 | (0.1) |
| TRANSPORT CONTAINER | UN1017 Chlorine: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 1000 | (3000) | 9.6 | (6.0) | 6.3 | (3.9) | 5.1 | (3.2) | 11.0+ | (7.0+) | 8.9 | (5.6) | 6.5 | (4.1) |
| Highway tank truck or trailer | 600 | (2000) | 5.6 | (3.5) | 3.3 | (2.1) | 2.5 | (1.6) | 6.4 | (4.0) | 4.7 | (2.9) | 3.8 | (2.4) |
| Multiple ton cylinders | 300 | (1000) | 1.9 | (1.2) | 1.3 | (0.8) | 1.0 | (0.6) | 3.5 | (2.2) | 2.3 | (1.4) | 1.3 | (0.8) |
| Multiple small cylinders or single ton cylinder | 150 | (500) | 1.3 | (0.9) | 0.7 | (0.5) | 0.5 | (0.3) | 2.4 | (1.5) | 1.2 | (0.8) | 0.6 | (0.4) |

Page 341

TABLE 3

Page 342

| TABLE 3 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES FOR LARGE SPILLS FOR DIFFERENT QUANTITIES OF SIX COMMON TIH (PIH IN THE US) GASES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DAY |  |  |  |  |  | NIGHT |  |  |  |  |  |
|  |  |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  |
| TRANSPORT CONTAINER | UN1040 Ethylene oxide: Large Spills UN1040 Ethylene oxide with nitrogen: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 200 | (600) | 1.5 | (1.0) | 0.8 | (0.5) | 0.7 | (0.4) | 3.0 | (1.8) | 1.4 | (0.9) | 0.8 | (0.5) |
| Highway tank truck or trailer | 100 | (300) | 0.9 | (0.6) | 0.5 | (0.3) | 0.4 | (0.3) | 2.0 | (1.3) | 0.7 | (0.4) | 0.4 | (0.3) |
| Multiple small cylinders or single ton cylinder | 30 | (100) | 0.4 | (0.3) | 0.2 | (0.1) | 0.1 | (0.1) | 0.8 | (0.5) | 0.3 | (0.2) | 0.2 | (0.1) |
| TRANSPORT CONTAINER | UN1050 Hydrogen chloride, anhydrous: Large Spills UN2186 Hydrogen chloride, refrigerated liquid: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 500 | (1500) | 3.7 | (2.3) | 2.0 | (1.3) | 1.7 | (1.1) | 9.7 | (6.1) | 3.3 | (2.1) | 2.2 | (1.4) |
| Highway tank truck or trailer | 200 | (600) | 1.5 | (0.9) | 0.8 | (0.5) | 0.6 | (0.4) | 3.7 | (2.3) | 1.5 | (0.9) | 0.8 | (0.5) |
| Multiple ton cylinders | 30 | (100) | 0.4 | (0.3) | 0.2 | (0.1) | 0.1 | (0.1) | 1.0 | (0.6) | 0.3 | (0.2) | 0.1 | (0.1) |
| Multiple small cylinders or single ton cylinder | 30 | (100) | 0.3 | (0.2) | 0.2 | (0.1) | 0.1 | (0.1) | 0.9 | (0.6) | 0.3 | (0.2) | 0.2 | (0.1) |

"+" means distance can be larger in certain atmospheric conditions

| TABLE 3 - INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES FOR LARGE SPILLS FOR DIFFERENT QUANTITIES OF SIX COMMON TIH (PIH IN THE US) GASES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | First ISOLATE in all Directions Meters (Feet) |  | Then PROTECT persons Downwind during |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DAY |  |  |  |  |  | NIGHT |  |  |  |  |  |
|  |  |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  | Low wind (< 6 mph = < 10 km/h) Kilometers (Miles) |  | Moderate wind (6-12 mph = 10 - 20 km/h) Kilometers (Miles) |  | High wind (> 12 mph = > 20 km/h) Kilometers (Miles) |  |
| TRANSPORT CONTAINER | UN1052 Hydrogen fluoride, anhydrous: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 500 | (1500) | 3.4 | (2.1) | 2.1 | (1.3) | 1.8 | (1.1) | 6.4 | (4.0) | 3.0 | (1.9) | 1.9 | (1.2) |
| Highway tank truck or trailer | 200 | (700) | 2.0 | (1.2) | 1.0 | (0.7) | 0.9 | (0.6) | 3.6 | (2.3) | 1.5 | (1.0) | 0.9 | (0.6) |
| Multiple small cylinders or single ton cylinder | 100 | (300) | 0.8 | (0.5) | 0.4 | (0.2) | 0.3 | (0.2) | 1.7 | (1.1) | 0.5 | (0.3) | 0.3 | (0.2) |
| TRANSPORT CONTAINER | UN1079 Sulfur dioxide / Sulphur dioxide: Large Spills |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rail tank car | 1000 | (3000) | 11.0+ | (7.0+) | 11.0+ | (7.0+) | 6.9 | (4.3) | 11.0+ | (7.0+) | 11.0+ | (7.0+) | 9.6 | (6.0) |
| Highway tank truck or trailer | 1000 | (3000) | 11.0+ | (7.0+) | 6.0 | (3.8) | 5.0 | (3.3) | 11.0+ | (7.0+) | 7.9 | (5.1) | 6.0 | (3.9) |
| Multiple ton cylinders | 500 | (1500) | 5.2 | (3.3) | 2.2 | (1.4) | 1.7 | (1.1) | 7.4 | (4.3) | 4.0 | (2.5) | 2.7 | (1.7) |
| Multiple small cylinders or single ton cylinder | 200 | (600) | 3.1 | (1.9) | 1.5 | (0.9) | 1.1 | (0.7) | 5.6 | (3.5) | 2.4 | (1.5) | 1.5 | (0.9) |

Page 343

TABLE 3

##### ERG2024 USER’S GUIDE

For the purposes of this guidebook, the terms hazardous materials/dangerous goods are synonymous.

The 2024 Emergency Response Guidebook (ERG2024) was developed jointly by Transport Canada (TC), the U.S. Department of Transportation (DOT), and the Secretariat of Infrastructures, Communications and Transport of Mexico (SICT), with help from CIQUIME (Centro de Información Química para Emergencias) of Argentina.

This guidebook is for firefighters, police and other emergency services personnel who may be first to arrive at the scene of a transportation incident involving dangerous goods.

It is primarily a guide to help first responders to quickly:

- identifythespecificor generichazardsof material(s) involvedinatransportation incident

- protect themselves and the general public during the initial response phase of the incident

For the purposes of this guidebook, “initial responsephase” isthe period after first responders arrive at the scene of an incident. During this phase, responders:

- confirm the presence and/or identification of dangerous goods

- start taking protective action and securing the area

- request the help of qualified personnel

This guide is designed for use at a dangerous goods incident on a highway or railroad. It may have limited value at fixed-facility locations, or onboard aircrafts or vessels.

This guide does not:

- provide information on the physical or chemical properties of dangerous goods

- replace emergency response training, knowledge, or sound judgment

- address all possible circumstances that may be associated with a dangerous goods incident

ERG2024 incorporates dangerous goods lists from the most recent United Nations Recommendations, and from other international and national regulations.

Explosives are not listed individually (by either proper shipping name or ID number) but, under the general heading “Explosives”, they do appear:

- at the beginning of the ID Number index (yellow section)

- alphabetically in the Name of Material index (blue section)

Chemical and biological warfare agents are now found in the “Criminal or Terrorist Use of Chemical, Biological and Radiological Agents” section.

Page 344

The letter (P) following the guide number in the yellow and blue sections identifies materials that present a polymerization hazard under certain conditions. For example: UN1092 - Acrolein, stabilized GUIDE 131P.

First responders at the scene of a dangerous goods incident should not solely rely on this guidebook. Always seek specific information about any material in question as soon as possible. To do so:

- Contact the appropriate emergency response agency listed on the inside back cover.

- Call the emergency response telephone number on the shipping paper.

- Consult information on or accompanying the shipping paper.

###### BEFORE AN EMERGENCY – BECOME FAMILIAR WITH THIS GUIDEBOOK! In the

U.S., according to the requirements of the U.S. Department of Labor’s Occupational Safety and Health Administration (OSHA, 29 CFR 1910.120) and regulations issued by the U.S. Environmental Protection Agency (EPA, 40 CFR Part 311), first responders must be trained in how to use this guidebook.

##### GUIDEBOOKCONTENTS

- Yellowsection: Index list of dangerous goods in order of IDnumber. The list displays the 4-digit ID followed by its assigned emergency response guide and material name.

| For example: | ID No. | GUIDE No. | Name of Material |
| --- | --- | --- | --- |
|  | 1090 | 127 | Acetone |

- Bluesection: Index list of dangerous goods inalphabetical order of material name. The list displays the namefollowed by its assigned emergency responseguide and 4-digit IDnumber.

| For example: | Name of Material | GUIDE No. | ID No. |
| --- | --- | --- | --- |
|  | Sulfuric acid | 137 | 1830 |

- Orange section: All safety recommendations are provided here. It is made up of 62 individual guides. Each guide recommends safety and emergency response procedures to protect yourself and the public. Each guide applies to a group of materials with similar chemical and toxicological characteristics. The guide title identifies the general hazards of the dangerous goods.

For example: GUIDE 124 - Gases - Toxic and/or Corrosive - Oxidizing.

Page 345

Each guide is divided into 3 main sections:

##### POTENTIAL HAZARDS:

- Displays the hazards in terms of FIRE OR EXPLOSION and HEALTH effects upon exposure.

- Primary potential hazard is listed first.

- Consult this section first to help you make decisions about how to protect the emergency response team and surrounding population.

##### PUBLIC SAFETY:

- Provides general information on initial precautionary measures to be taken by those first on scene.

- Providesgeneral guidanceonPROTECTIVECLOTHINGrequirementsandrespiratory protection.

- Lists suggested EVACUATION distances for immediate precautionary measures, spills, and for fires (fragmentation hazard).

- When the material is highlighted in green in the yellow and blue sections, it directs the reader to consult Table 1, which lists Toxic Inhalation Hazard (TIH) (PIH in the U.S.) materials and water-reactive materials (green section).

##### EMERGENCY RESPONSE:

- Outlines special precautions for incidents that involve FIRE, SPILL OR LEAK or chemical exposure.

- Lists several recommendations under eachpart tofurther assist your decision-making process.

- Provides specific FIRSTAIDguidancetousefor aproduct or aguideinadditiontothe general first aidguidancefor hazardousmaterials/dangerousgoodsincidents. General first aid guidance is found inthe “General First Aid” section situated immediately after the “How to use the Orange Guides” section.

- Green section: This section has 3 tables.

###### Table 1 - Initial Isolation and Protective Action Distances

Lists, by order of ID number:

  - TIH (PIH in the U.S.) materials

  - water-reactive materials which produce toxic gases upon contact with water

These materials are highlighted in green in the yellow and blue sections so you can easily identify them.

Page 346

Table 1 provides two types of recommended safety distances: “initial isolation distances”

and “protective action distances” for:

- small spills: 208 liters (55 US gallons) or less

- large spills: more than 208 liters (55 US gallons)

Within the “initial isolation distance”, protective clothing and respiratory protection is required. You should consider evacuating all people in all directions from the spill or leak source. This distance defines the radius of the “initial isolation zone” surrounding the spill in which people may be exposed to:

- dangerous concentrations upwind of the source

- life-threatening concentrations downwind of the source

The “protective action distances” are downwind distances from the spill or leak source, within which responders could carry out protective actions to:

- preserve the health and safety of emergency responders and the public

- evacuate and/or shelter-in-place people in this area (For more information, consult the “Protective Actions” section)

The “protective action distance” is divided into daytime and nighttime incidents because varying atmospheric conditions affect a hazardous area’s size. In fact, the quantity or concentration of the material’s vapor poses problems, not its merepresence. Duringthe night, the air is generally calmer. This causes the vapor to disperse less and therefore creates a greater toxic zone. In daytime, the atmosphere is more active, so the vapor disperses more. As a result, there is a lower concentration of vapor in the surrounding air and the area that reaches toxic levels is smaller. Daytime is after sunrise and before sunset. Nighttime is between sunset and sunrise.

For example, in the case of a small spill of UN1955 - compressed gas, toxic, n.o.s., the “initial isolation distance” is 150 meters (500 feet); therefore its “initial isolation zone” is 300 meters (1000 feet) in diameter. Its “protective action distance” is 1.0 kilometer (0.6 miles) for daytime and 3.9 kilometers (2.4 miles) for nighttime.

Note 1: Some water-reactive materials have 2 entries in Table 1. They are identified by (when spilled on land) since they are TIH products and (when spilled in water) because they produce additional toxic gases when spilled in water.

For example: UN1746 - Bromine trifluoride and UN1836 - Thionyl chloride.

Note 2: If a water-reactive material only has one entry in Table 1 for (when spilled in water) and the product is NOT spilled in water, Table 1 and Table 2 do not apply. You will find safe distances in the appropriate orange guide.

For example: UN1183 - Ethyldichlorosilane and UN1898 – Acetyl iodide.

Page 347

###### Table 2 - Water-Reactive Materials Which Produce Toxic Gases

Lists:

- by order of ID number, materials which produce large amounts of Toxic Inhalation Hazard (TIH) gases when spilled in water; and

- TIH gases produced by these materials.

You can easily identify water-reactive materials in Table 1, as their names are immediately followed by (when spilled in water).

NOTE: The TIH gases indicated in Table 2 are for information purposes only. These TIH gases have already been taken into consideration in the distances of Table 1.

For example, Table 2 indicates that UN1689 sodium cyanide, solid, when spilled in water, will generate hydrogen cyanide gas (HCN). In Table 1, you must refer to the distances for sodium cyanide, solid and not the distances for hydrogen cyanide gas.

###### Table 3 - Initial Isolation and Protective Action Distances for Large Spills for Different Quantities of Six Common TIH Gases

Lists the following 6 most common TIH materials:

- UN1005 - Ammonia, anhydrous

- UN1017 - Chlorine

- UN1040 - Ethylene oxide and UN1040 - Ethylene oxide with nitrogen

- UN1050- Hydrogenchloride, anhydrousandUN2186- Hydrogenchloride, refrigerated liquid

- UN1052 - Hydrogen fluoride, anhydrous

- UN1079 - Sulfur dioxide/Sulphur dioxide

Table 3 shows:

- initial isolation and protective action distances for large spills (more than 208 liters or 55 US gallons)

- different container types (therefore different volume capacities) for daytime and nighttime, and for three different wind speeds (low, moderate and high)

Page 348

##### HOW TO CHOOSE THE APPROPRIATE ISOLATION AND PROTECTIVE ACTION DISTANCES

ERG2024 lists isolation or evacuation distances in 2 places:

  - the individual guides (orange section)

  - Table 1 – Initial Isolation and Protective Action Distances (green section)

If you are dealing with a non-TIH material (not highlighted in green in the yellow or blue section),

  - Go to the assigned guide for the material (orange section).

  - Under EVACUATION, you will find:

− initial isolation distance as an immediate precautionary measure

− specific distances for spill or fire situations (fragmentation hazard)

− Please note that certain guides may also refer to Table 1. This is just a reminder for green highlighted materials only.

If you are dealing with a TIH or water-reactive material (green highlighted entries in the yellow or blue section):

###### If there is no fire:

  - Godirectly toTable1–Initial IsolationandProtectiveActionDistances (greensection).

  - Also, consult the assigned guide for the material (orange section).

###### If a fire is involved:

  - Go directly to the assigned guide (orange section) and apply the distances found under EVACUATION - Fire.

  - Also, consult Table 1 distances for residual material release.

Page 349

##### PROTECTIVE CLOTHING

STREET CLOTHING AND WORK UNIFORMS

Thesegarments, suchasuniformswornbypoliceandemergency medical servicespersonnel, providealmost noprotectionfromtheharmful effectsof hazardousmaterials/dangerousgoods.

##### STRUCTURAL FIREFIGHTERS’ PROTECTIVE CLOTHING (SFPC)

This category of clothing, often called turnout or bunker gear, is the protective clothing firefighters normally wear during structural firefighting operations. It includes a helmet, coat, pants, boots, gloves andahoodtocover parts of the headthat arenot protected by the helmet and facepiece. It can be used with full-facepiece positive pressure self-contained breathing apparatus (SCBA). It should, at minimum, meet the OSHA Fire Brigades Standard (29 CFR 1910.156) or NFPA 1851.

Structural firefighters’ protective clothing provides limited protection from heat and cold. It may not provide adequate protection from harmful vapors or liquids encountered during hazardous materials/dangerous goods incidents.

Each guide includes a statement about the use of SFPC in incidents involving the materials referenced by that guide. Some guides state that SFPC provides limited protection. In those cases, the responder wearing SFPC and SCBA may be able to performa quick “in-and-out” operation. However, this type of operation can place the responder at risk of exposure, injury or death. The incident commander makes the decision to do this only if there is an overriding benefit (for example, to performan immediate rescue, turn off avalve tocontrol a leak, etc.).

Pleasenotethat thecoverall-typeprotectiveclothingcustomarily worntofight fires inforests or wildlands is not SFPCand is not recommended nor referred to elsewhere in this guidebook.

##### POSITIVE PRESSURE SELF-CONTAINED BREATHING APPARATUS (SCBA)

This apparatus provides a constant, positive pressure flow of air within the facepiece.

You should always use an SCBA certified by NIOSH and the Department of Labor/Mine Safety and Health Administration, in accordance with:

- 42 CFR Part 84

- requirements for respiratory protection specified in OSHA 29 CFR 1910.134 (Respiratory Protection) and/or 29 CFR 1910.156 (f) (Fire Brigades Standard)

- NFPA 1852

Chemical-cartridge respirators or other filtering masks are not acceptable substitutes for positive pressure SCBA. Demand-type SCBA does not meet the OSHA 29 CFR 1910.156 (f)(1)(i) of the Fire Brigades Standard.

##### RESPIRATORS

If you suspect a chemical warfare agent is involved in an incident, use NIOSH-certified respirators with CBRN protection.

Page 350

N95 respirators are the most common of the seven types of particulate filtering facepiece respirators. This product filters at least 95% of airborne particles (0.3 microns), but is not resistant to oil. N95 filtering facepiece respirators do not protect against gases and vapors.

Powered air-purifying respirators (PAPR) force ambient air through the air-purifying cartridge or filter into the facepiece. A PAPR does not supply oxygen or air from a separate source (e.g., cylinders).

##### CHEMICAL PROTECTIVE CLOTHING AND EQUIPMENT

For you to safely use this type of protective clothing and equipment, you need specific skills developed through training and experience. This type of special clothing may protect against onechemical but bereadily permeated by chemicals for whichit was not designed. Therefore, do not use this type of protective clothing unless it is compatible with the released material. Also, be aware that it offers little or no protection against heat and/or cold.

Examples of this type of equipment have been described as:

- Vapor Protective Suits (NFPA1991), alsoknownas Totally-Encapsulating Chemical Protective Suits or Level A* protection (OSHA 29 CFR 1910.120, Appendix A & B)

- Liquid-SplashProtectiveSuits (NFPA1992), alsoknownas Level B* or C* protection (OSHA29 CFR1910.120, Appendix A&B), or suits for chemical/biological terrorism incidents (NFPA 1994), class 1, 2 or 3 Ensembles and Standard CAN/CGSB/ CSA-Z1610-11–Protectionof first respondersfromchemical, biological, radiological, and nuclear (CBRN) events

No single protective clothing material will protect you fromall hazardous materials/dangerous goods. Do not assume any protective clothing is resistant to cold and/or heat or flame exposure, unless certified by the manufacturer (NFPA 1991 5-3 Flammability Resistance Test and 5-6 Cold Temperature Performance Test).

*Consult the glossary for more information about protection levels under the heading “Protective Clothing.”

Page 351

##### DECONTAMINATION

###### The ways to decontaminate people and equipment can vary. If you need help with decontamination, contact the emergency response telephone number provided on the shipping papers or the agencies listed on the inside back cover. These resources may be able to put you in contact with the chemical manufacturer to determine the appropriate procedure if not otherwise available.

Decontamination is the process of removing or neutralizing hazardous materials/dangerous goods that have contaminated people and equipment during an incident.

Contamination happens in the area generally referred to as the Hot Zone. Everything and everyone entering this zone should be decontaminated when leaving, including emergency response personnel. This reduces the chances that more contamination will occur.

There are two main types of contamination:

- Direct contamination happens in the Hot Zone.

- Cross contamination happens when someone or something outside the Hot Zone was not properly decontaminated and comes incontact with another object or person, usually in the Warm or Cold Zone.

To decontaminate, you must:

- physically remove contaminants; and/or

- chemically neutralize contaminants*.

The NFPA 472, Chapter 3, describes the following four kinds of decontamination.

- Gross decontamination: Quickly removing surface contamination, which usually happens by mechanically removing the contaminant or rinsing with water from handheld hose lines, emergency showers, or other nearby water sources.

- Technical decontamination: Reducing contamination to a level as low as possible by chemical or physical methods. A hazmat team will perform this kind of decontamination.

- Mass decontamination: Reducing or removing surface contaminants as fast as possible froma large number of people in potentially life-threatening situations.

- Emergency decontamination: Immediately reducing contamination of people in potentially life-threatening situations with or without formally setting up a decontamination corridor. This process should be performed upwind and uphill fromvictims. Responders should avoid contact with victims, runoff or spray fromthe decontamination process.

Emergency and mass decontamination can be done with firefighting and rescue operations equipment. Nozzles can be put on wide-angle fog patterns and sprayed towards the ground to create a decontamination shower. Responders can also place nozzles on the discharge ports of engines.

Contaminated clothing and equipment must be removed after use and stored in a controlled area (Warm Zone) until cleanup procedures can begin. Sometimes protective clothing and equipment cannot be decontaminated and must be disposed of properly.

*Chemical neutralization releases heat. DO NOT PERFORM on a victim.

Page 352

##### FIRE AND SPILL CONTROL

FIRE CONTROL

Water is themost commonand generally most availablefireextinguishingagent. Use caution in selecting a fire extinguishing method, as there are many factors to consider. Water may be ineffective in fighting fires that involve some materials.

###### Fires Involving a Spill of Flammable Liquids

These fires are usually controlled by applying afirefighting foamto the surface of the burning material.

Fighting flammable liquid fires requires:

- foam concentrate that is chemically compatible with the burning material

- correct mixing of the foam concentrate with water and air

- careful application and maintenance of the foam blanket

There are two general types of firefighting foam: regular and alcohol-resistant. Examples of regular foam are protein-base, fluoroprotein, and aqueous film-forming foam (AFFF).

You can control some flammable liquid fires, including many petroleumproducts, by applying regular foam. Other flammable liquids, including polar solvents (flammable liquids that are water soluble), such as alcohols and ketones, have different chemical properties. You cannot easily control a fire that involves these materials with regular foam, and should use alcohol- resistant foam instead.

Polar solvent fires may be difficult to control and require a higher foamapplication rate than other flammable liquid fires (see NFPA Standards 11 for further information). Refer to the appropriate guide to determine which type of foamto use. For flammable liquids which have subsidiarycorrosiveor toxichazards, it isdifficult tomakespecificrecommendations. However, alcohol-resistant foam may be effective for many of these materials.

Contact the emergency responsetelephone number ontheshipping paper, or the appropriate emergencyresponseagency, assoonaspossiblefor guidanceontheproper fireextinguishing agent to use.

How you decide to control the fire depends on factors such as:

- incident location

- exposure hazards

- size of the fire

- environmental concerns

- availability of extinguishing agents and equipment at the scene

##### WATER-REACTIVE MATERIALS

Water is sometimes used to flush spills and reduce or direct vapors in spill situations. Some of the materials covered by this guidebook can react violently or even explosively with water. In these cases, consider letting the fire burn or leaving the spill alone (except to prevent its spreading by diking) until you can get more technical advice.

Page 353

The applicable guides clearly warn you of these potentially dangerous reactions. Technical advice is required for these materials since:

  - Water getting inside a ruptured or leaking container may cause an explosion.

  - You may need to cool adjoining containers with water to prevent themfromrupturing (exploding), or to prevent the fire spreading further.

  - Water may be effective in mitigating an incident involving a water-reactive material, but only if you can apply it at a sufficient flooding rate for a long period.

  - Products from the reaction with water may be more toxic, corrosive or undesirable than the product that caused the fire.

When you respond to an incident involving water-reactive materials, take into account:

  - existingconditions, suchaswind, precipitation, locationandaccessibilitytotheincident

  - availability of agents to control the fire or spill

Because there are variables to consider, base your decision to use water on fires or spills involving water-reactivematerials oninformationfromareliablesource. For example, consult the material’s manufacturer through the emergency response telephone number or the appropriate emergency response agency listed on the inside back cover.

##### VAPOR CONTROL

Limiting the amount of vapor released from a pool of flammable or corrosive liquids is an operational concern. It requires proper protectiveclothing, specializedequipment, appropriate chemical agents and skilled personnel. Before you engage in vapor control, seek advice on tactics to be used from qualified personnel.

There are several ways to minimize the amount of vapors escaping from pools of spilled liquids, such as special foams, adsorbing agents, absorbents, and neutralizing agents. To be effective, you must select a method for the specific material involved, and use it in a way that mitigates, not worsens, the incident.

Where specific materials are known, such as at a manufacturing or storage facilities, the hazardous materials/dangerous goods response team should prearrange with the facility operators to select and stockpile these control agents before a spill.

Inthefield, first respondersmaynot havethemost effectivevapor control agent for thematerial available. They will be more likely to have only water, and only one type of firefighting foam on their vehicles. If the available foamis not appropriate, they will probably use water spray. Because water is being used to forma vapor seal, care must be taken not tochurn or further spread the spill during application. Vapors that do not react with water may be directed away from the site using the air currents surrounding the water spray. Before using water spray or other methods to safely control vapor emission or suppress ignition, get technical advice based on a specific chemical name.

Page 354

##### LIQUID SPILL CONTROL

Spill control is an important part of any hazardous materials/dangerous goods incident. Spills can have serious health, safety, and environmental consequences. There are many ways to deal with a liquid spill, like:

- diking

- damming, and

- absorbing

A liquid spill can be controlled by setting up a barrier around the spill area. Depending on the product involved, the spill can be contained with either inert or non-combustible absorbent materials.

Inert absorbent materials are granular. The most common types are:

- sand

- diatomaceous earth (a fine powder made from sedimentary rock)

- vermiculite, or

- clay

Non-combustible absorbents are usually not very flammable and can absorb a lot of liquid. These materials are usually made from synthetic materials, like:

- polypropylene

- polyethylene, or

- other synthetic fibers

Other absorbent materials that are easy to find include sawdust or clay litter. Please note the following:

- Sawdust should not be used to absorb flammable liquids or oxidizers since it can catch fire

- Clay litter should not be used to absorb acids since it may contain baking soda, which will react with acids

Before using an absorbent material, get technical advice to confirm its compatibility or test a small amount on the spill.

Page 355

##### CONSIDERATIONS FOR LITHIUM BATTERY AND ELECTRIC VEHICLE (EV) FIRES

FIRE CONTROL

Water spray cools batteries and helps suppress and slow the release of toxic gases but does not stop the chemical reaction (thermal runaway). Other extinguishing agents (CO2, dry chemical, etc.) can trap heat instead of removing it and could result in false (lower temperature) readings.

During an electric vehicle (EV) fire, consult the manufacturer’s specific emergency response guide for help with identifying high voltage and medium voltage cabling. DO NOT CUT THESE CABLES.

Most electric vehicles have emergency cut loops which are low voltage wire loops that can be cut to disconnect the high voltage system from the rest of the vehicle. If it is safe to do so, followthe manufacturer’s directions to disconnect the 12-volt battery. This will isolate the power to the high voltage battery and reduce risk of electric shock.

##### DAMAGED, DEFECTIVE, OR RECALLED LITHIUM BATTERIES

All lithium batteries can pose a fire risk, whether they are lithium metal or lithium ion, new or used. However, damaged, defective, or recalled (DDR) lithium batteries pose a higher risk than non-DDR lithium batteries because they are more likely to catch fire in a process known as “thermal runaway”.

Thermal runaway is a chain reaction that leads to a violent release of stored energy and flammable gas. This reaction can spread to other batteries or combustible materials that are nearby, which could lead to a large-scale thermal event with severe consequences.

Signs that a battery is damaged, defective, or recalled include:

- leaking electrolytes

- swollen or discolored battery casing

- odor or corrosion

- burn marks

- known conditions of use or misuse

- being recalled

Page 356

##### BLEVE AND HEAT INDUCED TEAR

BLEVE (BOILING LIQUID EXPANDING VAPOR EXPLOSION)

The following section presents important safety-related information on BLEVEs, including a table, to consider in a situation involving Liquefied Petroleum Gases (LPG), UN1075.

LPGs include the following flammable gases:

- UN1011 - Butane	• UN1077 - Propylene

- UN1012 - Butylene	• UN1969 - Isobutane

- UN1055 - Isobutylene	• UN1978 - Propane

ABLEVEoccurswhenafireimpingedor damagedtankcar failstocontainitsinternal pressure and explodes with a sudden product release. This catastrophic failure is more likely to occur with damaged pressure tank cars, even in the absence of an active fire.

The main hazards from a LPG BLEVE are:

- Fire: If the released substance is ignited, there is an immediate fireball.

- Thermal radiation: At a distance of about 4 times the radius of a fireball, the heat radiatedfromafireball isenoughtoburnexposedskinin2seconds. Wearingprotective clothing limits the thermal radiation dose.

- Blast: Aconcussiveforcecaused by thesuddenreleaseof the pressurizedsubstance. For a BLEVE occurring out in the open, the blast strength at a distance of 4 times the radius of afireball can break windowglass and may causeminor damagetobuildings.

- Projectiles: Tank failure can throw metal fragments over large distances. These fragments can and have been deadly.

The danger decreases as you move away from the BLEVE centre. The furthest-reaching hazard is projectiles.

For a video with information on critical safety issues concerning BLEVEs, please visit

https://www.tc.gc.ca/eng/tdg/publications-menu-1238.html.

##### HEAT INDUCED TEAR (HIT)

A heat induced tear (HIT) is a rupture of a NON-PRESSURE tank car containing flammable liquids when exposed to the intense heat of a fire. The metal will soften and the pressure in the tank car will increase which can lead to containment failure. The tear generally occurs at the vapor space (upper side) of the container, venting large quantities of flammable liquid and vapors at high speed. A fireball and an intense heat wave will occur.

Comparedto BLEVEs, HITs rarely result intheprojection of tank car fragments. Heat induced tearing has occurred within 20 minutes of the derailment and as long as 10+ hours following the initial fire.

Responding to these types of incidents (BLEVE and HIT) requires specialized training, equipment and a tactical approach.

Page 357

##### BLEVE – SAFETY PRECAUTIONS

Use with caution. The following table gives a summary of tank properties, critical times, critical distances and cooling water flow rates for various tank sizes. This table is provided to give responders some guidance but it should be used with caution.

Tank dimensions are approximate and can vary depending on the tank design and application.

Minimum time to failure is based on severe torch fire impingement on the vapor space of a tank in good condition, and is approximate. Tanks may fail earlier if they are damaged or corroded. Tanks may fail minutes or hours later than these minimum times depending on the conditions. It has been assumed here that the tanks are not equipped with thermal barriers or water spray cooling.

Minimum time to empty is based on an engulfing fire with a properly sized pressure relief valve. If the tank is only partially engulfed, then time to empty will increase (i.e., if tank is 50% engulfed, then the tanks will take twice as long to empty). Once again, it has been assumed that the tank is not equipped with a thermal barrier or water spray.

Tanks equipped with thermal barriers or water spray cooling significantly increase the times to failure and the times to empty. A thermal barrier can reduce the heat input to a tank by a factor of ten or more. This means it could take ten times as long to empty the tank through the Pressure Relief Valve (PRV).

Fireball radius and emergency response distance is based on mathematical equations and is approximate. They assume spherical fireballs and this is not always the case.

Two safety distances for public evacuation. The minimum distance is based on tanks that are launched with a small elevation angle (i.e., a fewdegrees above horizontal). This is most common for horizontal cylinders. The preferred evacuation distance has more margin of safety since it assumes the tanks are launched at a 45 degree angle to the horizontal. This might be more appropriate if a vertical cylinder is involved.

It is understood that these distances are very large and may not be practical in a highly populated area. However, it should be understood that the risks increase rapidly the closer you are to a BLEVE. Keep in mind that the furthest reaching projectiles tend to come off in the zones 45 degrees on each side of the tank ends.

###### Water flow rate is based on 5 (√capacity (USgal) ) = USgal/min needed to cool tank metal.

Warning: the data given are approximate and should only be used with extreme caution. For example, where times are given for tank failure or tank emptying through the pressure relief valve – these times are typical but they can vary fromsituation to situation. Therefore, never risk life based on these times.

Page 358

Page 359

WARNING:

The data given are approximate and should only be used with extreme caution. These times can vary fromsituation to situation. LPG tanks have been known to BLEVE within minutes. Therefore, never risk life based on these times.

| BLEVE (USE WITH CAUTION) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Capacity | Diameter |  | Length | Propane Mass |  | Minimum time to failure for severe torch | Approximate time to empty for engulfing fire | Fireball radius | Emergency response distance |  | Minimum evacuation distance | Preferred evacuation distance |  | Cooling water flowrate |  |
| Litres (Gallons) | Meters (Feet) |  | Meters (Feet) | Kilograms(Pounds) |  | Minutes | Minutes | Meters (Feet) | Meters | (Feet) | Meters (Feet) | Meters (Feet) |  | Litres/min | USgal/min |
| 100 (26.4) | 0.3 | (1) | 1.5 (4.9) | 40 | (88) | 4 | 8 | 10 (33) | 90 | (295) | 154 (505) | 307 | (1007) | 97 | 26 |
| 400 (106) | 0.61 | (2) | 1.5 (4.9) | 160 | (353) | 4 | 12 | 16 (52) | 90 | (295) | 244 (801) | 488 | (1601) | 195 | 52 |
| 2000 (528) | 0.96 | (3.1) | 3 (9.8) | 800 | (1764) | 5 | 18 | 28 (92) | 111 | (364) | 417 (1368) | 834 | (2736) | 435 | 115 |
| 4000 (1057) | 1 | (3.3) | 4.9 (16.1) | 1600 | (3527) | 5 | 20 | 35 (115) | 140 | (459) | 525 (1722) | 1050 | (3445) | 615 | 163 |
| 8000 (2113) | 1.25 | (4.1) | 6.5 (21.3) | 3200 | (7055) | 6 | 22 | 44 (144) | 176 | (577) | 661 (2169) | 1323 | (4341) | 870 | 230 |
| 22000 (5812) | 2.1 | (6.9) | 6.7 (22) | 8800 | (19401) | 7 | 28 | 62 (203) | 247 | (810) | 926 (3038) | 1852 | (6076) | 1443 | 381 |
| 42000 (11095) | 2.1 | (6.9) | 11.8 (38.7) | 16800 | (37038) | 7 | 32 | 77 (253) | 306 | (1004) | 1149 (3770) | 2200 | (7218) | 1994 | 527 |
| 82000 (21662) | 2.75 | (9) | 13.7 (45) | 32800 | (72312) | 8 | 40 | 96 (315) | 383 | (1257) | 1435 (4708) | 2200 | (7218) | 2786 | 736 |
| 140000 (36984) | 3.3 | (10.8) | 17.2 (56.4) | 56000 (123459) |  | 9 | 45 | 114 (374) | 457 | (1499) | 1715 (5627) | 2200 | (7218) | 3640 | 962 |

##### CRIMINAL OR TERRORIST USE OF CHEMICAL, BIOLOGICAL AND RADIOLOGICAL AGENTS

If you suspect an intentional release of a chemical, biological or radiological agent (CBRN), youshouldimmediately contact your local emergency responseauthorities (911). Additionally, for CBRN incidents occurring:

- within the United States, call the National Response Center at 1-800-424-8802

- within Canada, call CANUTEC at 613-996-6666 (1-888-226-8832)

- within Mexico, call CENACOM at 555128-0000 extensions 36428, 36422, 36469,

37807, 37810

- in other countries, consult the “24-hour emergency response telephone numbers” section.”

The following is general guidance and does not serve as specialized incident response training. Do not enter the scene without appropriate training and equipment.

Initial actions to consider in a potential CBRN/terrorism event:

- First responders must ensure their own safety.

- Avoid using cell phones, radios, etc. within 100 meters (300 feet) of asuspect device.

- If known, request trained specialist resources.

- Set up incident command upwind and uphill of the area.

- Do not touch or move suspicious packages or containers.

- Be cautious about the potential presence of secondary devices (e.g., improvised explosive devices (IEDs)).

- Avoid contamination.

- Limit access toonly thoseresponsiblefor rescue of victims or assessment of unknown materials or devices.

- Evacuate and isolate people who were potentially exposed to hazardous materials/ dangerous goods to an area away fromthe scene, preferably upwind and uphill while avoiding physical contact to the extent possible.

- Isolate contaminated areas and secure the scene for analysis of material.

First responders can usethefollowinginformationtomakeaninitial assessment of asituation they suspect involves criminal or terrorist use of chemical agents, biological agents and/ or radioactive materials (CBRN). To help with this, the following paragraphs have a list of observable indicators that a CB agent or radioactive material has been used or is present. This section ends with a Safe Stand-Off Distance Chart for various threats when improvised explosive devices (IEDs) are involved.

##### DIFFERENCES BETWEEN A CHEMICAL, BIOLOGICAL AND RADIOLOGICAL AGENT

Chemical and biological agents as well as radioactive materials can be dispersed in the air we breathe, the water we drink, or on surfaces we physically contact. Dispersion methods may be as simple as opening a container or using conventional (garden) spray devices, or as elaborate as detonating an improvised explosive device.

Page 360

Chemical incidents are characterized by the rapid onset of medical symptoms (in minutes to hours) and easily observed signatures (colored residue, dead foliage, pungent odor, dead insects and animals).

Biological incidents are characterized by the onset of symptoms in hours to days. Typically, there will be no characteristic signatures because biological agents are usually odorless and colorless. Because of the delayed onset of symptoms, the affected area may be greater due to the movement of infected people.

Radiological incidentsare characterized by the onset of symptoms, if any, in days to weeks or longer. Typically, there will be no characteristic signatures because radioactive materials are usually odorless and colorless. Specialized equipment is needed to determine the size of the affected area, and if the level of radioactivity is an immediate or long-termhealth hazard. Because it is impossible to detect radioactivity without special equipment, the affected area may be greater due to the migration of contaminated people.

Themost probablesources wouldnot generateenoughradiationtokill peopleor causesevere illness. Inaradiological incident generated by a“dirty bomb,” or radiological dispersal device (RDD), in which a conventional explosive is detonated to spread radioactive contamination, the primary hazard is from the explosion. However, certain radioactive materials dispersed in the air could contaminate up to several city blocks, creating fear and possibly panic, and needing potentially costly cleanup.

##### INDICATORS OF A POSSIBLE CHEMICAL INCIDENT

| Dead animals/birds/fish | Not just an occasional road kill, but numerous animals (wild and domestic, small and large), birds, and fish in the same area. |
| --- | --- |
| Lack of insect life | If normal insect activity (ground, air, and/or water) is missing, check the ground, water surface or shore line for dead insects. If near water, check for dead fish and/ or aquatic birds. |
| Unexplained odors | Possible odors include fruity, flowery, sharp, pungent, garlic, horseradish-like, bitter almonds, peach kernels, or newly mown hay. The odor is completely out of character with its surroundings. |
| Unusual numbers of dying or sick people (mass casualties) | Health problems including nausea, disorientation, difficulty in breathing, convulsions, localized sweating, conjunctivitis (reddening of eyes), erythema (reddening of skin) and death. |
| Pattern of casualties | Casualties will likely be distributed downwind, or if indoors, by the air ventilation system. |

Page 361

| Blisters or rashes | Numerous people experiencing unexplained water-like blisters, weals (like bee stings), and/or rashes. |
| --- | --- |
| Illness in confined area | Different casualtyratesfor peopleworkingindoorsversus outdoors dependent on where the agent was released. |
| Unusual liquid droplets | Numerous surfaces show oily droplets or film; numerous water surfaces have an oily film (no recent rain). |
| Different-looking areas | Not just apatchof deadweeds, but trees, shrubs, bushes, food crops, and/or lawns that are dead, discolored, or withered (no current drought). |
| Low-lying clouds | Low-lying cloud or fog-like condition not consistent with its surroundings. |
| Unusual metal debris | Unexplained bomb or munitions-like material, especially if it contains a liquid. |

##### INDICATORS OF A POSSIBLE BIOLOGICAL INCIDENT

| Unusual numbers of sick or dying people or animals | Any number of symptoms may occur. Casualties may occur hours to days after an incident has occurred. The time required before symptoms are observed is dependent on the agent. |
| --- | --- |
| Unscheduled and unusual spray being disseminated | Especially if outdoors during periods of darkness. |
| Abandoned spray devices | Devices may not have distinct odors. |

INDICATORS OF A POSSIBLE RADIOLOGICAL INCIDENT

| Radiation Symbols | Containers may display a “propeller” radiation symbol. |
| --- | --- |
| Unusual metal debris | Unexplained bomb or munitions-like material. |
| Heat-emitting material | Material that is hot or seems to emit heat without any sign of an external heat source. |
| Glowing material | Strongly radioactive material may emit or cause radioluminescence. |
| Sick people/animals | In very improbable scenarios there may be unusual numbers of sick or dying people or animals. Casualties may occur hours to days or weeks after an incident has occurred. The time required before symptoms are observed is dependent on the radioactive material used, and the dose received. Possible symptoms include skin reddening or vomiting. |

Page 362

##### PERSONAL SAFETY CONSIDERATIONS

When you approach a scene that may involve CB agents or radioactive materials, the most critical thing to consider is your safety and that of other responders.

Useprotectiveclothing andrespiratory protection of an appropriatelevel of safety. Inincidents where you suspect that CBRN materials have been used as weapons, NIOSH-certified respirators with CBRN protection are highly recommended. Be aware that you may not be abletoverify or identify CBagents or radioactivematerials, especially inthecaseof biological or radiological agents.

The following actions apply to a chemical, biological or radiological incident. This guidance is general. Responders will need to apply it on a case-by-case basis.

###### Approach and response strategies:

- Minimize exposure time.

- Maximize the distance between you and the item that is likely to harm you.

- Use cover as protection.

- Wear appropriate personal protective equipment and respiratory protection.

- Identify and estimate the hazard by using the indicators above.

- Isolate the area and secure the scene.

- Isolate and decontaminate potentially contaminated people as soon as possible.

- To the extent possible, take measures to limit the spread of contamination.

Inthe event of a chemical incident, thefading of chemical odors does not necessarily indicate reduced vapor concentrations. Some chemicals deaden the senses, giving you the false perception that the chemical is no longer present.

If there is any indication that an area may be contaminated with radioactive materials, including the site of any non-accidental explosion, responders:

- should be equipped with radiation detection equipment

- should have adequate training in how to use this equipment

This equipment should be designed to also alert responders when an unacceptable ambient dose rate or ambient dose has been reached.

Page 363

##### DECONTAMINATION MEASURES

For chemical and biological agents: Emergency responders should follow standard decontamination procedures (flush-strip-flush). Mass casualty decontamination should begin as soon as possible by stripping all clothing, and flushing with soap and water. For further information, contact the agencies listed on the inside back cover of this guidebook.

For people contaminated with radioactive material: Take care to minimize the spread of the contamination to the extent possible. Move them to a low radiation area if necessary, and if it can be done safely. Remove their clothing and place it in a clearly marked and sealed receptacle, such as a plastic bag, for later testing. Use decontamination methods described above, but avoid breaking the skin (e.g., vigorous brushing). External radiological contamination on intact skin rarely causes a high enough dose to be a hazard, to either the contaminatedindividual or thefirst responders. For this reason, prioritizemedical stabilization for a contaminated injured individual.

NOTE: The aboveinformation was developedinpart by the Department of National Defence (Canada), the U.S. Department of the Army, Aberdeen Proving Ground and the Federal Bureau of Investigation (FBI).

##### CHEMICAL AND BIOLOGICAL WARFARE AGENTS

Chemical and biological warfare agents do not have an assigned ID number because they are not commercially transported. In an emergency situation, the assigned guide (orange section) will provide guidance for the initial response.

The volumes used for the chemical warfare agents’ distances are:

- Small release consists of a discharge up to 2 kg (4.4 lbs.)

- Large release consists of a discharge up to 25 kg (55 lbs.)

###### Biological Warfare Agents:

| Biological agents | Pathogens (bacteria, viruses, etc.) that are dispersed with criminal intent. They can cause disease or death in otherwise healthy humans. Examples: Anthrax, plague, smallpox virus. Refer to GUIDE 158. |
| --- | --- |
| Toxins | Poisonous or toxic material from a plant, animal, or bacterial source. Examples: Botulinum toxin, ricin. Refer to GUIDE 152. |

Page 364

###### Chemical Warfare Agents:

| Blister agents (vesicants) | Substances that cause blistering of the skin. Exposure is through liquid or vapor contact with any exposed tissue (eyes, skin, lungs). Examples: Lewisite, Mustard. Symptoms: Red eyes, skin irritation, burning of skin, blisters, upper respiratory damage, cough, hoarseness. |
| --- | --- |
| Blood agents | Substances that interfere with cell respiration (the exchange of oxygen and carbon dioxide between blood and tissues). Examples: Arsine, cyanogen chloride, hydrogen cyanide. Symptoms: Respiratory distress, headache, unresponsiveness, seizures, coma. |
| Choking agents | Substances that cause physical injury to the lungs. Exposure is through inhalation. In extreme cases, membranes swell, and lungs become filled with liquid (pulmonary edema). Death results from asphyxiation. Examples: Diphosgene, phosgene. Symptoms: Irritation to eyes, nose, and throat, respiratory distress, nausea, vomiting, burning of exposed skin. |
| Incapacitating agents | Materials that make people unable to think clearly or that cause an altered state of consciousness (possibly unconsciousness). Examples: 3-Quinuclidinyl benzilate (Buzz). Symptoms: Hallucinations, confusion, agitation, dilated pupils, blurred vision, dry/flushed skin, diarrhea, elevated heart rate, high blood pressure, elevated temperature. |
| Nerve agents | Substances that interfere with the central nervous system. Exposure is primarily through contact with the liquid (via skin and eyes) and secondarily through inhalation of the vapor. Examples: Sarin, Tabun, VX. Symptoms: Pinpoint pupils, extreme headache, severe tightness in the chest, dyspnea, runny nose, coughing, salivation, unresponsiveness, seizures. |

Page 365

| Tear gas agents | Chemical compounds that temporarily make people unable to function by causing irritation to the eyes, mouth, throat, lungs, and skin. Examples: Bromobenzylcyanide, chloroacetophenone. Symptoms: Excessive tearing, burning eyes, blurred vision, redness of the eyes, burning and irritation to mouth, difficulty swallowing, chest tightness, coughing, choking sensation, skin burns and rash. |
| --- | --- |
| Vomiting agents | Chemicals that cause rapid onset of irritation of the eyes, upper airway, and skin, and also nausea and vomiting. Examples: Adamsite, diphenylchloroarsine. Symptoms: Irritation of the eyes, noses, burning in throat, chest tightness, nausea, vomiting, abdominal cramps. |

##### INITIAL ISOLATION AND PROTECTIVE ACTION DISTANCES

| Chemical warfare agents | Guide | Initial isolation Meters (Feet) |  | Small release Kilometers (Miles) |  | Large release Kilometers (Miles) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Blister agents (vesicants) | 153 | 200 | (600) | 0.4 | (0.3) | 1.6 | (1.0) |
| Blood agents | 117 | 400 | (1200) | 0.9 | (0.6) | 3.2 | (2.0) |
| Choking agents | 125 | 100 | (300) | 0.3 | (0.2) | 1.1 | (0.7) |
| Incapacitating agents | 153 | 1000 | (3000) | 1.7 | (1.1) | 7.8 | (4.8) |
| Nerve agents | 153 | 400 | (1200) | 1.0 | (0.6) | 4.0 | (2.5) |
| Tear gas agents | 159 | 30 | (100) | 0.2 | (0.1) | 0.6 | (0.4) |
| Vomiting agents | 153 | 100 | (300) | 0.6 | (0.4) | 1.1 | (0.7) |

For biological warfare agents, refer to the respective Guide for distances.

##### IMPROVISED EXPLOSIVE DEVICE (IED)

AnIEDisa“homemade” bomband/or destructivedeviceusedtodestroy, incapacitate, harass, or distract. Because they are improvised, IEDs can come in many forms, ranging froma small pipe bomb to a sophisticated device capable of causing massive damage and loss of life.

The following table predicts the damage radius based on the volume or weight of explosive (TNT equivalent) and the type of bomb.

Page 366

Page 367

Improvised Explosive Device (IED) SAFE STAND-OFF DISTANCE

| Threat Description Explosives Capacity1 Mandatory Shelter-in-Place Zone Preferred Evacuation Distance2 Evacuation Distance3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Explosives (TNT Equivalent) | Pipe Bomb | 2.3 kg | 5 lbs | 21 m | 70 ft | 22 - 365 m | 71 - 1,199 ft | 366+ m | 1,200+ ft |
|  | Suicide Bomber | 9 kg | 20 lbs | 34 m | 110 ft | 35 - 518 m | 111 - 1,699 ft | 519+ m | 1,700+ ft |
|  | Briefcase/Suitcase | 23 kg | 50 lbs | 46 m | 150 ft | 47 - 563 m | 151 - 1,849 ft | 564+ m | 1,850+ ft |
|  | Car | 227 kg | 500 lbs | 98 m | 320 ft | 99 - 579 m | 321 - 1,899 ft | 580+ m | 1,900+ ft |
|  | SUV/Van | 454 kg | 1,000 lbs | 122 m | 400 ft | 123 - 731 m | 401 - 2,399 ft | 732+ m | 2,400+ ft |
|  | Small Delivery Truck | 1,814 kg | 4,000 lbs | 195 m | 640 ft | 196 - 1,158 m | 641 - 3,799 ft | 1,159+ m | 3,800+ ft |
|  | Container/Water Truck | 4,536 kg | 10,000 lbs | 263 m | 860 ft | 264 - 1,554 m | 861 - 5,099 ft | 1,555+ m | 5,100+ ft |
|  | Semi-Trailer | 27,216 kg | 60,000 lbs | 475 m | 1,570 ft | 476 - 2,834 m | 1,571 - 9,299 ft | 2,835+ m | 9,300+ ft |

1 Based on the maximum amount of material that could reasonably fit into a container or vehicle. Variations possible.

2 Governed by the ability of an unreinforced building to withstand severe damage or collapse.

3 Governed by the greater of fragment throw distance or glass breakage/falling glass hazard distance. These distances can be reduced for personnel wearing ballistic protection. Note that the pipe bomb, suicide bomb, and briefcase/suitcase bomb are assumed to have a fragmentation characteristic that requires greater stand-off distances than an equal amount of explosives in a vehicle.

Page 368

Improvised Explosive Device (IED) SAFE STAND-OFF DISTANCE

| Threat Description LPG Mass / Volume1 Fireball Diameter2 Safe Distance3, 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LPG- Butane or Propane | Small LPGTank | 9 kg / 19 L | 20 lbs / 5 gal | 12 m | 40 ft | 48 m | 160 ft |
|  | Large LPGTank | 45 kg / 95 L | 100 lbs / 25 gal | 21 m | 69 ft | 84 m | 276 ft |
|  | Commercial/Residential LPGTank | 907 kg / 1,893 L | 2,000 lbs / 500 gal | 56 m | 184 ft | 224 m | 736 ft |
|  | Small LPGTruck | 3,630 kg / 7,570 L | 8,000 lbs / 2,000 gal | 89 m | 292 ft | 356 m | 1,168 ft |
|  | Semitanker LPG | 18,144 kg / 37,850 L | 40,000 lbs / 10,000 gal | 152 m | 499 ft | 608 m | 1,996 ft |

1 Based on the maximum amount of LPGthat could reasonably fit into a container or vehicle. Variations possible.

2 Assuming efficient mixing of the flammable gas with ambient air.

3 Determined by U.S. firefighting practices wherein safe distances are approximately 4 times the flame height.

4 This table is for a loaded LPGtank with explosives on the exterior. Note that an LPGtank filled with high explosives would require a significantly greater stand-off distance than if it were filled with LPG.

|  | GLOSSARY |
| --- | --- |
| Adsorbed gas | Agas which sticks (adsorbs) tothe surface of asolid and porous material (such as activated charcoal) contained within a metal cylinder. This results in an internal cylinder pressure of less than 101.3kPaat 20°C(14 psi at 68°F) andless than 300kPaat 50°C (43 psi at 122°F). These pressures are much lower than those of conventional cylinderscontainingcompressedor liquefiedgases. |
| AEGL(s) | Acute Exposure Guideline Level(s), AEGLs represent threshold exposure limits for the general public after a once-in-a-lifetime, or rare, exposure and are applicable to emergency exposure periods ranging from10 minutes to 8 hours. Three levels AEGL- 1, AEGL-2 and AEGL-3 are developed for each of five exposure periods (10 and 30 minutes, 1 hour, 4 hours, and 8 hours) and are distinguished by varying degrees of severity of toxic effects; see AEGL-1, AEGL-2 and AEGL-3. |
| AEGL-1 | AEGL-1 is the airborne concentration (expressed as parts per million or milligrams per cubic meter [ppm or mg/m3]) of a substanceabovewhichit is predictedthat thegeneral population, including susceptible individuals, could experience notable discomfort, irritation, or certain asymptomatic, non-sensory effects. However, the effects are not disabling and are transient and reversible upon cessation of exposure. |
| AEGL-2 | AEGL-2 is the airborne concentration (expressed as ppmor mg/ m3) of a substance above which it is predicted that the general population, including susceptible individuals, could experience irreversible or other serious, long-lasting adverse health effects or an impaired ability to escape. |
| AEGL-3 | AEGL-3 is the airborne concentration (expressed as ppmor mg/ m3) of a substance above which it is predicted that the general population, including susceptible individuals, could experience life-threatening health effects or death. |
| Air-Reactive | See “Pyrophoric”. |
| Alcohol-resistant foam | A foamthat is resistant to polar chemicals such as ketones and esters which may break down other types of foam. |
| BLEVE | Boiling Liquid Expanding Vapor Explosion |
| Boil over | Asudden increase in fire intensity associated with the expulsion of burning flammable liquid caused by the boiling of water that has accumulated in the bottom of a tank car. |

Page 369

##### GLOSSARY

| Burn | Refers to either a chemical or thermal burn, the former may be caused by corrosive substances and the latter by liquefied cryogenic gases, hot molten substances, or flames. |
| --- | --- |
| Carcinogen | A substance or mixture which induces cancer or increases its incidence. |
| Category A | An infectious substance that poses a high risk to the health of individualsand/or animalsor publichealth. Thesesubstancescan causeserious diseaseandcanleadtodeath. Effectivetreatment and preventative measures may not be available. |
| Category B | An infectious substance that poses a low to moderate risk to individuals and/or animals and/or public health. These substances are unlikely to cause serious disease. Effective treatment and preventative measures are available. |
| CBRN | Chemical, biological, radiological or nuclear agent. |
| CO2 | Carbon dioxide gas. |
| Cold zone | Area where the command post and support functions that are necessary to control the incident are located. This is also referred to as the clean zone, green zone or support zone in other documents. (EPA Standard Operating Safety Guidelines, OSHA 29 CFR 1910.120, NFPA 472). |
| Combustible | In this guidebook, a solid or liquid capable of burning but does not catch fire as easily as a flammable liquid. See “Combustible Liquid”. |
| Combustible liquid | Liquids which have a flash point greater than 60°C (140°F) and below 93°C (200°F). U.S. regulations permit a flammable liquid with a flash point between 38°C(100°F) and 60°C(140°F) to be reclassed as a combustible liquid. |

Page 370

##### GLOSSARY

| Compatibility Group | Letters identify explosives that are deemed to be compatible. The definition of these Compatibility Groups in this Glossary are intended to be descriptive. Please consult the transportation of hazardous materials/dangerous goods or explosives regulations of your jurisdiction for the exact wording of the definitions. Class 1 materials are considered to be “compatible” if they can be transported together without significantly increasing either the probability of an incident or, for a given quantity, the magnitude of the effects of such an incident. - Substances which are expected to mass detonate very soon after fire reaches them. - Articles which are expected to mass detonate very soon after fire reaches them. - Substances or articles which may be readily ignited and burn violently without necessarily exploding. - Substances or articles which may mass detonate (with blast and/or fragment hazard) when exposed to fire. E & F Articles which may mass detonate in a fire. - Substances and articles which may mass explode and give off smoke or toxic gases. - Articles which in a fire may eject hazardous projectiles and dense white smoke. J Articles which may mass explode. K Articles which in a fire may eject hazardous projectiles and toxic gases. L Substances and articles which present aspecial risk and could be activated by exposure to air or water. N Articles which contain only extremely insensitive detonating substances and demonstrate a negligible probability of accidental ignition or propagation. S Packaged substances or articles which, if accidentally initiated, produce effects that are usually confined to the immediate vicinity. |
| --- | --- |
| Control temperature | The maximum temperature at which a temperature-controlled substance can be safely transported. Above this temperature, self-accelerating decomposition or polymerization may occur. |

Page 371

##### GLOSSARY

| Control zones | Designated areas at hazardous materials/dangerous goods incidents, based onsafety andthe degree of hazard. Many terms are used to describe control zones; however, in this guidebook, thesezones aredefined as the hot/exclusion/red/restrictedzone, warm/contamination reduction/yellow/limited access zone, and cold/support/green/cleanzone. (EPAStandardOperating Safety Guidelines, OSHA 29 CFR 1910.120, NFPA 472). |
| --- | --- |
| Criticality safety index (CSI) | A number value assigned to packages and overpacks containing fissilematerialsthat limitsthenumber of packagescontainingfissile materials during transportation. |
| Cryogenic liquid | A refrigerated, liquefied gas that has a boiling point colder than -90°C(-130°F) at atmosphericpressureor ishandledor transported at a temperature equal to or less than -100°C (-148°F). |
| Decomposition products | Products of a chemical or thermal break-down of a substance. |
| Decontamination | The removal of hazardous materials/dangerous goods from personnel and equipment to the extent necessary to prevent potential adversehealtheffects. For moreinformation, consult the “Decontamination” section. |
| Dry chemical | A preparation designed for fighting fires involving flammable liquids, pyrophoric substances and electrical equipment. Common types contain sodium bicarbonate or potassium bicarbonate. |
| Edema | The accumulation of an excessive amount of watery fluid in cells and tissues. Pulmonary edema is an excessive buildup of water in the lungs, for instance, after inhalation of a gas that is corrosive to lung tissue. |
| ERPG(s) | Emergency Response Planning Guideline(s). Values intended to provide estimates of concentration ranges above which one could reasonably anticipate observing adverse health effects; see ERPG-1, ERPG-2 and ERPG-3. |
| ERPG-1 | The maximumairborne concentration below which it is believed nearly all individuals could be exposed for up to 1 hour without experiencing more than mild, transient adverse health effects or without perceiving a clearly defined objectionable odor. |
| ERPG-2 | The maximumairborne concentration below which it is believed nearly all individuals could be exposed for up to 1 hour without experiencing or developing irreversible or other serious health effects or symptoms that could impair an individual’s ability to take protective action. |

Page 372

##### GLOSSARY

| ERPG-3 | The maximumairborne concentration below which it is believed nearly all individuals could be exposed for up to 1 hour without experiencing or developing life-threatening health effects. |  |
| --- | --- | --- |
| Evacuate | Evacuate aims to protect as many people as possible by removing persons from inside a zone safely. If removal is too risky, sheltering-in-place can also be considered in this zone. |  |
| Flammable liquid | A liquid that has a flash point of 60°C (140°F) or lower. |  |
| Flash point | Lowest temperature at which a liquid or solid gives off vapor in suchaconcentrationthat, whenthevapor combines withair near the surface of the liquid or solid, a flammable mixture is formed. Hence, thelower theflashpoint, themoreflammablethematerial. |  |
| Flooding quantities | Minimum of 1900 L/min (500 US gal/min) of water. |  |
| Hazard zones (Inhalation Hazard Zones) | HAZARD ZONE A: | Gases: LC50 of less than or equal to 200 ppm, Liquids: V equal to or greater than 500 LC50 and LC50 less than or equal to 200 ppm. |
|  | HAZARD ZONE B: | Gases: LC50 greater than 200 ppm and less than or equal to 1000 ppm, Liquids: V equal to or greater than 10 LC50; LC50 less than or equal to 1000 ppm and criteria for Hazard Zone A are not met. |
|  | HAZARD ZONE C: | LC50 greater than 1000 ppm and less than or equal to 3000 ppm. |
|  | HAZARD ZONE D: | LC50 greater than 3000 ppm and less than or equal to 5000 ppm. |
| Please note: even though the term“zone” is used, hazard zones are not an actual area or distance. How zones are assigned is strictly a function of the lethal concentration 50 (LC50) of the product. For example, TIH Zone A is more toxic than Zone D. |  |  |
| High expansion foam | Foams that have a high expansion ratio (over 1:200) with a low water content. |  |
| Hot zone | Areaimmediately surroundingahazardous materials/dangerous goods incident which extends far enough to prevent adverse effects fromthe released product to personnel outside the zone. This zone is also referred to as exclusion zone, red zone or restricted zone in other documents. (EPA Standard Operating Safety Guidelines, OSHA 29 CFR 1910.120, NFPA 472). |  |
| IED | See “Improvised Explosive Device”. |  |
| Immiscible | In this guidebook, means that a material does not mix readily with water. |  |

Page 373

##### GLOSSARY

| Improvised Explosive Device | A bomb that is manufactured from commercial, military or homemade explosives. |
| --- | --- |
| Isolate | Isolate indicates azone of no entry that applies tothe public and first responders who are not equipped, trained, and prepared to mitigate the incident. |
| Large spill | Aspill that involves quantities that are greater than 208 liters (55 US gallons). This usually involves a spill froma large package, or multiple spills from many small packages. |
| LC50 | Lethal concentration 50. The concentration of a material administered by inhalation that is expected to cause the death of 50% of an experimental animal population within a specified time. (Concentration is reported in either ppm or mg/m3). |
| Mass explosion | Explosion which affects almost the entire load virtually instantaneously. |
| MAWP | MaximumAllowableWorking Pressure: The maximumallowable internal pressure that the tank may experience during normal operations. |
| mg/m3 | Milligrams of a material per cubic meter of air. |
| Miscible | Inthis guidebook, means that amaterial mixes readily withwater. |
| mL/m3 | Milliliters of a material per cubic meter of air. (1 mL/m3 equals 1 ppm). |
| Mutagen | An agent giving rise to an increased occurrence of mutations in populations of cells and/or organisms. Mutation means a permanent change in the amount or structure of the genetic material in a cell. |
| Narcotic | A substance which acts as a central nervous system depressor producing effects such as drowsiness, narcosis, reduced alertness, loss of reflexes, lack of coordination, and vertigo. These effects can also be manifested as severe headache or nausea, and can lead toreduced judgment, dizziness, irritability, fatigue, impaired memory function, deficit in perception and coordination, reaction time, or sleepiness. |
| n.o.s. | These letters refer to “not otherwise specified”. The entries which use this description are generic names such as “Corrosive liquid, n.o.s.” This means that the actual chemical name for that corrosiveliquidis not listedintheregulations; therefore, ageneric name must be used to describe it on shipping papers. |
| Noxious | In this guidebook, means that a material may be harmful or injurious to health or physical well-being. |

Page 374

##### GLOSSARY

| Organic peroxide | An organic (carbon-containing) compound having two oxygen atoms joined together. Organic peroxides are thermally unstable chemicals. They may have one or more of the following properties: be liable to explosive decomposition; burn rapidly; be sensitive to impact or friction; react dangerously with other substances. |
| --- | --- |
| Oxidizer | Achemical which supplies its own oxygen and which helps other combustible material burn more readily. |
| P | See “Polymerization”. |
| Packing Group | The Packing Group (PG) is assigned based on the degree of danger presented by the hazardous material/dangerous good: PG I : Great danger PG II : Medium danger PG III : Minor danger |
| PG | See “Packing Group”. |
| pH | pH is a value that represents the acidity or alkalinity of a water solution. Pure water has a pHof 7. ApHvalue below7 indicates an acid solution (a pH of 1 is extremely acidic). A pH above 7 indicates an alkaline solution (a pH of 14 is extremely alkaline). Acids and alkalies (bases) arecommonly referredtoas corrosive materials. |
| PIH | Poison Inhalation Hazard. See “TIH”. |
| Polar | A molecule in which one side of the molecule has a partial positive charge while another side has a partial negative charge. Examples includes alcohols and ketones. |
| Polymerization | A chemical reaction that often produces heat and pressure. Once initiated, the reaction is accelerated by the heat that it produces. The uncontrolled buildup of heat and pressure can cause a fire or an explosion, or can rupture closed containers. The letter (P) following a guide number in the yellow and blue sections identifies a material that may polymerize violently under high temperature conditions or contamination with other products duringatransportationincident. It is alsousedtoidentify materials that have a strong potential for polymerization in the absence of an inhibitor due to depletion of this inhibitor caused by accident conditions. |
| ppm | Parts per million. (1 ppm equals 1 mL/m3). |

Page 375

##### GLOSSARY

| Protective clothing | In this guidebook, protective clothing includes both respiratory and physical protection. One cannot assign a level of protection to clothing or respiratory devices separately. These levels were accepted and defined by response organizations such as U.S. Coast Guard, NIOSH, and U.S. EPA. Level A: SCBA plus totally encapsulating chemical resistant clothing (permeation resistant). Level B: SCBAplus hooded chemical resistant clothing(splash suit). Level C: Full or half-face respirator plus hooded chemical resistant clothing (splash suit). Level D: Coverall, including structural firefighters’ protective clothing (SFPC), with no respiratory protection. SCBA: Self-contained breathing apparatus. For more information, consult the “Protective Clothing” section. |
| --- | --- |
| Pyrophoric | A material which ignites spontaneously upon exposure to air (or oxygen). |
| Radiation Authority | As referred to in GUIDES 161 through 166 for radioactive materials, the Radiation Authority is either a Federal, state/ provincial agency or state/province designated official. The responsibilities of this authority include evaluating radiological hazard conditions during normal operations and during emergencies. If theidentityandtelephonenumber of theauthority are not known by emergency responders, or included inthe local responseplan, theinformationcanbeobtainedfromtheagencies listed on the inside back cover. They maintain a periodically updated list of radiation authorities. |
| Radioactivity | Theproperty of somesubstances toemit invisibleandpotentially harmful radiation. |
| Refrigerated liquid | See “Refrigerated liquefied gas”. |
| Refrigerated liquefied gas | Agas which when packaged for transport is made partially liquid because of its low temperature. See “Cryogenic liquid”. |
| Respiratory sensitizer | Asubstancethat induces hypersensitivity of theairways following inhalation of the substance. |
| Right-of-way | A defined area on a property containing one or more high- pressure natural gas pipelines. |
| Self-heating material | Material that may spontaneously ignite or generate heat on contact with oxygen (in air) after long periods of time (hours or days). |

Page 376

##### GLOSSARY

| Self-reactive material | Material that is thermally unstable and produces heat upon decomposition, even without participation of air. |
| --- | --- |
| Shelter-in-place | People should seek shelter inside a building and remain inside until the danger passes. Sheltering-in-place is used when evacuatingthe public wouldcause greater risk thanstaying wheretheyare, or whenanevacuationcannot beperformed. Direct the people inside to close all doors and windows and to shut off all ventilating, heatingandcoolingsystems. In-place protection (shelter-in-place) may not be the best option if (a) the vapors are flammable; (b) if it will take a long time for the gas to clear thearea; or (c) if buildings cannot beclosedtightly. Vehicles can offer some protection for a short period if the windows are closed and the ventilating systems are shut off. Vehicles are not nearly as effective as buildings for in-place protection. |
| Skin corrosion | The production of irreversible damage to the skin following the application of a test substance for up to 4 hours. |
| Skin irritation | The production of reversible damage to the skin following the application of a test substance for up to 4 hours. |
| Skin sensitizer | A substance that will induce an allergic response following skin contact. |
| Small spill | Aspill that involves quantities that are 208 liters (55 US gallons) or less. This generally corresponds toaspill froma single small package (for example, a drum), a small cylinder, or a small leak from a large package. |
| Specific gravity | Weight of asubstancecomparedtotheweight of anequal volume of water at a given temperature. Specific gravity less than 1 indicatesasubstanceislighter thanwater; specificgravitygreater than 1 indicates a substance is heavier than water. |
| Spontaneously combustible material | In this guidebook, aspontaneously combustible material means apyrophoric (air-reactive) material or self-heating material. Refer to each term in the glossary. |

Page 377

##### GLOSSARY

| Straight or solid streams | Method used to apply or distribute water from the end of a hose. The water is delivered under pressure for penetration. In an efficient stream, approximately 90% of the water passes through an imaginary circle 38 cm (15 inches) in diameter at the breaking point. Hose solid streams are typically created by passing water through a non-adjustable bore tip. On the other hand, hose straight streams originate from fog nozzles that are dialed into astraight stream. Hose (solid or straight) streams are frequently used to cool tanks and other equipment exposed to flammable liquid fires, or for washing burning spills away from danger points. However, these streams will cause a spill fire to spread if improperly used or when directed into open containers of flammable and combustible liquids. |
| --- | --- |
| Thermal runaway | A chain reaction that leads to a violent release of stored energy and flammable gas. This reaction can spread to other batteries or combustible materials that are nearby, which could lead to a large-scale thermal event with severe consequences. |
| TIH | Toxic Inhalation Hazard. Term used to describe gases and volatile liquids that are toxic when inhaled (same as PIH). These materials pose aknown hazard to human health during transport or is presumed to be toxic to humans because of animal-based studies. |
| V | Saturatedvapor concentrationinair of amaterial inmL/m3 (ppm) at 20°C and standard atmospheric pressure. |
| Vapor density | Weight of a volume of pure vapor or gas (with no air present) compared tothe weight of an equal volume of dry air at the same temperature and pressure. A vapor density less than 1 (one) indicates that the vapor is lighter than air and will tend to rise. A vapor density greater than 1 (one) indicates that the vapor is heavier than air and may travel along the ground. |
| Vapor pressure | Pressure at which a liquid and its vapor are in equilibrium at a given temperature. Liquids with high vapor pressures evaporate rapidly. |
| Viscosity | Measure of a liquid’s internal resistance to flow. This property is important because it indicates how fast a material will leak out through holes in containers or tanks. |

Page 378

##### GLOSSARY

| Warm zone | Area between Hot and Cold zones where personnel and equipment decontamination and hot zone support take place. It includes control points for the access corridor and thus assists in reducing the spread of contamination. Also referred to as the contamination reduction corridor (CRC), contamination reduction zone (CRZ), yellow zone or limited access zone in other documents. (EPA Standard Operating Safety Guidelines, OSHA 29 CFR 1910.120, NFPA 472). |
| --- | --- |
| Water insoluble | A substance that does not easily dissolve in water. |
| Water-Reactive Material | In this guidebook, a material which produces a large amount of toxic gas when it comes in contact with water. |
| Water-sensitive | Substanceswhichmayproduceflammable, toxicand/or corrosive decomposition products upon contact with water. |
| Water soluble | Asubstance that easily dissolves in water. Polar substances are generally soluble in water. |
| Water spray (fog) | Method or way to apply or distribute water. The water is finely divided to provide for high heat absorption. Water spray patterns canrangefromabout 10to90degrees. Water spray streams can be used to extinguish or control the burning of afire or to provide exposureprotectionfor personnel, equipment, buildings, etc. This method can be used to absorb vapors, knock-down vapors or disperse vapors. Direct a water spray (fog), rather than a straight or solid stream, into the vapor cloud to accomplish any of the above. Water spray is particularly effective on fires of flammable liquids and volatile solids having flash points above 37.8°C (100°F). Regardless of the above, water spray can be used successfully on flammable liquids with low flash points. The effectiveness depends particularly on the method of application. With proper nozzles, even gasoline spill fires of some types have been extinguished when coordinated hose lines were used to sweep the flames off the surface of the liquid. Furthermore, water spray carefully applied has frequently been used with success in extinguishing fires involving flammable liquids with high flash points (or any viscous liquids) by causing frothing to occur only onthesurface, andthis foamingactionblankets andextinguishes the fire. |

Page 379

##### PUBLICATION DATA

The2024Emergency ResponseGuidebook(ERG2024) waspreparedbythestaff of Transport Canada, the U.S. Department of Transportation, and the Secretariat of Infrastructures, Communications and Transport (SICT) of Mexico with the assistance of many interested parties from government and industry including the collaboration of CIQUIME of Argentina. Printing and publication services are provided through U.S. DOT’s Pipeline and Hazardous Materials Safety Administration (PHMSA), Outreach, Engagement, and Grants Division.

ERG2024 is based on earlier Transport Canada, U.S. DOT, and Mexico’s Secretariat of Infrastructures, Communications and Transport emergency responseguidebooks. ERG2024 is published in three languages: English, French and Spanish. The Emergency Response Guidebook has been translated and printed in other languages, including Mandarin, German, Hebrew, Japanese, Portuguese, Korean, Hungarian, Polish, Turkish and Thai.

We encourage countries that wish to translate this Guidebook to please contact any of the websites or telephone numbers in the next paragraph.

##### DISTRIBUTION OF THIS GUIDEBOOK

The primary objectiveis toplaceonecopy of the ERG2024ineachpublicly owned emergency service vehicle through distribution to Federal, state, provincial and local public safety authorities. The distribution of this guidebook is being accomplished through the voluntary cooperation of a network of key agencies. Emergency service organizations that have not yet received copies of ERG2024 should contact the respective distribution center in their country, state or province. In the U.S., information about the distribution center for your location may be obtained from the Office of Hazardous Materials Safety website at https:// www.phmsa.dot.gov/hazmat/erg/emergency-response-guidebook-erg or call 202-366-4900. In Canada, contact CANUTEC at 1-613-992-4624 or via the website at https://tc.canada.ca/ en/dangerous-goods/canutec for information. In Mexico, call SICT at +52 55-57-23-93-00 ext. 20010, 20119, and 20250, or via email at sabundiz@sct.gob.mx and jose.rayon@sct. gob.mx. In Argentina, call CIQUIME at +54-11-5199-1409, or via the website at https://www. ciquime.org or via email at gre@ciquime.org.

##### REPRODUCTION AND RESALE

Copies of this document whichareprovidedfree-of-chargetofire, policeandother emergency services may not be resold. ERG2024 may be reproduced without further permission subject to the following:

The names and the seals of the participating governments may not be reproduced on acopy of this document unless that copy accurately reproduces the entire content (text, format, and coloration) of this document without modification. In addition, the publisher's full name and address must be displayed on the outside back cover of each copy, replacing the wording placed on the center of the back cover.

Page 380

Constructive comments concerning ERG2024 are solicited; in particular, comments concerning its use in handling incidents involving hazardous materials/dangerous goods. Comments should be addressed to:

###### In Canada:

CANUTEC

Transport Canada 330 Sparks Street

Place de Ville, Tower C 14th floor

Ottawa, Ontario Canada K1A 0N5

Phone: 613-992-4624 (information) Email: canutec@tc.gc.ca

###### In the U.S.:

U. S. Department of Transportation

Pipeline and Hazardous Materials Safety Administration Outreach, Engagement, and Grants Division (PHH-50) Washington, DC 20590-0001

Phone: 202-366-4900

Fax: 202-366-7342

Email: ERGComments@dot.gov

###### In Mexico:

Secretaría de Infraestructura, Comunicaciones y Transportes Dirección General de Autotransporte Federal

Dirección General Adjunta de Normas y Especificaciones Técnicas y de Seguridad en el Autotransporte

Calzada de las Bombas No. 411-2 piso, Col. Los Girasoles,

Alcaldía de Coyoacán, Código Postal 04920, Ciudad de México

Phone: +52 55-57-23-93-00 ext. 20010, 20119, and 20250

Email: asanchzt@sct.gob.mx, sabundiz@sct.gob.mx, jose.rayon@sct.gob.mx

###### In Argentina:

Centro de Información Química para Emergencias (CIQUIME) Av. Alvarez Thomas 636

C1427CCT Buenos Aires, Argentina Phone: +54-11-5199-1409

Email: gre@ciquime.org

Page 381

The Emergency Response Guidebook is normally revised and reissued every four years. However, in the event of asignificant mistake, omission or change in the state of knowledge, special instructions to change this guidebook (in pen-and-ink, with paste-over stickers, or with a supplement) may be issued.

Users of this guidebook should check periodically (about every 6 months) to make sure their version is current. Changes should be annotated below. Contact:

DOT/PHMSA

https://www.phmsa.dot.gov/hazmat/erg/emergency-response-guidebook-erg

TRANSPORT CANADA

https://tc.canada.ca/en/dangerous-goods/canutec

CIQUIME

https://www.ciquime.org

This guidebook incorporates changes dated:

Page 382

##### NOTES

Page 383

##### CANADA AND UNITED STATES NATIONAL RESPONSE CENTERS

For the purposes of this guidebook, the terms hazardous materials/dangerous goods are synonymous.

##### CANADA

- CANUTEC

CANUTEC is the Canadian Transport Emergency Centre operated by the Transportation of Dangerous Goods Directorate of Transport Canada.

CANUTEC provides a national bilingual (French and English) advisory service and is staffed by professional scientists experienced and trained in interpreting technical information and providing emergency response advice.

In an emergency, CANUTEC may be called at 1-888-CANUTEC (226-8832)

or collect at 613-996-6666 (24 hours)

*666 cellular (Press Star 666, Canada only)

Inanon-emergencysituation, pleasecall theinformationlineat 613-992-4624(24hours).

The Transportation of Dangerous Goods Act, 1992 (TDG Act) requires reporting dangerous goods incidents which meet or exceed established reporting criteria listed in the Transportation of Dangerous Goods Regulations (TDG Regulations). For more information, consult the TDG website:

https://tc.canada.ca/en/dangerous-goods/canutec/reporting-requirements

##### PROVINCIAL/TERRITORIAL AGENCIES

Although technical information and emergency response assistance can be obtained from CANUTEC, there are federal, provincial and territorial regulations requiring the reporting of dangerous goods incidents to certain authorities.

Page 384

The following list of provincial/territorial agencies is supplied for your convenience.

Province Emergency Authority and/or Telephone Number

Alberta Local Police and Provincial Authority 1-800-272-9600 British Columbia Local Police and Provincial Authority 1-800-663-3456 Manitoba Local Police or Fire Department and Provincial Authority 1-855-944-4888 New Brunswick Local Police or Provincial Authority 1-800-565-1633 Newfoundland and Labrador Local Police and Provincial Authority 1-800-563-9089 Northwest Territories Local Police and Provincial Authority 1-867-920-8130 Nova Scotia Local Police or Provincial Authority 1-800-565-1633 Nunavut Local Police and Provincial Authority 1-867-920-8130 Ontario Local Police and Provincial Authority 1-800-268-6060 Prince Edward Island Local Police or Provincial Authority 1-800-565-1633 Quebec Local Police and Provincial Authority 1-866-694-5454 Saskatchewan Local Police and Provincial Authority 1-800-667-7525 Yukon Local Police and Provincial Authority 1-867-667-7244

###### EMERGENCY RESPONSE ASSISTANCE PLANS (Applies in Canada ONLY)

An ERAP or Emergency Response Assistance Plan is a Transport Canada approved plan that describes what is to be done in the event of a transportation incident involving certain high-risk dangerous goods that require specialized emergency response personnel and equipment.

Once implemented, a plan can be used to assist local emergency responders either remotely or on site, by offering technical and emergency response advice and resources. ERAP resources are often integrated with plans from carriers and local or provincial authorities to help mitigate the consequences of an incident.

For shipments that require an ERAP, an ERAP reference number and telephone number to obtain assistance will be included on the shipping paper. Anyone can call the ERAP telephone number. If additional information is required, or to determine if the product involved in the emergency requires an ERAP, contact CANUTEC.

###### CANUTEC may be called at 1-888-CANUTEC (226-8832)

or collect at 613-996-6666 (24 hours)

*666 on cellular phone (Press star 666) In Canada Only

Page 385

##### UNITEDSTATES

NATIONAL RESPONSE CENTER(NRC)

The NRC, which is operated by the U.S. Coast Guard, receives reports required when hazardous materials are spilled. After receiving notification of an incident, the NRC will immediately notify the appropriate Federal On-Scene Coordinator and concerned Federal agencies. Federal law requires that anyone who releases into the environment a reportable quantity of a hazardous material (including oil when water is, or may be affected) or a material identified as a marine pollutant, must immediately notify the NRC. When in doubt as to whether the amount released equals the required reporting levels for these materials, the NRC should be notified.

CALL NRC (24 hours)

###### 1-800-424-8802

(Toll-free in the U.S., Canada, and the U.S. Virgin Islands)

202-267-2675 in the District of Columbia

Calling the emergency response telephone number, CHEMTREC®, CHEMTEL, INC., INFOTRAC or 3E COMPANY, does not constitute compliance with regulatory requirements to call the NRC.

Scan for U.S. Incident Reporting

Page 386

##### NOTES

Page 387

##### 24-HOUR EMERGENCY RESPONSE TELEPHONE NUMBERS

MEXICO

- CENACOM

555128-0000 extensions 36428, 36422, 36469, 37807, 37810

##### CONASENUSA

800-11-131-68 in the Republic of Mexico

##### SETIQ

###### 800-00-21-400 or 55-5559-1588

For calls originating elsewhere, call: +52-55-5559-1588

##### ARGENTINA

- CIQUIME

0-800-222-2933 in the Republic of Argentina

For calls originating elsewhere, call: +54-11-4552-8747*

##### BRAZIL

- PRÓ-QUÍMICA

0-800-1108270 in Brazil

For calls originating elsewhere, call: +55-19-3833-5310*

##### COLOMBIA

- CISPROQUIM

01-800-091-6012 in Colombia

For calls originating in Bogotá, Colombia call: 288-6012

For calls originating elsewhere call: +57-1-288-6012 / +57-1-919-1919

##### CHILE

- CITUC QUÍMICO

2-2247-3600 in the Republic of Chile

For calls originating elsewhere call +56-2-2247-3600

* Collect calls are accepted

Page 388

24-HOUR EMERGENCY RESPONSE TELEPHONE NUMBERS

CANADA

1. CANUTEC

UNITED STATES

- CHEMTREC

1-888-CANUTEC (226-8832) (in Canada and U.S.) or 613-996-6666 *

*666 (STAR 666) cellular (in Canada only)

###### 1-800-424-9300

(in the U.S., Canada and the U.S. Virgin Islands) For calls originating elsewhere: 703-527-3887 *

##### CHEMTEL, INC.

###### 1-888-255-3924

(in the U.S., Canada, Puerto Rico and the U.S. Virgin Islands) For calls originating elsewhere: 813-248-0573 *

##### INFOTRAC

###### 1-800-535-5053

(in the U.S., Canada and the U.S. Virgin Islands) For calls originating elsewhere: 352-323-3500 *

##### VERISK 3E

###### 1-800-451-8346

(in the U.S., Canada and the U.S. Virgin Islands) For calls originating elsewhere: 760-602-8703 *

The emergency response information services shown above maintain periodically updated lists of state and Federal radiation authorities who provide information and technical assistance on handling incidents involving radioactive materials.

- MILITARY SHIPMENTS, for assistance at incidents involving materials being shipped by, for, or to the Department of Defense (DOD), call one of the following numbers:

703-695-4695/4696 * - Explosives/ammunition incidents (U.S. Army Operations Center)

1-800-851-8061 - All other hazardous materials/dangerous goods incidents (Defense Logistics Agency)

- NATIONWIDE POISON CONTROL CENTER (United States only)

###### 1-800-222-1222

* Collect calls are accepted

A guidebook intended for use by first responders during the initial phase of a transportation incident involving hazardous materials/dangerous goods

THIS DOCUMENT SHOULD NOT BE USED TO DETERMINE COMPLIANCE WITH THE HAZARDOUS MATERIALS/

DANGEROUS GOODS REGULATIONS OR

TO CREATE WORKER SAFETY DOCUMENTS FOR SPECIFIC CHEMICALS

NOT FOR SALE

This document is intended for distribution free of charge to Public Safety Organizations by the US Department of Transportation and Transport Canada. This copy may not be resold by commercial distributors.

SCAN FOR

U.S. INCIDENT REPORTING

SCAN FOR CANUTEC

SCAN FOR SICT

www.phmsa.dot.gov/hazmat

www.tc.gc.ca/TDG

www.sct.gob.mx